| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0068-v1 |
| --- | --- |

| Title: | AHG16/Non-CE7: A study of bin to bit ratio for VTM5.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo DR San Diego, CA-92127, USA | Tel: Email: | +1 858 521 5838 minhua.zhou@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution reports the bin to bit ratio of the VTM5.0 for both the CTC and low QP setting (QP =2, 7, 12, 17).  The summary results are as follows:

- On average for  CTC AI/RA/LD_B configurations,

  - the weighted bin to bit ratio in the VTM5.0 is 19.71%/20.88% /16.99% higher than the HM16.19, up from 15.72%/17.39%/13.34% in the VTM4.0, respectively.

  - the unweighted bin to bit ratio in the VTM5.0 is 14.21%/15.25%/12.88% higher than the HM16.19, up from 12.38%/13.75%/11.25% in the VTM4.0, respectively.

- On average for low-QP AI/RA/LD_B configurations,

  - the weighted bin to bit ratio in the VTM5.0 is 20.24%/19.00%/19.67% higher than the HM16.19,  up from 18.11%/17.17%/17.67% in the VTM4.0, respectively.

  - the unweighted bin to bit ratio in the VTM5.0  is 11.40%/8.39%/10.28% higher than the HM16.19, up from 10.45%/7.63%/9.54% in the VTM4.0, respectively.

The newly adopted residual coding for transform skip mode has significantly increased the bin to bit ratio for screen content coding. From VTM4.0 to VTM5.0, the weighted bin to bit ratio (averaged over AI/RA/LD_B) in class F sequences is increased by 11.10% for CTC and 5.61% for low-QP setting, respectively. The same average number for other classes is 3.71% for CTC and 1.98% for low-QP setting, respectively.

The BD-rate reduction of the VTM5.0 relative to the HM16.19 at low QPs (QP =2, 7, 12, 17) is -12.99%, -14.46%, -13.77% and -14.94% in AI, RA, LD_B and LD_P configurations, respectively. When compared to HM16.19, the incremental BD-rate reduction from VTM4.0 to VTM5.0 is -0.60% in AI, -0.47% in RA, -0.08% in LD_B and -0.25%, respectively.

In the weighted bin to bit ratio, a bypass bin is counted at 0.25 context coded bins; in the unweight bin to bit ratio, a bypass bin and a context coded bin carry an equal weight (1:1). All the percentage numbers are measured against the HM16.19.

# Introduction

It was reported in JVET-N0049 that the bin to bit ratio of the VTM4.0 is double digits percentage higher than that of the HM16.19, for both the CTC and low QP coding (QP =2, 7, 12, 17).  This contribution reports the bin to bit ratio in VTM5.0 relative to HM16.19.

# Experimental results

Tests were carried out by using the VTM5.0 and HM16.19 reference software. The CTC (common test conditions) defined in [1] was followed. Additional tests were conducted for low QPs of 2, 7, 12 and 17 with a sequence length of 100 frames. The same test sequences and coding parameter settings were used for HM16.19.

The following data are collected on the decoder side:

- The peak context coded bin rate, bypass bin rate and bit-rate of a coded sequence

- The average context coded bin rate, bypass bin rate and bit-rate over a coded sequence.

- The average context coded bin rate, bypass bin rate and bit-rate consumed by the transform coefficients coding over a coded sequence.

In addition to the BD-rate, the following measurements are computed based on the collected data:

- Weighted BD-binrate for the peak case.

- Weighted BD-binrate for the average case.

- Weighted BD-binrate for the transform coefficients coding part.

- Weighted bin to bit ratio for the peak case.

- Unweighted bin to bit ratio for the peak case, for the average case, and for the transform coefficients coding part.

In the weighted measurements, a bypass bin is counted as 0.25 context coded bins. In the unweighted measurements, a bypass bin and a context coded bin carry an equal weight (1:1).

It has been observed that the bin to bit ratio for the peak case, for the average case and for the transform coefficients coding part is roughly the same.

As summarized in Table 1, from VTM4.0 to VTM5.0 the weighted bin to bit ratio (averaged over AI/RA/LD_B) exhibits an incremental percentage increase of 3.71% for CTC and 1.98% for low-QP setting, respectively.

|  | Bin2bit ratio (peak, weighted) |  |  |  |  |  | Bin2bit ratio (peak, unweighted) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HM16.19 | VTM4.0 | VTM5.0 | diff (%, VMT4.0 vs. HM16.19) | diff (%, VMT5.0 vs. HM16.19) | diff* (%, VMT5.0 vs. VTM4.0) | HM16.19 | VTM4.0 | VTM5.0 | diff (%, VMT4.0 vs. HM16.19) | diff (%, VMT5.0 vs. HM16.19) | diff* (%, VMT5.0 vs. VTM4.0) |
|  | QP = (22, 27, 32, 37), CTC |  |  |  |  |  |  |  |  |  |  |  |
| AI | 0.95 | 1.10 | 1.13 | 15.72% | 19.71% | 3.99% | 1.23 | 1.38 | 1.40 | 12.38% | 14.21% | 1.82% |
| RA | 0.93 | 1.09 | 1.12 | 17.39% | 20.88% | 3.49% | 1.21 | 1.38 | 1.40 | 13.75% | 15.25% | 1.50% |
| LB | 0.96 | 1.09 | 1.13 | 13.34% | 16.99% | 3.64% | 1.24 | 1.37 | 1.39 | 11.25% | 12.88% | 1.63% |
| LP | 0.96 | 1.09 | 1.13 | 13.24% | 17.30% | 4.06% | 1.23 | 1.37 | 1.39 | 11.08% | 13.02% | 1.94% |
|  | QP = (2, 7, 12, 17), 100 frames |  |  |  |  |  |  |  |  |  |  |  |
| AI | 0.79 | 0.94 | 0.95 | 18.11% | 20.24% | 2.12% | 1.19 | 1.31 | 1.33 | 10.45% | 11.40% | 0.95% |
| RA | 0.74 | 0.87 | 0.88 | 17.17% | 19.00% | 1.83% | 1.18 | 1.28 | 1.28 | 7.63% | 8.39% | 0.76% |
| LB | 0.78 | 0.92 | 0.93 | 17.67% | 19.67% | 2.00% | 1.18 | 1.30 | 1.31 | 9.54% | 10.28% | 0.74% |
| LP | 0.78 | 0.92 | 0.93 | 17.46% | 19.46% | 2.00% | 1.18 | 1.30 | 1.31 | 9.50% | 10.24% | 0.74% |
| Note*: Percentage difference between VTM5.0 and VTM4.0 is measured against HM16.19 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 1: Bin to bit ratio increase from VTM4.0 to VTM5.0 (Overall)

The AI/RA/LD_B summary results for the VVC common testing conditions (CTC) and the low-QP setting are provided in Table 2, and Table 3, respectively.

Table 2: Summary results for CTC (VTM5.0 vs. HM16.19)

Table 3: Summary results for low QPs (VTM5.0 vs. HM16.19)

As summarized in Table 4 below, the bin to bit ratio has a relatively large increase in class F sequences. This might be due to the newly adopted residual coding for the transform skip mode, which uses more context coded bins and is mainly effective for screen content coding.   From VTM4.0 to VTM5.0 the weighted bin to bit ratio (averaged over AI/RA/LD_B) is increased by 11.10% for CTC and 5.61% for low-QP setting, respectively. Note the weighted bin to bit ratio in class F sequences is already higher than the average.

|  | Bin2bit ratio (peak, weighted) |  |  |  |  |  | Bin2bit ratio (peak, unweighted) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HM16.19 | VTM4.0 | VTM5.0 | diff (%, VMT4.0 vs. HM16.19) | diff (%, VMT5.0 vs. HM16.19) | diff* (%, VMT5.0 vs. VTM4.0) | HM16.19 | VTM4.0 | VTM5.0 | diff (%, VMT4.0 vs. HM16.19) | diff (%, VMT5.0 vs. HM16.19) | diff* (%, VMT5.0 vs. VTM4.0) |
|  | QP = (22, 27, 32, 37), CTC |  |  |  |  |  |  |  |  |  |  |  |
| AI | 0.87 | 1.06 | 1.15 | 22.43% | 32.89% | 10.46% | 1.17 | 1.32 | 1.36 | 12.75% | 16.25% | 3.50% |
| RA | 0.84 | 1.03 | 1.11 | 23.01% | 31.82% | 8.81% | 1.16 | 1.31 | 1.35 | 12.98% | 15.81% | 2.83% |
| LB | 0.86 | 1.06 | 1.18 | 22.95% | 36.98% | 14.03% | 1.17 | 1.32 | 1.38 | 12.26% | 17.57% | 5.31% |
| LP | 0.86 | 1.06 | 1.18 | 22.95% | 36.98% | 14.03% | 1.17 | 1.32 | 1.38 | 12.26% | 17.57% | 5.31% |
|  | QP = (2, 7, 12, 17), 100 frames |  |  |  |  |  |  |  |  |  |  |  |
| AI | 0.73 | 0.93 | 0.96 | 26.95% | 32.04% | 5.09% | 1.14 | 1.28 | 1.30 | 11.76% | 13.50% | 1.74% |
| RA | 0.71 | 0.89 | 0.93 | 25.89% | 31.31% | 5.42% | 1.14 | 1.26 | 1.28 | 10.29% | 11.79% | 1.50% |
| LB | 0.71 | 0.89 | 0.94 | 25.38% | 31.70% | 6.31% | 1.14 | 1.26 | 1.28 | 10.16% | 12.41% | 2.24% |
| LP | 0.71 | 0.89 | 0.94 | 25.38% | 31.70% | 6.31% | 1.14 | 1.26 | 1.28 | 10.16% | 12.41% | 2.24% |
| Note*: Percentage difference between VTM5.0 and VTM4.0 is measured against HM16.19 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4: Bin to bit ratio increase from VTM4.0 to VTM5.0 (Class F)

# Discussions

A higher bin to bit ratio generally requires a higher throughput CABAC engine for real time decoding. It has been observed that the bin to bit ratio continues to increase from VTM4.0 to VTM5.0, and the newly adopted residual coding for transform skip mode has significantly increased the bin to bit ratio in class F sequences.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH

[2] M. Zhou, “AHG16/Non-CE7: A study of bin to bit ratio in VTM5.0 and HM16.19”, JVET-N0049, the 14th JVET meeting, March 2019, Geneva, CH

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.