| 0Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0070 |
| --- | --- |

| Title: | CE4: Prediction refinement with optical flow for affine mode (Test 2.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiancong (Daniel) Luo Yuwen He 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4819 Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of core experiment (CE) 4 test 2.1 on prediction refinement with optical flow (PROF) for affine mode. PROF is proposed to refine the sub-block based affine motion compensated prediction with optical flow in pixel level granularity without increasing the memory access bandwidth. The CE test has 3 configurations for different combinations sub-block size and interpolation filter for bi-prediction. Test a applies 4x4 sub-block and 6-tap interpolation filter for bi-prediction; test b applies 8x8 sub-block and 6-tap interpolation filter for bi-prediction; test c applies 8x8 sub-block and 8-tap interpolation filter for bi-prediction. For uni-prediction, all three tests apply 4x4 sub-block size and 6-tap interpolation filter. The simulation results reportedly show that average BD rate savings for RA and LD respectively are {-0.48%, -0.09%, -0.07%} and {-0.32%, -0.04%, -0.37%} in test a; and {-0.40%, -0.02%, 0.02%} and {-0.27%, 0.02%, -0.21%} in test b; and {-0.46%, -0.05%, -0.06%} and {-0.35%, -0.06%, -0.21%} in test c.

# Introduction

Affine motion model parameters can be used to derive the motion vector of each pixel in a CU. However, due to the high complexity and memory access bandwidth for generating pixel-based affine motion compensated prediction, the current VVC adopted a sub-block based affine motion compensation method, where a CU is divided into 4x4 sub-blocks, each of which is assigned with a MV derived from the affine CU’s control point MVs. The sub-block based affine motion compensation is a trade-off between coding efficiency, complexity and memory access bandwidth. It loses prediction accuracy due to sub-block based prediction.

# Prediction refinement with optical flow

To achieve a finer granularity of motion compensation, this contribution proposes a method to refine the sub-block based affine motion compensated prediction with optical flow. After the sub-block based affine motion compensation is performed, luma prediction sample is refined by adding a difference derived by the optical flow equation. The proposed PROF is described as following four steps.

Step 1) The sub-block-based affine motion compensation is performed to generate sub-block prediction Ii, j.

Step2) The spatial gradients gx(i,j) and gy(i,j) of the sub-block prediction are calculated at each sample location using a 3-tap filter [-1, 0, 1].

gxi, j=Ii+1, j-Ii-1, j

gyi, j=Ii, j+1-Ii, j-1

The sub-block prediction is extended by one pixel on each side for the gradient calculation. To reduce the memory bandwidth and complexity, the pixels on the extended borders are copied from the nearest integer pixel position in the reference picture. Therefore, additional interpolation for padding region is avoided.

Step 3) The luma prediction refinement is calculated by the optical flow equation.

ΔI(i,j)= gxi,j*Δvx(i,j)+gyi,j*Δvy(i,j)

where the Δv(i,j) is the difference between pixel MV computed for sample location (i,j), denoted by vi,j, and the sub-block MV of the sub-block to which pixel (i,j) belongs, as shown in Figure 1.

Figure 1. Sub-block MV VSB and pixel Δv(i,j) (red arrow)

Since the affine model parameters and the pixel location relative to the sub-block center are not changed from sub-block to sub-block, Δvi,j can be calculated for the first sub-block, and reused for other sub-blocks in the same CU.  Let x and y be the horizontal and vertical offset from the pixel location to the center of the sub-block, Δvx,y can be derived by the following equation,

Δvxx,y=c*x+d*yΔvy(x, y)=e*x+f*y

For 4-parameter affine model,

c=f=v1x-v0xwe=-d=v1y-v0yw

For 6-parameter affine model,

c=v1x-v0xwd=v2x-v0xhe=v1y-v0ywf=v2y-v0yh

where (v0x,v0y), (v1x,v1y), (v2x,v2y) are the top-left, top-right and bottom-left control point motion vectors, w and h are the width and height of the CU.

Step 4) Finally, the luma prediction refinement is added to the sub-block prediction Ii, j. The final prediction I’ is generated as the following equation.

I'(i,j)= Ii,j+ΔI(i,j)

# Simulation results

The proposed PROF has been implemented on top of VTM-5.0. The test condition described in JVET-N1010 [1] is used in the simulation.	 3 sub-tests are included in the CE test. In test a, affine sub-block size is 4x4 in both uni-prediction and bi-prediction. In test b, affine sub-block size is 4x4 for uni-prediction and 8x8 for bi-prediction and 6-tap interpolation filter is used for 8x8 sub-block. Test c has same setting as test b except 8-tap interpolation filter is used for 8x8 sub-block.

Table 1.  Test 2.1a Affine sub-block size 4x4 for both uni-prediction and bi-prediction

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.16% | 0.11% | 101% | 100% | - | - | - | - | - |
| Class A2 | -0.85% | 0.27% | 0.16% | 102% | 101% | - | - | - | - | - |
| Class B | -0.61% | -0.28% | -0.19% | 101% | 100% | -0.41% | -0.05% | -0.28% | 102% | 101% |
| Class C | -0.31% | -0.07% | -0.21% | 101% | 101% | -0.23% | 0.05% | -0.15% | 102% | 100% |
| Class E | - | - | - | - | - | -0.30% | -0.14% | -0.80% | 103% | 99% |
| Overall | -0.48% | -0.09% | -0.07% | 101% | 100% | -0.32% | -0.04% | -0.37% | 102% | 100% |
| Class D | -0.54% | -0.13% | 0.04% | 101% | 99% | -0.38% | -0.35% | 1.14% | 101% | 100% |
| Class F | -0.86% | -0.45% | -0.47% | 101% | 100% | -1.33% | -1.34% | -0.97% | 102% | 99% |

Table 2. Test 2.1b Affine sub-block size 4x4 for uni-prediction and 8x8 for bi-prediction, 6-tap filter for bi-prediction

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.16% | 0.15% | 100% | 101% | - | - | - | - | - |
| Class A2 | -0.73% | 0.40% | 0.14% | 100% | 100% | - | - | - | - | - |
| Class B | -0.52% | -0.18% | -0.03% | 100% | 100% | -0.34% | -0.07% | -0.29% | 101% | 99% |
| Class C | -0.22% | -0.03% | -0.13% | 99% | 101% | -0.18% | 0.04% | -0.26% | 101% | 102% |
| Class E | - | - | - | - | - | -0.26% | 0.15% | -0.01% | 101% | 100% |
| Overall | -0.40% | -0.02% | 0.02% | 100% | 101% | -0.27% | 0.02% | -0.21% | 101% | 100% |
| Class D | -0.39% | -0.16% | 0.13% | 100% | 99% | -0.38% | -0.48% | 0.02% | 100% | 100% |
| Class F | -0.67% | -0.23% | -0.24% | 100% | 100% | -0.98% | -1.22% | -1.21% | 101% | 100% |

Table 3. Test 2.1c Affine sub-block size 4x4 for uni-prediction and 8x8 for bi-prediction, 8-tap filter for bi-prediction

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.13% | 0.12% | 101% | 100% | - | - | - | - | - |
| Class A2 | -0.75% | 0.42% | 0.15% | 100% | 100% | - | - | - | - | - |
| Class B | -0.59% | -0.15% | -0.18% | 100% | 100% | -0.36% | 0.06% | -0.17% | 101% | 99% |
| Class C | -0.32% | -0.21% | -0.22% | 100% | 100% | -0.36% | -0.10% | -0.05% | 101% | 102% |
| Class E | - | - | - | - | - | -0.31% | -0.22% | -0.51% | 101% | 100% |
| Overall | -0.46% | -0.05% | -0.06% | 100% | 100% | -0.35% | -0.06% | -0.21% | 101% | 100% |
| Class D | -0.53% | -0.26% | 0.04% | 100% | 99% | -0.76% | -0.82% | 0.46% | 100% | 100% |
| Class F | -0.70% | -0.24% | -0.25% | 100% | 100% | -1.21% | -0.81% | -0.54% | 101% | 100% |

# Reference

- J. Luo, Y. He, “CE2-related: Prediction refinement with optical flow for affine mode”, JVET-N0236, Geneva, CH, Mar. 2019

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Mar. 2019.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).