| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0073 |
| --- | --- |

| Title: | CE8-1.6: Dedicated IBC Buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Li Zhang, Kai Zhang, Hongbin Liu, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document shows the results of the IBC buffer design tested in CE8. In CE8-1.6b, a dedicated buffer with 96x128 pixels in 8-bit precision is presented. In CE8-1.6c, a dedicated buffer with 96x128 pixels in 10-bit precision is presented. In addition, a dedicated buffer with 120x128 pixels in 8-bit precision is also presented. For 8-bit designs, lowQP (QP=2, 7, 12, 17) results are also shown, in addition to results under CE8 test conditions.

The summary results for normal QPs are:

For CE8-1.6b

Class F/4:2:0 TGM – AI: 0.14%/0.68%; RA: 0.13%/0.47%; LDB: 0.14%/0.43%;

For CE8-1.6c

Class F/4:2:0 TGM – AI: 0.13%/0.52%; RA: 0.11%/0.25%; LDB: 0.16%/0.24%;

For 120x128 8-bit

Class F/4:2:0 TGM – AI: -0.86%/-2.13%; -0.66%/-1.17%; LDB: -0.25%/-0.50%.

The summary results for low QPs are:

For CE8-1.6b

Class F/4:2:0 TGM – AI: 0.78%/0.08%; RA: 0.85%/-1.73%; LDB: 0.86%/4.35%;

For 120x128 8-bit

Class F/4:2:0 TGM – AI: 0.26%/-2.42%; RA: 0.35%/-3.49%; LDB: 0.64%/3.10%.

In addition, for low QPs and LMCS is off, summary results are:

For CE8-1.6b

Class F/4:2:0 TGM – AI: -0.34%/0.08%; RA: -1.44%/-4.51%; LDB: -0.95%/0.55%;

For 120x128 8-bit

Class F/4:2:0 TGM – AI: -0.88%/-2.42%; RA: -1.94%/-4.16%; LDB: -1.27%/-0.81%.

The proposed design can reduce the number of operations for luma block vector checking to 13 and also simplify the text.

# Introduction

An dedicated IBC buffer design was presented in JVET-N0472 [1] and tests with different buffer parameters were included into CE8 [2]. In the current VVC design, three additional VPDU’s on-chip memory are required to support IBC. That is 3x64x64x10x1.5/(1024x8)=22.5Kbytes.

The proposed design can reduce the on-chip memory by 20%; simplify the block vector checking from 29 operations to 13; and also simplify the specification text.

# Proposed methods

## Method description

Suppose the buffer size is Wx128 in 8-bit precision, after decoding each luma block, the buffer is updated as

ibcRef[x][y] = clip3(0, 255, (recSample[x%W][y%128] + 512)>>2)		   (1)

When IBC mode is used, the luma predition is generated as

predSample[x][y] = ibcRef[x%W][y%128]<<2					   (2)

If the buffer size is in 10-bit precision, there is no need to performe bit-shifting in (1) and (2).

## On-chip memory reduction

For CE8-1.6b, the buffer is configured as 96x128 pixels in 8-bit precision. Compared with the current design. The memory requirement is 96x128x1.5/(1024x8)=18Kbyes, with 20% on-chip memory saving.

For CE8-1.6c, the buffer is configured as 96x128 pixels in 10-bit precision. Compared with the current design, the memory requirement is the same.

For a configuration of 120x128 pixels in 8-bit precision, compared with the current design, the memory requirement is the same.

## Operation reduction for block vector checking

Because the reference is bounded by the buffer size, there is no need to perform some condition checks for a block vector.

Suppose that CTU size is 128x128 and a WxH block at (X, Y) related to the upper-left of the picture has a block vetor (xBv, yBv). Then the reference block’s four cornors can be calculated as (with 6 operations)

- refLeftX = X + xBv

- refRightX = refLeftX + W - 1

- refTopY = Y + yBv

- refBottomY = refTopY + H – 1

In the current VTM implementation, there are five groups of condition checks, i.e.

- If a reference block is outside of picture boundary. (4 operations)

  - refLeftX >=0

  - refRightX < picWidth

  - refTopY >=0

  - refBottomY < picHeight

- If a reference block is outside of current CTU row. (5 operations)

  - (refToY >> 7) >= (Y>>7)

  - (refBottom >> 7) <= (Y>>7)

- If a reference block is outside of VPDU memory reused area (12 operations)

  - isDecomp((refLeftX + (1<<7))/64*64, refTopY/64*64) == false

  - ((refLeftX + (1<<7))/64*64, refTopY/64*64) != (X, Y)

- If the upper left sample of the reference block has been reconstructed and accessible. (1 operation)

  - isDecomp((refLeftX, refTopY) == true

- If the bottom right sample of the reference block has been reconstructed and accessible. (1 operation)

  - isDecomp((refRightX, refBottomY) == true

In total, to tell if a block vector is valid or not, 29 operations might be needed given that 4 registers are used to store the four corners of a reference block.

With the proposed design, we propose to keep only 2) and 4) and condition (refLeftX >=0). That is to say, when a reference block is within the current CTU row and has been reconstruted, it can be used in IBC mode. The number of operations can be reduced from 29 to 13.

## Summary of complexity analysis

|  | Current design | 96x128 8-bit | 96x128 10-bit | 120x128 8-bit |
| --- | --- | --- | --- | --- |
| On-chip memory | 22.5Kbytes | 18Kbytes | 22.5Kbytes | 22.5Kbytes |
| Number of operations for luma BV checking | 29 | 13 | 13 | 13 |

# Experimental results

Simulations have been performed at normal QPs (22, 27, 32 and 37) and also at low QPs (2, 7, 12, 17) for 8-bit configurations. For low QPs, the 1st 100 frames are used. It is noted that there are some losses (e.g. 5.16% for SlideEditing sequence, CE8-1.6b in RA) for low QPs in RA and LDB cases. It might be because of LMCS. When we turn off LMCS in both the anchor and test, large gains (e.g. -3.37% for SlideEditing sequence, CE8-1.6b in RA) are shown instead of losses. Other large positive BD-rate numbers might be due to abnormal RD performance at low QPs for screen sequences (e.g. Desktop). In conclusion, there is no obvious degradation caused by the proposed design.

Normal QP results are summarized in table 1-3.

Table 1. Results of 96x128 8-bit, i.e. CE-8.1.6b, at normal QPs.

|  | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.11% | 0.05% | 141% | 100% | 0.11% | -0.24% | 0.17% | 106% | 100% |  |  |  |  |  |
| Class A2 | -0.67% | -0.82% | -0.84% | 146% | 100% | 0.01% | -0.06% | -0.06% | 106% | 100% |  |  |  |  |  |
| Class B | -0.71% | -0.64% | -0.75% | 154% | 100% | -0.10% | -0.37% | -0.50% | 108% | 100% | 0.05% | 0.25% | -0.12% | 118% | 100% |
| Class C | -0.26% | -0.28% | -0.25% | 154% | 100% | 0.14% | -0.22% | -0.26% | 113% | 100% | 0.15% | 0.09% | -0.13% | 124% | 97% |
| Class E | -1.12% | -1.05% | -1.07% | 158% | 98% |  |  |  |  |  | 0.09% | -0.16% | -0.10% | 115% | 91% |
| Overall | -0.56% | -0.57% | -0.57% | 151% | 100% | 0.03% | -0.24% | -0.21% | 109% | 100% | 0.09% | 0.10% | -0.12% | 119% | 97% |
| Class D | -0.49% | -0.53% | -0.53% | 149% | 102% | 0.10% | -0.31% | -0.07% | 113% | 99% | 0.11% | -0.02% | 1.09% | 128% | 101% |
| Class F | 0.14% | 0.16% | 0.21% | 102% | 101% | 0.13% | 0.15% | 0.27% | 101% | 99% | 0.14% | -0.41% | -0.50% | 103% | 100% |
| 4:2:0 TGM | 0.68% | 0.71% | 0.71% | 101% | 101% | 0.47% | 0.44% | 0.43% | 103% | 101% | 0.43% | 0.35% | 0.34% | 100% | 103% |

Table 2. Results of 96x128 10-bit, i.e. CE-8.1.6c, at normal QPs.

|  | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.04% | 0.07% | 140% | 99% | 0.12% | -0.21% | 0.28% | 106% | 100% |  |  |  |  |  |
| Class A2 | -0.68% | -0.93% | -0.84% | 146% | 99% | -0.03% | -0.13% | -0.07% | 107% | 100% |  |  |  |  |  |
| Class B | -0.72% | -0.65% | -0.81% | 154% | 98% | -0.10% | -0.30% | -0.55% | 108% | 99% | 0.02% | 0.28% | -0.27% | 118% | 99% |
| Class C | -0.27% | -0.30% | -0.22% | 155% | 97% | 0.16% | -0.32% | -0.24% | 113% | 97% | 0.08% | 0.10% | 0.08% | 126% | 98% |
| Class E | -1.13% | -1.06% | -1.19% | 157% | 97% |  |  |  |  |  | -0.01% | -0.10% | -0.11% | 114% | 97% |
| Overall | -0.57% | -0.57% | -0.60% | 151% | 98% | 0.03% | -0.25% | -0.21% | 109% | 99% | 0.04% | 0.13% | -0.12% | 119% | 98% |
| Class D | -0.50% | -0.57% | -0.57% | 149% | 95% | 0.10% | -0.27% | 0.04% | 112% | 98% | 0.15% | -0.21% | 0.24% | 127% | 100% |
| Class F | 0.13% | 0.07% | 0.08% | 102% | 98% | 0.11% | 0.16% | 0.12% | 101% | 98% | 0.16% | 0.04% | -0.38% | 103% | 99% |
| 4:2:0 TGM | 0.52% | 0.54% | 0.53% | 101% | 100% | 0.25% | 0.25% | 0.23% | 102% | 102% | 0.24% | 0.16% | 0.21% | 101% | 99% |

Table 3. Results of 120x128 8-bit at normal QPs.

|  | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.06% | 0.00% | 150% | 101% | 0.11% | -0.18% | 0.37% | 110% | 101% |  |  |  |  |  |
| Class A2 | -0.72% | -0.95% | -0.90% | 153% | 100% | -0.03% | -0.18% | -0.12% | 109% | 100% |  |  |  |  |  |
| Class B | -0.74% | -0.73% | -0.81% | 163% | 101% | -0.12% | -0.22% | -0.51% | 113% | 101% | 0.05% | 0.25% | 0.39% | 122% | 95% |
| Class C | -0.29% | -0.31% | -0.19% | 167% | 101% | 0.17% | -0.13% | -0.29% | 118% | 103% | 0.13% | 0.07% | -0.16% | 128% | 101% |
| Class E | -1.26% | -1.10% | -1.14% | 167% | 99% |  |  |  |  |  | 0.24% | 0.20% | -0.32% | 117% | 99% |
| Overall | -0.61% | -0.62% | -0.61% | 161% | 100% | 0.02% | -0.18% | -0.20% | 113% | 101% | 0.12% | 0.18% | 0.03% | 122% | 98% |
| Class D | -0.53% | -0.52% | -0.53% | 155% | 104% | 0.11% | -0.34% | -0.07% | 115% | 104% | 0.06% | 0.03% | 1.12% | 131% | 99% |
| Class F | -0.86% | -0.72% | -0.78% | 104% | 99% | -0.66% | -0.58% | -0.39% | 102% | 101% | -0.25% | -0.61% | -0.50% | 102% | 97% |
| 4:2:0 TGM | -2.13% | -2.15% | -2.17% | 102% | 98% | -1.17% | -1.12% | -1.22% | 102% | 100% | -0.50% | -0.64% | -0.60% | 100% | 102% |

Low QP results are summarized in table 4-5.

Table 4. Results of 96x128 8-bit, i.e. CE-8.1.6b, at low QPs.

|  | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.03% | -0.02% | 100% | 99% |  |  |  |  |  |  |  |  |  |  |
| Class A2 | 0.00% | 0.01% | 0.00% | 100% | 100% |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.02% | 0.01% | 0.04% | 101% | 97% | -0.03% | 0.06% | 0.06% | 100% | 100% | 0.01% | -0.01% | 0.03% | 100% | 100% |
| Class C | 0.01% | 0.01% | 0.00% | 100% | 98% | 0.00% | -0.02% | -0.01% | 100% | 98% | -0.01% | 0.00% | 0.05% | 100% | 98% |
| Class E | 0.02% | -0.03% | -0.02% | 101% | 98% |  |  |  |  |  | -0.03% | 0.07% | 0.08% | 101% | 99% |
| Overall | 0.02% | 0.01% | 0.01% | 100% | 98% |  |  |  |  |  | -0.01% | 0.01% | 0.05% | 100% | 99% |
| Class D | 0.08% | 0.10% | 0.12% | 101% | 92% | 0.02% | 0.07% | -0.04% | 100% | 94% | 0.04% | -0.06% | 0.17% | 101% | 100% |
| Class F | 0.78% | 0.67% | 0.67% | 101% | 100% | 0.85% | 0.94% | 0.87% | 100% | 96% | 0.86% | 0.53% | 0.62% | 101% | 99% |
| 4:2:0 TGM | 0.08% | 0.87% | 0.82% | 99% | 98% | -1.73% | 0.70% | 0.69% | 102% | 100% | 4.35% | 1.53% | 1.41% | 104% | 107% |

Table 5. Results of 120x128 8-bit at low QPs.

|  | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.03% | -0.02% | 104% | 100% |  |  |  |  |  |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 104% | 99% |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.02% | 0.03% | 0.03% | 107% | 99% | -0.03% | 0.06% | 0.04% | 104% | 101% | 0.01% | 0.00% | 0.05% | 103% | 101% |
| Class C | 0.01% | 0.00% | 0.01% | 104% | 102% | 0.01% | -0.01% | 0.01% | 104% | 102% | -0.01% | -0.02% | 0.00% | 104% | 100% |
| Class E | 0.00% | -0.04% | -0.02% | 106% | 100% |  |  |  |  |  | -0.03% | 0.03% | 0.13% | 104% | 100% |
| Overall | 0.01% | 0.01% | 0.00% | 105% | 100% |  |  |  |  |  | -0.01% | 0.00% | 0.05% | 104% | 101% |
| Class D | 0.07% | 0.12% | 0.03% | 104% | 103% | 0.02% | 0.05% | 0.03% | 102% | 96% | 0.02% | 0.00% | 0.04% | 105% | 100% |
| Class F | 0.26% | 0.15% | 0.20% | 104% | 102% | 0.35% | 0.57% | 0.50% | 104% | 98% | 0.64% | 0.27% | 0.29% | 105% | 101% |
| 4:2:0 TGM | -2.42% | -1.68% | -1.75% | 101% | 98% | -3.49% | -0.73% | -0.70% | 106% | 101% | 3.10% | 0.07% | -0.01% | 108% | 107% |

Low QP results, when LMCS is off, are summarized in table 6-7.

Table 6. Results of of 96x128 8-bit, i.e. CE-8.1.6b, at low QPs when LMCS is off.

|  | AI |  |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y |  | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -0.34% | 0.39% | 0.38% | 100% | 100% | -1.44% |  | 0.18% | 0.15% | 100% | 101% | -0.95% | 0.04% | 0.17% | 100% | 98% |
| 4:2:0 TGM | 0.08% | 0.87% | 0.82% | 100% | 99% | -4.51% |  | 0.49% | 0.47% | 100% | 99% | 0.55% | 0.59% | 0.53% | 100% | 96% |

Table 7. Results of 120x128 8-bit at low QPs when LMCS is off.

|  | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -0.88% | -0.14% | -0.10% | 104% | 101% | -1.94% | -0.20% | -0.24% | 101% | 103% | -1.27% | -0.20% | -0.20% | 102% | 102% |
| 4:2:0 TGM | -2.42% | -1.68% | -1.75% | 102% | 98% | -4.16% | -1.06% | -1.06% | 101% | 101% | -0.81% | -0.87% | -0.88% | 101% | 96% |

# Conclusions

A dedicated IBC buffer design is proposed in this CE test. With different settings, the design can bring significant on-chip memory reduction or remarkable coding gains. The design also significantly reduces number of operations for block vector checking and simplifies the specification text. Thus, it is proposed to adopt the proposed scheme into the VVC design.

# Change to the working draft

For CE8-1.6b, ibcBufferWidth is set to 96 and bdIbcRef is set to 10.

For CE8-1.6.c, ibcBufferWidth is set to 96 and bdIbcRef is set to 10.

For 120x128 8-bit configuration, ibcBufferWidth is set to 120 and bdIbcRef is set to 8.

The following changes, marked in light blue, are based on JVET-N1001-v8.

8.6.2.1 General

It is a requirement of bitstream conformance that the luma motion vector mvL shall obey the following constraints:

- The motion vector mvL shall point to a reference block that is available, i.e. for ( xRefTL, yRefTL ) and ( xRefBR, yRefBR ), all of the following conditions shall be true:

  - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xRefTL, yRefTL ) as inputs, the output shall be equal to TRUE.

  - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xRefBR, yRefBR ) as inputs, the output shall be equal to TRUE.

- The motion vector mvL shall point to a reference block that is outside (left and/or above) and not overlapping with the current coding bock, i.e. one or both the following conditions shall be true:

  - The value of ( mvL[ 0 ] >> 4 ) + cbWidth is less than or equal to 0.

  - The value of ( mvL[ 1 ] >> 4 ) + cbHeight is less than or equal to 0.

- The motion vector mvL shall point to a reference block that is fully contained in the same coding tree block as the current block or fully contained in a block to the left with the same height of the current coding tree block and a width equal to 128 luma samples, i.e. all of the following conditions shall be true:

yRefTL >> CtbLog2SizeY = yCb >> CtbLog2SizeY	(8-893)

yRefBR >> CtbLog2SizeY = yCb >> CtbLog2SizeY	(8-894)

xRefTL >> CtbLog2SizeY >= ( xCb >> CtbLog2SizeY ) + Min( 1, 7 − CtbLog2SizeY )	(8-895)
	− ( 1 << ( (7 − CtbLog2SizeY ) << 1) ) )

xRefBR >> CtbLog2SizeY <= ( xCb >> CtbLog2SizeY )	(8-896)

- When the motion vector mvL points to a reference block where the top-left sample is located inside the neighbouring coding tree block to the left, i.e. ( xRefTL ) >> CtbLog2SizeY is equal to ( xCb >> CtbLog2SizeY ) − 1, and CtbSizeY is equal to 128, the motion vector mvL shall not point to a reference block where the top-left sample is located left to or inside the same 64x64 area as the current block but inside the neighbouring coding tree block to the left, i.e. the following conditions shall be true:

  - The derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( ( xRefTL + 128 ) / 64 * 64, yRefTL / 64 * 64 ) as inputs, and the output shall be equal to FALSE.

  - The luma location ( ( xRefTL + 128 ) / 64 * 64, yRefTL / 64 * 64 ) shall not be equal to ( xCb, yCb ).

8.6.3 Decoding process for ibc blocks

8.6.3.1 General

This process is invoked when decoding a coding unit coded in ibc prediction mode.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- variables numSbX and numSbY specifying the number of luma coding subblocks in horizontal and vertical direction,

- the motion vectors mv[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- a variable cIdx specifying the colour component index of the current block.

- an (nIbcRefW)x(ctbSize) array ibcRef

Outputs of this process are:

- an array predSamples of prediction samples.

Let predSamplesL0L, and predSamplesL1L be (cbWidth)x(cbHeight) arrays of predicted luma sample values and, predSamplesL0Cb, predSamplesL1Cb, predSamplesL0Cr and predSamplesL1Cr be (cbWidth / 2)x(cbHeight / 2) arrays of predicted chroma sample values.

The reference indices refIdxL0 and refIdxL1 are set equal to −1.

The prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] are set equal to 1 and predFlagL1[ xSbIdx ][ ySbIdx ] are set equal to 0 with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

The bi-prediction weight index bcwIdx is set equal to 0,

The width and the height of the current coding sublock sbWidth and sbHeight in luma samples are derived as follows:

sbWidth  =  cbWidth / numSbX		(8-913)

sbHeight  =  cbHeight / numSbY		(8-914)

For each coding subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1, the following applies:

- The luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows:

( xSb, ySb )  =  ( xCb + xSbIdx * sbWidth, yCb + ySbIdx * sbHeight )	(8-915)

If cIdx is equal to 0, nIbcRefW is set to ibcBufferWidth, otherwise nIbcRefW is set to ( ibcBufferWidth / SubWidthC ). Let bitDepth be the bit depth for the current color component. The folling applies:

predSamples[ xSb + x ][ ySb + y ] = ibcRef[ ( xSb + x + (mv[ xSb ][ ySb ][ 0 ] >> 4 )) % nIbcRefW ][ ySb + y + (mv[ xSb ][ ySb ][ 1 ] >> 4) ] << (bitDepth - bdIbcRef)

for x = 0..sbWidth − 1 and y = 0..sbHeight – 1.

- The variable bdofFlag is set equal to FALSE.

The current decoded picture consists of a pic_width_in_luma_samples by pic_height_in_luma_samples array of luma samples currPicL and two PicWidthInSamplesC by PicHeightInSamplesC arrays of chroma samples currPicCb and currPicCr. The current decoded picture sample arrays currPicL, currPicCb and currPicCr correspond to decoded sample arrays SL, SCb and SCr derived in clause 8.8 for the current decoded picture.

- If cIdx is equal to 0, the following applies:

  - The array predSamplesL0L is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth, the coding subblock height sbHeight in luma samples, the motion vector offset mvOffset set equal to ( 0, 0 ), the luma motion vector mv[ xSb ][ ySb ], the reference array currPicL, bdofFlag, and cIdx as inputs.

- Otherwise if cIdx is equal to 1, the following applies:

  - The array predSamplesL0Cb is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth / 2, the coding subblock height sbHeight / 2, the motion vector offset mvOffset set equal to ( 0, 0 ), the chroma motion vector mv[ xSb ][ ySb ], the reference array currPicCb, bdofFlag, and cIdx as inputs.

- Otherwise (cIdx is equal to 2), the following applies:

  - The array predSamplesL0Cr is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth / 2, the coding subblock height sbHeight / 2, the motion vector offset mvOffset set equal to ( 0, 0 ), the chroma motion vector mv[ xSb ][ ySb ], the reference array currPicCr, bdofFlag, and cIdx as inputs.

- The array predSamples of prediction samples is derived as follows:

- If cIdx is equal to 0, the prediction samples inside the current luma coding subblock, predSamples[ xL + xSb ][ yL + ySb ] with xL = 0..sbWidth − 1 and yL = 0..sbHeight − 1, are derived by invoking the weighted sample prediction process as specified in clause 8.5.6.5 is invoked with the luma coding subblock width sbWidth, the luma coding subblock height sbHeight and the sample arrays predSamplesL0 L and predSamplesL1L, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, bcwIdx, and cIdx as inputs, and predSamples[ xL + xSb ][ yL + ySb ] as outputs.

- Otherwise, if cIdx is equal to 1, the prediction samples inside the current chroma component Cb coding block, predSamples [ xC + xCb / 2 ][ yC + yCb / 2 ] with xC = 0..cbWidth / 2 − 1 and yC = 0..cbHeight / 2 − 1, are derived by invoking the weighted sample prediction process specified in clause 8.5.6.5 with the coding block width nCbW set equal to cbWidth / 2, the coding block height nCbH set equal to cbHeight / 2, the sample arrays predSamplesL0Cb and predSamplesL1Cb, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, bcwIdx, and cIdx as inputs.

- Otherwise (cIdx is equal to 2), the prediction samples inside the current chroma component Cr coding block, predSamples[ xC + xCb / 2 ][ yC + yCb / 2 ] with xC = 0..cbWidth / 2 − 1 and yC = 0..cbHeight / 2 − 1, are derived by invoking the weighted sample prediction process specified in clause 8.5.6.5 with the coding block width nCbW set equal to cbWidth / 2, the coding block height nCbH set equal to cbHeight / 2, the sample arrays predSamplesL0Cr and predSamplesL1Cr, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, bcwIdx, and cIdx as inputs.

- When cIdx is equal to 0, the following assignments are made for x = 0..sbWidth − 1 and y = 0..sbHeight − 1:

MvL0[ xSb + x ][ ySb + y ] = mv[ xSbIdx ][ ySbIdx ]	(8-916)

MvL1[ xSb + x ][ ySb + y ] = 0		(8-917)

RefIdxL0[ xSb + x ][ ySb + y ] = refIdxL0		(8-918)

RefIdxL1[ xSb + x ][ ySb + y ] = refIdxL1		(8-919)

PredFlagL0[ xSb + x ][ ySb + y ] = predFlagL0[ xSbIdx ][ ySbIdx ]	(8-920)

PredFlagL1[ xSb + x ][ ySb + y ] = predFlagL1[ xSbIdx ][ ySbIdx ]	(8-921)

BcwIdx[ xSb + x ][ ySb + y ] = bcwIdx		(8-922)

8.7.5 Picture reconstruction process

8.7.5.1 General

Inputs to this process are:

- a location ( xCurr, yCurr ) specifying the top-left sample of the current block relative to the top-left sample of the current picture component,

- the variables nCurrSw and nCurrSh specifying the width and height, respectively, of the current block,

- a variable cIdx specifying the colour component of the current block,

- an (nCurrSw) x (nCurrSh) array predSamples specifying the predicted samples of the current block,

- an (nCurrSw) x (nCurrSh) array resSamples specifying the residual samples of the current block.

Output of this process is are

- a reconstructed picture sample array recSamples.

- an IBC reference array ibcRef.

Depending on the value of the colour component cIdx, the following assignments are made:

- If cIdx is equal to 0, recSamples corresponds to the reconstructed picture sample array SL and the function clipCidx1 corresponds to Clip1Y.

- Otherwise, if cIdx is equal to 1, tuCbfChroma is set equal to tu_cbf_cb[ xCurr ][ yCurr ], recSamples corresponds to the reconstructed chroma sample array SCb and the function clipCidx1 corresponds to Clip1C.

- Otherwise (cIdx is equal to 2), tuCbfChroma is set equal to tu_cbf_cr[ xCurr ][ yCurr ], recSamples corresponds to the reconstructed chroma sample array SCr and the function clipCidx1 corresponds to Clip1C.

Depending on the value of slice_lmcs_enabled_flag, the following applies:

- If slice_lmcs_enabled_flag is equal to 0, the (nCurrSw)x(nCurrSh) block of the reconstructed samples recSamples at location ( xCurr, yCurr ) is derived as follows for i = 0..nCurrSw − 1, j = 0..nCurrSh − 1:

recSamples[ xCurr + i ][ yCurr + j ] = clipCidx1( predSamples[ i ][ j ] + resSamples[ i ][ j ] ) 	(8-993)

- Otherwise (slice_lmcs_enabled_flag is equal to 1), the following applies:

- If cIdx is equal to 0, the following applies:

- The picture reconstruction with mapping process for luma samples as specified in clause 8.7.5.2 is invoked with the luma location ( xCurr, yCurr ), the block width nCurrSw and height nCurrSh, the predicted luma sample array predSamples, and the residual luma sample array resSamples as inputs, and the output is the reconstructed luma sample array recSamples.

- Otherwise (cIdx is greater than 0), the picture reconstruction with luma dependent chroma residual scaling process for chroma samples as specified in clause 8.7.5.3 is invoked with the chroma location ( xCurr, yCurr ), the transform block width nCurrSw and height nCurrSh, the coded block flag of the current chroma transform block tuCbfChroma, the predicted chroma sample array predSamples, and the residual chroma sample array resSamples as inputs, and the output is the reconstructed chroma sample array recSamples.

Denote bitDepth as the bitdepth of the bit depth of the current component, bdIbcRef as the bitdepth of ibcRef, nIbcRefW as the width of ibcRef, the following applies:

ibcRef[ ( xCurr + i ) % nIbcRefW ][ ( yCurr + j ) % ctbSize  ] =

clip3(( 0, ( 1  <<  bdIbcRef ) − 1, recSamples[ xCurr + i ][ yCurr + j ] + (1 << ( bitDepth – 1 )) >> (bitDepth - bdIbcRef))

for i = 0..nCurrSw − 1, j = 0..nCurrSh – 1.

# References

- J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang, “Non-CE8: On IBC reference buffer design,” JVET-N0472, Mar. 2019.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-N1028, Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).