| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0075 |
| --- | --- |

| Title: | CE11-1: GRA support by restriction to encoder operation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui | Tel: Email: | +81 44 754 2049 kazui.kimihiko@fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

- Abstract

This contribution presents a method to support gradual random access (GRA) by restriction to encoder operation.

In the proposed restriction, inter and intra prediction are controlled so that any prediction unit in the clean region does not refer to a sample in the dirty region. Moreover, a redundant intra-coded area is introduced to avoid contamination of the clean region from the dirty region through the in-loop filtering process. No changes of the decoding process are required.

It is reported that the proposed method results in 19.13%/15.50%/17.61% Y/Cb/Cr BD-rates increase with non-significant encoding and decoding time changes for VTM-5.0 low-delay B condition with CE11 specific setting.

Coding structure in GRA mode

Firstly, coding structure in GRA mode is described.

Access unit

Only the first picture of a sequence is encoded in intra mode. Remaining pictures are encoded in inter mode.

The first picture and the last picture in a refresh cycle are called a random access picture and a recovery point picture, respectively. Each picture is encoded so that the decoding result of a recovery picture when the decoding process starts from its immediately preceding random access picture is identical to that from the IDR picture.

The coding structure of a sequence in GRA mode is illustrated in Figure 1-1.

Figure 1-1 – Coding structure in AU level

Coding unit in non-IDR picture

A vertical refresh boundary is virtually defined in each picture. In this proposal, the refresh boundary shifts inside a picture from left to right. The left and right side of a picture separated by the boundary are named a clean area and dirty area, respectively. Each coding unit in the clean area is encoded so that it does not refer to any sample in the dirty area.

There is a vertical striped intra-coded area left to the refresh boundary. Basically each coding unit in the area is coded in intra, but it is possible to be coded in inter with a motion vector that points to a sample inside the clean area of a reference picture. Each coding unit in the right side of the refresh boundary is also encoded in intra in order to avoid contamination of samples in the in-loop filtering process. The width of the area is determined according to the refresh cycle and the picture width.

Each picture is divided into tiles. Each tile has a height of a CTU and a width of a picture. Each tile is associated to a decoding unit in order to reduce the size of the CPB buffer. It is noted that in-loop filtering across tile boundary is enabled.

The layout of the refresh boundary, intra-coded area, clean area and dirty area is illustrated in Figure 1-2.

Figure 1-2 – Coding structure in CU level

The refresh cycle and width of intra-coded area of each sequence are summarized in Table 1-1. The first t1 pictures and remaining ( r – t1 ) pictures in each refresh cycle are encoded with the intra-coded area whose width are ( w1 + w2 ) and ( w1 – 8 + w2 ), respectively. The size of horizontal shift of the boundary is w1 for the first t1 pictures and ( w1 – 8 ) for the remaining ( r – t1 ) pictures. It is noted that the refresh cycle is exceptionally set to 52, instead of 60, for “BQSquare” sequence (Class D, 60Hz) due to its small picture width and frame rate.

Table 1-1 – Refresh cycle and width of intra-coded area of sequence

| Class | Class B |  | Class C |  |  | Class D |  |  | E | Class F |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Picture width | 1920 |  | 832 |  |  | 416 |  |  | 1280 | 1920 | 832 | 1280 | 1280 |
| Frame rate [Hz] | 50 | 60 | 30 | 50 | 60 | 30 | 50 | 60 | 60 | 60 | 50 | 30 | 20 |
| Refresh cycle r | 50 | 60 | 30 | 50 | 60 | 30 | 50 | 52 | 60 | 60 | 50 | 30 | 20 |
| w1 | 40 | 32 | 32 | 24 | 16 | 16 | 16 | 8 | 24 | 32 | 24 | 48 | 64 |
| t1 | 40 | 60 | 14 | 4 | 44 | 22 | 2 | 52 | 40 | 60 | 4 | 10 | 20 |
| w2 | 16 | 16 | 16 | 16 | 8 | 8 | 8 | 8 | 16 | 16 | 16 | 16 | 16 |

Software implementation and setting

Software base

VTM-5.0 is used as the code base. The proposed GRA is enabled by setting macro JVET_N_CE11_1 equal to 1.

The code is available at https://vcgit.hhi.fraunhofer.de/jvet-n-ce11/VVCSoftware_VTM/tree/CE11-1 .

Restricted intra coding

Intra prediction modes which only use samples in the clean area for generating predicted samples are evaluated for a coding unit either in the clean area or intra-coded area in order to avoid contamination through the intra prediction process.

When IBC mode is enabled, the use of this mode is disabled for a coding unit which is inside the clean area and located near the refresh boundary.

Restricted inter coding

If a coding unit in the clean area is encoded in inter, then the search area of its motion vector is restricted inside the clean area of a reference picture. The derived motion vector in merge mode is subject to the same restriction. Those restriction is implemented using the corresponding code for MCTS mode.

Motion vector coding

TMVP mode is disabled for a GRA picture and its following three pictures in order to guarantee correct reconstruction of a motion vector in each inter coding unit.

Parameter setting

The following settings are applied to the encoder.

- HashME is set to 0 even for Class F sequences since the author could not implement the restriction for hash-based motion search mode by the deadline.

- IntraPeriod is set to −1.

- IntraRefreshPeriod is set to r in Table 1-1.

Experiment results

CE11 specific test conditions [4] are used for evaluation.

BD-rate and complexity

The results are summarized in Table 3-1.

Table 3-1 – Result of proposed encoder-only GRA

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE11 Anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 8.96% | 10.92% | 15.45% | 95% | 99% |
| Class C | 16.03% | 10.70% | 11.02% | 95% | 101% |
| Class E | 40.22% | 29.55% | 30.01% | 94% | 109% |
| Overall | 19.13% | 15.50% | 17.61% | 94% | 102% |
| Class D | 24.41% | 25.12% | 26.04% | 101% | 108% |
| Class F | 49.13% | 49.71% | 51.24% | 106% | 111% |

Exact match

Exact match at four different random access positions is tested. Four positions are the first GRA, second GRA, third GRA and fourth GRA. This test is performed using the associated decoder with the setting RandomAccessPos equal to M, where M is set to the POC value of the random access picture. If the hash value of the corresponding recovery point picture is matched to the coded value in the SEI, it is regarded as matched.

Since there is a bug of improper chroma intra mode selection and LMCS control at specific condition, four cases out of ( 80*4 ) bitstreams and positions combination do not achieve exact match. Mismatch only happens in chroma samples. The author asserts the effect of the bug is minor and this proposal with the test results still serve as a reference to measure the performance of other proposed tools in CE11.

Bitrate fluctuation

The average and standard deviation of bits and PSNR of access units are measured for each bitstream. Please refer to the attached Excel sheets.

The author asserts that no additional bitrate fluctuation is introduced to the bitstreams in the access unit level, considering the similar values are detected from the anchor bitstreams.

Conclusion

This contribution presents a method to support gradual random access (GRA) by restriction to encoder operation.

It is reported that the proposed method results in 19.13%/15.50%/17.61% Y/Cb/Cr BD-rates increase with small encoding and decoding time changes for VTM-5.0 low-delay B condition with CE11 specific setting.

References

[1] K.Kazui, “Revised software for ultra low-latency encoding”, JVET-N0080, March 2019

[2] VTM-5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

[3] “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019

[4] “Description of Core Experiment 11 (CE11): Gradual random access”, JVET-N1031, March 2019

Patent rights declaration(s)

Fujitsu Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).