| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0076 |
| --- | --- |

| Title: | CE6-1.1: An Explicit MTS Design with Fast Encoder |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao-Hsiung Hung, Hilmi E. Egilmez, Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-658-1293 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This CE proposes to replace DCT-8 in MTS by identity transform (IDT) or DCT-2 in intra-CUs based on a pre-defined transform length T. Also, another two encoder-only modification methods are introduced. Over VTM-5.0, with common test configuration. the proposed methods lead to minor gain of 0.00%, 0.01% and 0.03% luma BD-rate with 104%, 102% and 100% encoding time in AI, RA and LDB at similar decoding time.

# CE Description

It is proposed to replace DCT-8 in MTS modes from 3 to 5 depending on transform length T. Specifically, if the transform length T ≤ 16, DCT-8 is replaced by IDT. Otherwise, DCT-8 is replaced by DCT-2. This approach is only applied to intra CUs.

In addition, the following two encoder-only methods are proposed to improve the trade-off between coding performance and encoding complexity.

Encoder change 1: Size and mode-dependent scaling factor for L1 norm

In VTM 5.0, for each MTS mode sum of absolute values (L1-norm) of transformed coefficients are calculated for fast encoding. Let S be the calculated L1-norm corresponding to mode 0. For MTS mode of 1 to 5, if the L1 norm is less than X × S, this mode will be included in the candidate list for full R-D checking, where X is a scaling factor that is set as X = 1 for mode 1, and it is in the range of [1.2, 1.5] defined based on maximum transform length of the TU for the rest of the modes.

This contribution replaces the threshold X scaled by (X / Y). For MTS mode 1, Y = 1. Otherwise, Y is determined as follows. If horizontal (resp. vertical) transform is DST-7 or DCT-2 after DCT-8 replacement, YH (resp. YV) is set to 1. Otherwise, horizontal (resp. vertical) transform is IDT after DCT-8 replacement and YH (resp. YV) is determined as in Table 1-1. Then, Y is set as YH × YV.

Table 1-1. The value of YH or YV for different transform length.

| horizontal or vertical transform length | YH or YV |
| --- | --- |
| 4 | 1.0 |
| 8 | 1.3 |
| 16 | 1.7 |
| 32 | 2.0 |

Encoder change 2: Additional RD checks for MTS modes

In VTM 5.0, after checking RD cost of mode 0 and mode 1, if mode 1 is better than mode 0, RD cost of other modes will not be checked. In this contribution, this speed-up algorithm is disabled.

Table 1-2 shows the description of each test.

Table 1-2. Test descriptions.

| Test # | Description |
| --- | --- |
| 6-1.1a | MTS design with both encoder changes |
| 6-1.1b | Both encoder changes |
| 6-1.1c | Encoder change 1 only |
| 6-1.1d | Encoder change 2 only |
| 6-1.1d | MTS design without encoder changes |

# Simulation results

## Test sequences 4:2:0

The proposed method is evaluated according to the CE6 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. Table 2-1 ~ Table 2-5 shows the result of CE6.1.1. compared with VTM 5.0 under CTC. Table 2-6 ~ Table 2-10 shows the results with InterMTS enabled.

Table 2-1. Test 6-1.1a, CTC.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.00% | 105% | 100% |
| Class A2 | -0.01% | -0.01% | 0.01% | 104% | 99% |
| Class B | 0.00% | -0.02% | 0.01% | 105% | 105% |
| Class C | 0.00% | 0.00% | 0.03% | 103% | 98% |
| Class E | 0.00% | -0.01% | 0.02% | 105% | 101% |
| Overall | 0.00% | 0.00% | 0.01% | 104% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 111% | 124% |
| Class F | 0.00% | 0.00% | -0.01% | 104% | 103% |
| Class TGM | 0.00% | 0.00% | 0.00% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | 0.00% | 103% | 101% |
| Class A2 | 0.00% | -0.02% | 0.02% | 102% | 100% |
| Class B | -0.01% | -0.09% | -0.15% | 102% | 101% |
| Class C | -0.05% | -0.09% | 0.02% | 103% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.07% | -0.04% | 102% | 102% |
| Class D | -0.02% | -0.09% | 0.13% | 109% | 117% |
| Class F | 0.00% | 0.04% | 0.05% | 102% | 102% |
| Class TGM | -0.02% | -0.02% | -0.03% | 103% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.22% | -0.04% | 100% | 101% |
| Class C | -0.03% | 0.26% | -0.10% | 100% | 98% |
| Class E | -0.08% | 0.42% | 0.01% | 100% | 99% |
| Overall | -0.03% | 0.29% | -0.05% | 100% | 99% |
| Class D | -0.03% | -0.08% | 0.64% | 105% | 109% |
| Class F | -0.15% | -0.31% | -0.37% | 100% | 101% |
| Class TGM | -0.03% | -0.05% | 0.00% | 101% | 102% |

Table 2-2. Test 6-1.1b, CTC.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, CTC |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.00% | 0.00% |  | 0.00% |  | 105% |  | 100% |  |
| Class A2 | 0.00% | 0.00% |  | 0.00% |  | 104% |  | 98% |  |
| Class B | 0.00% | 0.00% |  | 0.00% |  | 104% |  | 103% |  |
| Class C | 0.00% | 0.00% |  | 0.00% |  | 104% |  | 99% |  |
| Class E | 0.00% | 0.00% |  | 0.00% |  | 105% |  | 102% |  |
| Overall | 0.00% | 0.00% |  | 0.00% |  | 104% |  | 101% |  |
| Class D | 0.00% | 0.00% |  | 0.00% |  | 110% |  | 122% |  |
| Class F | 0.00% | 0.00% |  | 0.00% |  | 103% |  | 104% |  |
| Class TGM | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 100% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.00% | 0.00% |  | 0.00% |  | 102% |  | 100% |  |
| Class A2 | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 100% |  |
| Class B | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 101% |  |
| Class C | 0.00% | 0.00% |  | 0.00% |  | 102% |  | 104% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% |  | 0.00% |  | 102% |  | 101% |  |
| Class D | 0.00% | 0.00% |  | 0.00% |  | 108% |  | 117% |  |
| Class F | 0.00% | 0.00% |  | 0.00% |  | 102% |  | 102% |  |
| Class TGM | 0.00% | 0.00% |  | 0.00% |  | 102% |  | 102% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.04% |  | 0.01% |  | -0.11% |  | 100% |  | 103% |
| Class C | -0.01% |  | 0.22% |  | -0.08% |  | 99% |  | 94% |
| Class E | -0.05% |  | 0.17% |  | 0.96% |  | 100% |  | 100% |
| Overall | 0.00% |  | 0.12% |  | 0.17% |  | 100% |  | 99% |
| Class D | 0.04% |  | -0.07% |  | 1.13% |  | 104% |  | 106% |
| Class F | 0.05% |  | 0.05% |  | -0.02% |  | 101% |  | 102% |
| Class TGM | -0.13% |  | -0.11% |  | -0.04% |  | 100% |  | 100% |

Table 2-3. Test 6-1.1c, CTC.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, CTC |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.18% | 0.07% |  | 0.22% |  | 97% |  | 101% |  |
| Class A2 | 0.18% | 0.17% |  | 0.11% |  | 97% |  | 98% |  |
| Class B | 0.25% | 0.08% |  | 0.23% |  | 95% |  | 103% |  |
| Class C | 0.25% | 0.05% |  | 0.22% |  | 94% |  | 100% |  |
| Class E | 0.32% | 0.23% |  | 0.32% |  | 96% |  | 101% |  |
| Overall | 0.24% | 0.11% |  | 0.22% |  | 96% |  | 101% |  |
| Class D | 0.27% | 0.24% |  | 0.12% |  | 100% |  | 126% |  |
| Class F | 0.16% | 0.14% |  | 0.09% |  | 97% |  | 103% |  |
| Class TGM | 0.03% | 0.03% |  | 0.04% |  | 92% |  | 102% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.08% | 0.39% |  | 0.68% |  | 99% |  | 101% |  |
| Class A2 | 0.10% | 0.20% |  | 0.17% |  | 96% |  | 99% |  |
| Class B | 0.16% | 0.14% |  | 0.10% |  | 96% |  | 101% |  |
| Class C | 0.18% | 0.24% |  | 0.02% |  | 94% |  | 103% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.14% | 0.23% |  | 0.21% |  | 96% |  | 101% |  |
| Class D | 0.24% | 0.35% |  | 0.07% |  | 100% |  | 119% |  |
| Class F | 0.18% | 0.15% |  | 0.24% |  | 93% |  | 102% |  |
| Class TGM | 0.04% | 0.04% |  | 0.00% |  | 96% |  | 103% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% |  | 0.23% |  | 0.01% |  | 100% |  | 104% |
| Class C | 0.02% |  | -0.07% |  | -0.20% |  | 99% |  | 97% |
| Class E | -0.02% |  | -0.38% |  | 0.66% |  | 101% |  | 98% |
| Overall | 0.01% |  | -0.02% |  | 0.10% |  | 100% |  | 100% |
| Class D | 0.05% |  | -0.10% |  | 0.80% |  | 104% |  | 108% |
| Class F | -0.06% |  | -0.02% |  | 0.32% |  | 99% |  | 100% |
| Class TGM | -0.01% |  | -0.05% |  | 0.07% |  | 100% |  | 101% |

Table 2-4. Test 6-1.1d, CTC.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, CTC |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 101% |  |
| Class A2 | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 97% |  |
| Class B | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 104% |  |
| Class C | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 99% |  |
| Class E | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 100% |  |
| Overall | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 100% |  |
| Class D | 0.00% | 0.00% |  | 0.00% |  | 109% |  | 122% |  |
| Class F | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 101% |  |
| Class TGM | 0.00% | 0.00% |  | 0.00% |  | 99% |  | 100% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 101% |  |
| Class A2 | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 100% |  |
| Class B | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 100% |  |
| Class C | 0.00% | 0.00% |  | 0.00% |  | 102% |  | 104% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 101% |  |
| Class D | 0.00% | 0.00% |  | 0.00% |  | 108% |  | 116% |  |
| Class F | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 102% |  |
| Class TGM | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 103% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.01% |  | 0.27% |  | 0.33% |  | 101% |  | 101% |
| Class C | -0.02% |  | -0.04% |  | -0.04% |  | 100% |  | 93% |
| Class E | -0.12% |  | -0.08% |  | 1.74% |  | 100% |  | 99% |
| Overall | -0.04% |  | 0.08% |  | 0.56% |  | 100% |  | 97% |
| Class D | -0.03% |  | 0.17% |  | 0.72% |  | 106% |  | 108% |
| Class F | 0.00% |  | 0.25% |  | -0.26% |  | 99% |  | 99% |
| Class TGM | -0.06% |  | -0.03% |  | -0.01% |  | 101% |  | 100% |

Table 2-5. Test 6-1.1e, CTC.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.00% | 102% | 102% |
| Class A2 | -0.01% | -0.01% | 0.01% | 101% | 99% |
| Class B | 0.00% | -0.02% | 0.01% | 102% | 104% |
| Class C | 0.00% | 0.00% | 0.03% | 101% | 98% |
| Class E | 0.00% | -0.01% | 0.02% | 101% | 99% |
| Overall | 0.00% | 0.00% | 0.01% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 109% | 122% |
| Class F | 0.00% | 0.00% | -0.01% | 102% | 103% |
| Class TGM | 0.00% | 0.00% | 0.00% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | 0.00% | 102% | 100% |
| Class A2 | 0.00% | -0.02% | 0.02% | 101% | 100% |
| Class B | -0.01% | -0.09% | -0.15% | 100% | 101% |
| Class C | -0.05% | -0.09% | 0.02% | 102% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.07% | -0.04% | 101% | 101% |
| Class D | -0.02% | -0.09% | 0.13% | 106% | 111% |
| Class F | 0.00% | 0.04% | 0.05% | 101% | 102% |
| Class TGM | -0.02% | -0.02% | -0.03% | 102% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.23% | -0.35% | 100% | 102% |
| Class C | 0.04% | 0.04% | -0.07% | 100% | 97% |
| Class E | 0.03% | -0.15% | 1.70% | 100% | 98% |
| Overall | 0.03% | 0.07% | 0.26% | 100% | 99% |
| Class D | -0.02% | 0.07% | 0.65% | 104% | 106% |
| Class F | -0.09% | 0.51% | -0.20% | 102% | 103% |
| Class TGM | -0.05% | -0.09% | -0.05% | 100% | 98% |

Table 2-6. Test 6-1.1a, InterMTS Enabled.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, InterMTSEnabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.00% | 104% | 100% |
| Class A2 | -0.01% | -0.01% | 0.01% | 104% | 101% |
| Class B | 0.00% | -0.02% | 0.01% | 104% | 105% |
| Class C | 0.00% | 0.00% | 0.03% | 104% | 105% |
| Class E | 0.00% | -0.01% | 0.02% | 105% | 106% |
| Overall | 0.00% | 0.00% | 0.01% | 104% | 104% |
| Class D | 0.00% | 0.00% | 0.00% | 104% | 110% |
| Class F | 0.00% | 0.00% | -0.01% | 99% | 98% |
| Class TGM | 0.00% | 0.00% | 0.00% | 103% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0, InterMTSEnabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.06% | 0.06% | 102% | 101% |
| Class A2 | 0.00% | -0.07% | 0.03% | 102% | 101% |
| Class B | 0.02% | 0.00% | -0.03% | 102% | 101% |
| Class C | 0.02% | 0.14% | 0.13% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.04% | 102% | 102% |
| Class D | -0.03% | -0.03% | 0.09% | 100% | 98% |
| Class F | -0.01% | -0.02% | 0.02% | 102% | 101% |
| Class TGM | 0.00% | 0.02% | -0.03% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0, InterMTSEnabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.09% | 0.16% | 99% | 98% |
| Class C | -0.04% | 0.11% | 0.09% | 102% | 111% |
| Class E | -0.16% | -0.53% | 0.34% | 101% | 104% |
| Overall | -0.03% | -0.06% | 0.18% | 101% | 104% |
| Class D | 0.01% | -0.46% | 0.29% | 99% | 95% |
| Class F | -0.12% | 0.16% | -0.04% | 100% | 101% |
| Class TGM | -0.05% | -0.12% | -0.05% | 103% | 102% |

Table 2-7. Test 6-1.1b, InterMTS Enabled

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, InterMTSEnabled |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.00% | 0.00% |  | 0.00% |  | 104% |  | 101% |  |
| Class A2 | 0.00% | 0.00% |  | 0.00% |  | 104% |  | 104% |  |
| Class B | 0.00% | 0.00% |  | 0.00% |  | 104% |  | 103% |  |
| Class C | 0.00% | 0.00% |  | 0.00% |  | 104% |  | 105% |  |
| Class E | 0.00% | 0.00% |  | 0.00% |  | 104% |  | 102% |  |
| Overall | 0.00% | 0.00% |  | 0.00% |  | 104% |  | 103% |  |
| Class D | 0.00% | 0.00% |  | 0.00% |  | 103% |  | 109% |  |
| Class F | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 101% |  |
| Class TGM | 0.00% | 0.00% |  | 0.00% |  | 102% |  | 102% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0, InterMTSEnabled |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.00% | 0.00% |  | 0.00% |  | 102% |  | 101% |  |
| Class A2 | 0.00% | 0.00% |  | 0.00% |  | 102% |  | 102% |  |
| Class B | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 101% |  |
| Class C | 0.00% | 0.00% |  | 0.00% |  | 102% |  | 102% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 101% |  |
| Class D | 0.00% | 0.00% |  | 0.00% |  | 99% |  | 97% |  |
| Class F | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 100% |  |
| Class TGM | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 100% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0, InterMTSEnabled |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.04% |  | -0.10% |  | -0.07% |  | 99% |  | 100% |
| Class C | 0.01% |  | 0.07% |  | -0.15% |  | 101% |  | 111% |
| Class E | -0.19% |  | -0.06% |  | 0.78% |  | 100% |  | 103% |
| Overall | -0.03% |  | -0.03% |  | 0.11% |  | 100% |  | 105% |
| Class D | -0.07% |  | 0.07% |  | -0.02% |  | 98% |  | 93% |
| Class F | -0.02% |  | -0.22% |  | -0.17% |  | 101% |  | 100% |
| Class TGM | 0.09% |  | 0.20% |  | 0.08% |  | 102% |  | 100% |

Table 2-8. Test 6-1.1c, InterMTS Enabled.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, InterMTSEnabled |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.18% | 0.07% |  | 0.22% |  | 96% |  | 101% |  |
| Class A2 | 0.18% | 0.17% |  | 0.11% |  | 97% |  | 101% |  |
| Class B | 0.25% | 0.08% |  | 0.23% |  | 95% |  | 102% |  |
| Class C | 0.25% | 0.05% |  | 0.22% |  | 94% |  | 105% |  |
| Class E | 0.32% | 0.23% |  | 0.32% |  | 96% |  | 101% |  |
| Overall | 0.24% | 0.11% |  | 0.22% |  | 95% |  | 102% |  |
| Class D | 0.27% | 0.24% |  | 0.12% |  | 94% |  | 110% |  |
| Class F | 0.16% | 0.14% |  | 0.09% |  | 92% |  | 98% |  |
| Class TGM | 0.03% | 0.03% |  | 0.04% |  | 94% |  | 101% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0, InterMTSEnabled |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.07% | 0.47% |  | 0.59% |  | 99% |  | 100% |  |
| Class A2 | 0.06% | 0.18% |  | 0.15% |  | 97% |  | 101% |  |
| Class B | 0.13% | 0.15% |  | 0.14% |  | 97% |  | 102% |  |
| Class C | 0.18% | 0.21% |  | 0.12% |  | 95% |  | 102% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.12% | 0.24% |  | 0.23% |  | 97% |  | 101% |  |
| Class D | 0.16% | 0.41% |  | 0.10% |  | 92% |  | 98% |  |
| Class F | 0.14% | 0.13% |  | 0.22% |  | 93% |  | 101% |  |
| Class TGM | 0.11% | 0.10% |  | 0.07% |  | 94% |  | 100% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0, InterMTSEnabled |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.07% |  | 0.10% |  | 0.02% |  | 99% |  | 99% |
| Class C | 0.01% |  | 0.28% |  | 0.03% |  | 102% |  | 111% |
| Class E | -0.04% |  | -0.24% |  | 1.46% |  | 101% |  | 108% |
| Overall | 0.02% |  | 0.08% |  | 0.38% |  | 100% |  | 105% |
| Class D | 0.00% |  | -0.48% |  | -0.06% |  | 98% |  | 97% |
| Class F | 0.06% |  | 0.13% |  | -0.09% |  | 100% |  | 100% |
| Class TGM | 0.03% |  | -0.04% |  | 0.00% |  | 100% |  | 102% |

Table 2-9. Test 6-1.1d, InterMTS Enabled.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, InterMTSEnabled |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 101% |  |
| Class A2 | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 104% |  |
| Class B | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 101% |  |
| Class C | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 104% |  |
| Class E | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 102% |  |
| Overall | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 102% |  |
| Class D | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 108% |  |
| Class F | 0.00% | 0.00% |  | 0.00% |  | 96% |  | 97% |  |
| Class TGM | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 102% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0, InterMTSEnabled |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 101% |  |
| Class A2 | 0.00% | 0.00% |  | 0.00% |  | 102% |  | 102% |  |
| Class B | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 102% |  |
| Class C | 0.00% | 0.00% |  | 0.00% |  | 102% |  | 103% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 102% |  |
| Class D | 0.00% | 0.00% |  | 0.00% |  | 99% |  | 96% |  |
| Class F | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 100% |  |
| Class TGM | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 100% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0, InterMTSEnabled |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.02% |  | 0.01% |  | 0.03% |  | 100% |  | 98% |
| Class C | -0.02% |  | -0.11% |  | -0.18% |  | 102% |  | 113% |
| Class E | -0.19% |  | -0.05% |  | 1.72% |  | 101% |  | 104% |
| Overall | -0.05% |  | -0.05% |  | 0.39% |  | 101% |  | 104% |
| Class D | -0.09% |  | 0.09% |  | -0.60% |  | 99% |  | 94% |
| Class F | 0.00% |  | -0.32% |  | -0.27% |  | 101% |  | 98% |
| Class TGM | 0.02% |  | 0.11% |  | 0.05% |  | 102% |  | 100% |

Table 2-10. Test 6-1.1e, InterMTS Enabled.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, InterMTSEnabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.00% | 101% | 100% |
| Class A2 | -0.01% | -0.01% | 0.01% | 101% | 103% |
| Class B | 0.00% | -0.02% | 0.01% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.03% | 102% | 107% |
| Class E | 0.00% | -0.01% | 0.02% | 101% | 102% |
| Overall | 0.00% | 0.00% | 0.01% | 101% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 110% |
| Class F | 0.00% | 0.00% | -0.01% | 97% | 98% |
| Class TGM | 0.00% | 0.00% | 0.00% | 102% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0, InterMTSEnabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.06% | 0.06% | 101% | 100% |
| Class A2 | 0.00% | -0.07% | 0.03% | 101% | 101% |
| Class B | 0.02% | 0.00% | -0.03% | 101% | 102% |
| Class C | 0.02% | 0.14% | 0.13% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.04% | 101% | 101% |
| Class D | -0.03% | -0.03% | 0.09% | 99% | 97% |
| Class F | -0.01% | -0.02% | 0.02% | 101% | 101% |
| Class TGM | 0.00% | 0.02% | -0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0, InterMTSEnabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.19% | 0.30% | 100% | 99% |
| Class C | -0.02% | -0.19% | -0.08% | 102% | 113% |
| Class E | -0.04% | -0.29% | 1.55% | 102% | 105% |
| Overall | 0.01% | -0.06% | 0.49% | 101% | 105% |
| Class D | 0.02% | -0.61% | -0.72% | 99% | 94% |
| Class F | -0.10% | 0.03% | -0.18% | 101% | 103% |
| Class TGM | 0.12% | 0.16% | 0.06% | 101% | 101% |

# Reference

- C.-H. Hung, H. E. Egilmez, N. Hu, V. Seregin, M. Karczewicz, “CE6-related: An Explicit MTS Design with Fast Encoder”, JVET-N0424, 14th meeting, Geneva, CH, March 2019.

- X. Zhao, H. E. Egilmez, “Description of Core Experiment 6 (CE6): Transforms and transform signalling”, JVET-N1026, 14th meeting, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).