| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0078 |
| --- | --- |

| Title: | CE8-1.7: Single HMVP table for all CUs inside the shared merge list region for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Suhong Wang1, Xiaozhen Zheng2, Shanshe Wang1, Siwei Ma1 (Peking University1, DJI2) Li Zhang, Kai Zhang, Hongbin Liu, Jizheng Xu, Yue Wang (Bytedance) Xiaozhong Xu, Xiang Li, Guichun Li, Shan Liu (Tencent) | Email: | lizhang.idm@bytedance.com xiaozhongxu@tencent.com suhong.wang@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd., Bytedance Inc., Tencent |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of Core Experiment (CE) 8-1.7 for using single HMVP table for all CUs inside the shared merge list which is used for the IBC merge list. In addition, the HMVP table is not updated when coding a child node under the shared node. The simulation followed the CE8 test conditions. For class F and class TGM, the proposed method has 0.01%, 0.03% BD rate reduction in AI and RA configuration, respectively. In LDB configuration, the proposed method has 0.08% BD-rate reduction for class F.

Introduction

Shared merge list was adopted in JVET-14th meeting [1] and has been removed from regular merge list in the last meeting. In the current design, shared merge candidates are still used for IBC merge list.

In the design of shared merge list, when the shared ancestor node is met, a temporary HMVP table will be allocated and filled by all available HMVP candidates in the original HMVP table. In addition, all child nodes of the shared ancestor node uses the temporary HMVP table to derive HMVP candidate and decoded motion information is used to update the original HMVP table. Therefore, two HMVP tables need to be maintained for coding small blocks. Such a design needs more complex logic to handle the HMVP tables.

Proposed Method

In order to simplify the current design of harmonization for HMVP and share merge list, it is proposed to use a single HMVP table and the HMVP buffer is updated only when a block is not inside a shared merging candidate list region.

Experimental Results

The experiments were performed under the test conditions described in JVET-N1028[2]. More specifically, the following changes of test conditions compared to the common test conditions apply:

Class A~E: IBC off for Anchor; and IBC on for Test;

Class F and TGM: IBC, HashME and RDPCM on

BD-Rate results of the tests on top of the VTM-5.0[3] compared to the anchor with AI, RA and LDB configurations are shown in Table 1. Due to the unstable performance of the computer resource, the encoding/decoding time is not reliable. The full results are provided in the attached spreadsheets.

Table 1. Results of proposed method compared to VTM-5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.01% | 0.09% | 135% | 99% |
| Class A2 | -0.78% | -0.99% | -1.01% | 141% | 102% |
| Class B | -0.77% | -0.69% | -0.85% | 146% | 102% |
| Class C | -0.29% | -0.25% | -0.23% | 147% | 94% |
| Class E | -1.24% | -1.10% | -1.22% | 151% | 97% |
| Overall | -0.62% | -0.59% | -0.64% | 144% | 99% |
| Class D | -0.54% | -0.52% | -0.65% | 142% | 101% |
| Class F | 0.00% | 0.01% | 0.08% | 100% | 99% |
| TGM | -0.02% | -0.01% | -0.01% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.27% | 0.21% | 105% | 90% |
| Class A2 | -0.06% | -0.37% | -0.05% | 104% | 90% |
| Class B | -0.12% | -0.35% | -0.49% | 105% | 88% |
| Class C | 0.18% | -0.15% | -0.19% | 106% | 89% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.28% | -0.18% | 105% | 89% |
| Class D | 0.13% | 0.12% | 0.11% | 109% | 91% |
| Class F | 0.00% | 0.08% | 0.09% | 94% | 83% |
| TGM | -0.07% | -0.03% | -0.08% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.14% | -0.27% | 118% | 98% |
| Class C | 0.12% | 0.20% | -0.33% | 122% | 95% |
| Class E | 0.07% | 0.90% | 0.92% | 112% | 96% |
| Overall | 0.08% | 0.35% | 0.01% | 118% | 96% |
| Class D | 0.07% | 0.19% | 0.58% | 123% | 96% |
| Class F | -0.08% | 0.10% | 0.03% | 100% | 97% |
| TGM |  |  |  |  |  |

Conclusion

This proposal suggests using one single HMVP LUT for all CUs under a shared ancestor node without updating HMVP table. By doing this, the HMVP update process is simplified and the extra copy for HMVP is unnecessary. With the proposed method, there are minor coding gains under each configuration, including the AI configurations, RA configurations and LDB configurations for class F and class TGM. It is recommended to adopt it to VVC.

Reference

[1] C.-C. Chen, Y.-C. Lin, M.-S. Chiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei , JVET-M0170, CE4.3.1: Shared merging candidate list, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[2] X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, JVET-N1028, Description of Core Experiment 8 (CE8): Screen Content Coding Tools, 14th Meeting: Geneva, CH, 19–27 March 2019

- VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).