| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0079 |
| --- | --- |

| Title: | CE10: Integrated in-loop filter based on CNN (Tests 2.1, 2.2 and 2.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuai Wan Mingze Wang Yanzhuo Ma Junyan Huo Hao Gong Chengyi Zou YuanFang Yu Yang Liu | Email: | swan@nwpu.edu.cn roronoa@mail.nwpu.edu.cn yzma@mail.xidian.edu.cn jyhuo@mail.xidian.edu.cn gong_h@mail.nwpu.edu.cn cyzou@mail.nwpu.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Northwestern Polytechnical University (NPU), Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

This document reports the simulation results of Core Experiment 10 (CE10) tests 2.1, 2.2 and 2.3 on neural network based filter for video coding.

Experimental results of test 2.1a reportedly show -0.46%, -4.11%, -2.80%, test 2.1b reportedly show 0.79%, -5.06%, -4.73% BD-rate savings on Y, Cb, Cr respectively on Random Access configuration. It can be seen that converting weights from 32bit-float to 8bit-int leads to lower gains.

Experimental results of test 2.2a reportedly show -3.93%, -7.89%, -7.29% BD-rate savings on Y, Cb, Cr respectively on All Intra configuration, and -0.26%, -3.86%, -2.91% on Random Access configuration. Test 2.2b shows -3.05%, -9.02%, -10.47% BD-rate savings on Y, Cb, Cr respectively on All Intra configuration, and -1.89%, -11.91%, -8.90% on Random Access configuration.

Experimental results of test 2.3b reportedly show -1.52%, -6.15%, -4.31% BD-rate savings on Y, Cb, Cr respectively on Random Access configuration.

It is noted that the proposed NN-based filter completely replaces the original in-loop filters (i.e., DBF+SAO+ALF ) when test 2.2a and 2.3b are performed to evaluate the in-loop situation, while it is added as a post filter with DBF+SAO+ALF all on in 2.2.b. The proposed NN filter outperforms DBF+SAO+ALF if used as the only in-loop filter, and is also useful if used as an extra post-loop filter, and has the generalization capability over higher QPs at the same time.

Introduction

In the 14th JVET meeting, an in-loop filter based on convolutional neural network (CNNLF) named WSE-CNNLF(Wide-activated Squeeze-and-Excitation Convolutional Neural Network Loop Filter) was proposed in JVET-N0133[1] and defined as CE10-2.1, 2.2 and 2.3 in Core Experiment 10 (CE10) description[2].

The tested WSE-CNNLF shown in Fig. 1 has the following properties:

- It includes six inputs: three reconstructed components (Y, U, V) and three auxiliary inputs (QPmap, CUmap for luma, CUmap for chroma).

- It consists of three stages to make the luma and chroma components jointly processed and separately fused with the corresponding CUmap before generating outputs.

- It can replace and even outperform the multiple filters in current VVC.

Fig. 1. Main structure of the proposed CNNLF

Test description

Test 2.1 investigates the impact of the bit-depth settings of the weights in neural network. It includes 2 kinds of bit-depth settings: (2.1a) 32bit-float weights; (2.1b) 8bit-int weights.

Test 2.2 investigates the difference between in-loop (2.2a) and post processing (2.2b) filter.

Test 2.3 investigates generalization capability of the NN filter when the test QPs are different from the training QPs. We report the results (2.3b) with the test QP as CTC QP+5 (27,32,37,42) while the training QP is CTC QP (22,27,32,37).

The test configurations are summarized in Table 1. Since the conventional loop filter is replaced in this proposal, the loop filter chain is also clarified.

Table 1. Test configuration

| Test # | Test Part | Description | in loop filter chain | post loop filter chain |
| --- | --- | --- | --- | --- |
| anchor |  |  | DBF+SAO+ALF | None |
| CE10-2.1a | bit-depth | 32-bit(float) weights | NN | None |
| CE10-2.1b |  | 8-bit(int) weights | NN | None |
| CE10-2.2a | in-loop/ post filter | CNNF as in-loop filter | NN | None |
| CE10-2.2b |  | CNNF as post filter | DBF+SAO+ALF | NN |
| CE10-2.3b | generalization capability (QP) | CTC QP for training, CTC QP + 5 for testing | NN | None |

Experimental results

The proposed method has been implemented on top of VTM 5.0[3]. The simulations are conducted according to CE10 description[2] and the common test conditions (CTC)[4].

Test 2.1

Table 2. Results for test 2.1a (32bit float) with RA configuration (Short test)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.47% | 2.29% | -2.24% | 149% | 143155% |
| Class A2 | -0.71% | -6.72% | -4.34% | 143% | 131083% |
| Class B | 0.25% | -4.90% | -3.09% | 150% | 162185% |
| Class C | -1.16% | -5.96% | -1.71% | 126% | 104090% |
| Class E |  |  |  |  |  |
| Overall | -0.46% | -4.11% | -2.80% | 142% | 134684% |
| Class D | -3.54% | -6.87% | -8.68% | 126% | 78922% |
| Class F | 1.79% | -1.16% | -0.80% | 175% | 211001% |

Table 3. Results for test 2.1b (8bit int) with RA configuration (Short test)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 3.25% | 2.07% | -2.17% | 204% | 329912% |
| Class A2 | 1.33% | -7.90% | -5.54% | 197% | 303632% |
| Class B | 0.96% | -5.85% | -5.79% | 210% | 338474% |
| Class C | -1.66% | -7.27% | -4.73% | 162% | 257704% |
| Class E |  |  |  |  |  |
| Overall | 0.79% | -5.06% | -4.73% | 192% | 306401% |
| Class D | -3.63% | -8.42% | -10.93% | 164% | 255245% |
| Class F | 1.56% | 0.19% | -0.82% | 267% | 483718% |

Test 2.2

Table 4. Results for test 2.2a (as in-loop filter) with AI configuration (Full test)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.38% | -3.27% | -4.47% | 159% | 93687% |
| Class A2 | -3.22% | -10.47% | -9.11% | 132% | 74780% |
| Class B | -3.31% | -10.55% | -9.34% | 130% | 72834% |
| Class C | -4.42% | -8.23% | -7.89% | 119% | 54397% |
| Class E | -5.59% | -5.05% | -4.06% | 143% | 93337% |
| Overall | -3.93% | -7.89% | -7.29% | 134% | 74517% |
| Class D | -5.11% | -10.01% | -10.97% | 109% | 45424% |
| Class F | -0.02% | -6.47% | -4.69% | 128% | 72949% |

Table 5. Results for test 2.2a (as in-loop filter) with RA configuration (Full test)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.18% | 0.63% | -2.95% | 138% | 123657% |
| Class A2 | -0.98% | -7.16% | -3.34% | 138% | 121571% |
| Class B | 0.64% | -4.16% | -3.67% | 149% | 154962% |
| Class C | -1.17% | -4.38% | -1.61% | 129% | 122434% |
| Class E |  |  |  |  |  |
| Overall | -0.26% | -3.86% | -2.91% | 139% | 132513% |
| Class D | -3.13% | -6.26% | -5.15% | 122% | 104265% |
| Class F | 2.34% | -1.83% | -1.59% | 187% | 215591% |

Table 6. Results for test 2.2b (as post-loop filter) with AI configuration (Full test)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.72% | -7.23% | -8.40% | 159% | 90905% |
| Class A2 | -2.47% | -11.60% | -10.86% | 133% | 74438% |
| Class B | -2.39% | -10.79% | -12.33% | 131% | 71733% |
| Class C | -3.56% | -9.37% | -12.32% | 121% | 55092% |
| Class E | -4.37% | -4.81% | -6.57% | 148% | 94108% |
| Overall | -3.05% | -9.02% | -10.47% | 136% | 74083% |
| Class D | -4.24% | -10.85% | -15.28% | 115% | 45073% |
| Class F | -1.29% | -7.43% | -7.47% | 129% | 74310% |

Table 7. Results for test 2.2b (as post-loop filter) with RA configuration (Full test)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.87% | -4.93% | -6.33% | 136% | 138922% |
| Class A2 | -1.68% | -12.17% | -7.12% | 135% | 129100% |
| Class B | -1.47% | -15.14% | -9.80% | 149% | 150251% |
| Class C | -3.34% | -12.91% | -11.03% | 133% | 122434% |
| Class E |  |  |  |  |  |
| Overall | -1.89% | -11.91% | -8.90% | 139% | 135869% |
| Class D | -4.97% | -14.34% | -13.62% | 131% | 102395% |
| Class F | -0.36% | -6.96% | -5.21% | 197% | 215235% |

Test 2.3

Table 8. Results for test 2.3b (QP+5) with RA configuration (Short test)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.29% | 1.40% | -2.64% | 180% | 162566% |
| Class A2 | -1.48% | -6.57% | -4.16% | 169% | 149302% |
| Class B | -1.35% | -7.59% | -5.63% | 179% | 178542% |
| Class C | -1.94% | -9.69% | -4.03% | 140% | 126973% |
| Class E |  |  |  |  |  |
| Overall | -1.52% | -6.15% | -4.31% | 166% | 154379% |
| Class D | -4.51% | -9.04% | -11.39% | 140% | 101397% |
| Class F | 0.94% | -1.16% | -0.07% | 200% | 233995% |

Conclusions

For test 2.1, it can be seen that converting weights from 32bit-float to 8bit-int leads to lower gains (from -0.46% to 0.79% for BD-rate saving on luma).

For test 2.2, in AI configuration, the BD-rate saving on luma of in-loop filter (-3.93%) is higher than that of post filter (-3.05%). In RA configuration, the BD-rate saving on luma of post filter (-1.89%) is higher than that of in-loop filter (-0.26%). It is noted that the proposed NN filter completely replaces the original in-loop filter (i.e., DBF+SAO+ALF ) when test 2.2a is performed to evaluate the in-loop situation, while it is added as a post filter with DBF+SAO+ALF all on in 2.2.b. It is concluded that the proposed NN filter reportedly outperforms DBF+SAO+ALF if used as the only in-loop filter, and is also useful if used as an extra post-loop filter.

For test 2.3, it’s shown that the proposed WSE-CNNLF has generalization capability on higher QP. The BD-rate saving is -0.46%, -4.11%, -2.80% when the test QP is the same as training QP, and -1.52%, -6.15%, -4.31% when the test QP equals to training QP+5. The CNN-based loop filter seems to be more effective on lower video qualities.

References

- S. Wan, M. Wang, Y. Ma, J. Huo, H. Gong, C. Zou, Y. Yu, Y. Liu, “CE13-related: Integrated in-loop filter based on CNN”, document JVET-N0133, 14th JVET Meeting: Geneva, CH, 19–27 March 2019.

- Y. Li, S. Liu, K. Kawamura, “Description of Core Experiment 10 (CE10): Neural Network based Filter for Video Coding”, document JVET-N1030, 14th JVET Meeting: Geneva, CH, 19–27 March 2019.

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 5 (VTM 5)”, document JVET-N1002, 14th JVET meeting: Geneva, CH, 19–27 March 2019.

- F. Bossen, J. Boyce, X. Li, and V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, document JVET-N1010, 14th JVET meeting: Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).