| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0081-v1 |
| --- | --- |

| Title: | CE8: Chroma Transform Skip (CE8-3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This document reports results of CE8-3.2, where chroma transform skip is introduced. The motivation is to enable chroma transform skip to improve coding efficiency for SCC and support future extension of lossless coding. CE8-3.2 applies the same signaling scheme for TS/MTS used for luma to chroma; transform type is signaled per component (TB-level) rather than TU-level, but MTS for chroma is disallowed.

The simulations were conducted for YUV420 and YUV444 sequences with common QP condition and low QP condition (QP=2,7,12,17).

It is reported that average BD-rate differences (Y, U, V) with common QP condition are as follows:

- In Class A1/A2/B/C/E, average BD-rate differences are (0.02%,0.04% and 0.02%) for AI, (0.03%, -0.01%, 0.15%) for RA, and (0.06%, 0.84%, 0.64%) for LB, respectively.

- In Class F, average BD-rate differences are (0.02%, -0.10% and -0.09%) for AI, (0.05%, -0.09%, -0.11%) for RA, and (0.13%, 0.19%, 0.20%) for LB, respectively.

- In Class SCC, average BD-rate differences are (0.02%, -0.57%, -0.85%) for AI, (-0.03%, -0.72%, -0.98%) for RA, (0.09%, -0.76%, -1.18%) for LB, respectively.

- In YUV444 sequence, average BD-rate differences are (-0.19%, -1.96%, -2.19%) for AI, (-0.43%, -2.68%, -2.77%) for RA, and (-0.14%, -2.55%, -2.82%) for LB, respectively.

It is reported that average BD-rate differences (Y, U, V) with low QP condition are as follows:

- In Class A1/A2/B/C/E, average BD-rate differences are (0.03%, -0.05% and -0.08) for AI, (0.03%, -0.10%, -0.08%) for RA, and (0.07%, -0.23%, -0.36%) for LB, respectively.

- In Class F, average BD-rate differences are (-0.21%, -0.89% and -0.83%) for AI, (-0.19%, -1.56%, -1.33%) for RA, and (-0.18%, -1.55%, -1.46%) for LB, respectively.

- In Class SCC, average BD-rate differences are (-2.21%, -4.07%, -4.05%) for AI, (-6.00%, -2.88%, -2.95%) for RA, (-1.48%, -2.47%, -2.61%) for LB, respectively.

- In YUV444 sequence, average BD-rate differences are (-4.35%, -6.84%, -6.81%) for AI, (-4.23%, -8.25%, -8.24%) for RA, and (-4.75%, -8.07%, -7.92%) for LB, respectively.

Minor impacts of encoding time are observed: on average 1~2% increase in YUV420 under common QP condition, 2~4% increase in YUV420 under low QP condition, 3~4% increase in YUV444 under common QP condition, and 4~7% increase in YUV444 under low QP condition.

It is recommended to adopt proposed changes into the next VTM/VVC WD and enable chroma transform skip for SCC content (classF/TGM/YUV444).

Introduction

In the Marrakech meeting, unified transform type signaling (called uniMTS) in JVET-M0464 [1] was adopt in VTM-4.0. It includes following modifications:

- Unified signalling for transform skip and MTS

- Luma transform skip size extension up to 32x32

- No chroma transform skip

- Fast transform type selection as non-normative changes: limits # of candidates for RDO to 3 for intra MTS and 4 for inter MTS

JVET-N0123 [2] reported that “no chroma TS” resulted in small chroma bdrate degradation, and it requested to study chroma TS in CE.

Proposal

CE8-3.2 applies following modifications into VTM-5.0:

- Signal transform type (transform_skip_flag and mts_idx) per component with minimum changes:

  - Additional single context model for chroma TS

  - No MTS for chroma

  - No changes of both context model and binarization for MTS signaling

- Interaction with LFNST (JVET-N0193 [3])

  - When chroma TS is applied, both primary transform and secondary transform for chroma is skipped as well as luma TS case. Checking number of non-zero coefficients is skipped when transform_skip_flag_flag associated with cIdx is equal to 1.

- Interaction with TS residual coding (JVET-N0214 [4])

  - TS residual coding is straightforwardly used for chroma TS as well as luma TS case.

Syntax changes are as bellow:

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| … |  |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| if( transform_skip_enabled_flag && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( (( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag )) && ( !transform_skip_flag[ x0 ][ y0 ] ) ) |  |
| tu_mts_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| transform_mode (x0, y0, tbWidth, tbHeight, 0) |  |
| if( !transform_skip_flag[ x0 ][ y0 ][ 0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ){ |  |
| transform_mode(x0,y0, wC, hC, 1) |  |
| if( !transform_skip_flag[ x0 ][ y0 ][ 1 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| else |  |
| residual_ts_coding(xC, yC, Log2( wC), Log2( hC ), 1 ) |  |
| } |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) { |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| tu_joint_cbcr_residual[ x0 ][ y0 ] | ae(v) |
| if( !tu_joint_cbcr_residual[ x0 ][ y0 ] ){ |  |
| transform_mode(x0, y0, wC, hC, 2) |  |
| if ( !transform_skip_flag[ x0 ][ y0 ][ 2 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| else |  |
| residual_ts_coding(xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| } |  |
| } |  |

| transform_mode( x0, y0, tbWidth, tbHeight, cIdx ) { | Descriptor |
| --- | --- |
| tsAllowed = (transform_skip_enabled_flag && tbWidth<=MaxTsSize && tbHeight<=MaxTsSize) && (IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ¦¦ cIdx!=0 ) && !cu_sbt_flag && !(BdpcmFlag[ x ][ y ] && cIdx==0 && tbWidth<=32 && tbHeight <=32) |  |
| if ( tsAllowed ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| mtsAllowed = (cIdx == 0) &&(( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag )) && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && !cu_sbt_flag && !(BdpcmFlag[ x ][ y ] && tbWidth<=32 && tbHeight <=32) |  |
| if ( mtsAllowed && !trasnfrom_skip_flag[x0][y0][cIdx]) |  |
| mts_idx [ x0 ][ y0 ][ cIdx ] | ae(v) |
| } |  |

transform_skip_flag[ x0 ][ y0 ][cIdx] specifies whether a transform is applied to the luma transform block associated with cIdx or not. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. transform_skip_flag[ x0 ][ y0 ][ cIdx ] equal to 1 specifies that no transform is applied to the luma associated transform block. transform_skip_flag[ x0 ][ y0 ][ cIdx ] equal to 0 specifies that the decision whether transform is applied to the luma associated transform block or not depends on other syntax elements.

When transform_skip_flag[ x0 ][ y0 ][ cIdx ] is not present, it is inferred as follows:

- If BdcpmFlag[ x0 ][ x0 ] is equal to 1 and cIdx is equal to 0, transform_skip_flag[ x0 ][ y0 ][ cIdx ] is inferred to be equal to 1.

- Else if Otherwise (BdcpmFlag[ x0 ][ x0 ] is equal to 0) and cIdx is equal to 0, transform_skip_flag[ x0 ][ y0 ][ cIdx ] is inferred to be equal to 0.

- Otherwise (cIdx is not equal to 0), transform_skip_flag[ x0 ][ y0 ][ cIdx ] is inferred to be equal to 0.

tu_mts_idx[ x0 ][ y0 ][ cIdx ] specifies which transform kernels are applied to the residual samples along the horizontal and vertical direction of the associated transform block with cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture.

When tu_mts_idx[ x0 ][ y0 ][cIdx] is not present, it is inferred to be equal to 0.

Detail changes relative to JVET-N1001v8 are found in JVET-O0081_chromaTS_WD.docx.

Encoder search for chroma transform type selection

Similar fast transform type selection algorithm described in [1] is used for chroma TS search as follow:

- Firstly, calculate the L1 norm for all 2 candidates ( DCT-II, TS )

- Secondly, TS with L1 norm greater than a specific multiple of the DCT-II L1 norm are excluded for further RDO testing. Additionally, RDO for TS is skipped when cbf with DCT-II case is equal to zero.

Modified chroma intra mode decision

In addition, the similar cost measurement for luma intra mode decision in [3] is used for chroma intra mode decision. As described in [3], transform-domain cost (SATD) may not match with TS efficiently since TS applies quantization and entropy coding directly on the spatial-domain. Thus, minimum value between SAD and SATD is used instead of SATD for chroma intra mode decision as bellow:

- Firstly, derive cost per chroma intra mode candidate

  - CE8-3.2: cost = min( SAD*2, SATD)

  - VTM-5.0: cost = SATD

- Secondly, sort cost in descending order and exclude top two modes with larger cost for further RDO testing

Simulation Results

The proposed chroma TS has been implemented on top of VTM-5.0 and following simulations were conducted under CE8 test condition [6]:

- Test1:	VTM-5.0 vs CE8-3.2 for YUV420 under common QP condition

- Test2: VTM-5.0 vs CE8-3.2 for YUV420 under low QP condition (QP=2,7,12,17)

- Test3: VTM-5.0 vs CE8-3.2 for YUV444 under common QP condition

- Test4: VTM-5.0 vs CE8-3.2 for YUV444 under low QP condition (QP=2,7,12,17)

Table 3-1 to 3-4 summarizes results of each Test1, Test2, Test3 and Test4, respectively.

Table 3-1: Results of YUV420 under common QP condition (Test1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.03% | -0.03% | 101% | 99% |
| Class A2 | 0.01% | -0.02% | 0.01% | 101% | 100% |
| Class B | 0.01% | 0.02% | 0.07% | 101% | 100% |
| Class C | 0.02% | 0.08% | 0.11% | 101% | 100% |
| Class E | 0.03% | 0.11% | -0.11% | 101% | 100% |
| Overall | 0.02% | 0.04% | 0.02% | 101% | 100% |
| Class D | 0.02% | 0.40% | 0.08% | 101% | 100% |
| Class F | 0.02% | -0.10% | -0.09% | 101% | 100% |
| Class SCC | 0.02% | -0.57% | -0.85% | 102% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.34% | 0.27% | 101% | 100% |
| Class A2 | 0.06% | -0.08% | -0.07% | 102% | 100% |
| Class B | 0.02% | 0.17% | 0.14% | 101% | 100% |
| Class C | -0.01% | 0.08% | 0.22% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.01% | 0.15% | 102% | 100% |
| Class D | 0.02% | 0.19% | 0.52% | 102% | 100% |
| Class F | 0.05% | -0.09% | -0.11% | 102% | 100% |
| Class SCC | -0.03% | -0.72% | -0.98% | 103% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.69% | 0.09% | 101% | 100% |
| Class C | 0.07% | 0.71% | 0.57% | 102% | 100% |
| Class E | 0.11% | 1.26% | 1.65% | 100% | 99% |
| Overall | 0.06% | 0.84% | 0.64% | 101% | 100% |
| Class D | 0.09% | 1.27% | 1.40% | 101% | 99% |
| Class F | 0.13% | 0.19% | 0.20% | 103% | 103% |
| Class SCC | 0.09% | -0.76% | -1.18% | 105% | 112% |

Table 3-2: Results of YUV420 under lowQP condition (Test2)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.01% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 101% |
| Class B | 0.03% | -0.07% | -0.07% | 102% | 100% |
| Class C | 0.02% | -0.06% | -0.10% | 104% | 102% |
| Class E | 0.09% | -0.07% | -0.21% | 102% | 100% |
| Overall | 0.03% | -0.05% | -0.08% | 102% | 100% |
| Class D | 0.03% | -0.14% | -0.11% | 103% | 101% |
| Class F | -0.21% | -0.89% | -0.83% | 102% | 100% |
| Class SCC | -2.21% | -4.07% | -4.05% | 103% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.03% | 104% | 100% |
| Class A2 | 0.02% | -0.02% | -0.01% | 104% | 99% |
| Class B | 0.04% | -0.14% | -0.11% | 103% | 100% |
| Class C | 0.03% | -0.20% | -0.18% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.10% | -0.08% | 104% | 100% |
| Class D | 0.04% | -0.21% | -0.20% | 104% | 100% |
| Class F | -0.19% | -1.56% | -1.33% | 103% | 100% |
| Class SCC | -6.00% | -2.88% | -2.95% | 104% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.20% | -0.17% | 103% | 100% |
| Class C | 0.03% | -0.19% | -0.16% | 102% | 99% |
| Class E | 0.10% | -0.33% | -0.93% | 101% | 99% |
| Overall | 0.07% | -0.23% | -0.36% | 102% | 100% |
| Class D | 0.06% | -0.20% | -0.04% | 103% | 99% |
| Class F | -0.18% | -1.55% | -1.46% | 102% | 98% |
| Class SCC | -1.48% | -2.47% | -2.61% | 106% | 98% |

Table 3-3: Results of YUV444 under common QP condition (Test3)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 Dual Tree On |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.89% | -4.16% | -4.56% | 107% | 98% |
| TGM 720p | 0.05% | -1.64% | -2.03% | 104% | 99% |
| Animation | 0.07% | -0.18% | -0.18% | 102% | 100% |
| Mixed content | 0.09% | -1.98% | -2.08% | 104% | 100% |
| Camera-Captured content | 0.04% | 0.00% | 0.02% | 102% | 100% |
| Overall | -0.19% | -1.96% | -2.19% | 104% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 Dual Tree On |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.64% | -5.29% | -5.54% | 106% | 99% |
| TGM 720p | -0.07% | -2.22% | -2.28% | 103% | 100% |
| Animation | 0.30% | -0.86% | -0.90% | 102% | 100% |
| Mixed content | -0.11% | -2.78% | -2.73% | 103% | 100% |
| Camera-Captured content | 0.03% | -0.09% | -0.13% | 102% | 100% |
| Overall | -0.43% | -2.68% | -2.77% | 103% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 Dual Tree On |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.02% | -4.60% | -5.04% | 105% | 99% |
| TGM 720p | 0.11% | -2.23% | -2.46% | 102% | 101% |
| Animation | 0.57% | -0.69% | -0.76% | 103% | 100% |
| Mixed content | 0.08% | -3.04% | -3.61% | 102% | 99% |
| Camera-Captured content | 0.08% | -0.20% | 0.03% | 102% | 100% |
| Overall | -0.14% | -2.55% | -2.82% | 103% | 100% |

Table 3-4: Results of YUV444 under low QP condition (Test4)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 DualTree On |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -9.91% | -13.53% | -13.36% | 111% | 93% |
| TGM 720p | -3.95% | -7.07% | -6.75% | 107% | 97% |
| Animation | -0.05% | -0.55% | -0.58% | 102% | 101% |
| Mixed content | -3.21% | -6.35% | -6.85% | 106% | 96% |
| Camera-Captured content | -0.01% | 0.00% | 0.01% | 102% | 99% |
| Overall | -4.35% | -6.84% | -6.81% | 107% | 96% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 DualTree On |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -10.19% | -16.14% | -16.00% | 108% | 96% |
| TGM 720p | -3.34% | -6.94% | -6.52% | 105% | 100% |
| Animation | -1.79% | -6.63% | -6.25% | 105% | 98% |
| Mixed content | -1.91% | -5.93% | -6.70% | 104% | 97% |
| Camera-Captured content | -0.04% | -0.18% | -0.49% | 103% | 100% |
| Overall | -4.23% | -8.25% | -8.24% | 105% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 DualTree On |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -12.36% | -16.13% | -15.69% | 106% | 96% |
| TGM 720p | -3.02% | -6.82% | -6.08% | 104% | 98% |
| Animation | -2.12% | -5.31% | -5.10% | 104% | 96% |
| Mixed content | -1.79% | -6.12% | -6.97% | 104% | 98% |
| Camera-Captured content | -0.04% | -0.14% | -0.28% | 102% | 101% |
| Overall | -4.75% | -8.07% | -7.92% | 104% | 98% |

Conclusion

It is recommended to adopt proposed changes into the next VTM/VVC WD and enable chroma transform skip for SCC content (classF/TGM/YUV444).

Reference

[1] B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand, “Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip, ” JVET-M0464, Jan. 2019

[2] T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, “AHG13/Non-CE6/CE8: Chroma Transform Skip,” JVET-N0123, Mar. 2019

[3] M. Koo, M. Salehifar, J. Lim, S. Kim, “CE6: Reduced Secondary Transform (RST) (CE6-3.1), ”, JVET-N0193, Mar. 2019

[4] B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand, “CE8: Residual Coding for Transform Skip Mode (CE8-4.3a, CE8-4.3b, CE8-4.4a, and CE8-4.4b), ” JVET-N0280, Mar. 2019

[5] X. Zhao, X. Li, S. Liu, “CE6-related: Modified encoder decision for transform skip, ” JVET-N0363, Mar. 2019

[6] X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-N1028, Mar. 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).