| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0082 |
| --- | --- |

| Title: | CE4-2.4: Simplifications of interweaved affine mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, Xiang Li, Xiaozhong Xu, Shan Liu 2747 Park Blvd., Palo Alto, CA 94306 | Email: | guichunli@tencent.com xlxiangli@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This document reports the test result of CE4-2.4, simplifications of interweaved affine mode. Two simplifications were proposed on top of interweaved affine mode: 1) to regard interweaved affine mode as a special bi-predicted mode so that weighted bi-prediction process is reused; 2) to simplify the weighting process by using equal weighting. Two tests were conducted. In CE4-2.4.a, weighted bi-prediction process is reused for interweaved affine mode; In CE4-2.4.b, weighted bi-prediction process is reused and equal weighting is used for interweaved affine mode. Simulation results are as reported below:

CE4-2.4.a: RA -0.32%, 105%, 102%, LB -0.11%, 108%, 104%;

CE4-2.4.b: RA -0.28%, 106%, 102%, LB -0.15%, 108%, 103%.

# Introduction

Interweaved affine mode was proposed in JVET-M0268 [2]. First, a coding block is divided into sub-blocks with two different dividing patterns. The first dividing pattern is the same as that in VTM, while the second dividing pattern also divides the coding-block into 4×4 sub-blocks but with a 2×2 offset. Then, two auxiliary predictions are generated by affine motion compensation. The MV for each sub-block in a dividing pattern is derived from CPMVs by the affine model. The final prediction is calculated as a weighted-sum of the two auxiliary predictions, formulated as:

| . |  |
| --- | --- |

The weighting is position based. Basically, an auxiliary prediction sample located at the center of a sub-block is associated with a weighting value 3, while an auxiliary prediction sample located at the boundary of a sub-block is associated with a weighting value 1.

# Proposed Methods

The simplification methods were previously proposed in JVET-N0399 [1]. In the interweaved affine mode, the two predictions are both uni-predicted, and then a weighted averaging process is invoked to generate the final prediction. To keep higher interpolation accuracy and simple control logic, it is proposed to regard interweaved affine mode as a special bi-predicted mode such that the weighted averaging may reuse the current weighted bi-prediction process with interweaved affine weighting table as input.

Moreover, it is proposed to further simplify the weighting process: Always using 2:2 weighting for all pixels except for the shaded region (as shown in Fig. 1) which keeps using 4:0 weighting as only one prediction is available for such a region.

Fig. 1 Interweaved prediction is not applied on the shaded regions.

Two CE tests are as listed below:

CE4-2.4.a, weighted bi-prediction process is reused for interweaved affine mode;

CE4-2.4.b, weighted bi-prediction process is reused and equal weighting (1/2, 1/2) is used for interweaved affine mode.

## Complexity Analysis

The following table shows the complexity analysis of the proposed methods comparing with VTM-5.0. Since the uni-prediction is performed twice with shifted subblock position, and extra addition and averaging operations are also involved, the number of operations are increased compared with VTM-5.0. The number of additional local buffer in this table is counting the storage for extra 4x4 luma samples with 10-bit bit depth. But it depends on the way it’s implemented. If it’s implemented in a way that the interweaved affine reuses the buffer of bi-prediction, no additional buffer is necessary.

| Test# | Number of mults | Number of adds | Number of shifts | Additional memory access | Additional local buffer | Others |
| --- | --- | --- | --- | --- | --- | --- |
| VTM-5.0 | 400/800 | 350/700 | 74/172 | - | - | Results are shown for uni/bi-prediction separately |
| 4-2.4a | 728 / 800 | 626 / 700 | 164 / 172 | 0 | 20 bytes / 0 |  |
| 4-2.4b | 712 / 800 | 626 / 700 | 164 / 172 | 0 | 20 bytes / 0 |  |

# Test results

The proposed methods were implemented on top of the interweaved prediction software as in CE4-2.3, which is based on VTM-5.0. And VTM-5.0 is used as anchor, and the original version of CE4-2.3 was used as second anchor.

Table 1-Test results of CE4-2.4.a

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  | Over VTM-5.0+Interweaved Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.08% | 0.09% | 106% | 101% | 0.01% | 0.20% | -0.04% | 101% | 100% |
| Class A2 | -0.57% | 0.13% | 0.09% | 107% | 101% | -0.03% | -0.14% | 0.05% | 101% | 99% |
| Class B | -0.41% | -0.17% | -0.16% | 105% | 105% | -0.01% | -0.05% | -0.01% | 101% | 102% |
| Class C | -0.21% | 0.00% | -0.06% | 103% | 101% | -0.02% | 0.08% | 0.04% | 100% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.32% | -0.05% | -0.03% | 105% | 102% | -0.01% | 0.02% | 0.01% | 101% | 100% |
| Class D | -0.31% | 0.06% | 0.04% | 103% | 100% | 0.03% | 0.01% | -0.09% | 99% | 97% |
| Class F (optional) | -0.57% | -0.25% | -0.22% | 104% | 103% | -0.01% | 0.06% | -0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  | Over VTM-5.0+Interweaved Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.15% | 0.17% | 0.05% | 109% | 101% | -0.01% | 0.14% | 0.43% | 102% | 98% |
| Class C | -0.09% | -0.45% | -0.21% | 106% | 107% | 0.03% | -0.31% | -0.15% | 102% | 103% |
| Class E | -0.05% | 0.32% | -0.50% | 112% | 107% | 0.13% | 0.26% | -0.13% | 104% | 103% |
| Overall | -0.11% | 0.00% | -0.17% | 108% | 104% | 0.04% | 0.02% | 0.09% | 102% | 101% |
| Class D | -0.17% | 0.04% | 0.30% | 104% | 106% | 0.07% | 0.22% | -0.19% | 100% | 101% |
| Class F (optional) | -0.83% | -0.92% | -0.75% | 106% | 103% | 0.00% | -0.03% | 0.18% | 101% | 102% |

Table 2 - Test results of CE4-2.4.b

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  | Over VTM-5.0+Interweaved Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.15% | 0.05% | 106% | 101% | 0.02% | 0.13% | -0.07% | 101% | 100% |
| Class A2 | -0.50% | 0.12% | 0.04% | 107% | 101% | 0.04% | -0.14% | 0.00% | 101% | 100% |
| Class B | -0.37% | -0.23% | -0.18% | 105% | 103% | 0.03% | -0.11% | -0.03% | 101% | 100% |
| Class C | -0.18% | -0.05% | -0.11% | 105% | 103% | 0.02% | 0.02% | -0.01% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.28% | -0.09% | -0.07% | 106% | 102% | 0.03% | -0.03% | -0.03% | 101% | 100% |
| Class D | -0.28% | -0.05% | 0.09% | 104% | 103% | 0.05% | -0.10% | -0.04% | 101% | 100% |
| Class F (optional) | -0.45% | -0.24% | -0.15% | 105% | 103% | 0.12% | 0.07% | 0.03% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  | Over VTM-5.0+Interweaved Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.16% | 0.09% | -0.18% | 109% | 103% | -0.01% | 0.07% | 0.19% | 102% | 100% |
| Class C | -0.08% | -0.05% | -0.25% | 106% | 103% | 0.03% | 0.09% | -0.20% | 101% | 99% |
| Class E | -0.22% | 0.16% | -0.08% | 111% | 103% | -0.04% | 0.09% | 0.28% | 103% | 100% |
| Overall | -0.15% | 0.06% | -0.18% | 108% | 103% | -0.01% | 0.08% | 0.08% | 102% | 100% |
| Class D | -0.14% | -0.15% | 0.33% | 105% | 103% | 0.10% | 0.03% | -0.15% | 101% | 98% |
| Class F (optional) | -0.73% | -0.65% | -0.34% | 107% | 102% | 0.10% | 0.24% | 0.61% | 102% | 100% |

# Conclusions

It was observed that the proposed simplification methods have minor coding performance impact on interweaved affine prediction.

# References

- X. Li, G. Li, X. Xu, S. Liu, “CE2-related: Simplifications of interweaved affine mode”, JVET-N0399, Mar. 2019.

- K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong, “Non-CE2: Interweaved Prediction for Affine Motion Compensation,” JVET-M0268, Jan. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).