| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0083-v1 |
| --- | --- |

| Title: | CE5-1.1: Modification of sub-pu deblocking and longer tap filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra Semih Esenlik Biao Wang Han Gao Elena Alshina | Tel: Email: | anand.meher.kotra@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

The current contribution reports the results for CE5-1.1. This test reportedly simplifies the deblocking design when longer tap filter is applied to a coding unit which consists of internal transform or prediction boundary edges.

The objective results for CTC configuration are as follows: Over VTM5.0 Anchor (AI, RA, LDB, LDP): Luma BD-Rate of -0.01%, 0.01%, 0.04%, 0.03% is achieved.

The objective results with ALF OFF configuration are as follows: Over VTM5.0 Anchor (AI, RA, LDB, LDP): Luma BD-Rate of -0.01%, 0.02%, 0.04%, 0.07% is achieved.

Subjective evaluation reportedly showed no visual quality degradation when compared with the Anchor.

# Introduction

The deblocking filter in VTM-5.0 [1] uses a longer tap filter whenever the size of the block is >= 32 samples in the direction of deblocking. However when the given coding unit uses sub-pu tools such as Affine, ATMVP or uses sub-block transform tools such as SBT (sub-block transform) or ISP (intra sub-partition tool), then the longer tap filter application is constrained to avoid filtering overlaps. Therefore VTM-5.0 uses filters which modify different number of samples (7+5, 5+7, 5+5 on either side of the edge based on the mode of the coding unit. Also the filter tap length for the internal edges which are close to the coding unit edge or an internal transform edge (where longer tap filter can be applied) are also constrained. Such edges are deblocked using a filter which modifies a maximum of two samples.

CE5-1.1 targets simplifying the deblocking design when longer tap filter is applied to coding units which uses sub-pu tools or use sub- block transform tools.

# Proposed Solution

CE5-1.1 proposed two modifications to the deblocking design: the first modification disallows longer tap filter to be applied for the internal edges inside a coding unit.

The second modification disallows application of the deblocking filter for the first and last internal edges which overlap on the 8 x 8 grid beginning at the top-left corner of the given coding unit.

As a result of both the above modifications, the longer tap filter can now be applied directly to the coding unit edge without additional restrictions on the longer tap filter length. Therefore the longer tap filter applied is always 7+7 (except for the horizontal edges overlapping with the CTU boundaries).  Moreover the filter tap length restriction of 2 samples for the internal edges close to the coding unit edge or internal transform edge is also removed.

The subjective evaluation reportedly showed no visual artifacts when compared with the Anchor.

# Complexity evaluation

Complexity of the proposal is evaluated based on the following criteria described in the CE5 document, JVET-N1025 [3].

Luma deblocking complexity:

- Number of samples from the block boundary that can be modified by deblocking (P+Q):

  - Same as in VTM5.0, Except:

    - Longer tap filter variants of 7+5, 5+7, 5+5 does not exist anymore

    - Internal edges which maximum 2 samples modified also does not exist anymore

- Number of samples from the block boundary that are needed for deblocking decisions:

  - same as in VTM5.0

- Number of filtering operations per sample including decisions and filtering (such as additions, multiplications, comparisons)

  - Same as in VTM5.0

- Number of line buffers:

  - 4 lines (same as VTM5.0)

- Parallelization capabilities, such as size of a unit of a picture that can be processed in parallel to other units

  - All edges overlapping with 8 x 8 grid can be deblocked in parallel

- Whether proposals increase the worst case complexity and if so, by how much: No

Chroma deblocking complexity:

- Chroma filtering is not changed

# Simulation Results

The test is implemented on top of VTM-5.0 [1] and the simulation is performed under common test conditions as described in JVET-N1010 [2].

## Results with CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | -0.01% | 0.00% | 0.00% | 101% | 100% |
| Class B | -0.02% | 0.00% | 0.00% | 101% | 100% |
| Class C | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class E | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class D | -0.01% | 0.00% | 0.00% | 100% | 99% |
| Class F | -0.01% | 0.00% | 0.00% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.21% | 0.03% | 100% | 101% |
| Class A2 | 0.00% | 0.06% | -0.03% | 100% | 101% |
| Class B | 0.02% | 0.02% | -0.06% | 100% | 100% |
| Class C | -0.02% | -0.19% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.07% | -0.03% | 100% | 100% |
| Class D | -0.02% | -0.06% | 0.37% | 100% | 100% |
| Class F | -0.01% | 0.05% | 0.04% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.25% | 0.32% | 100% | 102% |
| Class C | 0.03% | 0.19% | -0.02% | 100% | 101% |
| Class E | 0.10% | 0.10% | -0.12% | 100% | 102% |
| Overall | 0.04% | 0.19% | 0.10% | 100% | 102% |
| Class D | 0.04% | 0.06% | 0.85% | 100% | 100% |
| Class F | 0.00% | -0.09% | -0.57% | 100% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.27% | 0.45% | 100% | 102% |
| Class C | 0.01% | 0.02% | 0.11% | 100% | 101% |
| Class E | 0.10% | 0.09% | -0.72% | 101% | 101% |
| Overall | 0.03% | 0.14% | 0.04% | 100% | 101% |
| Class D | 0.03% | -0.11% | 0.16% | 100% | 100% |
| Class F | 0.05% | 0.93% | -0.35% | 101% | 101% |

## Results with ALF=0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 (ALF OFF) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | -0.01% | 0.00% | 0.00% | 100% | 99% |
| Class C | -0.01% | 0.00% | 0.00% | 100% | 97% |
| Class E | -0.01% | 0.00% | 0.00% | 100% | 99% |
| Overall | -0.01% | 0.00% | 0.00% | 100% | 99% |
| Class D | -0.01% | 0.00% | 0.00% | 100% | 99% |
| Class F | -0.01% | 0.00% | 0.00% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 5.0 (ALF OFF) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.03% | -0.09% | 100% | 100% |
| Class A2 | 0.02% | -0.01% | -0.04% | 100% | 100% |
| Class B | 0.03% | -0.11% | -0.09% | 100% | 100% |
| Class C | 0.01% | 0.19% | -0.09% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.01% | -0.08% | 100% | 99% |
| Class D | -0.02% | -0.06% | 0.00% | 99% | 98% |
| Class F | 0.00% | -0.02% | 0.02% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0 (ALF OFF) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.06% | -0.26% | 100% | 100% |
| Class C | -0.01% | 0.06% | -0.16% | 100% | 100% |
| Class E | 0.13% | -0.40% | 0.12% | 100% | 100% |
| Overall | 0.04% | -0.11% | -0.13% | 100% | 100% |
| Class D | 0.03% | -0.07% | -0.39% | 100% | 100% |
| Class F | -0.05% | -0.35% | 0.51% | 100% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 5.0 (ALF OFF) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.28% | -0.44% | 100% | 100% |
| Class C | 0.05% | 0.02% | -0.21% | 100% | 99% |
| Class E | 0.18% | 0.74% | 0.59% | 100% | 101% |
| Overall | 0.07% | 0.08% | -0.10% | 100% | 100% |
| Class D | 0.03% | 0.26% | 0.03% | 100% | 99% |
| Class F | -0.03% | -0.27% | -0.48% | 99% | 100% |

# Conclusion

In order to simplify the current deblocking design, two changes are proposed. The first modification disallows application of longer tap filter to the internal boundaries and the second modification disables application of the deblocking filter to the first and last internal edges overlapping on the internal 8 x 8 sample gird for a given coding unit. These two modifications allow longer tap filter to be applied with reduced conditional checks. Specification text changes are attached along with the contribution.

# References

- Versatile Video Coding (Draft 5), Document JVET-N1001_v7, B. Bross, J. Chen, S. Liu

- “JVET common test conditions and software reference configurations”, JVET-N1010

- C.-Y. Chen and A. Norkin, "Description of Core Experiment 5 (CE5): Loop filtering," Document of Joint Video Experts Team, JVET-N1025.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).