| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 |  |  |  | Document: JVET-O0086-v4 |  |
| --- | --- | --- | --- | --- | --- |
| Title: | CE4-3.2a and CE4-3.2b: Unidirectional local illumination compensation with affine prediction and simplified linear model parameter derivation based on min / max method |  |  |  |  |  |
| Status: | Input document to JVET |  |  |  |  |  |
| Purpose: | Proposal |  |  |  |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Elena Alshina | Email: | alexey.filippov@huawei.com, vasily.rufitskiy@huawei.com, elena.alshina@huawei.com |  |  |  |
| Source: | Huawei Technologies Co. |  |  |  |  |  |

_____________________________

# Abstract

This document reportedly provides the results of testing compression performance of tests CE4-3.2a and CE4-3.2b, in which LIC is applied to affine motion compensated blocks and linear model parameters are estimated using min/max method instead of using linear regression. The document also reportedly provides the results of the tests when substitution of template samples of a predicted CU is enabled, when template filter is not applied and when the value of b is derived by averaging the two minimum and the two maximum values.

Test CE4-3.2a reportedly shows the following overall results over VTM-5.0: -0.41% (Y), -0.37% (U), -0.48% (V) with encoding time of 117% and decoding time of 100% for RA configuration; -0.31% (Y), -0.12% (U), -0.40% (V) with encoding time of 119% and decoding time of 101% for LDB configuration.

Test CE4-3.2b reportedly shows the following overall results over VTM-5.0: -0.24% (Y), -0.23% (U), -0.23% (V) with encoding time of 105% and decoding time of 101% for RA configuration; -0.13% (Y), -0.19% (U), -0.24% (V) with encoding time of 107% and decoding time of 101% for LDB configuration.

When substitution of template samples of a predicted CU is enabled, test CE4-3.2a reportedly shows the following overall results over VTM-5.0: -0.37% (Y), -0.33% (U), -0.43% (V) with encoding time of 117% and decoding time of 100% for RA configuration; -0.27% (Y), -0.10% (U), -0.44% (V) with encoding time of 119% and decoding time of 100% for LDB configuration. Under the same conditions, test CE4-3.2b reportedly shows the following overall results over VTM-5.0: -0.22% (Y), -0.14% (U), -0.26% (V) with encoding time of 105% and decoding time of 101% for RA configuration; -0.09% (Y), -0.10% (U), -0.34% (V) with encoding time of 108% and decoding time of 101% for LDB configuration.

When substitution of template samples of a predicted CU is enabled and template filter is not applied test CE4-3.2a reportedly shows the following overall results over VTM-5.0: -0.37% (Y), -0.33% (U), -0.44% (V) with encoding time of 118% and decoding time of 100% for RA configuration; -0.27% (Y), -0.06% (U), -0.32% (V) with encoding time of 120% and decoding time of 101% for LDB configuration. Under the same conditions, test CE4-3.2b reportedly shows the following overall results over VTM-5.0: -0.22% (Y), -0.19% (U), -0.26% (V) with encoding time of 105% and decoding time of 100% for RA configuration; -0.14% (Y), -0.10% (U), -0.38% (V) with encoding time of 108% and decoding time of 101% for LDB configuration.

# Introduction

Unidirectional LIC method proposed in [1] uses linear regression to estimate linear model parameters. The input to the linear regression are reconstructed template samples of the current CU and corresponding template samples of the reference block that are used to inter-predict current CU.

Proposal [2] introduced the following modification to the test. Particularly:

- Usage of reconstructed intra coded, CIIP and IBC blocks, as this would have impact on complexity and parallel implementation pipelining (test with and without, and further study the impact)

- Disabling LIC over CTU boundaries is not necessary

- Disable for CIIP and IBC blocks

- Disable LIC for the blocks with less than 64 luma samples.

- No temporal inheritance of LIC flag

- No pruning based on LIC flag in merging candidate list generation

- Disable LIC for all subblock modes, including triangle mode and BDOF

- It is not applied to 128xN and Nx128 blocks

- It is not applied to bi-prediction

# Proposal

The proposal provides the result of the Core Experiment 4, tests 3.2a and 3.2b, which are based on JVET-N0716 proposal and Core Experiment 4 tests 3.1a and 3.1b. The tests propose to replace linear regression method by the min/max method to estimate linear model parameters of LIC. The difference between test 3.2a and test 3.2b is the same as between tests 3.1a and test 3.1b and consists in enabling fast decision-making at the encoder side as well as restricting LIC to non-GBi CUs.

Additionally, samples used to estimate linear model parameters of LIC are restricted by the VPDU size. Specifically, maximum of 16 luma samples adjacent to top or left side of a CU could be used and 8 chroma samples (when chroma format is YUV4:2:0) adjacent to top or left side of a CU.

In test 3.1a and test 3.1b of the Core Experiment 4 linear model parameters are estimated without using reconstructed intra coded, CIIP and IBC blocks. Special handling of the template samples belonging to these blocks is required when linear model parameters are estimated using linear regression, since this method has efficient shift-based division implementation only when the number of samples used in the estimation is a 2n value. Hence, instead of skipping restricted template samples of the predicted CU, it was proposed to substitute them with the values of corresponding template samples of the referenced block.

The proposed min/max method does not have this implementation constraint and the restricted samples of the template blocks are skipped. When minimum and maximum positions are determined in the template samples of the reference block, samples that have positions corresponding to the restricted blocks are not considered. Hence, positions of minimum and maximum sample values in the reference picture will not correspond to the positions of the restricted template samples of the predicted block.

Proposal [3] used the above-described sample substitution because of unification of CCLM and LIC linear model parameter derivation processes. However, in the meeting minutes [4] it was noted that this was not urgently necessary – the more relevant aspect is the complexity reduction of LIC.

## No template downsampling

In CE4-3.2 test, the step of selecting samples from the template is set equal to 1 for both current and reference blocks and without consideration of block aspect ratio. The MinMax method does not require that the number of processed template samples would be a power of 2, and therefore template downsampling is not performed.

## Filtering of template samples

When affine motion compensation is used in CU prediction process, template samples of a reference block are being gathered from neighbor samples of the corresponding base unit areas. Each of these base unit areas are specified by motion vectors derived from affine prediction parameters. This gathering operation can introduce discontinuities between template samples corresponding to the two consequent base unit areas.

Prior to applying min/max method to estimate linear model parameters of LIC, the template samples of reference block and template samples of predicted CU are processed by an FIR filter. This filter applies only when a CU is predicted in affine mode and is required to remove the outliers. Filter cores are specified subject to the size of a block. If a block has more than 32 samples, filter coefficients are [1 0 2 0 1] / 4. If a block has 32 and less samples, filter coefficients are defined as [1 2 1] / 4.

Sampling of the FIR filter is adapted not to use restricted template samples, i.e. position of a FIR coefficient equal to “1” is accordingly changed to be closer to the central coefficient that is equal to “2” when this position corresponds to a restricted template sample.

## Derivation of Mean values

Instead of using minimum and maximum values to derive parameter b, the following operation is used:

b=ymean-a*xmean,

where ymean and xmean are template average values for the current template and the reference template, respectively.

## Slope value derivation

Not only for the smallest luma value A and largest luma value B but also for the second-smallest luma value A' and the second-largest luma value B' are searched as shown in Fig. 1 in JVET-N0716. Then, the trend line can be fitted between the average of the data points at A and A' (by using yA = (A + A') >>  1) and the average of the data points at B and B' (by using yB = (B + B') >> 1), which generally leads to a better data-fit:

a=yB-yAxB-xA   (1)

    - Derivation of the slope is performed in accordance with MinMax method.

    - In this method the following first and second minimum and maximum values are used:

    - maxRa = (maxR + maxR2)  >> 1,

    - minRa = (minR + minR2)  >> 1,

    - maxCa = (maxC + maxC2)  >> 1,

    - minCa = (minC + minC2)  >> 1,

where maxR is the first maximum in the reference template, maxR2 is the second maximum in the reference template, minR is the first minimum in the reference template, minR2 is the second minimum in the reference template, maxC is the first maximum in the current template, maxC2 is the second maximum in the current template, minC is the first minimum in the current template, minC2 is the second minimum in the current template.

    - The input values for MinMax parameter derivation method are as follows:

    - diffC = maxCa − minCa,

    - diff = maxRa – minRa,

where diff corresponds to xB-xA and diffC corresponds to yB-yA in equation (1).

# Experiment Results

Compression performance was evaluated under the JVET CTC conditions [5].

The results for test CE4-3.2a are reportedly presented in Table 1 for AI and RA configurations.

Table 1. Experimental results for test CE4-3.2a

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.60% | -0.58% | -0.55% | 123% | 100% |
| Class A2 | -0.35% | -0.24% | -0.43% | 112% | 100% |
| Class B | -0.43% | -0.30% | -0.52% | 116% | 100% |
| Class C | -0.31% | -0.39% | -0.41% | 118% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.41% | -0.37% | -0.48% | 117% | 100% |
| Class D | -0.15% | -0.17% | 0.01% | 114% | 101% |
| Class F | -0.52% | -0.34% | -0.44% | 111% | 100% |
|  | Low Delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | 0.04% | -0.29% | 125% | 100% |
| Class C | -0.34% | -0.43% | -0.63% | 125% | 102% |
| Class E | -0.09% | 0.04% | -0.25% | 102% | 100% |
| Overall | -0.31% | -0.12% | -0.40% | 119% | 101% |
| Class D | -0.11% | -0.08% | 0.33% | 120% | 101% |
| Class F | -1.56% | -1.74% | -1.81% | 118% | 101% |

The results for test CE4-3.2b are reportedly presented in Table 2 for AI and RA configurations.

Table 2. Experimental results for test CE4-3.2b

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.44% | -0.23% | 105% | 100% |
| Class A2 | -0.21% | -0.27% | -0.30% | 105% | 101% |
| Class B | -0.28% | -0.12% | -0.22% | 105% | 101% |
| Class C | -0.14% | -0.19% | -0.20% | 105% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.24% | -0.23% | -0.23% | 105% | 101% |
| Class D | 0.02% | 0.09% | 0.18% | 104% | 101% |
| Class F | -0.38% | -0.25% | -0.25% | 103% | 101% |
|  | Low Delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.25% | -0.26% | -0.19% | 107% | 100% |
| Class C | -0.18% | -0.38% | -0.22% | 108% | 101% |
| Class E | 0.14% | 0.16% | -0.34% | 106% | 100% |
| Overall | -0.13% | -0.19% | -0.24% | 107% | 101% |
| Class D | 0.05% | -0.54% | -0.40% | 107% | 101% |
| Class F | -1.41% | -1.89% | -2.24% | 105% | 101% |

Results for test CE4-3.2a and CE4-3.2b when substitution of template samples of a predicted CU is enabled are reportedly presented in Table 3 and Table 4 for AI and RA configurations.

Results for test CE4-3.2a and CE4-3.2b when substitution of template samples of a predicted CU is enabled and template filter is not applied are reportedly presented in Table 5 and Table 6 for AI and RA configurations.

Results for test CE4-3.2a and CE4-3.2b when substitution of template samples of a predicted CU is enabled and template filter is not applied and the value of b is derived by averaging the two minimum and the two maximum values are reportedly presented in Table 7 and Table 8 for AI and RA configurations.

Table 3. Experimental results for test CE4-3.2a when substitution of template samples of a predicted CU is enabled

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.55% | -0.54% | -0.54% | 123% | 99% |
| Class A2 | -0.32% | -0.27% | -0.39% | 112% | 100% |
| Class B | -0.39% | -0.29% | -0.48% | 116% | 101% |
| Class C | -0.25% | -0.26% | -0.31% | 118% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.37% | -0.33% | -0.43% | 117% | 100% |
| Class D | -0.14% | -0.09% | -0.05% | 114% | 101% |
| Class F | -0.46% | -0.31% | -0.39% | 111% | 101% |
|  | Low Delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.38% | 0.00% | -0.23% | 125% | 100% |
| Class C | -0.33% | -0.46% | -0.61% | 127% | 100% |
| Class E | -0.01% | 0.22% | -0.56% | 102% | 99% |
| Overall | -0.27% | -0.10% | -0.44% | 119% | 100% |
| Class D | -0.08% | -0.11% | 0.01% | 120% | 101% |
| Class F | -1.36% | -1.52% | -1.48% | 120% | 103% |

Table 4. Experimental results for test CE4-3.2b when substitution of template samples of a predicted CU is enabled

|  | Random Access |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 | -0.29% |  | -0.27% |  | -0.27% |  | 105% |  | 100% |
| Class A2 | -0.20% |  | -0.17% |  | -0.29% |  | 105% |  | 100% |
| Class B | -0.25% |  | -0.04% |  | -0.27% |  | 105% |  | 101% |
| Class C | -0.14% |  | -0.14% |  | -0.24% |  | 106% |  | 101% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.22% |  | -0.14% |  | -0.26% |  | 105% |  | 101% |
| Class D | 0.06% |  | 0.17% |  | 0.15% |  | 105% |  | 102% |
| Class F | -0.36% |  | -0.17% |  | -0.19% |  | 104% |  | 101% |
|  | Low Delay B |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.25% |  | 0.04% |  | -0.39% |  | 108% |  | 100% |
| Class C | -0.13% |  | -0.28% |  | -0.45% |  | 108% |  | 100% |
| Class E | 0.21% |  | -0.10% |  | -0.10% |  | 107% |  | 102% |
| Overall | -0.09% |  | -0.10% |  | -0.34% |  | 108% |  | 101% |
| Class D | 0.07% |  | -0.66% |  | 0.09% |  | 107% |  | 101% |
| Class F | -1.38% |  | -1.31% |  | -1.75% |  | 106% |  | 102% |

Table 5. Experimental results for test CE4-3.2a when substitution of template samples of a predicted CU is enabled and template filter is not applied

|  | Random Access |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 | -0.56% |  | -0.60% |  | -0.48% |  | 125% |  | 101% |
| Class A2 | -0.31% |  | -0.31% |  | -0.36% |  | 113% |  | 100% |
| Class B | -0.39% |  | -0.24% |  | -0.47% |  | 116% |  | 100% |
| Class C | -0.26% |  | -0.26% |  | -0.44% |  | 118% |  | 100% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.37% |  | -0.33% |  | -0.44% |  | 118% |  | 100% |
| Class D | -0.10% |  | -0.08% |  | 0.15% |  | 115% |  | 101% |
| Class F | -0.45% |  | -0.24% |  | -0.31% |  | 111% |  | 100% |
|  | Low Delay B |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.38% |  | -0.11% |  | -0.03% |  | 125% |  | 100% |
| Class C | -0.33% |  | -0.24% |  | -0.66% |  | 128% |  | 101% |
| Class E | -0.03% |  | 0.29% |  | -0.36% |  | 103% |  | 103% |
| Overall | -0.27% |  | -0.06% |  | -0.32% |  | 120% |  | 101% |
| Class D | -0.09% |  | -0.21% |  | -0.04% |  | 119% |  | 100% |
| Class F | -1.36% |  | -1.52% |  | -1.22% |  | 118% |  | 102% |

Table 6. Experimental results for test CE4-3.2b when substitution of template samples of a predicted CU is enabled and template filter is not applied

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | -0.26% | -0.24% | 105% | 99% |
| Class A2 | -0.17% | -0.19% | -0.23% | 105% | 100% |
| Class B | -0.25% | -0.08% | -0.23% | 105% | 100% |
| Class C | -0.15% | -0.29% | -0.32% | 106% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.19% | -0.26% | 105% | 100% |
| Class D | 0.03% | 0.24% | 0.22% | 105% | 101% |
| Class F | -0.32% | -0.27% | -0.20% | 103% | 100% |
|  | Low Delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.23% | 0.00% | -0.27% | 108% | 100% |
| Class C | -0.16% | -0.36% | -0.50% | 108% | 102% |
| Class E | 0.03% | 0.06% | -0.38% | 106% | 100% |
| Overall | -0.14% | -0.10% | -0.38% | 108% | 101% |
| Class D | 0.07% | -0.38% | -0.65% | 107% | 100% |
| Class F | -1.35% | -1.45% | -1.70% | 105% | 100% |

Table 7. Experimental results for test CE4-3.2a when substitution of template samples of a predicted CU is enabled and template filter is not applied and the value of b is derived by averaging the two minimum and the two maximum values

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | -0.50% | -0.48% | 124% | 100% |
| Class A2 | -0.27% | -0.20% | -0.28% | 113% | 100% |
| Class B | -0.34% | -0.19% | -0.38% | 116% | 100% |
| Class C | -0.22% | -0.24% | -0.25% | 118% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.27% | -0.35% | 117% | 100% |
| Class D | -0.08% | -0.10% | 0.22% | 114% | 100% |
| Class F | -0.43% | -0.26% | -0.25% | 111% | 100% |
|  | Low Delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.34% | 0.04% | -0.21% | 125% | 99% |
| Class C | -0.23% | -0.15% | -0.60% | 127% | 100% |
| Class E | -0.05% | 0.05% | 0.24% | 102% | 100% |
| Overall | -0.23% | -0.02% | -0.23% | 119% | 100% |
| Class D | -0.05% | -0.24% | 0.07% | 119% | 101% |
| Class F | -1.38% | -1.53% | -1.24% | 118% | 100% |

Table 8. Experimental results for test CE4-3.2b when substitution of template samples of a predicted CU is enabled and template filter is not applied and the value of b is derived by averaging the two minimum and the two maximum values

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.36% | -0.20% | 105% | 100% |
| Class A2 | -0.14% | -0.13% | -0.18% | 105% | 101% |
| Class B | -0.19% | -0.03% | -0.20% | 105% | 100% |
| Class C | -0.06% | -0.21% | -0.24% | 105% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.16% | -0.21% | 105% | 100% |
| Class D | 0.10% | 0.08% | 0.37% | 104% | 100% |
| Class F | -0.30% | -0.19% | -0.20% | 103% | 100% |
|  | Low Delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.17% | 0.13% | 0.20% | 107% | 100% |
| Class C | 0.00% | -0.13% | -0.20% | 107% | 100% |
| Class E | 0.13% | -0.22% | -1.07% | 106% | 101% |
| Overall | -0.04% | -0.05% | -0.25% | 107% | 100% |
| Class D | 0.14% | -0.64% | 0.08% | 106% | 101% |
| Class F | -1.19% | -1.13% | -0.66% | 106% | 103% |

Parameters a and b are clipped as follows:

If (abs(b) > (bThr << (bitDepthcIdx - 8))), the following applies:

1. a = b < 0 ? min(a, aMax) : max(a, aMin);

2. b=ymean-a*xmean

Clipping thresholds are defined for the tests above as follows:

aMax = 10240 >> (13 - shift); aMin = 7373 >> (13 - shift); and bThr =25.

Based on test CE4-3.2b, an additional test have been performed. It modifies the value of aMax:

# aMax = 8601 >> (13 - shift).

# Results of this test is given in Table 9.

Table 9. Experimental results for test CE4-3.2b when aMax = 8601 >> (13 - shift)

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.26% | -0.14% | -0.24% | 105% | 101% |
| Class C | -0.14% | -0.15% | -0.32% | 106% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.02% | 0.21% | 0.06% | 105% | 101% |
| Class F | -0.37% | -0.21% | -0.24% | 103% | 101% |
|  | Low Delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.12% | -0.26% | -0.32% | 108% | 99% |
| Class E | 0.04% | -0.20% | -0.53% | 107% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.02% | -0.61% | -0.58% | 107% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# Conclusion

The major difference between the methods studied in CE4-3.1 and CE4-3.2 is that test CE4-3.2 uses min / max linear parameter derivation, while CE4-3.1 uses least mean squares (LMS) linear model parameter estimation. The min / max linear parameter derivation method has lower computational complexity in comparison with the LMS-based method, especially on the decoder side. As compared with CE4-3.1, compression performance degradation for CE4-3.2 is minor. Therefore, in case LIC is adopted into the next version of VTM software and the VVC spec draft, we propose to adopt modifications studied in CE4-3.2 as a part of the adopted LIC method.

# References

- V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, M. Karczewicz, “CE10-related: Unidirectional illumination compensation” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M0500, Jan. 2019.

- V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, M. Karczewicz, “CE1: Unidirectional illumination compensation (CE1-5.1)” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N0343, Mar. 2019.

- A. Filippov, V. Rufitskiy, J. Chen, C. Helmrich, H. Schwarz, D. Marpe, T. Wiegand, “CE1-related: Simplified and robust LIC parameter derivation unified with CCLM” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N0716, Mar. 2019.

- G. J. Sullivan, J.-R. Ohm, “Meeting Report of the 14th JVET Meeting” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1000, Mar. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, 13th JVET Meeting, Marrakesh, MA, January 2018.

# Patent rights declaration(s)

Huawei Technologies Co. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).