| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O087 |
| --- | --- |

| Title: | CE2: report of CE2 experiments from InterDigital |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. François, T. Poirier | Tel: Email: | edouard.francois@interdigital.com tangi.poirier@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports test results of CE2 related to luma-chroma dependency reduction. Results are reported for CE2-1 related to the chroma residual coding in Single tree case, CE2-2 related to the chroma residual coding in Dual tree case, and CE2-3 related to the CCLM restrictions. It is reported that enabling Chroma residual scaling in Dual tree case provides around 0.6% to 0.7% BD-rate for PSNR-U and V for SDR content, in RA configuration. For HDR PQ content, activating Chroma residual scaling in Dual tree when LMCS is used, reportedly provides a closer luma/chroma balance compared to the HDR PQ CTC anchors that do not use LMCS. In RA configuration, BD-rate wPSNR variations of around -1.3% for Y, 5% for U and -1.8 to -2% for V against the HDR PQ CTVC anchors are reported when LMCS is on. The different variants tested in this contribution lead reportedly to similar performance. For CCLM, the proposed restrictions lead to around 0.1% loss in luma, and 1% loss in chroma, in RA configuration.

# Introduction

CE2 addresses the issue of luma-chroma dependency reduction. It relates to two coding tools, chroma residual scaling (CRS) and CCLM. CE2 is split into three sets of tests:

- CE2-1: Chroma residual scaling luma-chroma dependency reduction – Single-tree case

- CE2-2: Chroma residual scaling luma-chroma dependency reduction – Dual-tree case

- CE2-3: CCLM luma-chroma dependency reduction in Dual-tree case

This contribution reports the experiments described in JVET-N1022 and performed by InterDigital, addressing these 3 sub-CE categories, based on contributions JVET-N0389 “Chroma residual scaling with separate luma/chroma tree” and JVET-N0390 “AHG16/non-CE3: Study of CCLM restrictions in case of separate luma/chroma tree”.

# Tests description

## CE2-1: Chroma residual scaling luma-chroma dependency reduction – Single-tree case

CE2-1 relates to the usage of chroma residual scaling factor in the single-tree case. The tested changes apply to inter-slices only.

In CE2-1.3, the tested solution consists in computing the chroma residual scaling from the reconstructed luma samples neighboring the luma block co-located with the chroma block.

Various numbers of reconstructed luma samples used to derive the chroma residual scaling factor are reported in this contribution (see also Figure 1):

- CE2-1.3a: top-left, or left, or top reconstructed luma samples

- CE2-1.3b: average of 1 sample from the top line (if available) and 1 sample from the left column (if available)

- CE2-1.3c: average of 2 samples from the top line (if available) and 2 sample from the left column (if available)

- CE2-1.3d: average of 4 samples from the top line (if available) and 4 sample from the left column (if available)

- CE2-1.3e: average of all samples from the top line (if available) and all sample from the left column (if available)

Figure 1. Examples of neighboring samples used for chroma residual scaling.

## CE2-2: Chroma residual scaling luma-chroma dependency reduction – Dual-tree case

CE2-2 relates to the usage of chroma residual scaling factor in the dual-tree case. In these tests, chroma residual scaling is activated both for intra- and inter-slices. In both cases, the chroma residual scaling factor is derived from luma samples neighboring the luma block co-located with the top-left sample of the chroma block.

Three CE2-2 cases are reported in this contribution:

- CE2-2.1: For all slices, chroma residual scaling is derived from the neighboring reconstructed luma samples

- CE2-2.2: For all slices, chroma residual scaling is derived from the neighboring reference luma samples used for the intra prediction of the luma block.

- CE2-2.3: For intra slices, chroma residual scaling is derived from the neighboring reconstructed luma samples. For inter slices, chroma residual scaling is derived from the neighboring reference luma samples used for the intra prediction of the luma block.

CE2-2.1.a to CE2-2.1e and CE2-2.2.a to CE2-2.2e use various numbers of luma samples to derive the chroma residual scaling factor are tested. CE2-2.3 uses all the neighboring samples to derive the chroma residual scaling factor.

## CE2-3: CCLM luma-chroma dependency reduction in Dual-tree case

CE2-3 aims at reducing the latency for luma-chroma latency for CCLM for the dual-tree case. Two tests are considered in CE2-3.

CE2-3.1 is based on contribution JVET-N0390. In the test, in intra-slices with dual tree, CCLM is enabled only if the chroma block is inside a 16x16 non-overlapping area in a 16x16 sample grid, and if all its co-located luma samples belong to luma blocks included in the corresponding 32x32 area.

CE2-3.2 applies to CCLM the same approach as CE2-2. In intra-slices with dual tree, the CCLM parameters for a chroma block are derived from the reconstructed luma/chroma samples neighboring the luma block collocated with the chroma block top-left sample.

# SDR results

## CE2-1.3

The table below summarizes the BD-rate results for CE2-1.3a to CE2-1.3e, for SDR content. Some tests are still running or will not be performed as it is observed that the different tests already available lead to extremely close results. Encoder and decoder times are close to 100% and are therefore not reported.

The following observations can be made:

- Deriving the chroma residual factor from neighboring reconstructed luma samples, instead of using the collocated prediction luma samples, has almost no impact on the coding performance.

- The number of luma samples has also a very limited impact on the results.

|  | All Intra Main10 |  |  | Random Access Main10 |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| CE2-1.3a | 0.00% | 0.00% | 0.00% | 0.01% | -0.09% | -0.03% |  |  |  |
| CE2-1.3c | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | -0.07% |  |  |  |
| CE2-1.3d | 0.00% | 0.00% | 0.00% | 0.00% | -0.01% | -0.02% |  |  |  |
| CE2-1.3e | 0.00% | 0.00% | 0.00% | -0.01% | -0.02% | -0.06% | 0.00% | 0.11% | -0.13% |

## CE2-2

The table below summarizes the BD-rate results for CE2-2. As for previous CE2 subset, due to the extremely close performance observed in the various considered cases, some tests or configurations have not been performed. Encoder and decoder times are close to 100% and are therefore not reported.

The following observations can be made:

- activating CRS in dual tree leads to a gain for chroma in all configurations, without noticeable impact on luma. For instance, for RA configuration, a gain of 0.6%-0.7% in U and V components for SDR content is observed.

- The type of luma samples (reconstructed or reference) has a very limited impact on the results.

- The number of luma samples has also a very limited impact on the results.

|  | All Intra Main10 |  |  | Random Access Main10 |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| CE2-2.1a | 0.03% | -0.46% | -0.62% | 0.03% | -0.71% | -0.59% | not done |  |  |
| CE2-2.1b | 0.03% | -0.39% | -0.55% | still running |  |  | not done |  |  |
| CE2-2.1c | 0.03% | -0.38% | -0.55% | 0.02% | -0.62% | -0.61% | not done |  |  |
| CE2-2.1d | 0.02% | -0.37% | -0.53% | 0.02% | -0.64% | -0.59% | not done |  |  |
| CE2-2.1e | 0.03% | -0.37% | -0.52% | 0.01% | -0.64% | -0.66% | -0.01% | -0.39% | -0.50% |
| CE2-2.2a | 0.04% | -0.44% | -0.60% | 0.02% | -0.66% | -0.61% | not done |  |  |
| CE2-2.2b | 0.03% | -0.36% | -0.52% | still running |  |  | not done |  |  |
| CE2-2.2c | 0.03% | -0.38% | -0.55% | still running |  |  | not done |  |  |
| CE2-2.2d | 0.03% | -0.36% | -0.51% | 0.02% | -0.67% | -0.67% | not done |  |  |
| CE2-2.2e | 0.03% | -0.34% | -0.52% | 0.03% | -0.58% | -0.65% | 0.00% | -0.48% | -0.50% |
| CE2-2.3 | 0.03% | -0.37% | -0.52% | 0.03% | -0.61% | -0.68% | 0.02% | -0.45% | -0.41% |

## CE2-3

The table below summarizes the BD-rate results for CE2-3.1 and CE2-3.2.

|  | All Intra Main10 |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| CE2-3.1 | 0.28% | 2.37% | 2.53% | 0.07% | 1.08% | 0.96% |
| CE2-3.2 | 0.33% | 2.17% | 2.28% | 0.10% | 1.16% | 0.98% |

# HDR PQ results

## Preamble – LMCS performance compared to HDR PQ anchors

The HDR PQ CTCs anchors do not use LMCS. The To assess the performance of the CE2 tests for HDR PQ, another anchor named “HDR_Anchor_Bis” has been also used.

HDR_Anchor_Bis activates LMCS, and corresponds to the following settings:

- LumaLevelToDeltaQPMode : 0

- LumaReshapeEnable : 1

- ReshapeSignalType : 1

Compared to the HDR CTC anchors, HDR_Anchor_Bis has the following BD-rate performance. The chroma loss is due in a large part to the de-activation of chroma residual scaling in Intra slices. This results in a significant different balance between luma and chroma, compared to the HDR CTC anchors. One interest of CE2-2 is to evaluate the impact of re-activating chroma residual scaling in Intra slices.

|  | All Intra Main10 |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | wY | wU | wV |
| LMCS VTM5 | -1.64% | 14.15% | 8.35% | -1.60% | 14.02% | 6.96% |

## CE2-1.3

The wPSNR BD-rate for CE2-1.3a, CE2-1.3d and CE2-1.3e are reported below. As in SDR case, the impact of the change is minor.

|  | All Intra Main10 |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | wY | wU | wV |
| CE2-1.3a | 0.00% | 0.00% | 0.00% | -0.01% | -0.10% | 0.10% |
| CE2-1.3d | 0.00% | 0.00% | 0.00% | -0.01% | -0.01% | 0.03% |
| CE2-1.3e | 0.00% | 0.00% | 0.00% | -0.01% | -0.04% | 0.02% |

## CE2-2

The wPSNR BD-rate for CE2-2 are reported below, compared to HDR CTC anchors (that is, with LMCS off), and compared to HDR_anchor_Bis (that is, with LMCS on).

Using HDR CTC anchors (LMCS off)

|  | All Intra Main10 |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | wY | wU | wV |
| CE2-2.1a | -1.18% | 4.72% | -1.46% | -1.29% | 5.48% | -1.37% |
| CE2-2.1d | -1.25% | 4.25% | -2.41% | -1.33% | 5.11% | -1.80% |
| CE2-2.1e | -1.24% | 4.36% | -2.31% | -1.34% | 5.01% | -2.10% |
| CE2-2.2a | -1.19% | 4.82% | -1.47% | -1.29% | 5.42% | -1.25% |
| CE2-2.2d | -1.24% | 4.29% | -2.43% | -1.32% | 5.19% | -1.68% |
| CE2-2.2e | -1.24% | 4.29% | -2.27% | -1.33% | 5.00% | -1.78% |
| CE2-2.3 | -1.24% | 4.36% | -2.31% | -1.33% | 5.01% | -2.06% |

Using HDR_anchor_Bis (LMCS on)

|  | All Intra Main10 |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wY | wU | wV | wY | wU | wV |
| CE2-2.1a | 0.46% | -7.29% | -7.68% | 0.31% | -7.08% | -7.25% |
| CE2-2.1d | 0.39% | -7.67% | -8.54% | 0.27% | -7.38% | -7.61% |
| CE2-2.1e | 0.40% | -7.55% | -8.44% | 0.26% | -7.49% | -7.91% |
| CE2-2.2a | 0.45% | -7.20% | -7.69% | 0.31% | -7.15% | -7.10% |
| CE2-2.2d | 0.40% | -7.63% | -8.55% | 0.28% | -7.30% | -7.52% |
| CE2-2.2e | 0.41% | -7.63% | -8.44% | 0.27% | -7.50% | -7.58% |
| CE2-2.3 | 0.40% | -7.55% | -8.44% | 0.27% | -7.49% | -7.85% |

The following observations can be made:

- activating CRS is very beneficial for chroma; the balance luma/chroma is therefore closer to what is obtained in HDR PQ CTC anchors.

- The type of luma samples (reconstructed or reference) has a very limited impact on the results.

- The number of luma samples has also a very limited impact on the results.

# Summarizing tables

The table below summarizes the results presented in this contribution for the SDR case.

|  | All Intra Main10 |  |  | Random Access Main10 |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| CE2-1.3a | 0.00% | 0.00% | 0.00% | 0.01% | -0.09% | -0.03% | not done |  |  |
| CE2-1.3b | Withdrawn |  |  | withdrawn |  |  | withdrawn |  |  |
| CE2-1.3c | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | -0.07% | not done |  |  |
| CE2-1.3d | 0.00% | 0.00% | 0.00% | 0.00% | -0.01% | -0.02% | not done |  |  |
| CE2-1.3e | 0.00% | 0.00% | 0.00% | -0.01% | -0.02% | -0.06% | 0.00% | 0.11% | -0.13% |
| CE2-2.1a | 0.03% | -0.46% | -0.62% | 0.03% | -0.71% | -0.59% | not done |  |  |
| CE2-2.1b | 0.03% | -0.39% | -0.55% | still running |  |  | not done |  |  |
| CE2-2.1c | 0.03% | -0.38% | -0.55% | 0.02% | -0.62% | -0.61% | not done |  |  |
| CE2-2.1d | 0.02% | -0.37% | -0.53% | 0.02% | -0.64% | -0.59% | not done |  |  |
| CE2-2.1e | 0.03% | -0.37% | -0.52% | 0.01% | -0.64% | -0.66% | -0.01% | -0.39% | -0.50% |
| CE2-2.2a | 0.04% | -0.44% | -0.60% | 0.02% | -0.66% | -0.61% | not done |  |  |
| CE2-2.2b | 0.03% | -0.36% | -0.52% | still running |  |  | not done |  |  |
| CE2-2.2c | 0.03% | -0.38% | -0.55% | still running |  |  | not done |  |  |
| CE2-2.2d | 0.03% | -0.36% | -0.51% | 0.02% | -0.67% | -0.67% | not done |  |  |
| CE2-2.2e | 0.03% | -0.34% | -0.52% | 0.03% | -0.58% | -0.65% | 0.00% | -0.48% | -0.50% |
| CE2-2.3 | 0.03% | -0.37% | -0.52% | 0.03% | -0.61% | -0.68% | 0.02% | -0.45% | -0.41% |
| CE2-3.1 | 0.28% | 2.37% | 2.53% | 0.07% | 1.08% | 0.96% | not done |  |  |
| CE2-3.2 | 0.33% | 2.17% | 2.28% | 0.10% | 1.16% | 0.98% | not done |  |  |

The table below summarizes the results presented in this contribution for the HDR PQ case.

| anchor HDR PQ CTC | All Intra Main10 |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (LQP on LMCS off) | wY | wU | wV | wY | wU | wV |
| LMCS VTM5 | -1.64% | 14.15% | 8.35% | -1.59% | 14.12% | 6.85% |
| CE2-1.3a | -1.64% | 14.15% | 8.35% | -1.60% | 14.02% | 6.96% |
| CE2-1.3d | -1.64% | 14.15% | 8.35% | -1.60% | 14.11% | 6.89% |
| CE2-1.3e | -1.64% | 14.15% | 8.35% | -1.60% | 14.09% | 6.90% |
| CE2-2.1a | -1.18% | 4.72% | -1.46% | -1.29% | 5.48% | -1.37% |
| CE2-2.1d | -1.25% | 4.25% | -2.41% | -1.33% | 5.11% | -1.80% |
| CE2-2.1e | -1.24% | 4.36% | -2.31% | -1.34% | 5.01% | -2.10% |
| CE2-2.2a | -1.19% | 4.82% | -1.47% | -1.29% | 5.42% | -1.25% |
| CE2-2.2d | -1.24% | 4.29% | -2.43% | -1.32% | 5.19% | -1.68% |
| CE2-2.2e | -1.24% | 4.29% | -2.27% | -1.33% | 5.00% | -1.78% |
| CE2-2.3 | -1.24% | 4.36% | -2.31% | -1.33% | 5.01% | -2.06% |

| anchor HDR PQ LMCS | All Intra Main10 |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (LQP off LMCS on) | wY | wU | wV | wY | wU | wV |
| CE2-1.3a | 0.00% | 0.00% | 0.00% | -0.01% | -0.10% | 0.10% |
| CE2-1.3d | 0.00% | 0.00% | 0.00% | -0.01% | -0.01% | 0.03% |
| CE2-1.3e | 0.00% | 0.00% | 0.00% | -0.01% | -0.04% | 0.02% |
| CE2-2.1a | 0.46% | -7.29% | -7.68% | 0.31% | -7.08% | -7.25% |
| CE2-2.1d | 0.39% | -7.67% | -8.54% | 0.27% | -7.38% | -7.61% |
| CE2-2.1e | 0.40% | -7.55% | -8.44% | 0.26% | -7.49% | -7.91% |
| CE2-2.2a | 0.45% | -7.20% | -7.69% | 0.31% | -7.15% | -7.10% |
| CE2-2.2d | 0.40% | -7.63% | -8.55% | 0.28% | -7.30% | -7.52% |
| CE2-2.2e | 0.41% | -7.63% | -8.44% | 0.27% | -7.50% | -7.58% |
| CE2-2.3 | 0.40% | -7.55% | -8.44% | 0.27% | -7.49% | -7.85% |

# Patent rights declaration

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).