| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 Jul. 2019 | Document: JVET-O0089-v1 |
| --- | --- |

| Title: | CE7-3.3/4: Context Modelling of sign for TS residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tung Nguyen, Benjamin Bross, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-230 tung.nguyen@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the results for CE7-3.3 and CE7-3.4 that modify the context modelling for the sign syntax element of the residual coding stage for the transform skip mode. The experimental results are as follows.

CE7-3.3 QP 22 – 37	(Luma BD-rates for CTC, Class F, TGM, encoding times for CTC, Class F, TGM)

All-Intra:			 0.00%	-0.14%	-0.14%	100%	100%	100%

Random-Access:	-0.01%	-0.04%	-0.19%	100%	100%	  99%

CE7-3.3 QP 2 – 7	(Luma BD-rates for CTC, Class F, TGM, encoding times for CTC, Class F, TGM)

All-Intra:			 0.00%	-0.01%	-0.12%	100%	101%	100%

Random-Access:	 0.00%	-0.12%	-5.53%	100%	100%	100%

CE7-3.4 QP 22 – 37	(Luma BD-rates for CTC, Class F, TGM, encoding times for CTC, Class F, TGM)

All-Intra:			 0.00%	-0.10%	-0.13%	100%	100%	100%

Random-Access:	 0.01%	-0.05%	-0.14%	  99%	 99%	  99%

CE7-3.4 QP 2 – 7	(Luma BD-rates for CTC, Class F, TGM, encoding times for CTC, Class F, TGM)

All-Intra:			 0.00%	-0.02%	-0.08%	100%	101%	100%

Random-Access:	 0.00%	-0.19%	 1.14%	100%	101%	100%

# Introduction

VTM-5.0 introduces a modified residual coding process for cases where the encoder skips the transform, the so-called transform skip mode (TSM). The syntax specifies a single context model for the sign flag when the corresponding scan position is significant.

The experiments CE7-3.3 and CE7-3.4 modifies the context modelling of the sign syntax element as follows.

The proposed approach employs three context models as follows. For each transform block using TSM, the encoder/decoder tracks the number of sign syntax elements equal to one and the total number of sign syntax elements. The relative ratio r between the two is updated and evaluated resulting in three context models: the case r < a employs the first context model, the case r > b employs the second context model, and the third (and initial) context model is employed for the case where a ≤ r ≤ b, with a = 0.05 and b = 0.95, respectively.

In the experiment CE7-3.3, a = 0.05 and b = 0.95, respectively, and in the experiment CE7-3.4, a = 0.15 and b = 0.85, respectively.

# Experimental results

The following part presents four set of results: CE7-3.3 and CE7-3.4 using the Common Test Conditions (CTC), and both configurations using the CTC in the so-called low QP range with the QPs are 2, 7, 12, and 17.

## CE7-3.3: CTC QPs (22, 27, 32, 37)

Table I: Results for the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | -0.02% | -0.01% | 101% | 101% |
| Class B | 0.00% | -0.04% | 0.05% | 100% | 100% |
| Class C | 0.01% | -0.02% | -0.06% | 100% | 101% |
| Class E | 0.01% | -0.04% | -0.04% | 100% | 100% |
| Overall | 0.00% | -0.03% | -0.01% | 100% | 100% |
| Class D | 0.00% | 0.07% | 0.06% | 99% | 101% |
| Class F | -0.14% | -0.02% | -0.03% | 100% | 101% |
| Class TGM | -0.14% | -0.07% | -0.06% | 100% | 101% |

Table II: Results for the RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.25% | -0.02% | 100% | 102% |
| Class A2 | 0.02% | 0.01% | 0.07% | 100% | 102% |
| Class B | 0.01% | 0.17% | -0.08% | 99% | 99% |
| Class C | -0.05% | -0.21% | -0.21% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.05% | -0.07% | 100% | 100% |
| Class D | -0.02% | -0.22% | 0.23% | 100% | 100% |
| Class F | -0.04% | -0.08% | 0.08% | 100% | 102% |
| Class TGM | -0.19% | -0.02% | -0.11% | 99% | 101% |

## CE7-3.3: Low QPs (2, 7, 12, 17)

Table III: Results for the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.01% | 100% | 103% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.01% | -0.01% | 0.01% | 101% | 101% |
| Class C | -0.01% | 0.03% | 0.00% | 101% | 100% |
| Class E | 0.00% | 0.02% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class D | -0.02% | -0.02% | 0.02% | 100% | 100% |
| Class F | -0.01% | -0.05% | -0.05% | 101% | 101% |
| Class TGM | -0.12% | -0.07% | -0.06% | 100% | 101% |

Table IV: Results for the RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Class C | 0.00% | -0.05% | -0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.01% | 0.03% | -0.07% | 99% | 100% |
| Class F | -0.12% | 0.01% | -0.02% | 100% | 103% |
| Class TGM | -5.53% | -0.05% | -0.05% | 100% | 101% |

## CE7-3.4: CTC QPs (22, 27, 32, 37)

Table I: Results for the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | -0.02% | 99% | 100% |
| Class A2 | 0.00% | -0.04% | 0.00% | 101% | 101% |
| Class B | 0.00% | -0.03% | 0.06% | 100% | 99% |
| Class C | -0.01% | 0.07% | -0.05% | 100% | 100% |
| Class E | 0.02% | -0.02% | -0.07% | 100% | 100% |
| Overall | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class D | 0.00% | 0.09% | 0.05% | 100% | 99% |
| Class F | -0.10% | -0.01% | 0.06% | 100% | 101% |
| Class TGM | -0.13% | -0.07% | -0.06% | 100% | 102% |

Table II: Results for the RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.25% | -0.02% | 99% | 103% |
| Class A2 | 0.03% | -0.03% | 0.12% | 99% | 100% |
| Class B | 0.01% | 0.14% | -0.07% | 100% | 99% |
| Class C | -0.01% | -0.14% | -0.13% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.05% | -0.04% | 99% | 100% |
| Class D | 0.01% | 0.09% | 0.04% | 100% | 100% |
| Class F | -0.05% | -0.01% | 0.03% | 99% | 101% |
| Class TGM | -0.14% | -0.05% | -0.06% | 99% | 101% |

## CE7-3.4: Low QPs (2, 7, 12, 17)

Table III: Results for the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.01% | 100% | 103% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.01% | 0.01% | 100% | 101% |
| Class C | 0.00% | 0.01% | -0.02% | 100% | 99% |
| Class E | 0.00% | 0.03% | 0.01% | 99% | 98% |
| Overall | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class D | -0.01% | -0.03% | -0.01% | 101% | 99% |
| Class F | -0.02% | -0.01% | -0.06% | 101% | 102% |
| Class TGM | -0.08% | -0.07% | -0.06% | 100% | 102% |

Table IV: Results for the RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.00% | 0.01% | 0.01% | 100% | 101% |
| Class C | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.03% | 0.02% | -0.01% | 100% | 100% |
| Class F | -0.19% | 0.02% | -0.06% | 101% | 103% |
| Class TGM | 1.14% | -0.15% | -0.12% | 100% | 101% |

# Draft changes

| coeff_sign_flag[ ] transform_skip_flag[ x0 ][ y0 ] = = 1 | ( MaxCcbs > 0) ? ( intra_bdpcm_flag ? 1 : 0 ) : bypass | na | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |
| coeff_sign_flag[ ] transform_skip_flag[ x0 ][ y0 ] = = 1 | ( MaxCcbs > 0) ? clause x.x.x.x.x : bypass | na | na | na | na | na |

##### Derivation process of ctxInc for the syntax element coeff_sign_flag

Inputs to this process are the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the variable ctxInc.

Given the array AbsLevel[ x ][ y ] for the transform block and the top-left luma location ( x0, y0 ), and the array ScanPattern[ c ] with the current coefficient scan location ( xC, yC ) = ScanPattern[ p ], the variables numSign and sumSign are derived as follows:

p++
( xN, yN ) = ScanPattern[ p ]
sumSign = 0
numSign = 0
while( xN < ( 1 << log2TbWidth ) – 1 || yN < ( 1 << log2TbHeight ) – 1 ) {
	sumSign += AbsLevel[ xN ][ yN ]
	numSign += ( AbsLevel[ xN ][ yN ] != 0 ? 1 : 0 )
}

The variable ratio is dervied as follows:

ratio = sumSign / numSign

The variable ctxInc is derived as follows:

- If ratio is less than 0.05:

ctxInc = 1

- If ratio is greater than 0.95:

ctxInc = 2

- Otherwise, the following applies:

ctxInc = 0

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).