| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0090 |
| --- | --- |

| Title: | CE5-4: Alternative luma filter sets and alternative chroma filters for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Taquet Patrice Onno Christophe Gisquet Guillaume Laroche | Tel: Email: | +33 299876800 jonathan.taquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution gives the results of the 3 tests (CE5-4.1, CE5-4.2, CE5-4.3) related to proposal for enabling alternative Luma filter sets and alternative Chroma filters signaling for ALF which was presented in JVET-N0243. In JVET-N0243, alternative luma filter sets and alternative chroma filters have been signaled at tile group level, so that more than one luma filter set and more than one chroma filter can be signaled at once per tile group, (i.e. they could be put in a same APS). In JVET-N0243, the encoder selection of luma filters and of one chroma filter has been signaled at CTU level.

It is reported that for CE5-4.1, enabling only alternative luma filter sets, the average Y/Cb/Cr BDR gains and enc./dec. runtimes are respectively 0.01%/0.00%/-0.01% - 101%/98% for the AI configuration, -0.08%/0.09%/0.06% - 101%/101% for the RA configuration, -0.15%/0.28%/0.04% - 104%/98% for the Low Delay B configuration, and -0.19%/0.26%/0.04% - 105%/100% for the Low Delay P configuration.

It is reported that for CE5-4.2, enabling only alternative chroma filters, the average Y/Cb/Cr BDR gains and enc./dec. runtimes are respectively 0.08%/-0.88%/-1.23% - 100%/99% for the AI configuration, 0.09%/-1.96%/-2.36% - 100%/100% for the RA configuration, 0.08%/-2.43%/-2.78% - 101%/100% for the Low Delay B configuration, and 0.04%/-2.33%/-2.59% - 100%/101% for the Low Delay P configuration.

It is reported that for CE5-4.3, enabling both alternative luma filter sets and alternative chroma filters, the average Y/Cb/Cr BDR gains and enc./dec. runtimes are respectively 0.08%/-0.88%/-1.23% - 100%/100% for the AI configuration, 0.01%/-1.91%/-2.28% - 101%/100% for the RA configuration, -0.12%/-2.45%/-2.64% - 104%/100% for the Low Delay B configuration, and -0.17%/-2.51%/-2.62% - 104%/99% for the Low Delay P configuration.

# Introduction

In VTM5.0, with adoption of JVET-N0415 proposal [3], ALF process can now either use filter sets of previously received APS (temporal filter sets) or use predefined luma filter sets. However it is not possible to signal more than one new luma filter set in a given APS. For ALF parameters signaling, the filter coefficients of one new luma filter set can be predicted by using the coefficients of one of the predefined luma filter set. For chroma filter, it is possible to choose one temporal filter or signaling one new filter, but it is not possible to switch between chroma filters at CTU level.

In JVET-N0243 [2], alternative luma filter sets are signaled at tile group level, so that more than one luma filter set can be signaled at once per tile group (i.e. they could be put in a same APS). JVET-N0243 also contains some syntax reduction mechanism that enables to further merge filter indexes between successive filter sets. Filters from different filter sets defined at tile group level can be mixed in a CTU specific filter set, through an additional signaling in the CTU data that enables to indicate an alternative luma filter set index for each luma filter index (i.e. it enable mixing filters of various luma filter sets).

In JVET-N0243, alternative chroma filters can be also signaled at tile group level, so that more than one chroma filter can be signaled at once per tile group (i.e. they could be put in a same APS). One alternative chroma filter can be thus selected per chroma component and per CTU through an additional signaling in the CTU data that enables to indicate one alternative chroma filter index per chroma component processed by ALF.

In JVET-N0243, there are no pre-trained fixed filter sets, nor temporal filter sets, which are introduced in VTM5.0 with adoption of JVET-N0415. The following tests would allow clarifying gains of each sub-tool of JVET-N0243 now that JVET-N0415 has been adopted.

# Proposed changes

In this contribution, it is proposed to

- enable signaling and using alternative Luma filter sets, to provide more flexible Luma filters selection for NL-ALF processing of the Luma component,

- enable signaling and using alternative Chroma filters, to provide more than one Chroma filter for the NL-ALF processing of each Chroma component,

The Luma filters selection and mixing and the Chroma filter selection are performed and signaled on a per CTB basis.

## Alternative Luma filter sets

The number of alternative filter sets is indicated in the APS, as well as the filter coefficients (and the associated clipping values) of each filter for each alternative Luma filter set. For this contribution, the maximum number of alternative Luma filter sets is fixed to MAX_NUM_ALF_ALTERNATIVES_LUMA = 2 per APS, as good compromise for APS memory increase, encoder complexity increase and coding gains.

## Alternative Chroma filters

The number of alternative Chroma filters is indicated in the APS, as well as the filter coefficients (and the associated clipping values) of each alternative Chroma filter. For this contribution, the maximum number of alternative Chroma filters is fixed to MAX_NUM_ALF_ALTERNATIVES_CHROMA = 8 per tile group, as good compromise for APS memory increase, encoder complexity increase and coding gains.

# CE5-4 tests description

Three tests have been performed.

- CE5-4.1 provides results when enabling the signaling of alternative luma filter sets in APS, and mixing their filters at CTU level in VTM5.0.

- CE5-4.2 provides results when enabling alternative chroma filters in APS, and signaling the filter selection at CTB level in VTM5.0.

- CE5-4.3 provides results of the combination of tests CE5-4.1, and CE5-4.2.

# Results

The proposed tests are implemented on top of VTM-5.0 [1] and the simulation are performed under common test conditions as described in JVET-N1010 [4].

Results also includes a YUV column which is computed as (14*Y+U+V)/16, to provide an overall coding gains approximation.

## Test CE5-4.1

## Test CE5-4.2

## Test CE5-4.3

# Conclusion

These three CE5-4 tests shows individual gains of using alternative luma filter sets, or alternative chroma filters or using both of them as proposed in JVET-N0243.

It can be seen from CE5-4.3 that coding gains of CE5-4.1 and CE5-4.2 are additive for AI and RA configuration, while the coding gains of the combination is even better than additive of low delay configurations LB and LP.

Considering the coding gains illustrated by these CE tests, we recommend to adopt CE5-4.3 in the next version of the VVC draft.

# References

- Versatile Video Coding (Draft 5), Document JVET-N1001_v7, B. Bross, J. Chen, S. Liu

- Non-CE5: Complementary results of tests CE5-3 on Non-Linear ALF, JVET-N0243, J. Taquet, P. Onno, C. Gisquet, G. Laroche

- CE5: Coding tree block based adaptive loop filter (CE5-4), JVET-N0415, N. Hu, V. Seregin, H. E. Egilmez, M. Karczewicz

- “JVET common test conditions and software reference configurations”, JVET-N1010

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).