| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0091 |
| --- | --- |

| Title: | CE5-2.3 and CE5-2.4 : filter length decision which support parallel processing and aligned filtering block edge |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Jaehyun Lim Seunghwan Kim | Tel: Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In VTM-5.0, de-block filtering is applied on an 8x8 grid for both luma sample and chroma sample which means filtering boundary between luma and chroma is not aligned. Moreover, filter length decision is made depending on various factors.

In these subtests, a simplified filter length decision, which only depends on the distance between neighboring edges, is adaptively applied to reduce the worst case complexity of the filtering process and also to support parallel processing for various chroma formats. Specifically, two sub-tests are performed as follows:

- CE5.2.3: 4x4 and 8x8 sample grids are applied for luma and chroma, respectively

- CE5.2.4: 4x4 and 2x2 sample grids are applied for luma and chroma, respectively

Since filtering operation for the above two sub-tests is skipped on egdes which has 4 luma sample distance for neighboring edges, the worst case filtering complexity is the same as VTM5.0.

Proposed method show BD-rate change as below.

Table 1. CE5-2.3 with CTC configuration

|  | Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| AI | -0.06% | -0.14% | -0.13% | 102% | 97% |
| RA | -0.01% | -0.33% | -0.41% | 100% | 101% |
| LDB | -0.01% | 0.17% | -0.16% | 102% | 99% |
| LDP | -0.04% | 0.10% | -0.14% | 101% | 100% |

Table 2. CE5-2.4 with CTC configuration

|  | Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| AI | -0.06% | 0.43% | 0.37% | 101% | 95% |
| RA | -0.01% | 0.57% | 0.44% | 100% | 101% |
| LDB | 0.00% | 0.19% | -0.28% | 102% | 100% |
| LDP | -0.04% | 0.37% | -0.25% | 101% | 99% |

Proposed method

In VTM-5.0, filter length is determined based on transform edge condition and modified for prediction edge. During this process, filter length should check neighboring edges condition as figure 1.

Figure 1. filter length decision method of VTM-5.0

Chroma edge filtering condition in VTM-5.0 is depicted in Figure 2.

Figure 2. chroma edge filtering condition of VTM-5.0

As shown in figure 2, Chroma deblocking filter is performed at edges which satisfy following 2 conditions and strong filter is also applied at edges when the filter length is 3.

- boundary strength is equal to 2

- boundary strength is equal to 1 and filter length is 3

In this CE, a simplified filter length decision method is suggested. The filter length is determined by only distance between neighboring edges.

Filter length decision process for luma is as follows:

- If the distance of Q side is smaller than or equal to 4, filter length is set equal to 0.

- Otherwise, if the distance of the each side is smaller than or equal to 8, filter length is set equal to 3.

- Otherwise if the distance of the each side is smaller than or equal to 16, filter length is set equal to 5.

- Otherwise, filter length is set equal to 7.

Filter length decision for chroma is described as follows:

- If the distance of Q side is smaller than or equal to 2, filter length is set equal to 0

- Otherwise, if the distance of the each side is smaller than or equal to 4, filter length is set equal to 1.

- Otherwise, filter length is set equal to 3.

Figure 3. proposed filter length decision

In order to support various color formats, distance condition for filter length decision is adaptively applied. For example, chroma filter length decision for 4:4:4 chroma format is adaptive applied as bellow.

- If the distance of Q side is smaller than or equal to 4, filter length is set equal to 0

- Otherwise, if the distance of the each side is smaller than or equal to 8, filter length is set equal to 1.

- Otherwise, filter length is set equal to 3.

In this proposed method, chroma edge filtering is allowed only when boundary strength is not equal to 0 as shown in figure 4.

Figure 4. proposed chroma filtering condition.

From the suggested method, the proposed de-block filtering process can support following features:

- Aligned filtering boundary between luma and chroma component

- Uniform filter length

- Parallel processing

Aligned filtering boundary between Luma and Chroma component

In VTM-5.0 de-blocking filter process, filtering process is performed on 8x8 sample grid both for luma and chroma at block boundaries with consideration of the following conditions.

- The transform unit edges and prediction unit edges.

- The prediction unit edge except sub-partitioned chroma prediction unit edge.

As a result, filtered edges between luma and chroma component are not aligned as shown in the below figure.

Figure 5. Example of the VTM-5.0 which is not aligned edge filtering (QTBTTT partitioning)

Figure 6. Example of VTM-5.0 which is not aligned edge filtering (32x32 SbTMVP coded block)

In CE5-2.4, the proposed method suggests filtering with dense grid keeping edges aligned between luma and chroma component.

Figure 7. Example of the aligned edge filtering by the proposed filter length decision

Figure 8. Example of the aligned edge filtering by the proposed filter length decision

Uniform filter length

In VTM-5.0, filter length is decided by neighbor TU edge distance,neighbor PU edge position and distance as shown in Figure 1. Therefore, in some cases, different filter length can be derived even though each edge has same property as shown in figure 9 example.

Figure 9. Example non-uniform filter length for VTM-5.0.

However, filter length of the proposed method is decided by only distance between neighboring edges then filter length can be always uniform for the same example.

Figure 10. Example uniform filter length by the proposed method.

Parallel processing

To support parallel processing property, the filtering operation for block edges on narrow distance between neighboring edges is skipped. For example, filtering operation for narrow edges are skipped as shown in Figure 11 and Figure 12. Filtering is performed when Q side is larger than 4.

Figure 11. Quad sbt coded block

Figure 12. quad sbt and sub-block predicted block

Experimental result

Objective test result.

Table3 ~ 6 shows CE5-2.3 test result. In spite of luma filter grid have been dense, decoding time is reduced because of filter operation is skipped on narrow distance edge is skipped.

Table 3.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.13% | -0.11% | 100% | 99% |
| Class A2 | -0.04% | -0.12% | -0.15% | 103% | 99% |
| Class B | -0.08% | -0.14% | -0.15% | 102% | 99% |
| Class C | -0.08% | -0.19% | -0.17% | 101% | 93% |
| Class E | -0.07% | -0.13% | -0.05% | 102% | 96% |
| Overall | -0.06% | -0.14% | -0.13% | 102% | 97% |
| Class D | -0.07% | -0.20% | -0.17% | 102% | 96% |
| Class F | -0.04% | -0.15% | -0.17% | 102% | 96% |

Table 4

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.28% | -0.10% | 99% | 101% |
| Class A2 | 0.00% | -0.09% | -0.15% | 99% | 100% |
| Class B | -0.03% | -0.35% | -0.58% | 100% | 101% |
| Class C | -0.02% | -0.52% | -0.60% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.33% | -0.41% | 100% | 101% |
| Class D | 0.02% | -0.62% | -0.48% | 100% | 101% |
| Class F | -0.01% | -0.38% | -0.51% | 99% | 101% |

Table 5

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.45% | 0.09% | 102% | 98% |
| Class C | 0.04% | 0.03% | -0.17% | 102% | 97% |
| Class E | -0.05% | -0.09% | -0.55% | 101% | 101% |
| Overall | -0.01% | 0.17% | -0.16% | 102% | 99% |
| Class D | -0.01% | -0.28% | 0.41% | 101% | 99% |
| Class F | 0.07% | 0.40% | -0.09% | 102% | 99% |

Table 6

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.17% | -0.02% | 102% | 100% |
| Class C | 0.01% | 0.00% | 0.05% | 101% | 100% |
| Class E | -0.15% | 0.13% | -0.58% | 97% | 99% |
| Overall | -0.04% | 0.10% | -0.14% | 101% | 100% |
| Class D | 0.02% | -0.87% | -0.39% | 101% | 100% |
| Class F | 0.03% | 0.47% | 0.17% | 96% | 99% |

Table7 ~ 10 shows CE5-2.4 test result. Even though luma filter grid is 4x4 and chroma filter grid is 2x2 grid for CTC condition, decoding time is reduced compared to VTM-5.0.

Table 7

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.43% | 0.19% | 100% | 99% |
| Class A2 | -0.05% | 0.44% | 0.58% | 101% | 96% |
| Class B | -0.08% | 0.59% | 0.53% | 100% | 96% |
| Class C | -0.08% | 0.49% | 0.37% | 101% | 92% |
| Class E | -0.07% | 0.08% | 0.08% | 101% | 94% |
| Overall | -0.06% | 0.43% | 0.37% | 101% | 95% |
| Class D | -0.08% | 0.46% | 0.44% | 101% | 94% |
| Class F | -0.04% | 0.46% | 0.54% | 101% | 96% |

Table 8

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.42% | 0.23% | 99% | 100% |
| Class A2 | 0.02% | 0.45% | 0.88% | 98% | 99% |
| Class B | -0.01% | 0.86% | 0.54% | 100% | 101% |
| Class C | -0.03% | 0.39% | 0.15% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.57% | 0.44% | 100% | 101% |
| Class D | 0.01% | 0.58% | 0.50% | 101% | 102% |
| Class F | 0.00% | 0.90% | 1.00% | 100% | 103% |

Table 9

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.38% | 0.15% | 103% | 99% |
| Class C | 0.05% | 0.15% | -0.39% | 103% | 100% |
| Class E | -0.06% | -0.08% | -0.83% | 102% | 102% |
| Overall | 0.00% | 0.19% | -0.28% | 102% | 100% |
| Class D | 0.01% | 0.33% | 0.37% | 103% | 102% |
| Class F | 0.12% | 0.98% | 0.80% | 102% | 99% |

Table 10

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.45% | 0.18% | 102% | 98% |
| Class C | 0.02% | -0.07% | -0.31% | 101% | 99% |
| Class E | -0.09% | 0.80% | -0.89% | 99% | 101% |
| Overall | -0.04% | 0.37% | -0.25% | 101% | 99% |
| Class D | 0.02% | 0.18% | 0.23% | 101% | 100% |
| Class F | 0.08% | 1.14% | 2.10% | 99% | 102% |

Worst case complexity analysis

In spite of filter grid of the proposed method is 4x4, worst case filtering complexity is the same as an 8x8 grid filter processing because the proposed method skips filter operation on edge which has 4 luma sample distance between neighboring edges.

Even though chroma filtering is applied 2x2 grid in CE5-2.4, filtering is also skipped on edges which has 2 chroma sample distance for 4:2:0. Therefore worst case filtering complexity is the same as 4x4 grid chroma filtering.

For 4:4:4 color format, 4 chroma sample distance edge is skipped for filtering. From this reason chroma complexity is same with 8x8 chroma sample grid.

|  | Grid for luma | Grid for chroma(4:2:0) | Grid for chroma(4:4:4) |
| --- | --- | --- | --- |
| VTM-5.0 | 8x8 | 8x8 | 8x8 |
| CE5-2.3 | 4x4 | 8x8 | 8x8 |
| CE5-2.4 | 4x4 | 2x2 | 4x4 |

| Worst case complexity | luma | for chroma(4:2:0) | Grid for chroma(4:4:4) |
| --- | --- | --- | --- |
| VTM-5.0 | 8x8 | 8x8 | 8x8 |
| CE5-2.3 | Same with 8x8 | Same with 8x8 | Same with 8x8 |
| CE5-2.4 | Same with 8x8 | Same with 4x4 | Same with 8x8 |

In this subtest, encoding/decoding time for 4:4:4 color format sequences is also reported. Results are achieved by viewing test sequences with QP 32, 37 of RA, LDB and LDP respectively.

Table 13 ~ Table 15 show CE5-2.3 test result.

Table 13

| Random access condition | Encoding time | Decoding time |
| --- | --- | --- |
| Kimono 444 10bit | 98% | 99% |
| EBULupoandCandlelight 444 10bit | 101% | 102% |

Table 14

| Low delay B condition | Encoding time | Decoding time |
| --- | --- | --- |
| Kimono 444 10bit | 101% | 101% |
| EBULupoandCandlelight 444 10bit | 102% | 104% |

Table 15

| Low delay P condition | Encoding time | Decoding time |
| --- | --- | --- |
| Kimono 444 10bit | 101% | 101% |
| EBULupoandCandlelight 444 10bit | 99% | 101% |

Test results of CE5-2.4 are depicted in Table 16 ~ Table 18.

Table 16

| Random access condition | Encoding time | Decoding time |
| --- | --- | --- |
| Kimono 444 10bit | 99% | 98% |
| EBULupoandCandlelight 444 10bit | 100% | 101% |

Table 17

| Low delay B condition | Encoding time | Decoding time |
| --- | --- | --- |
| Kimono 444 10bit | 100% | 99% |
| EBULupoandCandlelight 444 10bit | 100% | 103% |

Table 18

| Low delay P condition | Encoding time | Decoding time |
| --- | --- | --- |
| Kimono 444 10bit | 100% | 98% |
| EBULupoandCandlelight 444 10bit | 98% | 97% |

Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).