| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0092 |
| --- | --- |

| Title: | CE5-2.5 : De-block filtering with 4x4 chroma sample grid |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Jaehyun Lim Seunghwan Kim | Tel: Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In VTM-5.0 de-block filtering for chroma component is performed at edges on 8x8 chroma sample grid except edges where chroma block is splitted by sub-block prediction mode. In this proposal, de-block filtering with 4x4 chroma sample grid is investigated for chroma component and chroma filtering is allowed for more edges which come from subblock predicted mode and also edges whose boundary strength are not equal to 0. The proposed method shows negligible bd-rate change and decoding time as shown in Table 1.

|  | Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| AI | 0.00% | 0.52% | 0.46% | 103% | 99% |
| RA | 0.01% | 1.02% | 0.91% | 100% | 100% |
| LDB | 0.02% | 0.67% | 0.02% | 102% | 97% |
| LDB | -0.01% | 0.66% | 0.19% | 101% | 97% |

Introduction

In VTM-5.0, de-block filtering is performed at edges which is aligned with 8x8 chroma sample grid.

However, chroma filtering is disallowed at chroma edges of which boundary is generated by subblock prediction mode while luma edges which generated by sub-block prediction is filtered. Figure 1 depicts filter length decision process of VTM-5.0. As shown in Figure 1, chroma edge of subblock prediction boundary is skipped.

Figure 1. Filter length decision of VTM-5.0

Figure 2 depicts chroma edge filtering process of VTM-5.0. As shown in Figure 2, chroma filter is applied when following conditions are satisfied.

- Edge is aligned with 8x8 chroma sample grid

- Boundary strength is equal to 2 or boundary strength is equal to 1 with filter length 3

Figure 2. Chroma edge filtering of VTM-5.0

Proposed method

In this proposal, three parts are modified compared with deblock filtering of VTM-5.0.

- Enable filtering for chroma sub-pu edge

- Enable filtering for chroma edge whose boundary strength is not 0

- Perform filtering on 4x4 chroma sample grid.

Figure 3 shows the first part of the proposed chroma deblocking filter.

Figure 3. Proposed chroma filter length decision for subblock predicted chroma block.

Figure 4 shows the second part of the proposed chroma deblocking filter.

Figure 4. Proposed chroma filtering enable condition

Aligned filtered edge for de-block processing for 4:2:0

From the proposed method, filtering process is applied at edges which are aligned with 8x8 grid for luma and edges which are aligned with 4x4 grid for chroma component. Consequently, filtering process is aligned between luma and chroma component for 4:2:0 format.

Experimental result

Objective test result

The proposed method shows negligible BD-rate change and also decoding time change in spite of chroma filter grid is denser compared to VTM-5.0.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.52% | 0.31% | 102% | 100% |
| Class A2 | 0.00% | 0.51% | 0.66% | 103% | 100% |
| Class B | 0.00% | 0.68% | 0.63% | 102% | 99% |
| Class C | 0.00% | 0.59% | 0.46% | 102% | 96% |
| Class E | 0.00% | 0.18% | 0.12% | 104% | 100% |
| Overall | 0.00% | 0.52% | 0.46% | 103% | 99% |
| Class D | 0.00% | 0.56% | 0.54% | 103% | 100% |
| Class F | 0.00% | 0.53% | 0.61% | 102% | 98% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.66% | 0.40% | 99% | 99% |
| Class A2 | 0.02% | 0.68% | 1.17% | 99% | 98% |
| Class B | 0.01% | 1.41% | 1.19% | 100% | 101% |
| Class C | -0.02% | 1.07% | 0.74% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 1.02% | 0.91% | 100% | 100% |
| Class D | 0.02% | 1.26% | 1.31% | 101% | 102% |
| Class F | 0.02% | 1.41% | 1.72% | 101% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 1.03% | 0.64% | 102% | 96% |
| Class C | 0.03% | 0.84% | 0.26% | 103% | 97% |
| Class E | 0.01% | -0.17% | -1.33% | 102% | 97% |
| Overall | 0.02% | 0.67% | 0.02% | 102% | 97% |
| Class D | -0.03% | 0.93% | 0.87% | 102% | 100% |
| Class F | 0.19% | 1.33% | 1.80% | 102% | 96% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 1.06% | 1.18% | 103% | 97% |
| Class C | 0.00% | 0.48% | 0.09% | 102% | 97% |
| Class E | -0.03% | 0.25% | -1.33% | 98% | 97% |
| Overall | -0.01% | 0.66% | 0.19% | 101% | 97% |
| Class D | -0.03% | 0.55% | 0.05% | 102% | 101% |
| Class F | 0.11% | 1.73% | 1.77% | 98% | 98% |

Worst case complexity analysis

The proposed method allows more filtering operation for chroma component compared with VTM-5.0.

|  | Grid for luma | Grid for chroma(4:2:0) | Grid for chroma(4:4:4) |
| --- | --- | --- | --- |
| VTM-5.0 | 8x8 | 8x8 | 8x8 |
| CE5-2.5 | 8x8 | 4x4 (4 times) | 4x4 (4 times) |

Decoding time changes are negligible for the 4:4:4 sequences.

| Random access condition | Encoding time | Decoding time |
| --- | --- | --- |
| Kimono 444 10bit | 98% | 96% |
| EBULupoandCandlelight 444 10bit | 102% | 104% |

| Low delay B condition | Encoding time | Decoding time |
| --- | --- | --- |
| Kimono 444 10bit | 102% | 101% |
| EBULupoandCandlelight 444 10bit | 102% | 104% |

|  | Encoding time | Decoding time |
| --- | --- | --- |
| Kimono 444 10bit | 101% | 101% |
| EBULupoandCandlelight 444 10bit | 100% | 100% |

Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).