| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0093-v2 |
| --- | --- |

| Title: | CE8-1.3: BVD coding method |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Hyeongmun Jang Jungah Choi Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {junghak.nam, hm.jang, jungah.choi, jin78.heo, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

- Abstract

This contribution reports the CE test results for CE8-1.3a and CE8-1.3b. In CE8-1.3a, an adaptive binarization process depending on absolute level and a context modeling method are applied for block vector coding. In CE8-1.3b, binarization and context modeling of CE8-1.3a is applied to MVD coding as well as BVD coding. From experimental results, following luma BD rate changes are observed over VTM-5.0 with IBC:

CE8-1.3a

- AI: -0.64% (CTC), -0.34% (Class F), -0.83% (TGM)

- RA: 0.01% (CTC), -0.30% (Class F), -0.46% (TGM)

CE8-1.3b

- AI: -0.64% (CTC), -0.34% (Class F), -0.83% (TGM)

- RA: 0.17% (CTC), -0.16% (Class F), -0.38% (TGM)

Introduction

In VVC-5.0, block vector difference (BVD) is signaled by the same coding method for motion vector difference (MVD). However, block vector has different statistical distribution compared with motion vector because the search range of block vector is bounded by certain region. In JVET-N0459 [1], modification of binarization process and additional contexts for BVD coding were proposed. In this CE, modification of binarization process and additional contexts are tested.

Proposed method

This contribution proposes a new BVD coding method of BVD based on adaptive binarization and context modeling. And the proposed BVD coding method is also tested as a unified vector coding where both BVD and MVD are coded with the proposed BVD coding.

CE8-1.3a

In this test, modification of binarization for the BVD coding is proposed. Depending on absolute level of BVD, binarization method is determined. Following syntaxes are introduced, i.e. abs_bvd_greater0_flag, parity_flag, abs_bvd_greater10_flag, abs_bvd_unary0, abs_bvd_unary1, abs_bvd_flc, abs_bvd_exceed, bvd_sign_flag.

Figure 1: Encoder flowchart of binarization process for syntax elements of BVD coding

Figure 1 shows encoder flowchart of the proposed binarization process. Firstly, abs_bvd_greater0_flag is signaled to specify whether the absolute value of BVD is greater than 0. If the level is greater than 0, parity_flag and abs_bvd_greater10_flag are signaled sequentially. Here, the parity_flag indicates whether the value of BVD minus 1 is even or odd. The abs_bvd_greater10_flag specifies whether the absolute value of BVD is greater than 10. If the absolute value of BVD is equal to or less than 10, abs_bvd_unary0 syntax is signaled. Otherwise, abs_bvd_unary1 syntax is signaled. If abs_bvd_unary1 syntax is less than 32, abs_bvd_flc syntax is coded. Otherwise (abs_bvd_unary1 syntax is equal to 32), abs_bvd_exceed syntax is signaled. Table 1 shows binarization process of those syntaxes.

Table 1: Binarization process for syntax elements of BVD coding

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| bvd_coding( ) | abs_bvd_greater0_flag | FL | cMax = 1 |
|  | parity_flag | FL | cMax = 1 |
|  | abs_bvd_greater10_flag | FL | cMax = 1 |
|  | abs_bvd_unary0 | TR | cMax = 4, cRiceParam = 0 |
|  | abs_bvd_unary1 | TR | cMax = 32, cRiceParam = 0 |
|  | abs_bvd_flc | FL | cMax = 7 |
|  | abs_bvd_exceed | EG1 | - |
|  | bvd_sign_flag | FL | cMax = 1 |

Assignment of ctxInc to syntax elements of proposed BVD coding is shown in Table 2. Each component of abs_bvd_greater0_flag and abs_bvd_greater10_flag have different context models. In addition, parity_flag is coded by a context model.

Table 2: Assignment of ctxInc to syntax elements for bvd coding

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| abs_bvd_greater0_flag[0] | 0 | na | na | na | na | na |
| abs_bvd_greater0_flag[1] | 0 | na | na | na | na | na |
| parity_flag | 0 | na | na | na | na | na |
| abs_bvd_greater10_flag[0] | 0 | na | na | na | na | na |
| abs_bvd_greater10_flag[1] | 0 | na | na | na | na | na |
| abs_bvd_unary0 | bypass | bypass | bypass | bypass | na | na |
| abs_bvd_unary1 | bypass | bypass | bypass | bypass | bypass | bypass |
| abs_bvd_flc | bypass | bypass | bypass | na | na | na |
| abs_bvd_exceed | bypass | bypass | bypass | bypass | bypass | bypass |
| bvd_sign_flag | bypass | na | na | na | na | na |

CE8-1.3b

CE8-1.3b tests proposed binarization and context modeling for MVD coding as well as BVD coding.

Experimental results

The proposed methods have been implemented on top of VTM-5.0 and tested under CE8 test conditions [2].

Table 1. Performance of test 8-1.3a with IBC in AI

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.04% | 0.02% | -0.03% | 136% | 101% |
| Class A2 | -0.79% | -1.04% | -1.00% | 142% | 97% |
| Class B | -0.79% | -0.72% | -0.85% | 149% | 99% |
| Class C | -0.30% | -0.30% | -0.22% | 150% | 95% |
| Class E | -1.29% | -1.17% | -1.31% | 154% | 97% |
| Overall | -0.64% | -0.63% | -0.67% | 147% | 98% |
| Class D | -0.56% | -0.45% | -0.66% | 145% | 100% |
| Class F | -0.34% | -0.24% | -0.28% | 101% | 97% |
| TGM | -0.83% | -0.82% | -0.83% | 99% | 99% |

Table 2. Performance of test 8-1.3a with IBC in RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.10% | -0.20% | 0.20% | 102% | 98% |
| Class A2 | -0.06% | -0.21% | -0.11% | 102% | 98% |
| Class B | -0.12% | -0.37% | -0.61% | 105% | 101% |
| Class C | 0.17% | -0.12% | -0.12% | 111% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.24% | -0.22% | 106% | 100% |
| Class D | 0.11% | -0.11% | 0.02% | 110% | 101% |
| Class F | -0.30% | -0.17% | -0.13% | 101% | 102% |
| TGM | -0.46% | -0.38% | -0.40% | 100% | 100% |

Table 3. Performance of test 8-1.3a with IBC in LD

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.24% | -0.22% | 119% | 98% |
| Class C | 0.10% | 0.06% | -0.15% | 124% | 98% |
| Class E | 0.05% | 0.48% | -0.15% | 115% | 99% |
| Overall | 0.06% | 0.24% | -0.18% | 120% | 98% |
| Class D | 0.06% | -0.10% | 1.12% | 126% | 101% |
| Class F | -0.20% | -0.15% | -0.05% | 103% | 96% |
| Class SCC | -0.23% | -0.14% | -0.16% | 100% | 102% |

Table 4. Performance of test 8-1.3b with IBC in AI

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.04% | 0.02% | -0.03% | 135% | 100% |
| Class A2 | -0.79% | -1.04% | -1.00% | 142% | 99% |
| Class B | -0.79% | -0.72% | -0.85% | 150% | 99% |
| Class C | -0.30% | -0.30% | -0.22% | 151% | 95% |
| Class E | -1.29% | -1.17% | -1.31% | 154% | 99% |
| Overall | -0.64% | -0.63% | -0.67% | 147% | 98% |
| Class D | -0.56% | -0.45% | -0.66% | 144% | 99% |
| Class F | -0.34% | -0.24% | -0.28% | 101% | 97% |
| Class SCC | -0.83% | -0.82% | -0.83% | 99% | 99% |

Table 5. Performance of test 8-1.3b with IBC in RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.21% | -0.17% | 0.25% | 102% | 99% |
| Class A2 | 0.08% | -0.22% | 0.05% | 102% | 99% |
| Class B | 0.04% | -0.28% | -0.41% | 105% | 100% |
| Class C | 0.38% | -0.03% | 0.04% | 110% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.17% | -0.18% | -0.07% | 105% | 100% |
| Class D | 0.41% | 0.09% | 0.07% | 110% | 101% |
| Class F | -0.16% | -0.15% | -0.04% | 100% | 101% |
| Class SCC | -0.38% | -0.38% | -0.41% | 100% | 101% |

Table 6. Performance of test 8-1.3b with IBC in LD

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.17% | 0.40% | -0.15% | 118% | 97% |
| Class C | 0.32% | 0.36% | -0.13% | 123% | 97% |
| Class E | 0.34% | 0.84% | 0.48% | 113% | 99% |
| Overall | 0.26% | 0.50% | 0.01% | 118% | 97% |
| Class D | 0.53% | 0.32% | 0.79% | 124% | 99% |
| Class F | 0.06% | -0.31% | -0.30% | 102% | 96% |
| Class SCC | -0.18% | -0.17% | -0.19% | 100% | 100% |

Conclusion

In this contribution, modification of binarization and using additional context models for BVD coding on IBC mode are tested in CE8-1.3. It is recommended to adopt the proposed BVD coding method.

Reference

- J. Nam, et al., “Non-CE8: Coding on block vector difference,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N0459, 14th Meeting: Geneva, CH, 19–27 March 2019.

- X. Xu, et al., “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1028, 14th Meeting: Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).