| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0094 |
| --- | --- |

| Title: | CE6-2.1: Simplification of Low Frequency Non-Separable Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mischa Siekmann Heiko Schwarz Detlev Marpe | Tel: Email: | +49 30 31002-440 mischa.siekmann@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

By restricting the LFNST to be applicable only if all coefficients outside the first 4x4 sub-group are non-significant, a simplified and cleaner design can be achieved which significantly decreases the encoder-runtime. When applied to the LFNST design of VTM-5.0 (CE6-2.1a) an encoder runtime of 90% (AI) and 97% (RA) can be achieved with luma BD-rates of 0.06% (AI) and 0.02% (RA), respectively. If further the number of LFNST candidates is reduced from two to one and the 16x48 transform matrices are replaced by 16x64 transforms (CE6-2.1b), the encoder runtime can be reduced to 76% (AI) and 93% (RA) relative to VTM-5.0, with luma BD-rates of 0.21% (AI) and 0.13% (RA), respectively.

# Introduction

From encoder perspective, LFNST is a secondary transform which is applied to the top-left sub-set of primary transform coefficients, while, depending on the residual block size, only a sub-set of the obtained secondary transform coefficients are retained, and the remaining secondary coefficients are set to be zero [1]. Only for block sizes with 4xN and Nx4 with N > 8, a full rank 16x16 LFNST is applied, and hence all obtained secondary coefficients are retained. In the current LFNST design, the 16x16 LFNST is applied to the two 4x4 coefficients with lowest frequency (Figure 1).

Figure 1. A 16x16 LFNST is applied to each of the two 4x4 coefficient blocks with lowest frequency (orange). The remaining coefficients are referred to as primary-only (green).

Figure 2. The 4x4 coefficient block with lowest frequency is transformed by a 16x48 LFNST to 48 coefficients (orange). Blue: enforced non-significant coefficients, Green: unaltered primary-only transform coefficients.

For all other block shapes some of the secondary transform coefficients are enforced to be zero, hence, the LFNST for those cases are presented by non-quadratic transform matrices.

From decoder perspective, it can be inferred that no LFNST is applied to a block if at any of the restricted positions a coefficient is significant. The LFNST index is signaled at CU level, after the transform coefficients have been coded, and hence, only if none of the TUs have significant coefficients at the restricted coefficient positions.

# Proposed Restriction

In this core experiment, the LFNST is restricted to be applicable only if all coefficients outside the first coefficient sub-group are non-significant. Hence, all primary-only transform coefficients have to be zero when LFNST is applied. This allows a conditioning of the LFNST index signalling on the last-significant position, and hence avoids the extra coefficient scanning in the current LFNST design, which is needed for checking for significant coefficients at specific positions only. The worst-case handling of LFNST (in terms of multiplications per pixel) restricts the non-separable transforms for 4x4 and 8x8 blocks to 8x16 and 8x48 transforms, respectively. In those cases, the last-significant scan position has to be less than 8 when LFNST is applied, for other sizes less than 16. For blocks with a shape of 4xN and Nx4 and N > 8, the proposed restriction implies that the LFNST is now applied only once, and that to the top-left 4x4 region only (Figure 3).

Figure 3

As all primary-only coefficients are zero when LFNST is applied, the number of operations needed for the primary transforms is reduced in such cases.

From encoder perspective, the quantization of coefficients is remarkably simplified when LFNST transforms are tested. A rate-distortion optimized quantization has to be done at maximum for the first 16 coefficients (in scan order), the remaining coefficients are enforced to be zero.

The core experiment CE6-2.1 defines two tests. The first (CE6-2.1a) applies the described restriction to the LFNST design of VTM-5.0, the second (CE6-2.1b) is based on a design with only one LFNST candidate and 16x64 non-separable transforms instead of 16x48 transforms of VTM-5.0.

Table 1

## CE6-2.1a

The described restriction on significant coefficients is applied on top of the LFNST design of VTM-5.0.

Figure 4

## CE6-2.1b

The 8 48x16 LFNS transform matrices of VTM5.0 are replaced by the 4 64x16 matrices tested in the last meeting cycle as CE6-3.1e and described in JVET-N0193 [1], which also implies that only one LFNST candidate is applicable to each TU.

Figure 5

# Experimental results

According to the common test conditions in JVET-N1010 [2], simulations were performed for all-intra (AI), random-access (RA) configuration with VTM-5.0 software. In addition to the CTC, also the configurations required by JVET-N1026 [3] are tested (i.e. low QP configuration and inter MTS enabled). The main parameters of the simulation environment are shown in Table 2.

Table 2 Simulation environment

| CPU | Turbo Boost | SIMD settings | Compiler | OS |
| --- | --- | --- | --- | --- |
| E5-2697A v4 | off | AVX2 | GCC 7.2.1 | Linux |

## CE6-2.1a

### Results for CTC

### Results with inter MTS on

### Results for low QPs (2,7,12,17)

## CE6-2.1b

### Results for CTC

### Results with inter MTS on

### Results for low QPs (2,7,12,17)

# Conclusion

With the presented restriction on significant coefficients a clean design of the low frequency non-separable transform (LFNST) is achieved. The encoder runtime could be significantly decreased. Both presented configurations show a reasonable trade off in terms of run-time decrease and loss of coding efficiency.

We propose to either adopt the described restriction on top of the LFNST design of VTM-5.0 (CE6-2.1a) or to reduce the number of LFNST candidates to one, replace the 8 48x16 LFNSTs to 4 64x16 LFNST and apply the described restriction of significant coefficients on top of it (CE6-2.1b).

# Reference

- M. Koo et al., “CE6: Reduced Secondary Transform (RST)(CE6-3.1)”, JVET-N0193, 14th meeting, Geneva, CH, 19-27 March 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET meeting, Geneva, 19–27 Mar. 2019.

- X. Zhao and H. E. Egilmez, “Description of Core Experiment 6 (CE6): Transforms and transform signalling”, JVET-N1026, 14th JVET meeting, Geneva, 19–27 Mar. 2019.

- M. Zhou, Y. Hu, “CE6-related: A study of primary transforms,” JVET-M0046, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).