| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0101-v1 |
| --- | --- |

| Title: | CE10: Dense Residual Convolutional Neural Network based In-Loop Filter (Tests 2.5 and 2.7) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yingbin Wang, Tong Ouyang, Cong Zou, Yiming Li, Zhenzhong Chen Wuhan University Liang Zhao, Shan Liu, Xiang Li Tencent | Tel: Email: | zzchen@whu.edu.cn shanl@tencent.com |
| Source: | Tencent, Wuhan University |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of Core Experiment 10 (CE10) tests 2.5 and 2.7 on neural network based filter for video coding. In test 2.5, the impact of bit-depth of model parameters is investigated. In test 2.3, the generalization capability of DRNLF when QP is different from the training QP is investigated.

Experimental results show that, -1.02%, -4.75% and -4.10% BD-rates are reported on Y, Cb, and Cr components, respectively with a Random Access configuration for test 2.5a, -0.84%, -3.88% and -2.84% for test 2.5b, -1.17%, -4.75% and -4.39% for test 2.7a, -0.63%, -3.65% and -3.23% for test 2.7b.

Introduction

Fig. 1 shows the network structure of dense residual convolutional neural network based in-loop filter (DRNLF) [1], where N and M denotes the number of dense residual unit (DRU) and convolution kernel, respectively. Specifically, N is set to 4 and M is set to 32 for the tradeoff between computational efficiency and performance. Normalized QP map is concatenated with the reconstructed frame as the input to DRN.

Figure 1. Network structure of DRNLF.

The main body of the proposed network consists of series of DRU. The structure of DRU is shown in Fig. 2. The DRU directly propagates its input to the subsequent unit through the shortcut. To further reduce the computational cost, 3x3 depth-wise separable convolutional (DSC) layer is applied in DRU.

Figure 2. Dense Residual Unit (DRU)

Finally, the output of the network has 3 channels, which corresponds to Y, Cb, Cr, respectively. The filter is applied for both intra and inter pictures. Additional flag is signaled for each CTUs to indicate the on/off of the DRNLF.

Test description

In test 2.5, the impact of bit-depth of model parameters, where 2 kinds of bit-depth are tested: (a) 32-bit(float) weights; and (b) 8-bit(int) weights.

Test 2.7 investigates the generalization capability of DRNLF when encoding QP is different from the training QP. It includes 2 kinds of QP settings: (a) CTC QP for training, CTC QP + 5 for testing; (b) CTC QP for training, CTC QP - 5 for testing.

The tests are summarized in Table 1. Note that test 2.5 uses CTC QP for training, CTC QP for testing and the bit-depth of test 2.7 is 32-bit(float).

Table 1 Tests summary

| Test # | Test Part | Description |
| --- | --- | --- |
| CE13-2.5a | bit-depth | 32-bit(float) weights |
| CE13-2.5b |  | 8-bit(int) weights |
| CE13-2.7a | generalization capability (QP) | CTC QP for training, CTC QP – 5 for testing |
| CE13-2.7b |  | CTC QP for training, CTC QP + 5 for testing |

Experimental results

The proposed method has been implemented on top of VTM 5.0 [2]. The simulation is conducted according to CE10 description [3] and the common test conditions (CTC) [4].

In the training phase, DIV2K dataset [5] is utilized to generate the training sets and validating set for the proposed network. There are 800 images in its training set and 100 images in its validation set. The images in DIV2K dataset are converted from RGB color space to YUV color space before compression. The images are compressed by VTM 4.0 with All Intra (AI) configuration using QP values in {22, 27, 32, 37}. L1 loss is used as the loss function for training. To balance the coding gain between different components, we set the weights of Y, Cb, Cr to 10, 1, 1, respectively. The loss is minimized by ADAM optimizer. 300 epochs of training are applied. For each epoch, 8 small patches with size of 64×64 are randomly extracted from the images to generate the training samples. The learning rate is initialized as 5×10-4 and is divided by half every 50 epochs.

In evaluation, VTM 5.0 serves as the reference software. All the sequences recommended in CTC are evaluated. The simulations are performed on Linux cluster without using GPUs. The experimental results of test 2.5a, 2.5b using RA configuration compared to VTM-5.0 baseline are shown in Table 2, Table 3, respectively. Table 4, Table 6 shows the result of test 2.7a, test 2.7b, respectively.

The following experiments use a different number of frames than the CE10 description specifies. The corresponding results will be updated in later version.

Test 2.5

Table 2 Results for test 2.5a with an RA configuration (Short test)

|  | Random Access Main10 Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.33% | -4.58% | -4.80% | 107% | 7607% |
| Class A2 | -1.31% | -5.96% | -2.52% | 107% | 4690% |
| Class B | -0.77% | -4.21% | -4.71% | 105% | 5350% |
| Class C | -0.90% | -4.65% | -3.99% | 103% | 5573% |
| Class E |  |  |  |  |  |
| Overall | -1.02% | -4.75% | -4.10% | 105% | 5652% |
| Class D | -0.80% | -2.12% | -2.64% | 102% | 2592% |
| Class F | -0.32% | -1.40% | -1.43% | 109% | 2113% |

Table 3 Results for test 2.5b with an RA configuration (Short test)

|  | Random Access Main10 Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.12% | -4.22% | -3.02% | 107% | 7021% |
| Class A2 | -0.95% | -4.81% | -2.10% | 106% | 4139% |
| Class B | -0.61% | -3.27% | -2.83% | 106% | 5238% |
| Class C | -0.83% | -3.70% | -3.27% | 103% | 5654% |
| Class E |  |  |  |  |  |
| Overall | -0.84% | -3.88% | -2.84% | 105% | 5408% |
| Class D | -0.69% | -1.41% | -2.19% | 103% | 2497% |
| Class F | -0.39% | -1.02% | -1.32% | 110% | 2098% |

Test 2.3

Table 4 Results for test 2.7a with a RA configuration (Short test)

|  | Random Access Main10 Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.31% | -3.63% | -4.69% | 110% | 7539% |
| Class A2 | -1.30% | -4.41% | -2.24% | 107% | 4081% |
| Class B | -0.99% | -4.50% | -4.83% | 108% | 4759% |
| Class C | -1.19% | -6.17% | -5.23% | 104% | 6461% |
| Class E |  |  |  |  |  |
| Overall | -1.17% | -4.75% | -4.39% | 107% | 5489% |
| Class D | -1.30% | -3.12% | -4.31% | 103% | 3720% |
| Class F | -0.47% | -2.43% | -1.98% | 111% | 2746% |

Table 5 Results for test 2.7b with a RA configuration (Short test)

|  | Random Access Main10 Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.91% | -4.39% | -3.81% | 103% | 7252% |
| Class A2 | -0.66% | -5.40% | -2.45% | 103% | 4514% |
| Class B | -0.44% | -2.88% | -4.17% | 102% | 4615% |
| Class C | -0.62% | -2.75% | -2.20% | 102% | 4732% |
| Class E |  |  |  |  |  |
| Overall | -0.63% | -3.65% | -3.23% | 103% | 5063% |
| Class D | -0.49% | -1.04% | -1.50% | 103% | 1934% |
| Class F | -0.16% | -0.65% | -0.83% | 107% | 1608% |

References

[1] Yingbin Wang, Zhenzhong Chen, Yiming Li, Liang Zhao, Shan Liu, Xiang Li, “CE13: Dense Residual Convolutional Neural Network based In-Loop Filter (Test 2.2 and 2.3)”, JVET-N0254, Geneva, CH, 19–27 March. 2019.

[2] VTM 5.0 software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-5.0.

[3] Yiming Li, Shan Liu, Kei Kawamura, “Description of Core Experiment 10 (CE10): Neural Network based Filter for Video Coding ”, JVET-N1030, Geneva, CH, 19–27 March. 2019.

[4] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, 19–27 March. 2019.

[5] DIV2K, https://data.vision.ee.ethz.ch/cvl/DIV2K/.

Patent rights declaration(s)

Wuhan University and Tencent America LLC do not have any current or pending patent rights relating to the technology described in this contribution.