| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0102-r1 |
| --- | --- |

| Title: | CE2-1.4: luma-chroma dependency reduction for chroma scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Ruling Liao, Yan Ye Radiance Builidng Beijing China | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

At the 13th JVET meeting, luma mapping with chroma scaling (LMCS) (previously referred to as adaptive in-loop reshaper in JVET-M0427) was adopted into the VVC draft 4. At the 14th JVET meeting, it is proposed to simplify the chroma scaling factor derivation by using top-left reference sample instead of averaged value of luma prediction block in JVET-M0477 and this method was included in CE2 to be further studied.

This contribution is the CE test report for CE2-1.4.a to CE2-1.4.e in which different numbers of luma reference samples are used to derive the scaling factor for chroma.

Test description

In VTM5, the predictors of the whole luma block are averaged and used to derive the scaling factor for chroma. In CE2-1.2, for intra coded block, luma reference samples are averaged and the average value is used to derive the scaling factor for chroma, so that the intra processing latency can be reduced. To simplify the average calculation, a subset of luma reference samples are averaged in CE2-1.4.a to CE2-1.4.d. The numbers of luma reference averaged are listed in Table 1.

Table 1 test description

| CE2-1.4.a | For intra coded blocks, derivation of the scaling factor from the top-left reference luma sample used for intra prediction |
| --- | --- |
| CE2-1.4.b | For intra coded blocks, derivation of the scaling factor from the averaged value of 2 reference luma samples |
| CE2-1.4.c | For intra coded blocks, derivation of the scaling factor from the averaged value of 4 reference luma samples |
| CE2-1.4.d | For intra coded blocks, derivation of the scaling factor from the averaged value of 8 reference luma samples |
| CE2-1.4.e | For intra coded blocks, derivation of the scaling factor from the averaged value of reference luma samples used for intra prediction |

- CE2-1.4.a

In this test, only left-top reference luma sample is used as shown in

- CE2-1.4.b

In this test, average of 2 reference luma samples is used as shown in

- CE2-1.4.c

In this test, average of 4 reference luma samples is used as shown in

- CE2-1.4.d

In this test, average of 8 reference luma samples is used as shown in

- CE2-1.4.e

In this test, average of all reference luma samples is used as shown in

Experimental Results

The tests are implemented on top of VTM5.0 and launched under common test conditions (CTC). Since chroma scaling is only enabled in single tree case, so only RA and LD results are reporte.

Simulation result for CE2-1.4.a

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.22% | 0.09% | 100% | 100% |
| Class A2 | 0.02% | 0.01% | 0.01% | 100% | 100% |
| Class B | -0.01% | -0.03% | -0.03% | 99% | 100% |
| Class C | -0.02% | -0.08% | -0.09% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.08% | -0.01% | 100% | 100% |
| Class D | 0.00% | -0.06% | 0.19% | 100% | 99% |
| Class F | -0.01% | 0.00% | -0.03% | 100% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.39% | -0.16% | 100% | 99% |
| Class C | 0.03% | 0.14% | -0.13% | 100% | 99% |
| Class E | -0.09% | 0.28% | -0.43% | 100% | 100% |
| Overall | -0.01% | 0.28% | -0.22% | 100% | 99% |
| Class D | 0.00% | 0.16% | 0.65% | 99% | 99% |
| Class F | 0.04% | 0.18% | 0.19% | 100% | 99% |

Simulation result for CE3-1.4.b

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.03% | 0.00% | 100% | 100% |
| Class A2 | -0.01% | 0.05% | 0.03% | 100% | 100% |
| Class B | 0.01% | -0.03% | -0.09% | 100% | 100% |
| Class C | -0.01% | -0.12% | -0.19% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | -0.07% | 100% | 100% |
| Class D | 0.00% | -0.03% | 0.09% | 100% | 99% |
| Class F | -0.01% | 0.03% | -0.03% | 100% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.15% | -0.05% | 100% | 99% |
| Class C | 0.00% | -0.10% | 0.07% | 100% | 99% |
| Class E | -0.03% | 0.05% | -0.26% | 100% | 100% |
| Overall | -0.01% | 0.04% | -0.06% | 100% | 99% |
| Class D | 0.00% | -0.16% | 0.81% | 100% | 99% |
| Class F | -0.04% | -0.05% | 0.18% | 100% | 99% |

Simulation result for CE3-1.4.c

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.06% | 0.06% | 100% | 100% |
| Class A2 | -0.01% | 0.02% | -0.01% | 100% | 100% |
| Class B | 0.01% | -0.05% | -0.06% | 100% | 100% |
| Class C | 0.00% | 0.03% | -0.08% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.03% | 100% | 100% |
| Class D | -0.01% | -0.05% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.01% | -0.04% | 100% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.31% | 0.19% | 100% | 99% |
| Class C | 0.02% | -0.08% | -0.05% | 100% | 99% |
| Class E | -0.07% | 0.14% | -0.13% | 100% | 100% |
| Overall | -0.01% | 0.14% | 0.03% | 100% | 99% |
| Class D | -0.07% | 0.30% | 0.57% | 100% | 99% |
| Class F | 0.01% | -0.01% | -0.12% | 99% | 99% |

Simulation result for CE3-1.4.d

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.07% | 0.04% | 100% | 100% |
| Class A2 | -0.01% | -0.04% | -0.03% | 100% | 100% |
| Class B | 0.01% | 0.01% | 0.07% | 100% | 100% |
| Class C | -0.01% | -0.03% | -0.19% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.02% | 100% | 100% |
| Class D | 0.01% | 0.12% | 0.01% | 100% | 99% |
| Class F | 0.01% | 0.10% | -0.01% | 100% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.12% | 0.08% | 100% | 98% |
| Class C | -0.02% | 0.00% | -0.04% | 100% | 99% |
| Class E | -0.03% | 0.18% | -0.49% | 100% | 100% |
| Overall | -0.01% | 0.10% | -0.10% | 100% | 99% |
| Class D | 0.04% | -0.13% | 0.53% | 99% | 99% |
| Class F | -0.02% | -0.16% | -0.04% | 100% | 99% |

Simulation result for CE3-1.4.e

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.09% | 0.08% | 100% | 100% |
| Class A2 | 0.01% | -0.16% | -0.15% | 100% | 99% |
| Class B | 0.02% | -0.11% | -0.17% | 100% | 100% |
| Class C | -0.02% | 0.02% | 0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.08% | -0.06% | 100% | 100% |
| Class D | 0.00% | -0.11% | 0.15% | 100% | 99% |
| Class F | 0.01% | 0.03% | 0.01% | 100% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.29% | -0.05% | 100% | 99% |
| Class C | 0.04% | 0.20% | 0.01% | 100% | 99% |
| Class E | 0.04% | 0.20% | 0.30% | 99% | 100% |
| Overall | 0.02% | 0.23% | 0.06% | 100% | 99% |
| Class D | -0.01% | -0.09% | 0.39% | 99% | 99% |
| Class F | -0.02% | -0.09% | 0.06% | 100% | 98% |

Conclusion

The contribution reports the results of CE2-1.4.a to CE2-1.4.e. In these tests, for intra coded block in single tree, the neighboring reference luma samples are used to derive the chroma scaling factor to reduce the latency between luma coding and chroma coding. Compared to VTM5.0, the coding performance change is negligible.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Draft Specification Changes

- 8.7.5.3 Picture reconstruction with luma dependent chroma residual scaling process for chroma samples

Inputs to this process are:

- a location ( xCurr, yCurr ) of the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable nCurrSw specifying the transform block width,

- a variable nCurrSh specifying the transform block height,

- a variable tuCbfChroma specifying the coded block flag of the current chroma transform block,

- an (nCurrSw)x(nCurrSh) array predSamples specifying the chroma prediction samples of the current block,

- an (nCurrSw)x(nCurrSh) array resSamples specifying the chroma residual samples of the current block.

- a variable refIdx specifying the intra prediction reference line index,

- the neighbouring samples p[ x ][ y ], with x = −1 − refIdx, y = −1 − refIdx..nTbH − refIdx − 1 and x = − refIdx..nTbW − 1 − refIdx, y = −1 − refIdx.

Output of this process is a reconstructed chroma picture sample array recSamples.

The reconstructed chroma picture sample recSamples is derived as follows for i = 0..nCurrSw − 1, j = 0..nCurrSh − 1:

- If one of the following conditions is true, recSamples[ xCurr + i ][ yCurr + j ] is set equal to Clip1C( predSamples[ i ][ j ] + resSamples[ i ][ j ] ):

- slice_chroma_residual_scale_flag is equal to 0

- nCurrSw * nCurrSh is less than or euqal to 4

- tu_cbf_cb [ xCurr ][ yCurr ] is equal to 0 and tu_cbf_cr [ xCurr ][ yCurr ] is equal to 0

- Otherwise, the following applies:

- For the derivation of the variable varScale the following ordered steps apply:

- The variable invAvgLuma is derived as follows:

  - If CuPredMode[ xCurr ][ yCurr ] is equal to MODE_INTRA:

invAvgLuma = p[-1-refIdx][-1-refIdx]     (for CE2-1.4.a)

invAvgLuma = (Clip1Y(p[-1-refIdx+( nCurrSw>>1)][-1-refIdx])+ Clip1Y(p[-1-refIdx][-1-refIdx+(nCurrSh>>1)])+1)>>1                 (for CE2-1.4.b)

invAvgLuma = (Clip1Y(p[-refIdx][-1-refIdx])+ Clip1Y(p[-1-refIdx+nCurrSw][-1-refIdx])+ Clip1Y(p[-1-refIdx][-refIdx])+ Clip1Y(p[-1-refIdx][-1-refIdx+nCurrSh])+2)>>2                                                      (for CE2-1.4.c )

invAvgLuma = (Clip1Y(p[-refIdx][-1-refIdx])+ Clip1Y(p[-1-refIdx+(nCurrSw>>2)][-1-refIdx])+ Clip1Y(p[-1-refIdx+(nCurrSw>>1)][-1-refIdx])+ Clip1Y(p[-1-refIdx+(3*nCurrSw>>4)][-1-refIdx])+ Clip1Y(p[-1-refIdx][-refIdx])+ Clip1Y(p[-1-refIdx][-refIdx+(nCurrSh>>2)])+ Clip1Y(p[-1-refIdx][-1-refIdx+(nCurrSh>>1)])+2)+ Clip1Y(p[-1-refIdx][-1-refIdx+(3*nCurrSh>>2)])+4)>>3          (for CE2-1.4.d)

invSumLuma=k=0nTbWClip1Y(p-1-refIdx+k-1-refIdx)+l=1nTbHClip1Y(p-1-refIdx-1-refIdx+l)

invAvgLuma=(invSumLuma*((0x4000+(nTbW+nTbH-1)/2)/ (nTbW+nTbH-1))+8192)>>14                         (for CE2-1.4.e)

  - Othersies

invAvgLuma = Clip1Y( ( k=0SubWidthC*nCurrSw-1l=0SubHeightC*nCurrSh-1PredMapSamples[ k ][ l ]
	 + nCurrSw * nCurrSh * ( ( SubWidthC * SubHeightC ) / 2 ) ) / 	(8-996)
		( nCurrSw * nCurrSh *SubWidthC * SubHeightC ) )

- The variable idxYInv is derived by invoking the identification of piece-wise function index process for a luma sample as specified in clause 8.8.2.3 with the variable lumaSample set equal to invAvgLuma as the input and idxYInv as the output.

- The variable varScale is derived as follows:

varScale = ChromaScaleCoeff[ idxYInv ]	(8-997)

- The recSamples is derived as follows:

- If tuCbfChroma is equal to 1, the following applies:

resSamples[ i ][ j ] = Clip3( −( 1 << BitDepthC ), ( 1 << BitDepthC ) − 1, resSamples[ i ][ j ] )	(8-998)

recSamples[ xCurr + i ][ yCurr + j ] = Clip1C( predSamples[ i ][ j ] + 	(8-999)
	Sign( resSamples[ i ][ j ] ) * ( ( Abs( resSamples[ i ][ j ] ) * varScale + ( 1 << 10 ) ) >> 11 ) )

- Otherwise (tu_cbf is equal to 0), the following applies:

recSamples[ xCurr + i ][ yCurr + j ] = Clip1C(predSamples[ i ][ j ] )	(8-1000)

References

- “JVET common test conditions and software reference configurations”, JVET-N1010, F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring

- “Description of Core Experiment 2 (CE2): luma-chroma dependency redcution,” JVET-N1022, G. Van der Auwera, L. Li, and A. Filippov

- “Versatile Video Coding (Draft 5)”, JVET-N1001-v7, B. Bross, J. Chen, and S. Liu