| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0103 |
| --- | --- |

| Title: | CE9-2.3: CIIP with disabled BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xiang Li, Xin Zhao, and Shan Liu | Email: | {leolzhao, xlxiangli, xinzzhao, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution presents the simulation results of CE9-2.3 test on top of VTM-5.0. In CE9-2.3, BDOF is always disabled for CIIP mode. It is reported that test CE9-2.3 obtains 0.00% and 0.00% luma BD-rate for the RA and LDB configurations respectively on top of VTM-5.0 with around 100% encoding and decoding time.

# Introduction

In VTM-5.0 [1], combined intra-inter prediction (CIIP) mode combines intra prediction and merge indexed inter prediction. For inter prediction in CIIP mode, BDOF is conditionally applied to refine the motion compensated samples. For intra prediction in CIIP mode, Planar intra prediction mode is always applied to generate the prediction samples for luma component and DM is always applied to generate the prediction samples for chroma component. The weight for intra and inter predicted samples are adaptively selected based on the number of neighbouring intra-coded blocks (Figure 1) [2]. The (wIntra, wInter) weights are adaptively set as follows. If both top and left neighbours are intra-coded, (wIntra, wInter) is set equal to (3, 1). Otherwise, if one of these blocks is intra-coded, these weights are identical, i.e., (2, 2), else the weights are set equal to (1, 3).

Figure 1 The position of the top and left neighbouring blocks that are checked to decide the weights for intra and inter predicted samples in blending.

# Proposed method in CE9-2.3

Bi-directional optical flow (BDOF) is always disabled for CIIP mode.

# Test results

The proposed method in CE9-2.3 is implemented on top of VTM5.0 software [3]. The simulations are performed on Linux cluster using RA and LDB configurations under the common test condition described in JVET-N1010 [4], which are listed below:

Table 1: Results of CE9-2.3 over VTM-5.0

|  | Random Access Main10 |  |  |  |  | Low Delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.09% | -0.04% | 101% | 99% |  |  |  |  |  |
| Class A2 | 0.00% | -0.01% | -0.03% | 101% | 100% |  |  |  |  |  |
| Class B | 0.01% | 0.04% | 0.02% | 99% | 98% | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | -0.02% | 0.00% | -0.05% | 98% | 94% | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class E |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | -0.01% | -0.02% | 99% | 98% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.02% | 0.05% | 0.04% | 99% | 94% | 0.00% | 0.00% | 0.00% | 99% | 94% |
| Class F | 0.01% | 0.00% | -0.02% | 99% | 96% | 0.00% | 0.00% | 0.00% | 100% | 99% |

# Spec text changes

The following text describes the proposed method. It is based on VVC draft5 v7 [5] section 8.5.6. Modifications related to this proposal are highlighted in yellow.

8.5.6	Decoding process for inter blocks

8.5.6.1	General

This process is invoked when decoding a coding unit coded in inter prediction mode.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- variables numSbX and numSbY specifying the number of luma coding subblocks in horizontal and vertical direction,

- the motion vectors mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the refined motion vectors refMvL0[ xSbIdx ][ ySbIdx ] and refMvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the bi-prediction weight index bcwIdx,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an array predSamples of prediction samples.

Let predSamplesL0L, predSamplesL1L and predSamplesIntraL be (cbWidth)x(cbHeight) arrays of predicted luma sample values and, predSamplesL0Cb, predSamplesL1Cb, predSamplesL0Cr and predSamplesL1Cr, predSamplesIntraCb, and predSamplesIntraCr be (cbWidth / 2)x(cbHeight / 2) arrays of predicted chroma sample values.

  - The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

    - If all of the following conditions are true, bdofFlag is set equal to TRUE.

      - sps_bdof_enabled_flag is equal to 1.

      - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

      - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

      - MotionModelIdc[ xCb ][ yCb ] is equal to 0.

      - merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

      - sym_mvd_flag[ xCb ][ yCb ] is equal to 0.

      - ciip_flag[ xCb ][ yCb ] is equal to 0

      - BcwIdx[ xCb ][ yCb ] is equal to 0.

      - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

      - cbHeight is greater than or equal to 8

      - cIdx is equal to 0.

    - Otherwise, bdofFlag is set equal to FALSE.

# Conclusion

This contribution presents the simulation results for CE9-2.3, wherein BDOF is disabled for CIIP mode to reduce the computational complexity. It is reported that the proposed method obtains 0.00% and 0.00% luma BD-rate change in RA and LDB configurations on top of VTM-5.0 respectively. It is proposed to adopt this CE test.

# References

- M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, and S.-M. Lei “CE10.1.1: Multi-hypothesis prediction for improving AMVP mode, skip or merge mode, and intra mode,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L0100, Macao, CN, 3–12 Oct. 2018.

- L. P. Van, G. V. Auwera A. K. Ramasubramonian, V. Seregin, and M. Karczewicz “CE10: CIIP with position-independent weights,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0302, Geneva, CH, 19–27 March 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 March 2019.

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 2),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1001, Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).