| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0105-v2 |
| --- | --- |

| Title: | CE7: Joint chroma residual coding with multiple modes (tests CE7-2.1, CE7-2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christian Helmrich, Heiko Schwarz, Tung Nguyen, Christian Rudat, Detlev Marpe, Thomas Wiegand, Bappaditya Ray, Geert Van der Auwera, Adarsh K. Ramasubramonian, Muhammed Coban, Marta Karczewicz | Email: | christian.helmrich@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI, Qualcomm Inc. |  |  |

_____________________________

changes in v2:

- added previously missing results for lowQP configuration, RA, class A2

- corrected calculation of HDR metrics (DE100, PSNR-L100) for HDR all intra configuration

# Abstract

This document reports the simulation results of the CE tests CE7-2.1 and CE7-2.2.  In these CE tests, extensions of the joint coding mode for chroma residuals are investigated.  In contrast to VTM-5, in which a single joint chroma coding mode is supported, the extensions investigated in CE7-2.1 and CE7-2.2 support three different joint coding modes for chroma residuals (which are indicated by the chroma cbfs).  In all joint chroma coding modes of CE7-2.1, the Cb and Cr residuals are derived from a single coded transform block.  In one of the modes supported in CE7-2.2, the Cb and Cr residuals are derived from two coded transform blocks (which represent the coefficients of a Hadamard transform).

For the SDR common test conditions, the following average BD rates (Y, Cb, Cr, in %) are reported relative to VTM-5.0:

CE7-2.1:	(AI) –0.18, –0.57,  0.07,    (RA) –0.11, –1.10, 0.18,   (LB) 0.08, –0.67, 1.07
CE7-2.2:	(AI) –0.22, –0.42,–0.06,    (RA) –0.15, –0.58, 0.55,   (LB) 0.10,   0.41, 1.60

For the HDR common test conditions, the following average BD rates (in %) are reported relative to VTM-5.0, for all intra (AI):

CE7-2.1:	(DE100) –3.05,  (L100) –0.05,  (wPSNR) –0.06, –4.26,   0.75,  (PSNR) –0.07, –3.33,   0.42
CE7-2.2:	(DE100) –2.98,  (L100) –0.06,  (wPSNR) –0.04, –3.85, –0.85,  (PSNR) –0.05, –2.81, –0.96

and, for random access (RA),

CE7-2.1:	(DE100) –2.58,  (L100) –0.01,  (wPSNR) –0.03, –3.98,   1.52,  (PSNR) –0.06, –3.06,   0.64
CE7-2.2:	(DE100) –2.84,  (L100) –0.03,  (wPSNR) –0.03, –3.85, –0.24,  (PSNR) –0.05, –2.63, –0.66

For the low-QP configuration (QP values 2, 7, 12, 17 and 100 coded frames) for SDR, the following BD rates (in %) are reported relative to VTM-5.0:

CE7-2.1:	(AI) –0.05, –0.14,   0.10,    (RA) –0.02, –0.24, –0.13,   (LB) –0.01, –0.26, –0.19
CE7-2.2:	(AI) –0.18,   0.00, –0.04,    (RA) –0.09, –0.19, –0.14,   (LB) –0.06, –0.13, –0.25

# Proposed Joint Chroma Residual Coding

The tests CE7-2.1/2 propose extensions of the joint chroma residual coding technique first described in JVET-M0305 [1] by offering a wider range of joint-coding parameters and modes. Specifically, the following is proposed:

- CE7-2.1: Extension of VTM 5.0 by allowing three instead of one joint chroma coding mode as in JVET-N0282 configuration 1, signaled using the chroma coded block flag (CBF) syntax elements. These coding modes apply simplified cross-component rotational transforms, as shown in Table 1.

- CE7-2.2: Similar to CE7-2.1, but allowing two joint chroma coded residual signals (instead of only one) to be transmitted in case of the joint chroma coding mode signaled via CBFCb = 1, CBFCr = 1. The rotational transform associated with this mode represents a Hadamard transform, see Table 2.

## Normative reconstruction of chroma residuals

Thus, in contrast to M0305, in which the addition of one single-channel joint chroma residual coding mode (given by Cr = −Cb) was suggested, in the extensions, three modes for joint chroma residual coding with different mixing factors are supported. The modes are additionally characterized by a sign (i. e., sign of the joint-decoding weights, CSign in JVET-N0282 [2]) used for deriving the second chroma residual, which is coded in the slice header. The usage (activation) of a joint chroma coding mode is indicated by a TU-level flag tu_joint_cbcr_residual_flag and the selected mode is implicitly indicated by the chroma CBFs. The flag tu_joint_cbcr_residual_flag is present if either or both chroma CBFs for a TU are equal to 1.

As in VTM 5.0, a chroma QP offset (coded at slice level) specifically for use with certain joint chroma coding modes can be signaled. When a corresponding joint chroma coding mode (modes 2 and 4 is the following description) is active in a TU, this chroma QP offset is added to the applied luma-derived chroma QP during quantization and decoding of that TU. For the other modes (modes 1 and 3 in the following description), the chroma QPs are derived in the same way as for conventional Cb or Cr blocks.

The normative reconstruction process of the chroma residuals (resCb and resCr) from the transmitted transform blocks (resJointC or, for CE7-2.2, resJointC1 and resJointC2) is summarized in the following tables.

Table 1.	Reconstruction of chroma residuals in CE7-2.1. The value CSign is a sign value (+1 or –1), which is specified in the slice header, resJointC[ ][ ] is the transmitted residual.

| tu_cbf_cb | tu_cbf_cr | reconstruction of Cb and Cr residuals | mode |
| --- | --- | --- | --- |
| 1 | 0 | resCb[ x ][ y ] = resJointC[ x ][ y ] resCr[ x ][ y ] = ( CSign * resJointC[ x ][ y ] ) >> 1 | 1 |
| 1 | 1 | resCb[ x ][ y ] = resJointC[ x ][ y ] resCr[ x ][ y ] = CSign * resJointC[ x ][ y ] | 2 |
| 0 | 1 | resCb[ x ][ y ] = ( CSign * resJointC[ x ][ y ] ) >> 1 resCr[ x ][ y ] = resJointC[ x ][ y ] | 3 |

Table 2.	Reconstruction of chroma residuals in in CE7-2.2. The value CSign is a sign value (+1 or –1) coded in the tile group header, resJointC1[ ][ ] and resJointC2[ ][ ] are the transmitted residuals. The reconstruction for the case tu_cbf_cb = 0, tu_cbf_cr = 1 depends on the value of the flag joint_cbcr_alt_mode_flag, also transmitted in the slice header.

| tu_cbf_cb | tu_cbf_cr | reconstruction of Cb and Cr residuals | mode |
| --- | --- | --- | --- |
| 1 | 0 | resCb[ x ][ y ] = resJointC1[ x ][ y ] resCr[ x ][ y ] = CSign * resJointC1[ x ][ y ] | 2 |
| 1 | 1 | resCb[ x ][ y ] = resJointC1[ x ][ y ] + resJointC2[ x ][ y ] resCr[ x ][ y ] = CSign * ( resJointC1[ x ][ y ] – resJointC2[ x ][ y ] ) | 4 |
| 0 | 1 | if ( joint_cbcr_alt_mode_flag ) { resCb[ x ][ y ] = ( CSign * resJointC2[ x ][ y ] ) >> 1 resCr[ x ][ y ] = resJointC2[ x ][ y ] } else { resCb[ x ][ y ] = resJointC2[ x ][ y ] resCr[ x ][ y ] = ( CSign * resJointC2[ x ][ y ] ) >> 1 } | 3 1 |

## Non-normative determination of joint chroma residuals in the encoder

In the investigated encoder implementation, the joint chroma components are derived as explained in the following. Depending on the mode (listed in the tables above), resJointC{1,2} are generated by the encoder as follows:

- If mode is equal to 4 (Hadamard transform, two residuals are transmitted), the joint residuals are determined according to

resJointC1[ x ][ y ] = ( resCb[ x ][ y ] + CSign * resCr[ x ][ y ] ) / 2
	resJointC2[ x ][ y ] = ( resCb[ x ][ y ] – CSign * resCr[ x ][ y ] ) / 2.

- Otherwise, if mode is equal to 2 (single residual with reconstruction Cb = C, Cr = CSign * C), the joint residual is determined according to

resJointC[ x ][ y ] = ( resCb[ x ][ y ] + CSign * resCr[ x ][ y ] ) / 2.

- Otherwise, if mode is equal to 1 (single residual with reconstruction Cb = C, Cr = (CSign * C) / 2), the joint residual is determined according to

resJointC[ x ][ y ] = ( 4 * resCb[ x ][ y ] + 2 * CSign * resCr[ x ][ y ] ) / 5.

- Otherwise (mode is equal to 3, i. e., single residual, reconstruction Cr = C, Cb = (CSign * C) / 2), the joint residual is determined according to

resJointC[ x ][ y ] = ( 4 * resCr[ x ][ y ] + 2 * CSign * resCb[ x ][ y ] ) / 5.

More details on how the encoder selects the TU-wise joint mode are given in Sec. 2.4 of JVET-N0282 [2].

# Specification Text

Changes of the specification text relative to the latest draft JVET-N1001-v8 are provided in the accompanying Word documents. All changes relative to JVET-N1001-v8 are marked using revision marks (editing ids “CE7-2.1” and “CE7-2.2”). Details on normative aspects can be found in these texts.

# Experimental Results

The proposed joint chroma coding configurations were tested on top of VTM 5.0 [4] according to the JVET common test conditions for SDR content [5] (stdQP), the low-QP configuration defined in the CE7 description [3] (lowQP, incl. restriction to 100 coded frames) as well as the common test conditions for HDR/WCG video [6]. Detailed results are provided on the following pages and in the accompanying Excel documents.

Table 3 summarizes the Bjøntegaard-delta (BD)-rate changes on top of VTM 5.0 due to the two CE subtests.

Table 3. Summary of average Bjøntegaard delta-rate results relative to VTM-5.0.

––––––––––––  CE7-2.1  ––––––––––––	 ––––––––––––  CE7-2.2  ––––––––––––

| Test | BDR-Y | BDR-Cb | BDR-Cr | Enc. time | BDR-Y | BDR-Cb | BDR-Cr | Enc. time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| stdQP, AI | –0.18% | –0.57% | 0.07% | 100% | –0.22% | –0.42% | –0.06% | 102% |
| stdQP, RA | –0.11% | –1.10% | 0.18% | 99% | –0.15% | –0.58% | 0.55% | 102% |
| stdQP, LB | 0.08% | –0.67% | 1.07% | 101% | 0.10% | 0.41% | 1.60% | 103% |
| lowQP, AI | –0.05% | –0.14% | 0.10% | 104% | –0.18% | 0.00% | –0.04% | 108% |
| lowQP, RA | –0.02% | –0.24% | –0.13% | 104% | –0.09% | –0.19% | –0.14% | 106% |
| lowQP, LB | –0.01% | –0.26% | –0.19% | 104% | –0.06% | –0.13% | –0.25% | 104% |
| DE100, AI | –3.05% |  |  | 100% | –2.98% |  |  | 103% |
| L100, AI | –0.05% |  |  | 100% | –0.06% |  |  | 103% |
| wPSNR, AI | –0.06% | –4.26% | 0.75% | 100% | –0.04% | –3.85% | –0.85% | 103% |
| PSNR, AI | –0.07% | –3.33% | 0.42% | 100% | –0.05% | –2.81% | –0.96% | 103% |
| DE100, RA | –2.58% |  |  | 99% | –2.84% |  |  | 101% |
| L100, RA | –0.01% |  |  | 99% | –0.03% |  |  | 101% |
| wPSNR, RA | –0.03% | –3.98% | 1.52% | 99% | –0.03% | –3.85% | –0.24% | 101% |
| PSNR, RA | –0.06% | –3.06% | 0.64% | 99% | –0.05% | –2.63% | –0.66% | 101% |

## SDR Common Test Conditions

The simulation results for the SDR common test conditions are summarized in Table 4. The BD rates and encoder/decoder time measurements are reported relative to VTM-5.0.

In order to evaluate the improvement of the VTM-5 joint chroma coding due to the suggested modifications, we additionally compared the results of CE7-2.1 and CE7-2.2 as well as the results of VTM-5.0 to VTM-5.0 with joint chroma coding disabled. This additional anchor results were obtained by setting the macro JVET_N0054_JOINT_CHROMA in the VTM-5.0 software equal to 0.  The average BD rates of VTM-5.0, CE7-2.1, and CE7-2.2 relative to this additional anchor are summarized in Table 5.

Table 4. Average Bjøntegaard delta-rate results relative to VTM-5.0 for SDR common test conditions.

|  |  |  | CE7-2.1 vs VTM-5.0 |  |  |  |  |  | CE7-2.2 vs VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | Cb | Cr | EncT | DecT |  | Y | Cb | Cr | EncT | DecT |
| AI | A1 |  | -0.23% | 0.49% | 0.68% | 100% | 97% |  | -0.25% | 0.08% | 0.75% | 101% | 96% |
|  | A2 |  | -0.53% | -0.58% | 0.23% | 101% | 99% |  | -0.72% | -0.96% | -0.01% | 104% | 99% |
|  | B |  | -0.08% | -0.13% | 0.19% | 100% | 99% |  | -0.09% | 0.30% | 0.16% | 103% | 99% |
|  | C |  | -0.06% | -1.17% | -0.24% | 100% | 97% |  | -0.07% | -0.71% | -0.29% | 101% | 100% |
|  | E |  | -0.09% | -1.59% | -0.49% | 100% | 98% |  | -0.10% | -1.17% | -0.99% | 102% | 99% |
|  | avg. |  | -0.18% | -0.57% | 0.07% | 100% | 98% |  | -0.22% | -0.42% | -0.06% | 102% | 99% |
|  | D |  | -0.05% | -1.13% | -0.14% | 100% | 97% |  | -0.07% | -0.74% | -0.20% | 102% | 98% |
|  | F |  | -0.16% | -0.94% | -0.71% | 101% | 98% |  | -0.17% | -0.64% | -0.95% | 101% | 98% |
| RA | A1 |  | -0.28% | -0.49% | 0.40% | 98% | 102% |  | -0.30% | -0.67% | 1.11% | 101% | 103% |
|  | A2 |  | -0.30% | -1.82% | 0.44% | 100% | 102% |  | -0.38% | -1.77% | 0.51% | 103% | 102% |
|  | B |  | 0.01% | -0.49% | 0.09% | 99% | 99% |  | -0.02% | 0.29% | 0.57% | 101% | 101% |
|  | C |  | 0.01% | -1.76% | -0.06% | 100% | 101% |  | -0.03% | -0.72% | 0.12% | 102% | 100% |
|  | avg. |  | -0.11% | -1.10% | 0.18% | 99% | 101% |  | -0.15% | -0.58% | 0.55% | 102% | 101% |
|  | D |  | 0.04% | -1.85% | 0.56% | 100% | 99% |  | -0.01% | -0.95% | 0.50% | 101% | 99% |
|  | F |  | -0.06% | -1.27% | -0.66% | 100% | 99% |  | -0.09% | -0.65% | -0.55% | 101% | 98% |
| LB | B |  | 0.05% | 0.15% | 1.74% | 100% | 102% |  | 0.03% | 0.92% | 1.93% | 103% | 101% |
|  | C |  | 0.09% | -0.95% | 0.50% | 101% | 103% |  | 0.07% | 0.21% | 1.26% | 102% | 105% |
|  | E |  | 0.10% | -1.65% | 0.72% | 100% | 103% |  | 0.25% | -0.17% | 1.52% | 103% | 108% |
|  | avg. |  | 0.08% | -0.67% | 1.07% | 101% | 102% |  | 0.10% | 0.41% | 1.60% | 103% | 104% |
|  | D |  | 0.07% | -1.05% | 2.79% | 100% | 99% |  | 0.08% | -0.29% | 3.32% | 101% | 107% |
|  | F |  | 0.06% | -1.34% | -1.52% | 99% | 99% |  | 0.13% | -0.36% | -0.71% | 102% | 103% |

Table 5. Average Bjøntegaard delta-rate results relative to VTM-5.0 without joint chroma residual coding (for SDR common test conditions).

|  |  | Reference: VTM-5.0 without joint Cb/Cr coding |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | VTM-5.0 |  |  |  |  | CE7-2.1 |  |  |  |  | CE7-2.2 |  |  |  |
|  |  | Y | Cb | Cr | EncT |  | Y | Cb | Cr | EncT |  | Y | Cb | Cr | EncT |
| AI | A1 | -0.26% | -0.03% | 0.72% | 103% |  | -0.49% | 0.51% | 1.40% | 104% |  | -0.51% | 0.11% | 1.47% | 105% |
|  | A2 | -0.63% | -1.44% | 0.48% | 106% |  | -1.15% | -2.02% | 0.70% | 107% |  | -1.34% | -2.37% | 0.46% | 111% |
|  | B | -0.17% | -0.27% | -1.80% | 102% |  | -0.25% | -0.40% | -1.60% | 103% |  | -0.26% | 0.02% | -1.62% | 105% |
|  | C | -0.26% | -1.09% | -2.89% | 105% |  | -0.32% | -2.23% | -3.13% | 105% |  | -0.33% | -1.78% | -3.18% | 106% |
|  | E | -0.18% | 0.03% | -2.43% | 103% |  | -0.27% | -1.55% | -2.92% | 103% |  | -0.28% | -1.14% | -3.41% | 105% |
|  | avg. | -0.28% | -0.56% | -1.35% | 104% |  | -0.46% | -1.12% | -1.28% | 104% |  | -0.50% | -0.95% | -1.40% | 106% |
|  | D | -0.24% | -1.34% | -2.63% | 105% |  | -0.29% | -2.45% | -2.77% | 105% |  | -0.31% | -2.07% | -2.82% | 107% |
|  | F | -0.29% | -2.40% | -3.76% | 102% |  | -0.45% | -3.31% | -4.47% | 103% |  | -0.46% | -3.01% | -4.68% | 103% |
| RA | A1 | -0.38% | -0.30% | 1.41% | 104% |  | -0.66% | -0.75% | 1.81% | 102% |  | -0.68% | -0.92% | 2.54% | 105% |
|  | A2 | -0.39% | -2.81% | 0.26% | 104% |  | -0.69% | -4.56% | 0.70% | 104% |  | -0.77% | -4.50% | 0.78% | 107% |
|  | B | -0.17% | -0.10% | -1.25% | 102% |  | -0.16% | -0.58% | -1.14% | 102% |  | -0.18% | 0.20% | -0.64% | 103% |
|  | C | -0.17% | 0.19% | -2.15% | 103% |  | -0.16% | -1.56% | -2.21% | 102% |  | -0.19% | -0.52% | -2.02% | 104% |
|  | avg. | -0.25% | -0.60% | -0.66% | 103% |  | -0.36% | -1.67% | -0.46% | 102% |  | -0.40% | -1.15% | -0.09% | 105% |
|  | D | -0.15% | -0.47% | -2.45% | 103% |  | -0.11% | -2.32% | -1.91% | 103% |  | -0.15% | -1.42% | -1.96% | 104% |
|  | F | -0.42% | -1.74% | -3.55% | 102% |  | -0.47% | -2.98% | -4.21% | 102% |  | -0.51% | -2.38% | -4.08% | 104% |
| LB | B | -0.14% | -3.70% | -4.35% | 102% |  | -0.09% | -3.54% | -2.75% | 102% |  | -0.12% | -2.81% | -2.54% | 105% |
|  | C | -0.11% | -3.34% | -4.93% | 102% |  | -0.02% | -4.26% | -4.45% | 103% |  | -0.04% | -3.15% | -3.72% | 105% |
|  | E | -0.08% | -2.40% | -5.22% | 99% |  | 0.02% | -4.05% | -4.55% | 99% |  | 0.17% | -2.58% | -3.77% | 102% |
|  | avg. | -0.12% | -3.26% | -4.76% | 101% |  | -0.04% | -3.91% | -3.77% | 102% |  | -0.02% | -2.87% | -3.24% | 104% |
|  | D | -0.06% | -4.44% | -4.97% | 101% |  | 0.01% | -5.44% | -2.37% | 101% |  | 0.02% | -4.70% | -1.84% | 102% |
|  | F | -0.30% | -3.40% | -4.13% | 102% |  | -0.24% | -4.70% | -5.62% | 101% |  | -0.17% | -3.74% | -4.84% | 104% |

## HDR Common Test Conditions

The simulation results for the HDR common test conditions are summarized in Table 6. The BD rates and encoder/decoder time measurements are reported relative to VTM-5.0.

Table 6. Average Bjøntegaard delta-rate results relative to VTM-5.0 for HDR common test conditions.

|  |  | All Intra (vs VTM-5.0) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DE100 | PSNR |  | wPSNR |  |  | PSNR |  |  |  |
|  |  |  | L100 | Y | U | V | Y | U | V | EncT | DecT |
| CE7-2.1 | H1 | -3.05% | -0.05% | -0.06% | -4.26% | 0.75% | -0.06% | -4.61% | 1.38% | 101% | 99% |
|  | H2 |  |  |  |  |  | -0.07% | -1.09% | -1.25% | 99% | 101% |
|  | avg. | -3.05% | -0.05% | -0.06% | -4.26% | 0.75% | -0.07% | -3.33% | 0.42% | 100% | 100% |
| CE7-2.2 | H1 | -2.98% | -0.06% | -0.04% | -3.85% | -0.85% | -0.04% | -4.26% | -0.33% | 103% | 100% |
|  | H2 |  |  |  |  |  | -0.08% | -0.25% | -2.07% | 102% | 101% |
|  | avg. | -2.98% | -0.06% | -0.04% | -3.85% | -0.85% | -0.05% | -2.81% | -0.96% | 103% | 100% |
|  |  | Random Access (vs VTM-5.0) |  |  |  |  |  |  |  |  |  |
|  |  | DE100 | PSNR |  | wPSNR |  |  | PSNR |  |  |  |
|  |  |  | L100 | Y | U | V | Y | U | V | EncT | DecT |
| CE7-2.1 | H1 | -2.58% | -0.01% | -0.03% | -3.98% | 1.52% | -0.06% | -4.15% | 2.33% | 99% | 101% |
|  | H2 |  |  |  |  |  | -0.05% | -1.16% | -2.32% | 100% | 100% |
|  | avg. | -2.58% | -0.01% | -0.03% | -3.98% | 1.52% | -0.06% | -3.06% | 0.64% | 99% | 100% |
| CE7-2.2 | H1 | -2.84% | -0.03% | -0.03% | -3.85% | -0.24% | -0.04% | -4.16% | 0.55% | 101% | 100% |
|  | H2 |  |  |  |  |  | -0.07% | 0.05% | -2.78% | 101% | 101% |
|  | avg. | -2.84% | -0.03% | -0.03% | -3.85% | -0.24% | -0.05% | -2.63% | -0.66% | 101% | 100% |

## Low QP Results for SDR

The simulation results for the low QP configuration (SDR with QP values 2, 7, 12, 17 and 100 encoded frames) are summarized in Table 7. The BD rates and encoder/decoder time measurements are reported relative to VTM-5.0.

Table 7. Average Bjøntegaard delta-rate results relative to VTM-5.0 for the low-QP configuration (SDR with QP values 2, 7, 12, 17 and 100 coded frames).

|  |  |  | CE7-2.1 vs VTM-5.0 |  |  |  |  |  | CE7-2.2 vs VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | Cb | Cr | EncT | DecT |  | Y | Cb | Cr | EncT | DecT |
| AI | A1 |  | 0.00% | -0.08% | -0.02% | 102% | 100% |  | -0.01% | -0.09% | 0.03% | 105% | 99% |
|  | A2 |  | -0.03% | 0.02% | 0.01% | 105% | 100% |  | -0.30% | 0.11% | 0.05% | 110% | 100% |
|  | B |  | -0.01% | -0.07% | -0.01% | 103% | 109% |  | -0.09% | -0.01% | -0.08% | 109% | 106% |
|  | C |  | -0.16% | -0.41% | 0.34% | 103% | 112% |  | -0.36% | -0.07% | -0.09% | 105% | 101% |
|  | E |  | -0.06% | -0.11% | 0.19% | 105% | 109% |  | -0.15% | 0.11% | -0.03% | 109% | 103% |
|  | avg. |  | -0.05% | -0.14% | 0.10% | 104% | 107% |  | -0.18% | 0.00% | -0.04% | 108% | 102% |
|  | D |  | -0.17% | -0.40% | 0.13% | 104% | 115% |  | -0.43% | -0.24% | -0.39% | 106% | 97% |
|  | F |  | -0.28% | -0.49% | 0.13% | 105% | 115% |  | -0.79% | -0.70% | -0.69% | 105% | 103% |
| RA | A1 |  | 0.01% | -0.21% | -0.01% | 101% | 99% |  | -0.02% | -0.36% | 0.15% | 104% | 98% |
|  | A2 |  | -0.05% | 0.03% | 0.04% | 102% | 101% |  | -0.24% | 0.02% | 0.01% | 108% | 100% |
|  | B |  | 0.03% | -0.20% | -0.16% | 104% | 101% |  | -0.02% | -0.12% | -0.20% | 107% | 102% |
|  | C |  | -0.08% | -0.53% | -0.29% | 109% | 106% |  | -0.13% | -0.30% | -0.39% | 106% | 100% |
|  | avg. |  | -0.02% | -0.24% | -0.13% | 104% | 102% |  | -0.09% | -0.19% | -0.14% | 106% | 100% |
|  | D |  | -0.05% | -0.56% | -0.30% | 107% | 118% |  | -0.17% | -0.37% | -0.61% | 104% | 100% |
|  | F |  | -0.21% | -0.67% | -0.78% | 106% | 110% |  | -0.56% | -0.74% | -0.98% | 108% | 106% |
| LB | B |  | 0.02% | -0.12% | -0.10% | 104% | 102% |  | -0.03% | -0.06% | -0.13% | 104% | 101% |
|  | C |  | -0.08% | -0.43% | -0.38% | 105% | 101% |  | -0.13% | -0.25% | -0.33% | 104% | 100% |
|  | E |  | 0.03% | -0.28% | -0.10% | 102% | 99% |  | -0.02% | -0.07% | -0.35% | 103% | 99% |
|  | avg. |  | -0.01% | -0.26% | -0.19% | 104% | 101% |  | -0.06% | -0.13% | -0.25% | 104% | 100% |
|  | D |  | -0.07% | -0.48% | -0.13% | 106% | 114% |  | -0.12% | -0.36% | -0.31% | 102% | 99% |
|  | F |  | -0.12% | -0.47% | -0.43% | 109% | 111% |  | -0.43% | -0.45% | -0.59% | 106% | 107% |

# References

- J. Lainema, “CE7-related: Joint coding of chrominance residuals,” Joint Video Experts Team, doc. JVET-M0305, Marrakech, MA, Jan. 2019.

- C. Helmrich, C. Rudat, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand, “CE7-related: Joint chroma residual coding with multiple modes,” Joint Video Experts Team, doc. JVET-N0282, Geneva, CH, Mar. 2019.

- H. Schwarz, M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding,” Joint Video Experts Team, doc. JVET-N1027, Geneva, CH, June 2019.

- JVET, “VTM version 5.0,” https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team, doc. JVET-N1010, Geneva, CH, May 2019.

- A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video,” Joint Video Experts Team, doc. JVET-N1011, Geneva, CH, Apr. 2019.

# Patent Rights Declarations

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).