| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0107-v1 |
| --- | --- |

| Title: | CE3-1.7: On 1xN and 2xN subblocks of ISP with vertical intra mode restriction for 4xN vertically split ISP-coded blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Geert Van der Auwera Ted Hsieh Vadim Seregin Luong Pham Van Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | aramasub@qti.qualcomm.com |
| Source: | Qualcomm Technologies Inc. |  |  |

_____________________________

# Abstract

This document reports the results of sub-test CE3-1.7 of CE3 Intra Prediction and Mode Coding. The sub-test studies the performance of allowing only vertical intra prediction mode for 4xN CUs that are coded using ISP with vertical split on top of the sub-test CE3-1.6 (which defines 4xN prediction regions for the ISP-coded CUs that contain 1xN or 2xN subblocks.) The changes from JVET WD5 are as follows: for 4xN coding blocks and 8xN coding blocks (N > 4) that are coded using ISP with vertical split, the prediction region is specified to be of size 4xN; for 4xN coding blocks that are coded using ISP with vertical split, only the vertical intra prediction mode is allowed. The BD-rate performance of the test is as follows: (0.01%, −0.01%, 0.03%) for (Y, Cb, Cr) at runtime of 100% (EncT) and 101% (DecT) for AI configuration and (0.02%, −0.02%, −0.04%) for (Y, Cb, Cr) at runtime of 100% (EncT) and 101% (DecT) for RA configuration

# Introduction

Intra sub-partition (ISP) coding is applied to luma samples, where a coding unit is split into two or four sub-units and each unit is predicted and coded into a subblock. Each subblock of the coding unit is dependent on the reconstructed values of the preceding subblock (if any) in the coding unit. The subblocks may be obtained by horizontal or vertical split of the coding units. Each subblock is restricted to contain at least 16 samples. All the subblocks in a coding block share the same intra prediction mode.

In previous meetings and in the JVET reflector, some experts have raised concerns regarding the handling of 1xN and 2xN subblocks of ISP. It has been mentioned that dependence of 1xN/2xN subblocks on the reconstructed values of preceding 1xN/2xN subblocks of the coding block may result in implementation issues. There was expressed desire to remove this reconstruction dependence.

# Test

This test studies the performance restricting the intra prediction mode to be vertical mode only for 4xN coding blocks that are coded with ISP and vertical split on top of the sub-test CE3-1.6. The test CE3-1.6 removes the dependence of 1xN/2xN subblock prediction on the reconstructed values of previously decoded 1xN/2xN subblocks of the coding block; the minimum width of prediction for subblocks is proposed to be set as four samples; there is no change in the transform block sizes. In this test, in addition to the changes in CE3-1.6, the intra prediction mode is not signalled and inferred to be VER mode for 4xN coding blocks that are coded with ISP and vertical split. Figure 1 shows how an 8xN (N > 4) coding block that is coded using ISP with vertical split is split into two prediction regions each of size 4xN and four transforms of size 2xN. Figure 2 shows how a 4xN coding block that is coded using ISP with vertical split is predicted using the full 4xN block; four transform each of 1xN is used and only VER intra mode is used for prediction.

Figure 1: An 8xN (N > 4) coding block coded with ISP and split vertically; prediction is done on 4xN prediction regions and the transform block (TB) size is 2xN.

Figure 2: A 4xN coding block coded with ISP and split vertically; prediction is done on 4xN prediction region and the transform block size is 1xN; only VER intra prediction mode is allowed.

The following table summarizes the proposed partition sizes for prediction and transform sizes; for other block sizes, the prediction and transform sizes are as currently defined for ISP. The changes from VTM-5.0 are highlighted in yellow. The entries for the sub-test CE3-1.6 are also provided for reference.

| Coding block sizes (WxH) | VTM-5.0 |  | CE3-1.6 |  | CE3-1.7 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Partition sizes for prediction | Transform sizes | Partition sizes for prediction | Transform sizes | Partition sizes for prediction | Transform sizes |
| 4x8 | 2x8 | 2x8 | 4x8 | 2x8 | 4x8 (only VER) | 2x8 |
| 4xN (N > 8) | 1xN | 1xN | 4xN | 1xN | 4xN (only VER) | 1xN |
| 8xN (N > 4) | 2xN | 2xN | 4xN | 2xN | 4xN | 2xN |

Although the transform sizes of 1xN and 2xN are still retained in the test, it is asserted that the transform of these blocks in 4xN regions can be performed in parallel. For example, when a 4xN prediction region contains four 1xN transforms, there is no transform in the horizontal direction; the transform in the vertical direction can be performed as a single 4xN transform in the vertical direction. Similarly, when a 4xN prediction region contains two 2xN transform blocks, the transform operation of the two 2xN blocks in each direction (horizontal and vertical) can be conducted in parallel. Thus, there is no delay added in processing these smaller blocks than processing 4x4 regular-coded intra blocks.

# Simulation results

The simulations for the test were conducted under common test conditions on VTM-5.0. The BD-rate performance of the test is as follows: (0.01%, −0.01%, 0.03%) for (Y, Cb, Cr) at runtime of 100% (EncT) and 101% (DecT) for AI configuration and (0.02%, −0.02%, −0.04%) for (Y, Cb, Cr) at runtime of 100% (EncT) and 101% (DecT) for RA configuration. The full results are reported in Table 1 and in the attached Excel sheet.

Table 1: Simulation results for sub-test CE3-1.7 for 4xN prediction regions for 1xN and 2xN ISP subblocks, and VER mode for 4xN ISP-coded blocks with vertical split

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | 0.03% | 100% | 98% |
| Class A2 | 0.01% | 0.02% | -0.02% | 100% | 100% |
| Class B | 0.02% | -0.03% | 0.03% | 101% | 102% |
| Class C | 0.01% | 0.04% | 0.06% | 99% | 102% |
| Class E | 0.01% | -0.03% | 0.06% | 99% | 102% |
| Overall | 0.01% | -0.01% | 0.03% | 100% | 101% |
| Class D | 0.03% | 0.21% | 0.03% | 98% | 101% |
| Class F | 0.00% | -0.03% | 0.09% | 101% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.08% | 0.10% | 100% | 101% |
| Class A2 | 0.04% | 0.10% | -0.01% | 101% | 102% |
| Class B | 0.03% | -0.01% | 0.02% | 101% | 102% |
| Class C | -0.02% | -0.09% | -0.24% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.02% | -0.04% | 100% | 101% |
| Class D | 0.04% | 0.46% | 0.24% | 97% | 99% |
| Class F | 0.00% | 0.00% | -0.01% | 100% | 101% |

# Conclusions

The reported test has minimal changes to VTM-5.0 to solve the problem of dependence on previously reconstructed 1xN/2xN subblocks of a coding block. The changes result in minimal performance loss. It is proposed to adopt this change in VVC.

# Patent rights declaration

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).