| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0109 |
| --- | --- |

| Title: | CE3-1.4: Intra Sub-Partitions Coding without Thin Partitions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-chuan Ma Xiao-yu Xiu Yi-Wen Chen Xianglin Wang Hong-Jheng Jhu | Email: | tsung-chuanma@kwai.com xiaoyuxiu@kwai.com yiwenchen@kwai.com xianglinwang@kwai.com jhuhong-jheng@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, we report CE results with 1xN or 2xN subblock partitions disabled for intra sub-partition (ISP) mode. The method being tested was proposed to improve the decoding throughput of the ISP mode. Based on the CE test conditions, simulation results are as follows:

- When disabling 1×N ISP sub-partitions, the BD-rate increases are 0.01%, 0.01% and 0.06% for AI, RA and LD, respectively.

- When disabling 1×N and 2×N ISP sub-partitions, the BD-rate increases are 0.08%, 0.02% and 0.04% for AI, RA and LD, respectively.

# Introduction

In VTM-5.0 [1], the ISP mode splits a current coding unit into 4 or 2 sub-blocks according to the size of the current block [2, 3]. When the ISP is enabled, the reconstructed samples of one ISP subblock can be used as references to predict the samples of its following subblock. In other words, the intra prediction of one ISP subblock may not be started until its preceding ISP subblock (according to the ISP coding order) is fully reconstructed. Such dependency causes decoding throughput issue, especially for the ISP CUs with small sub-partitions, e.g., 1xN, 2xN. For example, considering an ISP CU in the size of 4xN that is split into four 1xN subblocks, it may consume 4 times processing cycles than a non-ISP 4xN CU. In other words, the decoding throughput of the former is roughly just one-quarter of the latter.

In this contribution, the following two options are proposed and tested:

- First option, only removing ISP mode that introduces 1xN sub-blocks.

- Second option, removing ISP mode that introduces 1xN or 2xN sub-blocks.

# Results

The proposed modifications are integrated into VTM-5.0 [1] and the results of the proposed implementations are shown below against the VTM-5.0 under CTC [4]. Table 1 and Table 2 show the BD-rate results of the proposed option 1 and option 2 respectively. For more detailed results in each setting, please refer to the attached spreadsheets.

Table 1. Results of proposed option 1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.08% | 0.04% | 95% | 98% |
| Class A2 | -0.01% | -0.07% | -0.02% | 96% | 98% |
| Class B | 0.00% | -0.06% | 0.03% | 98% | 100% |
| Class C | 0.02% | -0.02% | 0.09% | 95% | 97% |
| Class E | 0.04% | 0.00% | -0.03% | 99% | 102% |
| Overall | 0.01% | -0.02% | 0.03% | 97% | 99% |
| Class D | 0.02% | 0.22% | -0.07% | 100% | 97% |
| Class F | 0.03% | 0.06% | 0.07% | 102% | 104% |

Table 2. Results of proposed option 2

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.02% | 97% | 100% |
| Class A2 | 0.01% | -0.08% | -0.08% | 100% | 102% |
| Class B | 0.04% | -0.06% | 0.02% | 96% | 99% |
| Class C | 0.13% | 0.00% | 0.10% | 97% | 101% |
| Class E | 0.24% | 0.19% | 0.16% | 100% | 105% |
| Overall | 0.08% | 0.00% | 0.04% | 98% | 101% |
| Class D | 0.17% | 0.29% | 0.02% | 97% | 94% |
| Class F | 0.12% | 0.17% | 0.13% | 101% | 102% |

# Conclusion

In this proposal, we provide two options to simplify ISP mode in order to increase its throughput. As shown in Table 1, removing only 1xN partition under ISP mode virtually no RD-rate impact; removing both 1xN and 2xN partitions under ISP mode causes 0.08% RD-rate loss. It is recommended to adopt one of the proposed modifications into VVC.

# References

| [1] | "VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM". |
| --- | --- |
| [2] | T.-C. Ma, X. Xiu, Y.-W. Chen and X. Wang, "Non-CE3: Intra Sub-Partitions Coding without Thin Partitions," Document JVET-N0330, Geneva, CH, 2019. |
| [3] | S. De-Luxán-Hernández, H. Schwarz, D. Marpe and T. Wiegand, "CE3: Line-based intra coding mode (Tests 1.1.1 and 1.1.2)," Document JVET-M0102, Marrakesh , MA, 2019. |
| [4] | F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, "JVET common test conditions and software reference configurations for SDR video," 2018. |

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).