| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0110 |
| --- | --- |

| Title: | CE7-4.1: Simplification of cbf coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Xiaoyu Xiu Tsung-Chuan Ma Xianglin Wang | Email: | yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

Reported in this document is the results of CE7-4.1 on the simplification of luma cbf coding. In current VVC, five contexts are utilized for the CABAC coding of the luma cbf. In this proposal, it is proposed to remove the logics of context selection based on the intra sub-partition mode. With the proposed modification, the logics for the context selection of luma cbf coding is reduced and the number of contexts for luma cbf coding is also reduced by 2. 

Based on the simulation results reported, the proposed modification incurs {0.01%, -0.05%, -0.01%}, {0.01%, -0.08%, 0.00%} and {0.00%, 0.04%, 0.17%} BD-rates changes on Y, Cb and Cr components for AI, RA and LDB over VTM-5.0 under CTC; for the low QPs conditions (17, 12, 7, 2), the proposed simplification shows {0.00%, -0.01%, 0.01%}, {0.00%, -0.01%, 0.00%} and {-0.01%, 0.01%, -0.02%} BD-rates changes for AI, RA and LDB.

# Introduction

In VVC, a luma coefficient coded block flag (cbf) is signaled for each luma transform block to indicate whether there are non-zero coefficients in this transform block. The corresponding context selection procedures for luma cbf coding are illustrated as below:

##### Derivation process of ctxInc for the syntax element tu_cbf_luma

Five CABAC contexts could be utilized to coding the tu_cbf_luma, with the context index (ctxInc) determined based on the following procedure. The variable trDepth specifies if the current transform block is smaller than the current CU.

- If intra_bdpcm_flag[ ][ ] is equal to 1
            ctxInc=4;

- Otherwise if IntraSubpartitionSplitType is equal to ISP_NO_SPLIT or current block is not luma block, the following applies:

ctxInc = trDepth = = 0 ?  1 : 0

- Otherwise (IntraSubpartitionSplitType is not equal to ISP_NO_SPLIT and current transform block is luma block), the following applies:

  - The variable prevTuCbfY is derived as follows:

    - If the current transform unit is the first one to be parsed in a coding unit, prevTuCbfY is set equal to 0.

    - Otherwise, prevTuCbfY is set equal to the value of tu_cbf_luma of the previous luma transform unit in the current coding unit.

  - The variable ctxInc is derived as follows:

ctxInc = 2 + prevTuCbfY

# Proposed Methods

It is proposed to remove the context selection based on the ISP mode for luma cbf coding. The simplified context selection is illustrated as below.

- If intra_bdpcm_flag[ ][ ] is equal to 1
            ctxInc=2;

- Otherwise, the following applies:

ctxInc = trDepth = = 0 ?  1 : 0

# Results

The proposed modification is integrated into VTM-5.0 [1] and the results of the proposed implementation under CTC [2] and the low QP condition (QP=17, 12, 7, 2) are shown below against VTM-5.0.

Table 1. The proposed modification under CTC

Table 2. The proposed modification under low QP condition

# Conclusion

In this proposal, a simplification on luma cbf coding is proposed to reduce the condition checking for context selection as well as the number of CABAC context models. The modification incurs {0.01%, -0.05%, -0.01%}, {0.01%, -0.08%, 0.00%} and {0.00%, 0.04%, 0.17%} BD-rates changes on Y, Cb and Cr components for AI, RA and LDB. It is recommended to adopt the proposed simplification into VVC for luma cbf coding.

# Reference

- VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-N1010, March 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).