| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0111-v1 |
| --- | --- |

| Title: | Non-CE3: Modified CABAC Coding for Intra Prediction Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | B. Wang, A. M. Kotra, S. Esenlik H. Gao, E. Alshina (Huawei) | Tel: Email: | biao.wang@huawei.com, |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

A planar flag is introduced in VTM5.0 due to the high occurrence of planar mode in intra prediction. The planar flag uses CABAC regular coding with two contexts based on whether current block is applied with Intra Sub-Partition (ISP) coding tool. In this contribution, four contexts are proposed based on the prediction mode of left and above blocks of the current block. Furthermore, the first bin of the MPM index is changed from bypass coding to regular CABAC coding, and the same context derivation logic to the planar flag is used. Compared to VTM5.0, the proposed method achieves a coding performance of -0.07% and -0.02% for all intra and random access configurations, respectively, with negligible runtime difference.

# Introduction

A new planar flag (intra_luma_not_planar_flag) is introduced in VTM5.0 to identify whether the intra mode of current block is Planar mode or not. Its syntax is as shown in Table 1:

Table 1: Syntax of planar flag and MPM index

| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| --- | --- |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |

When intra_luma_not_planar_flag is equal to 0, it indicates the current block is applied with planar mode. Otherwise (intra_luma_not_planar_flag is equal to 1), it indicates the current block is not applied with planar mode. In this case, the intra prediction mode of the current block is obtained from a 5-MPM list using an MPM index (intra_luma_mpm_idx) decoded from a bitstream.

The planar flag (intra_luma_not_planar_flag) uses regular CABAC coding with two contexts:

- the current block is applied with ISP mode

- the current block is not applied with ISP mode

The MPM index (intra_luma_mpm_idx) ranges from 0 to 4 and is binarized using truncated unary code. For each possible value of MPM index, its binarized codeword is shown in Table 2.

Table 2: Binarization of MPM index using Truncated unary code

| MPM index | Bin string of binarized mpm index |
| --- | --- |
| 0 | 0 |
| 1 | 10 |
| 2 | 110 |
| 3 | 1110 |
| 4 | 1111 |

Each bin in the binarized bin string of MPM index is bypass coded, as shown in Table 3:

Table 3: CABAC coding mode for each bin of MPM Index

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| intra_luma_mpm_idx[ ][ ] | bypass | bypass | bypass | bypass | na | na |

# Proposed Modification

With the introduction of not planar flag (intra_luma_not_planar_flag), it is possible to know whether an intra-predicted block uses planar mode or not without MPM list construction. This contribution introduces more contexts for coding the not planar flag and the first bin of the MPM index. Based on the not planar flag of left and above block, four contexts are used:

- current block is applied with ISP mode

- current block is not applied with ISP mode and neither left nor above block use planar mode.

- current block is not applied with ISP mode and either left or above block use planar mode.

- current block is not applied with ISP mode and both left or above blocks use planar mode.

The position of left and above blocks is shown in Figure 1:

Figure 1: position of left (L) and above (A) blocks

The neighboring left (or above) block is considered as if it uses planar mode if one of the following conditions is true:

- it is applied with Matrix-based intra prediction;

- it is applied with inter-prediction;

- it is applied with Intra block copy (IBC) mode;

- it is applied with normal intra prediction, and the prediction mode is planar mode.

For the first bin of MPM index, it is changed from bypass coding to CABAC regular coding, and the same four contexts are used.

In total 8 context models, 4 for the first bin of the MPM index and 4 for the not planar flag are used in the proposal. The same context switching conditions are used to select the context model of both bins. Which means, for instance, current block is ISP block, then first context model is selected for the not planar flag and at the same time first context model is selected for the first bin of the MPM index.

# Experimental Results

The simulations are performed according to common test conditions (CTC) as described in [1]. AI and RA results are reported.

## Simulation result for proposed modification

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.07% | -0.15% | 100% | 101% |
| Class A2 | -0.06% | -0.07% | -0.10% | 101% | 101% |
| Class B | -0.07% | -0.11% | -0.10% | 101% | 101% |
| Class C | -0.06% | -0.09% | -0.03% | 101% | 101% |
| Class E | -0.08% | -0.11% | -0.17% | 101% | 101% |
| Overall | -0.07% | -0.09% | -0.10% | 101% | 101% |
| Class D | -0.04% | 0.07% | -0.03% | 101% | 101% |
| Class F | -0.09% | -0.06% | -0.11% | 99% | 101% |
|  | Random Access |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.17% | 0.00% | 101% | 101% |
| Class A2 | 0.01% | -0.06% | 0.06% | 101% | 101% |
| Class B | -0.01% | 0.04% | -0.24% | 101% | 100% |
| Class C | -0.03% | -0.22% | -0.29% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.09% | -0.14% | 101% | 100% |
| Class D | -0.04% | -0.07% | 0.23% | 101% | 101% |
| Class F | -0.05% | 0.01% | 0.06% | 100% | 100% |

## Additional results with modification to planar flag only

Additional results is provided where only the CABAC coding of planar flag is changed (and the CABAC coding for the first bin of MPM index is not changed).

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.05% | -0.06% | 100% | 101% |
| Class A2 | -0.02% | -0.03% | -0.08% | 101% | 101% |
| Class B | -0.03% | -0.06% | -0.01% | 101% | 101% |
| Class C | -0.01% | -0.03% | 0.06% | 101% | 100% |
| Class E | -0.07% | -0.15% | -0.07% | 101% | 100% |
| Overall | -0.04% | -0.06% | -0.03% | 101% | 101% |
| Class D | -0.03% | 0.16% | -0.07% | 101% | 101% |
| Class F | -0.01% | -0.02% | 0.01% | 101% | 101% |
|  | Random Access |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.23% | 0.07% | 101% | 100% |
| Class A2 | 0.01% | 0.08% | 0.04% | 101% | 100% |
| Class B | 0.01% | -0.02% | -0.25% | 100% | 100% |
| Class C | -0.05% | -0.08% | 0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.06% | -0.04% | 101% | 100% |
| Class D | 0.03% | 0.02% | 0.08% | 101% | 101% |
| Class F | 0.04% | 0.02% | 0.09% | 100% | 100% |

# Conclusion

This contribution proposes four contexts for the newly introduced planar flag in VTM5.0 based on left and above blocks. Meanwhile, the bypass coding mode of the first bin of the MPM index is changed to CABAC regular coding. 4 context models are used for the planar flag and 4 additional context models are used for the first bin of the MPM index. It is noted that same context switching conditions are used for coding of both bins, which means context model of both bins can be decided at the same time using the same conditions. The proposed modification provides 0.07% and 0.02% coding gain over VTM5.0 on all intra and random access configurations, respectively, with no impact on runtime.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “JVET common test conditions and software reference configurations”, JVET-N1010, F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring

- “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-N1023, G. Van der Auwera, L. Li, and A. Filippov

- “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, B. Bross, J. Chen, and S. Liu