| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0112-v1 |
| --- | --- |

| Title: | CE1: CABAC Initialization, all experiments |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 647 745 1691 fbossen@sharpsec.com |
| Source: | Sharp Electronics of Canada Ltd |  |  |

_____________________________

# Abstract

Results for all CE1 experiments on CABAC initialization are reported. No substantial coding efficiency differences are observed between the different proposed methods and the VVC draft 5 method, except at high QP values. Initial state training times are found to be dominated by the generation of bitstreams and thus only marginally affected by the initialization method.

# Summary

## Coding efficiency

In addition to regular CTC conditions, the “random access” configuration was also tested as extremely low (2-17) and high (42-57) QP values. The table below captures BD rate difference averages for the luma component.

| Experiment | AI | RA | LB | LP | RA (low) | RA (high) |
| --- | --- | --- | --- | --- | --- | --- |
| CE1-1.1 | -0.02% | 0.01% | 0.04% | 0.00% |  | 0.66% |
| CE1-1.2 | -0.02% | -0.01% | 0.03% | -0.09% |  | 0.22% |
| CE1-2.1 | -0.02% | -0.02% | -0.01% | -0.07% |  | 0.74% |
| CE1-3.1.a | -0.01% | -0.02% | -0.01% | -0.02% | 0.00% | 0.33% |
| CE1-3.1.b | 0.00% | 0.00% | 0.00% | -0.04% | 0.01% | 0.65% |

Coding efficiency differences are minor. The largest differences are observed at very high QP values. Experiments with a larger init value space (2.1 and 3.1.a: 8 bits) do not yield substantially better performance.

## Training time

The initial state training code was run in a distributed fashion. The table below reports the (wall clock) run times obtained with the code provided in CE1 and also with proprietary code (both yield the same results). These run times exclude the time required to generate the bitstreams used for training. Training time is affected mainly by the init value space size. Results are thus not provided separately for each experiment, but by init value space size.

| InitValue | Experiments | CE1 code | Own code |
| --- | --- | --- | --- |
| 6 bits | 1.1, 1.2, 3.1.b | 89.4 minutes | 24.8 minutes |
| 8 bits | 2.1, 3.1.a | 261.9 minutes | 82.0 minutes |

Given the short amount of time required for training, the search space size does not appear to be a critical aspect. Indeed, the amount of time required to generate the bitstreams used for training is substantially higher (by at least an order of magnitude).

# Details

## Experiment 1.1: 6-bit init value, linear space

Regular QP values:

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.00% | 0.05% | -0.02% | 0.00% | 101% | 101% |
| Class A2 | -0.01% | 0.01% | 0.00% | -0.01% | 99% | 100% |
| Class B | -0.01% | 0.06% | 0.07% | -0.01% | 101% | 102% |
| Class C | -0.03% | 0.01% | 0.11% | -0.02% | 102% | 101% |
| Class E | -0.05% | 0.06% | -0.06% | -0.04% | 101% | 100% |
| Overall | -0.02% | 0.04% | 0.03% | -0.01% | 101% | 101% |
| Class D | -0.04% | 0.20% | 0.08% | -0.01% | 100% | 100% |
| Class F | 0.00% | 0.11% | 0.11% | 0.02% | 100% | 101% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.00% | 0.17% | 0.21% | 0.01% | 100% | 101% |
| Class A2 | 0.03% | 0.32% | 0.11% | 0.06% | 100% | 101% |
| Class B | 0.03% | 0.24% | -0.14% | 0.03% | 100% | 101% |
| Class C | -0.03% | 0.08% | 0.12% | -0.03% | 101% | 101% |
| Class E |  |  |  |  |  |  |
| Overall | 0.01% | 0.20% | 0.05% | 0.02% | 100% | 101% |
| Class D | 0.01% | 0.35% | 0.37% | 0.04% | 101% | 101% |
| Class F | 0.00% | 0.17% | 0.16% | 0.01% | 100% | 101% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.01% | 0.68% | -0.15% | 0.04% | 102% | 102% |
| Class C | -0.01% | 0.57% | 0.15% | 0.03% | 100% | 101% |
| Class E | 0.14% | 1.26% | 0.40% | 0.20% | 100% | 103% |
| Overall | 0.04% | 0.79% | 0.09% | 0.08% | 101% | 102% |
| Class D | 0.04% | 0.04% | 0.83% | 0.07% | 101% | 101% |
| Class F | -0.02% | -0.25% | -0.68% | -0.11% | 100% | 101% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.02% | 0.49% | 0.28% | 0.00% | 104% | 103% |
| Class C | -0.02% | 0.33% | 0.35% | 0.00% | 101% | 100% |
| Class E | 0.05% | 1.09% | 0.65% | 0.12% | 99% | 101% |
| Overall | 0.00% | 0.58% | 0.39% | 0.03% | 102% | 102% |
| Class D | 0.01% | 0.20% | 0.95% | 0.04% | 97% | 98% |
| Class F | -0.08% | -0.11% | -0.43% | -0.02% | 99% | 102% |

High QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.77% | 1.85% | 0.90% | 0.84% | 98% | 99% |
| Class A2 | 0.55% | 0.76% | 0.24% | 0.56% | 96% | 100% |
| Class B | 0.81% | 1.01% | 0.84% | 0.82% | 94% | 100% |
| Class C | 0.46% | 1.05% | 0.65% | 0.48% | 98% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | 0.66% | 1.14% | 0.68% | 0.68% | 96% | 100% |
| Class D | 0.97% | 2.17% | 1.29% | 0.98% | 98% | 100% |
| Class F | 0.12% | 0.43% | 0.45% | 0.15% | 98% | 100% |

Low QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.01% | 0.06% | 0.03% | 0.01% | 100% | 102% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 52% | 53% |
| Class B | -0.01% | 0.04% | 0.03% | 0.01% | 101% | 100% |
| Class C | -0.02% | 0.05% | 0.02% | -0.01% | 101% | 101% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 88% | 89% |
| Class D | -0.04% | 0.02% | 0.01% | -0.03% | 101% | 101% |
| Class F | 0.22% | 0.02% | 0.01% | -0.32% | 101% | 101% |

## Experiment 1.2: 6-bit init value, linear space

Regular QP values:

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.04% | 0.00% | 103% | 101% |
| Class A2 | 0.00% | 0.01% | -0.04% | 0.00% | 101% | 101% |
| Class B | -0.01% | 0.02% | 0.02% | -0.01% | 102% | 101% |
| Class C | -0.02% | -0.02% | 0.03% | -0.02% | 102% | 100% |
| Class E | -0.05% | -0.06% | -0.04% | -0.05% | 101% | 99% |
| Overall | -0.02% | -0.01% | -0.01% | -0.02% | 102% | 101% |
| Class D | -0.02% | 0.32% | -0.15% | -0.01% | 100% | 100% |
| Class F | -0.01% | 0.12% | -0.08% | 0.01% | 98% | 100% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.12% | -0.01% | 100% | 102% |
| Class A2 | -0.01% | 0.34% | 0.06% | 0.02% | 99% | 102% |
| Class B | 0.00% | 0.20% | -0.13% | 0.00% | 100% | 100% |
| Class C | -0.04% | 0.16% | -0.05% | -0.03% | 99% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | -0.01% | 0.18% | -0.02% | 0.00% | 100% | 101% |
| Class D | 0.02% | 0.29% | 0.00% | 0.03% | 101% | 101% |
| Class F | 0.01% | 0.05% | 0.08% | 0.02% | 100% | 100% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.01% | 0.34% | -0.12% | 0.00% | 100% | 102% |
| Class C | -0.01% | 0.37% | -0.25% | 0.00% | 101% | 101% |
| Class E | 0.16% | -1.00% | 0.72% | 0.12% | 95% | 100% |
| Overall | 0.03% | 0.01% | 0.05% | 0.03% | 99% | 101% |
| Class D | -0.03% | 0.50% | -1.76% | -0.06% | 100% | 101% |
| Class F | -0.07% | -0.34% | -0.30% | -0.13% | 98% | 99% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.05% | 0.02% | -0.31% | -0.06% | 104% | 104% |
| Class C | -0.07% | 0.02% | -0.15% | -0.07% | 102% | 101% |
| Class E | -0.16% | -0.65% | 0.35% | -0.16% | 96% | 100% |
| Overall | -0.09% | -0.15% | -0.09% | -0.09% | 101% | 102% |
| Class D | 0.05% | 0.21% | -1.66% | 0.01% | 99% | 100% |
| Class F | -0.12% | -0.01% | -0.11% | -0.07% | 100% | 102% |

High QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.54% | 1.46% | 0.65% | 0.52% | 100% | 99% |
| Class A2 | 0.36% | 1.06% | 0.30% | 0.39% | 98% | 99% |
| Class B | 0.13% | 0.65% | 0.37% | 0.16% | 95% | 100% |
| Class C | -0.02% | 0.54% | 0.56% | 0.02% | 97% | 99% |
| Class E |  |  |  |  |  |  |
| Overall | 0.22% | 0.87% | 0.46% | 0.24% | 97% | 100% |
| Class D | 0.32% | 0.78% | 1.80% | 0.33% | 93% | 100% |
| Class F | -0.04% | 0.26% | 0.45% | -0.02% | 98% | 100% |

Low QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 82% | 84% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 48% | 48% |
| Class B | -0.01% | 0.05% | 0.04% | 0.01% | 100% | 100% |
| Class C | -0.03% | 0.06% | 0.01% | -0.01% | 101% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 83% | 84% |
| Class D | -0.03% | 0.13% | -0.11% | -0.02% | 99% | 101% |
| Class F | 0.15% | 0.04% | -0.02% | -0.32% | 99% | 100% |

## Experiment 2.1

Regular QP values:

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.01% | -0.09% | -0.08% | 0.00% | 103% | 100% |
| Class A2 | -0.01% | -0.01% | -0.03% | -0.01% | 101% | 101% |
| Class B | -0.02% | -0.07% | -0.02% | -0.02% | 99% | 101% |
| Class C | -0.03% | -0.04% | 0.02% | -0.03% | 102% | 100% |
| Class E | -0.06% | 0.01% | 0.07% | -0.05% | 93% | 98% |
| Overall | -0.02% | -0.04% | -0.01% | -0.02% | 100% | 100% |
| Class D | -0.06% | 0.12% | -0.06% | -0.04% | 97% | 98% |
| Class F | -0.04% | 0.04% | -0.14% | -0.02% | 100% | 101% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.04% | 0.18% | 0.17% | -0.02% | 100% | 102% |
| Class A2 | 0.01% | 0.18% | 0.05% | 0.03% | 100% | 101% |
| Class B | 0.00% | 0.27% | -0.07% | 0.00% | 99% | 100% |
| Class C | -0.06% | 0.06% | 0.01% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | -0.02% | 0.18% | 0.02% | -0.01% | 100% | 101% |
| Class D | -0.05% | 0.31% | -0.08% | -0.03% | 102% | 101% |
| Class F | -0.02% | 0.13% | 0.05% | 0.00% | 100% | 101% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.01% | 0.52% | -0.10% | 0.02% | 100% | 101% |
| Class C | -0.10% | 0.13% | -0.09% | -0.09% | 100% | 101% |
| Class E | 0.12% | 0.16% | 0.29% | 0.12% | 99% | 101% |
| Overall | -0.01% | 0.30% | 0.00% | 0.01% | 100% | 101% |
| Class D | -0.09% | -0.60% | -0.29% | -0.12% | 100% | 100% |
| Class F | -0.15% | -0.19% | -0.41% | -0.24% | 98% | 100% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.04% | 0.26% | -0.19% | -0.04% | 104% | 102% |
| Class C | -0.10% | -0.16% | 0.19% | -0.10% | 98% | 99% |
| Class E | -0.09% | 0.44% | 0.20% | -0.05% | 99% | 100% |
| Overall | -0.07% | 0.16% | 0.03% | -0.06% | 100% | 100% |
| Class D | -0.04% | -0.83% | -0.53% | -0.09% | 99% | 100% |
| Class F | -0.12% | 0.47% | -0.25% | -0.04% | 99% | 102% |

High QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 1.26% | 2.71% | 1.37% | 1.28% | 99% | 98% |
| Class A2 | 1.00% | 1.91% | 0.94% | 1.04% | 97% | 98% |
| Class B | 0.80% | 1.34% | 1.17% | 0.82% | 94% | 99% |
| Class C | 0.07% | 0.45% | 0.13% | 0.08% | 95% | 99% |
| Class E |  |  |  |  |  |  |
| Overall | 0.74% | 1.49% | 0.89% | 0.76% | 96% | 99% |
| Class D | 0.35% | 0.93% | 0.45% | 0.37% | 93% | 99% |
| Class F | 0.10% | 0.40% | 0.30% | 0.13% | 97% | 100% |

Low QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 68% | 70% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | 0.03% | 0.02% | 0.00% | 102% | 103% |
| Class C | -0.04% | 0.01% | 0.00% | -0.03% | 100% | 99% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.08% | -0.10% | -0.13% | -0.09% | 100% | 100% |
| Class F | 0.23% | 0.00% | 0.01% | -0.32% | 99% | 100% |

## Experiment 3.1.a: 8-bit init value, log space

Regular QP values:

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.02% | 0.02% | -0.07% | 0.01% | 99% | 101% |
| Class A2 | 0.00% | -0.03% | -0.06% | -0.01% | 101% | 101% |
| Class B | -0.01% | -0.03% | -0.01% | -0.01% | 98% | 101% |
| Class C | -0.02% | 0.05% | 0.05% | -0.02% | 104% | 101% |
| Class E | -0.03% | -0.05% | -0.11% | -0.04% | 102% | 100% |
| Overall | -0.01% | -0.01% | -0.03% | -0.01% | 101% | 101% |
| Class D | -0.05% | 0.24% | -0.08% | -0.03% | 100% | 100% |
| Class F | -0.02% | -0.04% | 0.02% | -0.01% | 99% | 101% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.01% | -0.07% | 0.17% | -0.01% | 101% | 102% |
| Class A2 | -0.02% | 0.26% | -0.08% | 0.00% | 101% | 102% |
| Class B | 0.00% | 0.12% | -0.11% | 0.00% | 100% | 101% |
| Class C | -0.06% | 0.01% | 0.00% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | -0.02% | 0.08% | -0.02% | -0.02% | 100% | 101% |
| Class D | -0.02% | 0.30% | 0.21% | 0.00% | 101% | 100% |
| Class F | 0.00% | 0.17% | 0.24% | 0.01% | 99% | 101% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.01% | 0.68% | 0.12% | 0.02% | 101% | 102% |
| Class C | -0.08% | 0.46% | -0.19% | -0.06% | 100% | 101% |
| Class E | 0.08% | 0.01% | 0.02% | 0.08% | 98% | 102% |
| Overall | -0.01% | 0.44% | -0.01% | 0.01% | 100% | 102% |
| Class D | -0.11% | -0.44% | -0.05% | -0.12% | 102% | 101% |
| Class F | -0.11% | -0.07% | -0.45% | -0.15% | 99% | 101% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.05% | 0.32% | -0.13% | -0.04% | 105% | 103% |
| Class C | -0.07% | 0.06% | 0.25% | -0.05% | 102% | 103% |
| Class E | 0.08% | -0.69% | 0.42% | 0.06% | 97% | 101% |
| Overall | -0.02% | -0.02% | 0.14% | -0.02% | 102% | 102% |
| Class D | -0.07% | -0.01% | 0.04% | -0.06% | 100% | 101% |
| Class F | -0.09% | 0.16% | 0.05% | 0.03% | 100% | 103% |

High QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.44% | 1.60% | 0.82% | 0.52% | 101% | 100% |
| Class A2 | 0.41% | 0.87% | 0.43% | 0.42% | 99% | 101% |
| Class B | 0.50% | 1.11% | 0.67% | 0.53% | 99% | 100% |
| Class C | -0.05% | 0.30% | 0.28% | -0.03% | 100% | 101% |
| Class E |  |  |  |  |  |  |
| Overall | 0.33% | 0.94% | 0.55% | 0.36% | 100% | 100% |
| Class D | 0.29% | 0.89% | 0.78% | 0.28% | 97% | 99% |
| Class F | 0.15% | 0.17% | 0.39% | 0.17% | 100% | 100% |

Low QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.01% | 0.06% | 0.04% | 0.02% | 101% | 101% |
| Class A2 | -0.02% | 0.01% | 0.01% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.01% | 0.02% | 0.00% | 102% | 103% |
| Class C | -0.01% | -0.01% | 0.03% | -0.01% | 101% | 102% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.03% | 0.00% | 101% | 102% |
| Class D | -0.02% | -0.07% | 0.00% | -0.03% | 101% | 101% |
| Class F | 0.22% | 0.01% | 0.00% | -0.30% | 101% | 102% |

## Experiment 3.1.b: 8-bit init value, log space

Regular QP values:

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.02% | 0.06% | -0.02% | 0.02% | 101% | 100% |
| Class A2 | 0.00% | 0.06% | 0.03% | 0.01% | 100% | 101% |
| Class B | 0.00% | 0.08% | 0.10% | 0.01% | 98% | 100% |
| Class C | -0.01% | 0.00% | 0.21% | 0.01% | 101% | 97% |
| Class E | -0.02% | -0.05% | 0.06% | -0.02% | 98% | 95% |
| Overall | 0.00% | 0.03% | 0.09% | 0.01% | 99% | 99% |
| Class D | -0.01% | 0.23% | 0.14% | 0.03% | 93% | 96% |
| Class F | 0.01% | 0.13% | 0.00% | 0.02% | 99% | 99% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.03% | 0.08% | 0.27% | -0.01% | 100% | 102% |
| Class A2 | -0.02% | 0.42% | 0.13% | 0.02% | 100% | 102% |
| Class B | 0.03% | 0.36% | 0.02% | 0.05% | 97% | 98% |
| Class C | -0.01% | 0.23% | 0.00% | 0.00% | 99% | 98% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.28% | 0.09% | 0.02% | 99% | 100% |
| Class D | 0.04% | 0.72% | 0.49% | 0.09% | 101% | 100% |
| Class F | 0.07% | 0.35% | 0.29% | 0.09% | 98% | 98% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.01% | 0.80% | 0.31% | 0.05% | 99% | 102% |
| Class C | -0.01% | 0.67% | -0.06% | 0.03% | 98% | 102% |
| Class E | 0.02% | 1.37% | 1.50% | 0.15% | 96% | 101% |
| Overall | 0.00% | 0.90% | 0.48% | 0.07% | 98% | 102% |
| Class D | 0.06% | 0.27% | 0.14% | 0.06% | 99% | 96% |
| Class F | 0.00% | 0.27% | -0.31% | -0.02% | 98% | 99% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.03% | 0.67% | -0.31% | -0.01% | 103% | 102% |
| Class C | -0.02% | 0.39% | 0.21% | 0.01% | 99% | 100% |
| Class E | -0.10% | 1.56% | 1.27% | 0.05% | 97% | 100% |
| Overall | -0.04% | 0.80% | 0.26% | 0.01% | 100% | 101% |
| Class D | 0.08% | 0.41% | -0.38% | 0.07% | 99% | 97% |
| Class F | 0.01% | 0.56% | -0.04% | 0.10% | 99% | 101% |

High QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.58% | 1.70% | 0.93% | 0.60% | 101% | 99% |
| Class A2 | 0.47% | 1.08% | 0.28% | 0.48% | 100% | 99% |
| Class B | 0.88% | 1.61% | 1.38% | 0.91% | 101% | 100% |
| Class C | 0.55% | 1.13% | 0.59% | 0.55% | 103% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | 0.65% | 1.39% | 0.86% | 0.67% | 101% | 99% |
| Class D | 1.06% | 2.04% | 1.40% | 1.06% | 99% | 100% |
| Class F | 0.50% | 0.93% | 0.30% | 0.52% | 102% | 100% |

Low QP values:

|  |  |  | Random Access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.06% | 0.02% | 102% | 101% |
| Class A2 | 0.00% | 0.03% | 0.02% | 0.01% | 100% | 102% |
| Class B | 0.01% | 0.04% | 0.04% | 0.01% | 104% | 103% |
| Class C | 0.02% | 0.06% | 0.04% | 0.02% | 100% | 101% |
| Class E |  |  |  |  |  |  |
| Overall | 0.01% | 0.05% | 0.04% | 0.02% | 102% | 102% |
| Class D | 0.10% | 0.14% | 0.09% | 0.09% | 100% | 100% |
| Class F | 0.12% | 0.07% | 0.10% | -0.24% | 100% | 102% |

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft text for experiments 3.1

The draft text below is applicable to experiments 3.1.a. and 3.1.b. In the 3.1.b case, values for initValue are constrained such that slopeIdx and offsetIdx are even numbers.

The following describes changes to section 9.5.2.2 of the draft VVC specification.

For each context variable, the two variables pStateIdx0 and pStateIdx1 are initialized as follows:

- Table 9-6 to Table X-X contain the values of the 8 bit variable initValue used in the initialization of context variables that are assigned to all syntax elements in subclauses 7.3.7.1 through 7.3.7.11, except end_of_brick_one_bit and pcm_flag.

- From the 8 bit table entry initValue, the two 4 bit variables slopeIdx and offsetIdx are derived as follows:

slopeIdx = initValue  >>  4
offsetIdx = initValue & 15	(9-6)

- The variables m and n, used in the initialization of context variables, are derived from slopeIdx and offsetIdx as follows:

m = slopeIdx − 8* 5 − 45
n = ( offsetIdx  − 8<<  3 ) * 25− 16	(9-7)

- The two values assigned to pStateIdx0 and pStateIdx1 for the initialization are derived from SliceQpY, which is derived in Equation 7-60. Given the variables m and n, the initialization is specified as follows:

preCtxState = Clip3( 0, 127, ( ( m * Clip3( 0, 51, SliceQpY ) )  >>  4 ) + n
mps = preCtxState < 0 ? 0 : 1
preCtxState =  3 * min( 255, mps ? preCtxState : − preCtxState − 1 )
state = ( ( 255 – ( preCtxStateIdx & 127 ) ) * 32 ) >> ( preCtxStateIdx >> 7 )
state = mps ? 16383 – p : p	(9-8)

- The two values assigned to pStateIdx0 and pStateIdx1 for the initialization are derived as follows from initial state mapping table initStateIdxToState[ preCtxState ] specified in Table 9-4:

pStateIdx0 = stateinitStateIdxToState[preCtxState] >> 4
pStateIdx1 = stateinitStateIdxToState[preCtxState]	(9-9)

NOTE 1 – The variables pStateIdx0 and pStateIdx1 correspond to probability state indices as further described in subclause 9.5.4.3.

In Table 9-5, the ctxIdxOffset for which initialization is needed for each of the three initialization types, specified by the variable initType, are listed. Also listed is the table number that includes the values of initValue needed for the initialization for each value of ctxIdxInc. For P and B slice types, the derivation of initType depends on the value of the cabac_init_flag syntax element. The variable initType is derived as follows:

if( slice_type  = =  I )
	initType = 0
else if( slice_type  = =  P )
	initType = cabac_init_flag ? 2 : 1	(9-10)
else
	initType = cabac_init_flag ? 1 : 2

Table 9-4 – Initial state mapping table initStateIdxToState[ preCtxState ]

| preCtxState | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| initStateIdxToState | 307 | 323 | 340 | 359 | 378 | 398 | 419 | 442 | 466 | 491 | 517 | 544 | 574 | 604 | 637 | 671 |
| preCtxState | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| initStateIdxToState | 707 | 745 | 784 | 826 | 871 | 917 | 966 | 1018 | 1073 | 1130 | 1191 | 1254 | 1321 | 1392 | 1467 | 1545 |
| preCtxState | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| initStateIdxToState | 1628 | 1715 | 1807 | 1903 | 2005 | 2112 | 2226 | 2345 | 2470 | 2602 | 2741 | 2888 | 3043 | 3206 | 3377 | 3558 |
| preCtxState | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| initStateIdxToState | 3748 | 3949 | 4160 | 4383 | 4617 | 4864 | 5124 | 5399 | 5687 | 5992 | 6312 | 6650 | 7006 | 7381 | 7775 | 8192 |
| preCtxState | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| initStateIdxToState | 8192 | 8608 | 9002 | 9377 | 9733 | 10071 | 10391 | 10696 | 10984 | 11259 | 11519 | 11766 | 12000 | 12223 | 12434 | 12635 |
| preCtxState | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
| initStateIdxToState | 12825 | 13006 | 13177 | 13340 | 13495 | 13642 | 13781 | 13913 | 14038 | 14157 | 14271 | 14378 | 14480 | 14576 | 14668 | 14755 |
| preCtxState | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
| initStateIdxToState | 14838 | 14916 | 14991 | 15062 | 15129 | 15192 | 15253 | 15310 | 15365 | 15417 | 15466 | 15512 | 15557 | 15599 | 15638 | 15676 |
| preCtxState | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |
| initStateIdxToState | 15712 | 15746 | 15779 | 15809 | 15839 | 15866 | 15892 | 15917 | 15941 | 15964 | 15985 | 16005 | 16024 | 16043 | 16060 | 16076 |