| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0117 |
| --- | --- |

| Title: | CE8-1.4 and CE8-1.5: Default Processing for IBC Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu Han, Wei-Jung Chien, Marta Karczewicz (Qualcomm) Jingya Li, Che-Wei Kuo Chong Soon Lim (Panasonic) Hyeongmun Jang, Junghak Nam Jaehyun Lim (LGE) | Tel: Email: | +1-858-845-1793 yuhan@qti.qualcomm.com wchien@qti.qualcomm.com jingya.li@sg.panasonic.com hm.jang@lge.com |
| Source: | Qualcomm, Panasonic and LGE |  |  |

_____________________________

# Abstract

This contribution presents the tests results of “CE8-1.4: Default IBC mode in case of invalid vector” which use a default processing method for IBC mode to prevent un-defined decoder behavior, “CE8.1.5a: CE8-1.4a + CU based Chroma BV” and “CE8-1.5b: CE8-1.4 + Subblock based Chroma BV with default BV”. “CE8-1.4: Default IBC mode in case of invalid vector” didn’t change the mode decision in the encoder side and achieved the same results as anchor. The simulation results of “CE8.1.5a: CE8-1.4a + CU based Chroma BV” shows 0.16% gain in AI case, 0.01% loss in RA case and 0.00% gain in LB case in screen content coding materials. “CE8-1.5b: CE8-1.4 + Subblock based Chroma BV with default BV” shows 0.33% gain in AI case, 0.14% gain in RA case and 0.06% loss in LB case.

# Introduction

In the 13th JVET meeting, IBC mode as an independent mode was adopted into VTM4.0. In the current design of IBC mode, there are 3 encoder constraints as below:

- The IBC merge list may have zero BV. Therefore, encoder can only signal valid merge index.

- The encoder can only signal valid block vector whether it is skip/merge mode or AMVP mode.

- The encoder can only signal chroma IBC mode when collocated luma blocks are all coded with IBC mode and the block vectors are all valid.

These encoder constraints are heavily relied on the quality of the encoder. It is desirable to design a default processing method for IBC mode in order to prevent un-defined decoder behavior.

# Technical description

## CE8-1.4: Default IBC mode in case of invalid vector

In this method, a default processing method for IBC mode is designed which uses a default value of 1 << (internalBitDepth - 1) as the prediction samples.

In VTM5.0, for the constraint of invalid IBC merge index, the size of IBC merge list is set to be fixed to 6. If the available IBC merge candidates less than 6, fill the merge list with zero block vectors. At the encoder, zero block vector candidates will not be tested. Zero block vectors and other invalid block vectors may be signaled to the decoder. The decoder first examines the validity of the signal block vectors. If a block vector is not valid, the default value, 1 << (internalBitDepth - 1), is used to fill the prediction values.

## CE8.1.5a: CE8-1.4a + CU based Chroma BV

In VTM5.0, chroma IBC mode is sub-block based, each 2x2 chroma block inherits block vector from its collocated luma block. The encoder must check all of the prediction mode and block vectors of the collocated luma blocks. Only when collocated luma blocks are all coded with IBC mode and the block vectors are all valid, the chroma block can coded in IBC mode. In this test, CU based chroma IBC mode is presented to reduce the complexity. In this method, the whole chroma CU uses one block vector which is inherited from the collocated luma block using the center position of the chroma CU. If this block vector is invalid, the default value, 1 << (internalBitDepth - 1), is used to fill the prediction values.

## CE8-1.5b: CE8-1.4 + Subblock based Chroma BV with default BV

In the current VVC reference software VTM-5.0, IBC chroma Rate-Optimization is applied when the entire corresponding luma block of current chroma block is IBC coded. When the luma block is partially IBC coded or none IBC coded, the Chroma IBC Rate-Optimization is not applied and ibc_pred_mode is set as 0. However current VVC WD allows Chroma IBC mode despite of that the corresponding luma block is partially IBC coded. It causes a corner case which is corresponding luma block with being intra coded. In this case, current chroma subblock derives motion vector as (0, 0), predFlagL0 and predFlagL1 as 0 also. Using this information, current chroma IBC sub-block cannot derive prediction sample. In order to handle the corner case, the chroma block with corresponding luma block being coded with intra uses a default IBC motion vector. The default IBC motion vector is set to the IBC motion vector of center position in the corresponding luma block. In encoder side, only if the center position of corresponding luma block is IBC coded, the chroma block allows Rate-Optimization process for IBC mode. Based on CE8-1.4, if a block vector is not valid, the default value, 1 << (internalBitDepth - 1), is used to fill the prediction values.

# Simulation results

The simulation is based on VTM5.0 software. The anchor is generated by using VTM5.0 default configurations with IBC mode. For the test, IBC is enabled for class A to E, class F and class TGM followed the common test conditions for Class F. Results are shown in the following tables:

## CE8-1.4: Default IBC mode in case of invalid vector

In this test, mode decision didn’t change. The test results of class F and class TGM are the same as anchor.

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.01% | 0.08% | 139% | 99% |
| Class A2 | -0.78% | -0.99% | -1.00% | 143% | 100% |
| Class B | -0.79% | -0.72% | -0.82% | 153% | 100% |
| Class C | -0.27% | -0.30% | -0.21% | 151% | 97% |
| Class E | -1.25% | -1.15% | -1.34% | 156% | 100% |
| Overall | -0.62% | -0.62% | -0.65% | 149% | 99% |
| Class D | -0.54% | -0.53% | -0.67% | 145% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class SCC 1080p | 0.00% | 0.00% | 0.00% | 99% | 99% |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.24% | 0.11% | 103% | 99% |
| Class A2 | -0.07% | -0.40% | -0.09% | 104% | 99% |
| Class B | -0.10% | -0.40% | -0.62% | 107% | 100% |
| Class C | 0.14% | -0.23% | -0.18% | 111% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.32% | -0.25% | 107% | 99% |
| Class D | 0.10% | -0.17% | 0.18% | 112% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class SCC 1080p | 0.00% | 0.00% | 0.00% | 98% | 98% |

|  | Low delay B Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.36% | -0.41% | 115% | 96% |
| Class C | 0.07% | -0.03% | -0.37% | 123% | 98% |
| Class E | -0.04% | -0.69% | -0.19% | 111% | 98% |
| Overall | 0.03% | -0.03% | -0.34% | 117% | 97% |
| Class D | 0.06% | 0.06% | 0.66% | 127% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class SCC 1080p | 0.00% | 0.00% | 0.00% | 98% | 97% |

## CE8.1.5a: CE8-1.4a + CU based Chroma BV

Based on CE8-1.4, in this test, for IBC chroma mode, the whole chroma CU use one block vector which is inherited from the collocated luma block using the center position of the chroma CU. If the inherited block vector of the chroma block is invalid, use the default value 1 << (internalBitDepth - 1) as prediction.

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.00% | 0.06% | 140% | 99% |
| Class A2 | -0.82% | -1.05% | -0.99% | 143% | 99% |
| Class B | -0.81% | -0.79% | -0.92% | 154% | 100% |
| Class C | -0.29% | -0.35% | -0.31% | 151% | 97% |
| Class E | -1.23% | -1.25% | -1.49% | 157% | 99% |
| Overall | -0.64% | -0.68% | -0.73% | 150% | 99% |
| Class D | -0.54% | -0.52% | -0.66% | 147% | 102% |
| Class F | -0.04% | -0.07% | -0.06% | 101% | 99% |
| Class SCC 1080p | -0.16% | -0.21% | -0.19% | 98% | 99% |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | -0.15% | 0.21% | 104% | 99% |
| Class A2 | -0.09% | -0.26% | -0.21% | 105% | 98% |
| Class B | -0.14% | -0.60% | -0.72% | 106% | 100% |
| Class C | 0.14% | -0.36% | -0.19% | 112% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.38% | -0.29% | 107% | 99% |
| Class D | 0.07% | -0.32% | 0.12% | 112% | 102% |
| Class F | 0.00% | -0.10% | -0.02% | 98% | 97% |
| Class SCC 1080p | 0.01% | -0.06% | -0.11% | 98% | 97% |

|  | Low delay B Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.19% | -0.06% | 115% | 95% |
| Class C | 0.12% | 0.22% | -0.14% | 124% | 102% |
| Class E | -0.05% | 0.15% | -0.43% | 111% | 97% |
| Overall | 0.04% | 0.19% | -0.18% | 117% | 98% |
| Class D | 0.10% | -0.09% | 0.56% | 126% | 102% |
| Class F | 0.00% | 0.86% | 0.07% | 98% | 95% |
| Class SCC 1080p | 0.00% | -0.09% | -0.09% | 98% | 98% |

## CE8-1.5b: CE8-1.4 + Subblock based Chroma BV with default BV

Based on CE8-1.4, in this test, the chroma block with corresponding luma block being coded with intra uses a default IBC motion vector. The default IBC motion vector is set to the IBC motion vector of center position in the corresponding luma block.

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.01% | 0.05% | 141% | 100% |
| Class A2 | -0.82% | -1.04% | -0.97% | 146% | 101% |
| Class B | -0.80% | -0.78% | -0.87% | 154% | 99% |
| Class C | -0.30% | -0.32% | -0.31% | 152% | 96% |
| Class E | -1.23% | -1.25% | -1.43% | 156% | 98% |
| Overall | -0.64% | -0.67% | -0.70% | 150% | 99% |
| Class D | -0.53% | -0.55% | -0.68% | 145% | 99% |
| Class F | -0.06% | -0.13% | -0.14% | 101% | 98% |
| Class SCC 1080p | -0.33% | -0.43% | -0.47% | 99% | 98% |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.17% | 0.19% | 103% | 99% |
| Class A2 | -0.07% | -0.28% | -0.10% | 105% | 98% |
| Class B | -0.13% | -0.44% | -0.70% | 106% | 99% |
| Class C | 0.15% | -0.27% | -0.25% | 111% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.31% | -0.28% | 106% | 99% |
| Class D | 0.09% | -0.37% | -0.05% | 112% | 101% |
| Class F | -0.08% | -0.09% | -0.07% | 98% | 97% |
| Class SCC 1080p | -0.14% | -0.20% | -0.27% | 98% | 97% |

|  | Low delay B Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.26% | -0.23% | 116% | 96% |
| Class C | 0.12% | 0.30% | -0.05% | 124% | 100% |
| Class E | 0.02% | -0.19% | 0.45% | 110% | 97% |
| Overall | 0.06% | 0.16% | 0.00% | 117% | 97% |
| Class D | 0.06% | 0.08% | 0.71% | 126% | 102% |
| Class F | 0.04% | -0.57% | -0.34% | 99% | 97% |
| Class SCC 1080p | -0.06% | -0.11% | -0.10% | 97% | 95% |

# Conclusions

This contribution presents the tests results of “CE8-1.4: Default IBC mode in case of invalid vector” which use a default processing method for IBC mode to prevent un-defined decoder behavior, “CE8.1.5a: CE8-1.4a + CU based Chroma BV” and “CE8-1.5b: CE8-1.4 + Subblock based Chroma BV with default BV”. “CE8-1.4: Default IBC mode in case of invalid vector” didn’t change the mode decision in the encoder side and achieved the same results as anchor. The simulation results of “CE8.1.5a: CE8-1.4a + CU based Chroma BV” shows 0.16% gain in AI case, 0.01% loss in RA case and 0.00% gain in LB case in screen content coding materials. “CE8-1.5b: CE8-1.4 + Subblock based Chroma BV with default BV” shows 0.33% gain in AI case, 0.14% gain in RA case and 0.06% loss in LB case.

# References

- Y. Han, W.-J. Chien, M. Karczewicz, JVET-N0316, “CE8-related: Default Processing for IBC Mode”, Geneva, March 2019.

- J. Li, C.-W. Kuo, C. Lim, JVET-N0201, “Non-CE8: IBC modifications”, Geneva, March 2019.

- H. Jang, J. Nam, J. Lim, JVET-N0466, “CE8-related: The corner case handling regarding mv derivation for Chroma IBC in dual tree structure”, Geneva, March 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).