| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0117 |
| --- | --- |

| Title: | Suggested Draft Text changes for JVET-O0117 |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu Han, Wei-Jung Chien, Marta Karczewicz (Qualcomm) Jingya Li, Che-Wei Kuo Chong Soon Lim (Panasonic) Hyeongmun Jang, Junghak Nam Jaehyun Lim (LGE) | Tel: Email: | +1-858-845-1793 yuhan@qti.qualcomm.com wchien@qti.qualcomm.com jingya.li@sg.panasonic.com hm.jang@lge.com |
| Source: | Qualcomm, Panasonic and LGE |  |  |

_____________________________

# Abstract

This document describes the suggested specification changes on top of version 8 of JVET-N1001. The newly added parts are highlighted in yellow and the text to be removed is marked as red with strikethrough.

## CE8-1.4: Default IBC mode in case of invalid vector

## Decoding process for coding units coded in IBC prediction mode

### General decoding process for coding units coded in IBC prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

Output of this process is a modified reconstructed picture before in-loop filtering.

The derivation process for quantization parameters as specified in clause 8.7.1 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight, and the variable treeType as inputs.

The decoding process for coding units coded in ibc prediction mode consists of the following ordered steps:

- The motion vector components of the current coding unit are derived as follows:

    - If treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the following applies:

- The derivation process for motion vector components as specified in clause 8.6.2.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vector mvL[ 0 ][ 0 ] as output.

- When treeType is equal to SINGLE_TREE, the derivation process for chroma motion vectors in clause 8.6.2.5 is invoked with luma motion vector mvL[ 0 ][ 0 ] as input, and chroma motion vector mvC[ 0 ][ 0 ] as output.

- The number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY are both set equal to 1.

    - Otherwise, if treeType is equal to DUAL_TREE_CHROMA, the following applies:

- The number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY are derived as follows:

numSbX = ( cbWidth >> 2 )		(8-873)

numSbY = ( cbHeight >> 2 )		(8-874)

- The chroma motion vectors mvC[ xSbIdx ][ ySbIdx ] are derived as follows for xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1:

- The luma motion vector mvL[ xSbIdx ][ ySbIdx ] is derived as follows:

- The location ( xCuY, yCuY ) of the collocated luma coding unit is dervied as follows:

xCuY = xCb + xSbIdx*4		 	(8-875)

yCuY = yCb + ySbIdx*4		 	(8-876)

- If CuPredMode[ xCuY ][ yCuY ] is equal to MODE_INTRA, the following applies.

mvL[ xSbIdx ][ ySbIdx ][ 0 ] = 0	(8-877)

mvL[ xSbIdx ][ ySbIdx ][ 1 ] = 0	(8-878)

predFlagL0[ xSbIdx ][ ySbIdx ] = 0	(8-879)

predFlagL1[ xSbIdx ][ ySbIdx ] = 0	(8-880)

- Otherwise ( CuPredMode[ xCuY ][ yCuY ] is equal to MODE_IBC ), the following applies:

mvL[ xSbIdx ][ ySbIdx ][ 0 ]=MvL0[ xCuY ][ yCuY ][ 0 ] 	(8-881)

mvL[ xSbIdx ][ ySbIdx ][ 1 ]=MvL0[ xCuY ][ yCuY ][ 1 ] 	(8-882)

predFlagL0[ xSbIdx ][ ySbIdx ] = 1	(8-883)

predFlagL1[ xSbIdx ][ ySbIdx ] = 0	(8-884)

- The derivation process for chroma motion vectors in clause 8.6.2.5 is invoked with mvL[ xSbIdx ][ ySbIdx ] as inputs, and mvC[ xSbIdx ][ ySbIdx ] as output.

- It is a requirement of bitstream conformance that the chroma motion vector mvC[ xSbIdx ][ ySbIdx ] shall obey the following constraints:

- If the chroma motion vector mvC[ xSbIdx ][ ySbIdx ] doesn’t obey one of the following constrains, set defautIbcModeChroma[xSbIdx][ySbIdx] to be equal to true, apply IBC_Default_Mode for IBC coding block.

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( xCb / SubWidthC + ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ), yCb / SubHeightC + ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) ) as inputs, the output shall be equal to TRUE.

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( xCb / SubWidthC + ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ) + cbWidth / SubWidthC − 1, yCb / SubHeightC + ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) + cbHeight / SubHeightC − 1 ) as inputs, the output shall be equal to TRUE.

- One or both of the following conditions shall be true:

- ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ) + xSbIdx * 2 + 2 is less than or equal to 0.

- ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) + ySbIdx * 2 + 2 is less than or equal to 0.

- The prediction samples of the current coding unit are derived as follows:

- If  treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the prediction samples of the current coding unit are derived as follows:

      - The decoding process for ibc blocks as specified in clause 8.6.3.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the luma motion vectors mvL[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the variable cIdx set equal to 0 as inputs, and the ibc prediction samples (predSamples) that are an (cbWidth)x(cbHeight) array predSamplesL of prediction luma samples as outputs.

- Otherwise if treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the prediction samples of the current coding unit are derived as follows:

      - The decoding process ibc blocks as specified in clause 8.6.3.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the chroma motion vectors mvC[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1 and the variable cIdx set equal to 1 as inputs, and the ibc prediction samples (predSamples) that are an (cbWidth / 2)x(cbHeight / 2) array predSamplesCb of prediction chroma samples for the chroma components Cb as outputs.

      - The decoding process for ibc blocks as specified in clause 8.6.3.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the chroma motion vectors mvC[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1 and the variable cIdx set equal to 2 as inputs, and the ibc prediction samples (predSamples) that are an (cbWidth / 2)x(cbHeight / 2) array predSamplesCr of prediction chroma samples for the chroma components Cr as outputs.

- The variables NumSbX[ xCb ][ yCb ] and NumSbY[ xCb ][ yCb ] are set equal to numSbX and numSbY, respectively.

- The residual samples of the current coding unit are derived as follows:

- When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, the decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.5.8 is invoked with the location ( xTb0, yTb0 ) set equal to the luma location ( xCb, yCb ), the width nTbW set equal to the luma coding block width cbWidth, the height nTbH set equal to the luma coding block height cbHeight and the variable cIdxset equal to 0 as inputs, and the array resSamplesL as output.

- When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, the decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.5.8 is invoked with the location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / 2, yCb / 2 ), the width nTbW set equal to the chroma coding block width cbWidth / 2, the height nTbH set equal to the chroma coding block height cbHeight / 2 and the variable cIdxset equal to 1 as inputs, and the array resSamplesCb as output.

- When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, the decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.5.8 is invoked with the location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / 2, yCb / 2 ), the width nTbW set equal to the chroma coding block width cbWidth / 2, the height nTbH set equal to the chroma coding block height cbHeight / 2 and the variable cIdxset equal to 2 as inputs, and the array resSamplesCr as output.

- The reconstructed samples of the current coding unit are derived as follows:

- When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, the picture reconstruction process for a colour component as specified in clause 8.7.5 is invoked with the block location ( xB, yB ) set equal to ( xCb, yCb ), the block width bWidth set equal to cbWidth, the block height bHeight set equal to cbHeight, the variable cIdx set equal to 0, the (cbWidth)x(cbHeight) array predSamples set equal to predSamplesL and the (cbWidth)x(cbHeight) array resSamples set equal to resSamplesL as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, the picture reconstruction process for a colour component as specified in clause 8.7.5 is invoked with the block location ( xB, yB ) set equal to ( xCb / 2, yCb / 2 ), the block width bWidth set equal to cbWidth / 2, the block height bHeight set equal to cbHeight / 2, the variable cIdx set equal to 1, the (cbWidth / 2)x(cbHeight / 2) array predSamples set equal to predSamplesCb and the (cbWidth / 2)x(cbHeight / 2) array resSamples set equal to resSamplesCb as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, the picture reconstruction process for a colour component as specified in clause 8.7.5 is invoked with the block location ( xB, yB ) set equal to ( xCb / 2, yCb / 2 ), the block width bWidth set equal to cbWidth / 2, the block height bHeight set equal to cbHeight / 2, the variable cIdx set equal to 2, the (cbWidth / 2)x(cbHeight / 2) array predSamples set equal to predSamplesCr and the (cbWidth / 2)x(cbHeight / 2) array resSamples set equal to resSamplesCr as inputs, and the output is a modified reconstructed picture before in-loop filtering.

### Derivation process for motion vector components for IBC blocks

#### General

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vector in 1/16 fractional-sample accuracy mvL.

The luma motion vector mvL is derived as follows:

- The derivation process for IBC luma motion vector prediction as specified in clause 8.6.2.2 is invoked with the luma location ( xCb, yCb ), the variables cbWidth and cbHeight inputs, and the output being the luma motion vector mvL.

- When general_merge_flag[ xCb ][ yCb ] is equal to 0, the following applies:

- The variable mvd is derived as follows:

mvd[ 0 ] = MvdL0[ xCb ][ yCb ][ 0 ]		(8-885)

mvd[ 1 ] = MvdL0[ xCb ][ yCb ][ 1 ]		(8-886)

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to mvL, rightShift set equal to MvShift + 2, and leftShift set equal to MvShift + 2 as inputs and the rounded mvL as output.

- The luma motion vector mvL is modified as follows:

u[ 0 ] = ( mvL[ 0 ] + mvd[ 0 ] + 218 ) % 218		(8-887)

mvL[ 0 ] = ( u[ 0 ]  >=  217 ) ? ( u[ 0 ] − 218 ) : u[ 0 ]	(8-888)

u[ 1 ] = ( mvL[ 1 ] + mvd[ 1 ] + 218 ) % 218		(8-889)

mvL[ 1 ] = ( u[ 1 ]  >=  217 ) ? ( u[ 1 ] − 218 ) : u[ 1 ]	(8-890)

NOTE 1– The resulting values of mvL[ 0 ] and mvL[ 1 ] as specified above will always be in the range of −217 to 217 − 1, inclusive.

The updating process for the history-based motion vector predictor list as specified in clause 8.6.2.6 is invoked with luma motion vector mvL.

The top-left location inside the reference block ( xRefTL, yRefTL ) and the bottom-right location inside the reference block ( xRefBR, yRefBR ) are derived as follows:

( xRefTL, yRefTL ) = ( xCb + ( mvL[ 0 ] >> 4 ), yCb + ( mvL[ 1 ] >> 4 ) ) 	(8-891)

( xRefBR, yRefBR ) = ( xRefTL + cbWidth − 1, yRefTL + cbHeight − 1 ) 	(8-892)

It is a requirement of bitstream conformance that the luma motion vector mvL shall obey the following constraints:

If the luma motion vector mvL doesn’t obey one of the following constrains, set defautIbcModeLuma to be equal to true, apply IBC_Default_Mode for IBC coding block.

- The motion vector mvL shall point to a reference block that is available, i.e. for ( xRefTL, yRefTL ) and ( xRefBR, yRefBR ), all of the following conditions shall be true:

  - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xRefTL, yRefTL ) as inputs, the output shall be equal to TRUE.

  - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xRefBR, yRefBR ) as inputs, the output shall be equal to TRUE.

- The motion vector mvL shall point to a reference block that is outside (left and/or above) and not overlapping with the current coding bock, i.e. one or both the following conditions shall be true:

  - The value of ( mvL[ 0 ] >> 4 ) + cbWidth is less than or equal to 0.

  - The value of ( mvL[ 1 ] >> 4 ) + cbHeight is less than or equal to 0.

- The motion vector mvL shall point to a reference block that is fully contained in the same coding tree block as the current block or fully contained in a block to the left with the same height of the current coding tree block and a width equal to 128 luma samples, i.e. all of the following conditions shall be true:

yRefTL >> CtbLog2SizeY = yCb >> CtbLog2SizeY	(8-893)

yRefBR >> CtbLog2SizeY = yCb >> CtbLog2SizeY	(8-894)

xRefTL >> CtbLog2SizeY >= ( xCb >> CtbLog2SizeY ) + Min( 1, 7 − CtbLog2SizeY )	(8-895)
	− ( 1 << ( (7 − CtbLog2SizeY ) << 1) ) )

xRefBR >> CtbLog2SizeY <= ( xCb >> CtbLog2SizeY )	(8-896)

- When the motion vector mvL points to a reference block where the top-left sample is located inside the neighbouring coding tree block to the left, i.e. ( xRefTL ) >> CtbLog2SizeY is equal to ( xCb >> CtbLog2SizeY ) − 1, and CtbSizeY is equal to 128, the motion vector mvL shall not point to a reference block where the top-left sample is located left to or inside the same 64x64 area as the current block but inside the neighbouring coding tree block to the left, i.e. the following conditions shall be true:

  - The derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( ( xRefTL + 128 ) / 64 * 64, yRefTL / 64 * 64 ) as inputs, and the output shall be equal to FALSE.

  - The luma location ( ( xRefTL + 128 ) / 64 * 64, yRefTL / 64 * 64 ) shall not be equal to ( xCb, yCb ).

### Decoding process for ibc blocks

#### General

This process is invoked when decoding a coding unit coded in ibc prediction mode.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- variables numSbX and numSbY specifying the number of luma coding subblocks in horizontal and vertical direction,

- the motion vectors mv[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an array predSamples of prediction samples.

Let predSamplesL0L, and predSamplesL1L be (cbWidth)x(cbHeight) arrays of predicted luma sample values and, predSamplesL0Cb, predSamplesL1Cb, predSamplesL0Cr and predSamplesL1Cr be (cbWidth / 2)x(cbHeight / 2) arrays of predicted chroma sample values.

The reference indices refIdxL0 and refIdxL1 are set equal to −1.

The prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] are set equal to 1 and predFlagL1[ xSbIdx ][ ySbIdx ] are set equal to 0 with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

The bi-prediction weight index bcwIdx is set equal to 0,

The width and the height of the current coding sublock sbWidth and sbHeight in luma samples are derived as follows:

sbWidth  =  cbWidth / numSbX		(8-913)

sbHeight  =  cbHeight / numSbY		(8-914)

For each coding subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1, the following applies:

- The luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows:

( xSb, ySb )  =  ( xCb + xSbIdx * sbWidth, yCb + ySbIdx * sbHeight )	(8-915)

- The variable bdofFlag is set equal to FALSE.

The current decoded picture consists of a pic_width_in_luma_samples by pic_height_in_luma_samples array of luma samples currPicL and two PicWidthInSamplesC by PicHeightInSamplesC arrays of chroma samples currPicCb and currPicCr. The current decoded picture sample arrays currPicL, currPicCb and currPicCr correspond to decoded sample arrays SL, SCb and SCr derived in clause 8.8 for the current decoded picture.

- If cIdx is equal to 0, the following applies:

  - The array predSamplesL0L is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth, the coding subblock height sbHeight in luma samples, the motion vector offset mvOffset set equal to ( 0, 0 ), the luma motion vector mv[ xSb ][ ySb ], the reference array currPicL, bdofFlag, and cIdx as inputs.

- Otherwise if cIdx is equal to 1, the following applies:

  - The array predSamplesL0Cb is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth / 2, the coding subblock height sbHeight / 2, the motion vector offset mvOffset set equal to ( 0, 0 ), the chroma motion vector mv[ xSb ][ ySb ], the reference array currPicCb, bdofFlag, and cIdx as inputs.

- Otherwise (cIdx is equal to 2), the following applies:

  - The array predSamplesL0Cr is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth / 2, the coding subblock height sbHeight / 2, the motion vector offset mvOffset set equal to ( 0, 0 ), the chroma motion vector mv[ xSb ][ ySb ], the reference array currPicCr, bdofFlag, and cIdx as inputs.

- The array predSamples of prediction samples is derived as follows:

  - If defautIbcModeLuma is TRUE, the prediction samples inside the current luma coding subblock, predSamples[ xL + xSb ][ yL + ySb ] = 1 << (internalBitDepth - 1). Otherwise, if cIdx is equal to 0, the prediction samples inside the current luma coding subblock, predSamples[ xL + xSb ][ yL + ySb ] with xL = 0..sbWidth − 1 and yL = 0..sbHeight − 1, are derived by invoking the weighted sample prediction process as specified in clause 8.5.6.5 is invoked with the luma coding subblock width sbWidth, the luma coding subblock height sbHeight and the sample arrays predSamplesL0 L and predSamplesL1L, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, bcwIdx, and cIdx as inputs, and predSamples[ xL + xSb ][ yL + ySb ] as outputs.

- If defautIbcModeChroma[xSbIdx][ySbIdx] is TRUE, the prediction samples inside the current chroma component Cb coding block, predSamples [ xC + xCb / 2 ][ yC + yCb / 2 ] = 1 << (internalBitDepth - 1). Otherwise, if cIdx is equal to 1, the prediction samples inside the current chroma component Cb coding block, predSamples [ xC + xCb / 2 ][ yC + yCb / 2 ] with xC = 0..cbWidth / 2 − 1 and yC = 0..cbHeight / 2 − 1, are derived by invoking the weighted sample prediction process specified in clause 8.5.6.5 with the coding block width nCbW set equal to cbWidth / 2, the coding block height nCbH set equal to cbHeight / 2, the sample arrays predSamplesL0Cb and predSamplesL1Cb, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, bcwIdx, and cIdx as inputs.

- If defautIbcModeChroma[xSbIdx][ySbIdx] is TRUE, the prediction samples inside the current chroma component Cr coding block, predSamples [ xC + xCb / 2 ][ yC + yCb / 2 ] = 1 << (internalBitDepth - 1). Otherwise (cIdx is equal to 2), the prediction samples inside the current chroma component Cr coding block, predSamples[ xC + xCb / 2 ][ yC + yCb / 2 ] with xC = 0..cbWidth / 2 − 1 and yC = 0..cbHeight / 2 − 1, are derived by invoking the weighted sample prediction process specified in clause 8.5.6.5 with the coding block width nCbW set equal to cbWidth / 2, the coding block height nCbH set equal to cbHeight / 2, the sample arrays predSamplesL0Cr and predSamplesL1Cr, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, bcwIdx, and cIdx as inputs.

- When cIdx is equal to 0, the following assignments are made for x = 0..sbWidth − 1 and y = 0..sbHeight − 1:

MvL0[ xSb + x ][ ySb + y ] = mv[ xSbIdx ][ ySbIdx ]	(8-916)

MvL1[ xSb + x ][ ySb + y ] = 0		(8-917)

RefIdxL0[ xSb + x ][ ySb + y ] = refIdxL0		(8-918)

RefIdxL1[ xSb + x ][ ySb + y ] = refIdxL1		(8-919)

PredFlagL0[ xSb + x ][ ySb + y ] = predFlagL0[ xSbIdx ][ ySbIdx ]	(8-920)

PredFlagL1[ xSb + x ][ ySb + y ] = predFlagL1[ xSbIdx ][ ySbIdx ]	(8-921)

BcwIdx[ xSb + x ][ ySb + y ] = bcwIdx		(8-922)

## CE8.1.5a: CE8-1.4a + CU based Chroma BV

## Decoding process for coding units coded in IBC prediction mode

### General decoding process for coding units coded in IBC prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

Output of this process is a modified reconstructed picture before in-loop filtering.

The derivation process for quantization parameters as specified in clause 8.7.1 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight, and the variable treeType as inputs.

The decoding process for coding units coded in ibc prediction mode consists of the following ordered steps:

- The motion vector components of the current coding unit are derived as follows:

    - If treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the following applies:

- The derivation process for motion vector components as specified in clause 8.6.2.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vector mvL[ 0 ][ 0 ] as output.

- When treeType is equal to SINGLE_TREE, the derivation process for chroma motion vectors in clause 8.6.2.5 is invoked with luma motion vector mvL[ 0 ][ 0 ] as input, and chroma motion vector mvC[ 0 ][ 0 ] as output.

- The number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY are both set equal to 1.

    - Otherwise, if treeType is equal to DUAL_TREE_CHROMA, the following applies:

- The number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY are derived as follows:

numSbX = ( cbWidth >> 2 )		(8-873)

numSbY = ( cbHeight >> 2 )		(8-874)

- The chroma motion vectors mvC[ xSbIdx ][ ySbIdx ] are derived as follows for xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1:

- The luma motion vector mvL[ xSbIdx ][ ySbIdx ] is derived as follows:

- The location ( xCuY, yCuY ) of the collocated luma coding unit is dervied as follows:

xCuY = xCb + (cbHeight >> 1) xSbIdx*4		 	(8-875)

yCuY = yCb + (cbWidth >> 1)ySbIdx*4		 	(8-876)

- If CuPredMode[ xCuY ][ yCuY ] is equal to MODE_INTRA, the following applies.

mvL[ xSbIdx ][ ySbIdx ][ 0 ] = 0	(8-877)

mvL[ xSbIdx ][ ySbIdx ][ 1 ] = 0	(8-878)

predFlagL0[ xSbIdx ][ ySbIdx ] = 0	(8-879)

predFlagL1[ xSbIdx ][ ySbIdx ] = 0	(8-880)

- Otherwise ( CuPredMode[ xCuY ][ yCuY ] is equal to MODE_IBC ), the following applies:

mvL[ xSbIdx ][ ySbIdx ][ 0 ]=MvL0[ xCuY ][ yCuY ][ 0 ] 	(8-881)

mvL[ xSbIdx ][ ySbIdx ][ 1 ]=MvL0[ xCuY ][ yCuY ][ 1 ] 	(8-882)

predFlagL0[ xSbIdx ][ ySbIdx ] = 1	(8-883)

predFlagL1[ xSbIdx ][ ySbIdx ] = 0	(8-884)

- The derivation process for chroma motion vectors in clause 8.6.2.5 is invoked with mvL[ xSbIdx ][ ySbIdx ] as inputs, and mvC[ xSbIdx ][ ySbIdx ] as output.

- It is a requirement of bitstream conformance that the chroma motion vector mvC[ xSbIdx ][ ySbIdx ] shall obey the following constraints:

- If the chroma motion vector mvC doesn’t obey one of the following constrains, set defautIbcModeChroma to be equal to true, apply IBC_Default_Mode for IBC coding block.

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( xCb / SubWidthC + ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ), yCb / SubHeightC + ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) ) as inputs, the output shall be equal to TRUE.

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( xCb / SubWidthC + ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ) + cbWidth / SubWidthC − 1, yCb / SubHeightC + ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) + cbHeight / SubHeightC − 1 ) as inputs, the output shall be equal to TRUE.

- One or both of the following conditions shall be true:

- ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ) + xSbIdx * 2 + 2 is less than or equal to 0.

- ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) + ySbIdx * 2 + 2 is less than or equal to 0.