| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0119 |
| --- | --- |

| Title: | CE8-2.1: Palette mode in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Chao-Hsiung Hung, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz (Qualcomm) Yu-Chen Sun, Tsui-Shan Chang, Jian Lou (Alibaba) Weijia Zhu, Li Zhang, Jizheng Xu, Kai Zhang, Hongbin Liu, Yue Wang (Bytedance) | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com +1-858-349-2818 yuchen.s@alibaba-inc.com +1-360-5250535 weijia.zhu@bytedance.com |
| Source: | Qualcomm Incorporated Alibaba Cloud Computing Ltd Bytedance Inc. |  |  |

_____________________________

# Abstract

This document reports the results of CE test 8.2.1 on palette mode tested on top of VTM-5.0. The test included the syntax from palette in HEVC. The CE results show that:

For 4:2:0 test sequences,

- the proposed method provides 4.29%, 1.71%, and 0.43% BD-rate gains for luma in TGM sequences, with 102%, 102% and 104% in encoding time, and 102%, 102% and 102% in decoding time under AI, RA, and LD configurations, respectively.

- the proposed method provides 0.72%, 0.55%, and 0.28% BD-rate gains for luma in class F sequences, with 107%, 107% and 106% for encoding time, and 108%, 107% and 104% in decoding time under AI, RA, and LD configurations, respectively.

For 4:4:4 test sequences,

- when DualITree turned off, the proposed method achieves 16.41%, 10.64%, and 6.54% BD-rate gain in TGM 1080p sequences, and 5.76%, 4.15%, and 2.09% BD-rate gains overall for luma, with 102%, 99% and 102% in encoding time, and 97%, 97% and 98% in decoding time under AI, RA, and LD configurations, respectively.

- when DualITree turned on, the proposed method achieves 6.54%, 5.40%, and 5.43% BD-rate gain in TGM 1080p sequences, and 2.13%, 1.71%, and 1.79% BD-rate gains overall for luma with 106%, 100% and 103%  encoding time, and 98%, 99% and 98% in decoding time under AI, RA, and LD configurations, respectively.

# CE Description

The decoder syntax in this CE for palette table and indices parsing is the same as in HEVC palette. For palette mode signaling, the palette mode is coded as a prediction mode for a coding unit, i.e., the prediction modes for a coding unit can be MODE_INTRA, MODE_INTER, MODE_IBC and MODE_PLT [2]. If the palette mode is utilized, the pixels values in the CU are represented by a small set of representative colour values. The set is referred to as the palette. For pixels with values close to the palette colors, the palette indices are signalled. For pixels with values outside the palette, the pixel is denoted with an escape symbol and the quantized pixel values are signaled directly.

To decode a palette encoded block, the decoder needs to decode palette colors and indices. Palette colors are described by a palette table and encoded by palette table coding tools. An escape flag is signaled for each CU to indicate if escape symbols are present in the current CU. If escape symbols are present, the palette table is augmented by one and the last index is assigned to the escape mode. Palette indices of all pixels in a CU form a palette index map and are encoded by palette index map coding tools.

For coding of the palette table, a palette predictor is maintained. The predictor is initialized at the beginning of each slice where predictor is reset to 0. For each entry in the palette predictor, a reuse flag is signalled to indicate whether it is part of the current palette. The reuse flags are sent using run-length coding of zeros. After this, the number of new palette entries are signalled using exponential Golomb code of order 0. Finally, the component values for the new palette entries are signalled. After encoding the current CU, the palette predictor will be updated using the current palette, and entries from the previous palette predictor which are not reused in the current palette will be added at the end of new palette predictor until the maximum size allowed is reached (palette stuffing).

For coding the palette index map, the indices are coded using horizontal and vertical traverse scans as shown in 1. The scan order is explicitly signalled in the bitstream using the palette_transpose_flag.

Figure 1: Horizontal and vertical traverse scans

The palette indices are coded using two main palette sample modes: 'INDEX' and 'COPY_ABOVE'. The mode is signalled using a flag except for the top row when horizontal scan is used, the first column when the vertical scan is used, or when the previous mode was 'COPY_ABOVE'. In the 'COPY_ABOVE' mode, the palette index of the sample in the row above is copied. In the 'INDEX' mode, the palette index is explicitly signalled. For both 'INDEX' and 'COPY_ABOVE' modes, a run value is signalled which specifies the number pixels that are coded using the same mode.

The encoding order for index map is as follows: First, the number of index values for the CU is signalled. This is followed by signalling of the actual index values for the entire CU using truncated binary coding. Both the number of indices as well as the index values are coded in bypass mode. This groups the index-related bypass bins together. Then the palette mode (INDEX or COPY_ABOVE) and run are signalled in an interleaved manner. Finally, the component escape values corresponding to the escape samples for the entire CU are grouped together and coded in bypass mode. An additional syntax element, last_run_type_flag, is signalled after signalling the index values. This syntax element, in conjunction with the number of indices, eliminates the need to signal the run value corresponding to the last run in the block.

In VTM5.0, dual tree is enabled for I slice which separate the coding unit partitioning for Luma and Chroma. Hence, in this proposal, palette is applied on Luma (Y component) and Chroma (Cb and Cr components) separately. If dual tree is disabled, palette will be applied on Y, Cb, Cr components jointly, same as in HEVC palette.

# Simulation results

## Test sequences 4:2:0

The proposed method is evaluated according to the CE8 description [3] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations on YUV4:2:0 sequences. IBC, HashME, and RDPCM are turned on only for Class F and Class TGM. Table 1 shows the results of CE8.2.1 compared with VTM5.0.

Table 1: Results of the CE8.2.1 on test sequences 4:2:0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.02% | 107% | 105% |
| Class A2 | 0.04% | 0.07% | 0.06% | 108% | 102% |
| Class B | 0.05% | 0.02% | 0.06% | 107% | 105% |
| Class C | 0.05% | 0.06% | 0.06% | 108% | 106% |
| Class E | 0.06% | 0.04% | -0.08% | 109% | 103% |
| Overall | 0.04% | 0.03% | 0.02% | 108% | 105% |
| Class D | 0.03% | 0.26% | -0.02% | 106% | 105% |
| Class F | -0.72% | -0.56% | -0.83% | 107% | 108% |
| Class TGM | -4.29% | -4.53% | -3.89% | 102% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.10% | 0.13% | 102% | 102% |
| Class A2 | 0.06% | -0.11% | 0.12% | 100% | 99% |
| Class B | 0.05% | -0.02% | -0.29% | 102% | 102% |
| Class C | 0.00% | -0.03% | -0.11% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.06% | -0.08% | 102% | 102% |
| Class D | 0.04% | -0.17% | 0.15% | 101% | 102% |
| Class F | -0.55% | -0.83% | -0.77% | 107% | 107% |
| Class TGM | -1.71% | -2.54% | -2.15% | 102% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.60% | -0.12% | 101% | 100% |
| Class C | 0.04% | 0.23% | -0.12% | 100% | 104% |
| Class E | 0.06% | -0.05% | 0.02% | 100% | 98% |
| Overall | 0.04% | 0.31% | -0.09% | 101% | 101% |
| Class D | 0.06% | 0.26% | 0.36% | 104% | 110% |
| Class F | -0.28% | -0.75% | -0.72% | 106% | 104% |
| Class TGM | -0.43% | -0.57% | -0.47% | 104% | 102% |

## Test sequences 4:4:4

The proposed method is evaluated according to the CE8 description [3] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations on YUV4:4:4 sequences. The IBC, HashME, and RDPCM are turned on for all classes. Table 2 shows the results with DualITree turned off while Table 3 shows the results with DualITree turned on.

Table 2: Results of the CE8.2.1 on test sequences 4:4:4, DualITree = 0

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, DT0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| TGM 1080p | -16.41% |  | -22.86% |  | -22.49% |  | 99% |  | 87% |
| TGM 720p | -4.64% |  | -10.72% |  | -12.74% |  | 103% |  | 98% |
| Animation | 0.04% |  | -0.24% |  | -0.19% |  | 105% |  | 108% |
| Mixed content | -0.76% |  | -4.48% |  | -5.21% |  | 103% |  | 101% |
| Camera-Captured content | 0.01% |  | 0.01% |  | 0.04% |  | 104% |  | 102% |
| Overall | -5.76% |  | -9.88% |  | -10.46% |  | 102% |  | 97% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM 5.0, DT0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| TGM 1080p | -10.64% |  | -17.44% |  | -17.01% |  | 92% |  | 92% |
| TGM 720p | -4.62% |  | -9.89% |  | -12.21% |  | 100% |  | 98% |
| Animation | 0.05% |  | -0.16% |  | -0.17% |  | 103% |  | 100% |
| Mixed content | -0.42% |  | -3.11% |  | -4.17% |  | 103% |  | 100% |
| Camera-Captured content | -0.01% |  | 0.24% |  | 0.18% |  | 100% |  | 98% |
| Overall | -4.15% |  | -7.90% |  | -8.62% |  | 99% |  | 97% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM 5.0, DT0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| TGM 1080p | -6.54% | -11.48% |  | -11.07% |  | 100% |  | 96% |  |
| TGM 720p | -1.72% | -6.34% |  | -7.97% |  | 102% |  | 98% |  |
| Animation | 0.06% | -0.36% |  | -0.03% |  | 105% |  | 102% |  |
| Mixed content | 0.52% | -1.99% |  | -3.20% |  | 103% |  | 98% |  |
| Camera-Captured content | -0.03% | 0.04% |  | -0.12% |  | 101% |  | 99% |  |
| Overall | -2.09% | -5.19% |  | -5.74% |  | 102% |  | 98% |  |

Table 3: Results of the CE8.2.1 on test sequences 4:4:4, DualITree = 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -6.54% | -13.69% | -13.62% | 107% | 91% |
| TGM 720p | -1.58% | -7.03% | -8.55% | 106% | 100% |
| Animation | 0.06% | -1.10% | -0.75% | 104% | 104% |
| Mixed content | 0.11% | -5.74% | -5.58% | 105% | 101% |
| Camera-Captured content | 0.06% | 0.03% | 0.02% | 106% | 101% |
| Overall | -2.13% | -6.81% | -7.13% | 106% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -5.40% | -13.28% | -12.89% | 93% | 93% |
| TGM 720p | -0.74% | -7.07% | -8.29% | 101% | 99% |
| Animation | 0.09% | -0.46% | -0.36% | 105% | 104% |
| Mixed content | -0.45% | -6.84% | -6.12% | 103% | 99% |
| Camera-Captured content | 0.05% | 0.08% | -0.06% | 104% | 105% |
| Overall | -1.71% | -6.85% | -6.93% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -5.43% | -10.81% | -10.28% | 101% | 95% |
| TGM 720p | -1.13% | -5.67% | -6.86% | 102% | 99% |
| Animation | 0.03% | -0.22% | -0.26% | 108% | 97% |
| Mixed content | -0.24% | -6.25% | -5.92% | 104% | 97% |
| Camera-Captured content | 0.04% | -0.07% | 0.00% | 101% | 102% |
| Overall | -1.79% | -5.68% | -5.79% | 103% | 98% |

# Additional results

## Test sequences 4:2:0, Low QP

The proposed method is evaluated for low QP (QP = 2, 7, 12, 17) according to the CE8 description [3] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. IBC, HashME, and RDPCM are turned on only for Class F and Class TGM. Table 4 shows the results of CE8.2.1 compared with VTM5.0 on test sequences 4:2:0.

Table 4: Results of the CE8.2.1 on test sequences 4:2:0, Low QP

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, CTC, LQP |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 | 0.00% |  | -0.02% |  | -0.03% |  | 106% |  | 107% |
| Class A2 | 0.01% |  | 0.02% |  | 0.01% |  | 106% |  | 105% |
| Class B | -0.15% |  | -0.16% |  | -0.16% |  | 107% |  | 105% |
| Class C | -0.10% |  | -0.17% |  | -0.14% |  | 107% |  | 109% |
| Class E | -0.86% |  | -1.69% |  | -2.10% |  | 108% |  | 100% |
| Overall | -0.21% |  | -0.36% |  | -0.43% |  | 107% |  | 105% |
| Class D | -0.02% |  | -0.27% |  | -0.31% |  | 108% |  | 105% |
| Class F | -0.80% |  | -1.06% |  | -1.06% |  | 106% |  | 102% |
| Class TGM | -11.07% |  | -10.22% |  | -9.79% |  | 102% |  | 89% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0, CTC, LQP |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class A2 | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class B | -0.03% | -0.03% |  | 0.00% |  | 104% |  | 110% |  |
| Class C | -0.04% | -0.08% |  | -0.04% |  | 100% |  | 105% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class D | -0.05% | -0.01% |  | -0.02% |  | 98% |  | 100% |  |
| Class F | -0.69% | -0.90% |  | -1.08% |  | 102% |  | 99% |  |
| Class TGM | -5.30% | -4.79% |  | -4.71% |  | 92% |  | 96% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0, CTC, LQP |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% | -0.01% |  | -0.02% |  | 101% |  | 103% |  |
| Class C | 0.00% | 0.03% |  | 0.00% |  | 102% |  | 104% |  |
| Class E | -0.03% | -0.11% |  | -0.25% |  | 101% |  | 104% |  |
| Overall | -0.01% | -0.02% |  | -0.07% |  | 102% |  | 104% |  |
| Class D | 0.00% | -0.06% |  | 0.01% |  | 100% |  | 102% |  |
| Class F | -0.46% | -0.58% |  | -0.62% |  | 107% |  | 109% |  |
| Class TGM | -3.64% | -2.86% |  | -2.66% |  | 99% |  | 97% |  |

## Test sequences 4:4:4, Low QP

The proposed method is evaluated for low QP (QP = 2, 7, 12, 17) according to the CE8 description [3] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The IBC, HashME, and RDPCM are turned on for all classes. Table 5 shows the results with DualITree turned off while Table 6 shows the results with DualITree turned on.

Table 5: Results of the CE8.2.1 on test sequences 4:4:4, DualITree = 0, Low QP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, CTC, LQP, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -34.09% | -38.68% | -38.24% | 100% | 75% |
| TGM 720p | -15.38% | -17.13% | -18.09% | 106% | 91% |
| Animation | -0.11% | -0.23% | -0.23% | 106% | 107% |
| Mixed content | -3.60% | -4.50% | -4.42% | 105% | 100% |
| Camera-Captured content | 0.01% | 0.00% | 0.00% | 110% | 109% |
| Overall | -13.92% | -15.81% | -15.94% | 105% | 92% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, LQP, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM 720p | -12.94% | -15.09% | -16.70% | #NUM! | 97% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Mixed content | -2.30% | -2.83% | -3.31% | #NUM! | 97% |
| Camera-Captured content | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, LQP, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM 720p | -10.02% | -11.79% | -13.21% | #NUM! | 92% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Mixed content | -1.57% | -2.20% | -2.92% | #NUM! | 97% |
| Camera-Captured content | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 6: Results of the CE8.2.1 on test sequences 4:4:4, DualITree = 1, Low QP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, CTC, LQP, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -25.71% | -28.07% | -28.07% | 114% | 78% |
| TGM 720p | -9.08% | -11.97% | -12.20% | 117% | 96% |
| Animation | -0.70% | -1.09% | -1.16% | 111% | 105% |
| Mixed content | -3.90% | -3.75% | -3.60% | 116% | 99% |
| Camera-Captured content | 0.01% | 0.01% | 0.02% | 106% | 106% |
| Overall | -10.15% | -11.57% | -11.61% | 113% | 94% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, LQP, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM 720p | -8.57% | -11.68% | -12.62% | 94% | 99% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Mixed content | -2.02% | -1.62% | -1.35% | #NUM! | 99% |
| Camera-Captured content | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, LQP, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM 720p | -6.92% | -9.19% | -10.72% | #NUM! | 92% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Mixed content | -1.59% | -1.08% | -1.00% | #NUM! | 97% |
| Camera-Captured content | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Reference

- Y.-H. Chao, V. Seregin, M. Karczewicz, Y.-C. Sun, T.-S. Chang, J. Lou, “CE8: Palette Mode in HEVC (CE8-2.1)”, JVET-N0258, 14th meeting, Geneva, CH, March 2019.

- W. Zhu, L. Zhang, J. Xu, K. Zhang, H. Liu, Y. Wang, “CE8-related: Palette Mode Coding”, JVET-N0258, 14th meeting, Geneva, CH, March 2019.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-N1028, 14th meeting, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).