| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0119 |
| --- | --- |

| Title: | CE8-2.1: WD for palette mode in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Chao-Hsiung Hung, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz (Qualcomm) Yu-Chen Sun, Tsui-Shan Chang, Jian Lou (Alibaba) Weijia Zhu, Li Zhang, Jizheng Xu, Kai Zhang, Hongbin Liu, Yue Wang (Bytedance) | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com +1-858-349-2818 yuchen.s@alibaba-inc.com +1-360-5250535 weijia.zhu@bytedance.com |
| Source: | Qualcomm Incorporated Alibaba Cloud Computing Ltd Bytedance Inc. |  |  |

_____________________________

      - Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_affine_enabled_flag ) |  |
| sps_affine_type_flag | u(1) |
| sps_palette_enabled_flag | u(1) |
| sps_bcw_enabled_flag | u(1) |
| sps_ibc_enabled_flag | u(1) |
| sps_ciip_enabled_flag | u(1) |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| sps_triangle_enabled_flag | u(1) |
| sps_lmcs_enabled_flag | u(1) |
| sps_lfnst_enabled_flag | u(1) |
| sps_ladf_enabled_flag | u(1) |
| if ( sps_ladf_enabled_flag ) { |  |
| sps_num_ladf_intervals_minus2 | u(2) |
| sps_ladf_lowest_interval_qp_offset | se(v) |
| for( i = 0; i < sps_num_ladf_intervals_minus2 + 1; i++ ) { |  |
| sps_ladf_qp_offset[ i ] | se(v) |
| sps_ladf_delta_threshold_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

      - Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ¦¦ sps_plt_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && !( cbWidth = = 4 && cbHeight = = 4 && !sps_ibc_enabled_flag ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && sps_ibc_enabled_flag && ( cbWidth != 128 ¦ ¦ cbHeight != 128 ) ) |  |
| pred_mode_ibc_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && pred_mode_ibc_flag[ x0][ y0 ] = = 0 && sps_plt_enabled_flag && cbWidth <= 64 && cbHeight <= 64) |  |
| pred_mode_plt_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_PLT) { |  |
| if( treeType != SINGLE_TREE) { |  |
| if( treeType = = DUAL_TREE_LUMA ) |  |
| palette_coding( x0, y0, cbWidth, cbHeight, 0, 1 ) |  |
| if( treeType = = DUAL_TREE_CHROMA ) { |  |
| SubWidthC = 1 |  |
| SubHeightC = 1 |  |
| if(ChromaFormat == CHROMA_420) { |  |
| SubWidthC = 2 |  |
| SubHeightC = 2 |  |
| } |  |
| if(ChromaFormat == CHROMA_422) { |  |
| SubWidthC = 2 |  |
| } |  |
| palette_coding( x0, y0, cbWidth / SubWidthC, cbHeight / SubHeightC, 1, 2 ) |  |
| } |  |
| } else { |  |
| palette_coding( x0, y0, cbWidth, cbHeight, 0, 3 ) |  |
| } |  |
| } else { |  |
| if( sps_pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( cbWidth <= 32 && cbHeight <= 32 ) |  |
| intra_bdpcm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_bdpcm_flag[ x0 ][ y0 ] ) |  |
| intra_bdpcm_dir_flag[ x0 ][ y0 ] | ae(v) |
| else { |  |
| if( sps_mip_enabled_flag && ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) <= 2 ) && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_mip_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_flag[ x0 ][ y0 ] ) { |  |
| intra_mip_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_mip_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_mip_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( sps_pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| … |  |
| } |  |
| } |  |
| palette_coding( x0, y0, cbWidth, cbHeight, startComp, numComps) { | Descriptor |
| /* Parsing palette reuse flags */ |  |
| palettePredictionFinished = 0 |  |
| NumPredictedPaletteEntries = 0 |  |
| for( predictorEntryIdx = 0; predictorEntryIdx < PredictorPaletteSize [startComp] && !palettePredictionFinished && NumPredictedPaletteEntries [startComp] < palette_max_size; predictorEntryIdx++ ) { |  |
| palette_predictor_run | ae(v) |
| if( palette_predictor_run != 1 ) { |  |
| if( palette_predictor_run > 1 ) |  |
| predictorEntryIdx += palette_predictor_run − 1 |  |
| PalettePredictorEntryReuseFlags[ predictorEntryIdx ] = 1 |  |
| NumPredictedPaletteEntries++ |  |
| } else |  |
| palettePredictionFinished = 1 |  |
| } |  |
| /* Parsing number of new palette entries */ |  |
| num_signalled_palette_entries | ae(v) |
| CurrentPaletteSize = NumPredictedPaletteEntries + num_signalled_palette_entries |  |
| /* Parsing new palette entries */ |  |
| for( i = 0; i < num_signalled_palette_entries[ startComp ]; i++ ) |  |
| for( cIdx = startComp; cIdx < ( startComp + numComps); cIdx++ ) |  |
| new_palette_entries[ cIdx][ i ] | ae(v) |
| /* Parsing escape flag */ |  |
| if( CurrentPaletteSize > 0 ) |  |
| palette_escape_val_present_flag | ae(v) |
| else |  |
| palette_escape_val_present_flag = 1 |  |
| /* Parsing palette indices following scanning order */ |  |
| MaxPaletteIndex = CurrentPaletteSize + palette_escape_val_present_flag |  |
| if( MaxPaletteIndex > 1) { |  |
| num_palette_indices_minus1 | ae(v) |
| for( i = 0; i <= num_palette_indices_minus1; i++ ) { |  |
| palette_index_idc | ae(v) |
| PaletteIndexIdc [ i ] = palette_index_idc |  |
| } |  |
| copy_above_indices_for_final_run_flag | ae(v) |
| palette_transpose_flag | ae(v) |
| } |  |
| else |  |
| palette_transpose_flag = 0 |  |
| remainingNumIndices = num_palette_indices_minus1 + 1 |  |
| PaletteScanPos = 0 |  |
| while( PaletteScanPos <cbWidth*cbHeightt) { |  |
| xC = TraverseScanOrder[cbWidth][cbHeight][ PaletteScanPos ][ 0 ] |  |
| yC = TraverseScanOrder[cbWidth][cbHeight][ PaletteScanPos ][ 1 ] |  |
| if( PaletteScanPos > 0) { |  |
| xcPrev = TraverseScanOrder[cbWidth][cbHeight][ PaletteScanPos − 1 ][ 0 ] |  |
| ycPrev = TraverseScanOrder[cbWidth][cbHeight][ PaletteScanPos − 1 ][ 1 ] |  |
| } |  |
| CopyAboveIndicesFlag[ xC ][ yC ] = 0 |  |
| /* Parsing run types */ |  |
| if( MaxPaletteIndex > 1 ) |  |
| if( ( ( !palette_transpose_flag && yC > 0 ) ¦ ¦ ( ( palette_transpose_flag && xC > 0 ) ) && CopyAboveIndicesFlag[ xcPrev ][ ycPrev ] = = 0 ) |  |
| if( remainingNumIndices > 0 && PaletteScanPos < cbWidth* cbHeight − 1 ) { |  |
| copy_above_palette_indices_flag | ae(v) |
| CopyAboveIndicesFlag[ xC ][ yC ] = copy_above_palette_indices_flag |  |
| } else { |  |
| if( PaletteScanPos = = cbWidth * cbHeight − 1 && remainingNumIndices > 0 ) |  |
| CopyAboveIndicesFlag[ xC ][ yC ] = 0 |  |
| else |  |
| CopyAboveIndicesFlag[ xC ][ yC ] = 1 |  |
| } |  |
| /* Palette Index adjustment */ |  |
| if ( CopyAboveIndicesFlag[ xC ][ yC ] = = 0 ) { |  |
| currNumIndices = num_palette_indices_minus1 + 1 − remainingNumIndices |  |
| PaletteIndexMap[ xC ][ yC ] = PaletteIndexIdc[ currNumIndices ] |  |
| RefLevel = MaxPaletteIndex |  |
| if ( PaletteScanPos > 0 ) { |  |
| if ( CopyAboveIndicesFlag[ xcPrev ][ ycPrev ] = = 0 ) { |  |
| RefLevel = PaletteIndexMap[ xcPrev ][ ycPrev ] |  |
| } else { |  |
| if (!palette_transpose_flag ) |  |
| RefLevel = PaletteIndexMap[ xC ][ yC - 1 ] |  |
| else |  |
| RefLevel = PaletteIndexMap[ xC - 1 ][ yC ] |  |
| } |  |
| } |  |
| if (PaletteIndexIdc[ currNumIndices ] >= RefLevel ) |  |
| PaletteIndexMap[ xC ][ yC ] ++ |  |
| } |  |
| /* Parsing run length */ |  |
| PaletteRun = cbWidth * cbHeight − PaletteScanPos |  |
| if( MaxPaletteIndex > 1 ) { |  |
| if ( CopyAboveIndicesFlag[ xC ][ yC ] = = 0 ) |  |
| remainingNumIndices − = 1 |  |
| PaletteMaxRun = nCbS * nCbS − PaletteScanPos − 1 − remainingNumIndices − copy_above_indices_for_final_run_flag |  |
| if( remainingNumIndices > 0 ¦ ¦ CopyAboveIndicesFlag[ xC ][ yC ] != copy_above_indices_for_final_run_flag ) |  |
| if( PaletteMaxRun > 0 ) { |  |
| palette_run_prefix | ae(v) |
| if( ( palette_run_prefix > 1 ) && ( PaletteMaxRun != ( 1 << ( palette_run_prefix − 1 ) ) ) ) |  |
| palette_run_suffix | ae(v) |
| } else { |  |
| PaletteRun = cbWidth*cbHeightt - PaletteScanPos |  |
| } |  |
| for ( runPos = 1; runPos < PaletteRun; runPos++ ) { |  |
| xR =TraverseScanOrder[cbWidth][cbHeight][ PaletteScanPos + runPos ][ 0 ] |  |
| yR = TraverseScanOrder[cbWidth][cbHeight][ PaletteScanPos + runPos ][ 1 ] |  |
| if( CopyAboveIndicesFlag[ xC ][ yC ] = = 0 ) { |  |
| CopyAboveIndicesFlag[ xR ][ yR ] = 0 |  |
| PaletteIndexMap[ xR ][ yR ] = PaletteIndexMap[ xC ][ yC ] |  |
| } else { |  |
| CopyAboveIndicesFlag[ xR ][ yR ] = 1 |  |
| if (!palette_transpose_flag) |  |
| PaletteIndexMap[xR][yR] = PaletteIndexMap [xR][yR - 1] |  |
| else |  |
| PaletteIndexMap [xR][yR] = PaletteIndexMap [xR - 1][yR] |  |
| } |  |
| } |  |
| PaletteScanPos += PaletteRun |  |
| } |  |
| /* Parsing escape values */ |  |
| ScaleY = 1 |  |
| ScaleX = 1 |  |
| if( ChromaFormat = = CHROMA_420 ) { |  |
| ScaleY = 2 |  |
| ScaleX = 2 |  |
| } |  |
| if( ChromaFormat = = CHROMA_422 ) |  |
| ScaleX = 2 |  |
| if( palette_escape_val_present_flag ) { |  |
| for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++ ) |  |
| for(sPos = 0; sPos < cbWidth* cbHeight; sPos++ ) { |  |
| xC = TraverseScanOrder[ cbWidth][cbHeight][ sPos ][ 0 ] |  |
| yC = TraverseScanOrder[ cbWidth][cbHeight][ sPos ][ 1 ] |  |
| if( PaletteIndexMap[ startComp ] [ xC ][ yC ] = = (MaxPaletteIndex – 1)){ |  |
| if( cIdx == 0 ¦ ¦ startComp != 0){ |  |
| palette_escape_val | ae(v) |
| PaletteEscapeVal[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| if( startComp = = 0 && cIdx != 0 && xC % ( 1 << ScaleX ) = = 0 && yC % ( 1 << ScaleY ) = = 0 ){ |  |
| palette_escape_val | ae(v) |
| PaletteEscapeVal[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### Palette semantics

In the following semantics, the array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. The array indices xC, yC specify the location ( xC, yC ) of the sample relative to the top-left luma sample of the picture.

pred_mode_plt_flag specifies the use of palette mode in the current coding unit. pred_mode_plt_flag = = 1 indicates that palette mode is applied in the current coding unit. pred_mode_plt_flag = = 0 indicates that palette mode is not applied for the current coding unit.

treeType specifies whether joint tree is applied in the current coding tree. treeType == SINGLE_TREE indicates that the same coding unit partition is applied for both luma and chorma. treeType != SINGLE_TREE indicates that separate coding unit partitions are applied for luma and chroma (dual tree).  treeType = = DUAL_TREE_LUMA indicates that the current partition is for luma component in dual tree and treeType = = DUAL_TREE_CHROMA indicates that the current partition is for luma component in dual tree.

startComp specifies the first colour component of the current palette table. startComp = = 0 indicates the Y component and startComp == 1 and 2 indicates Cb and Cr components. numComps specifies the number of colour components in the current palette table.

The predictor palette consists of palette entries from previous coding units that are used to predict the entries in the current palette.

The variable PredictorPaletteSize [startComp] specifies the size of the predictor palette for the first colour component of the current palette table startComp.

The variable PalettePredictorEntryReuseFlags[ i ] equal to 1 specifies that the i-th entry in the predictor palette is reused in the current palette. PalettePredictorEntryReuseFlags[ i ] equal to 0 specifies that the i-th entry in the predictor palette is not an entry in the current palette. All elements of the array PalettePredictorEntryReuseFlags[ i ] are initialized to 0.

palette_predictor_run is used to determine the number of zeros that precede a non-zero entry in the array PalettePredictorEntryReuseFlags.

It is a requirement of bitstream conformance that the value of palette_predictor_run shall be in the range of 0 to ( PredictorPaletteSize − predictorEntryIdx ), inclusive, where predictorEntryIdx corresponds to the current position in the array PalettePredictorEntryReuseFlags. The variable NumPredictedPaletteEntries specifies the number of entries in the current palette that are reused from the predictor palette. The value of NumPredictedPaletteEntries shall be in the range of 0 to palette_max_size, inclusive.

num_signalled_palette_entriesspecifies the number of entries in the current palette that are explicitly signalled for the first colour component of the current palette table startComp.

When num_signalled_palette_entriesis not present, it is inferred to be equal to 0.

The variable CurrentPaletteSize specifies the size of the current palette and is derived as follows:

CurrentPaletteSize NumPredictedPaletteEntries + num_signalled_palette_entries [startComp]	(XX)

The value of CurrentPaletteSize [startComp] shall be in the range of 0 to palette_max_size, inclusive.

new_palette_entries[ cIdx ][ i ] specifies the value for the i-th signalled palette entry for the colour component cIdx.

The variable PredictorPaletteEntries[ cIdx ][ i ] specifies the i-th element in the predictor palette for the colour component cIdx.

The variable CurrentPaletteEntries[ cIdx ][ i ] specifies the i-th element in the current palette for the colour component cIdx and is derived as follows:

numPredictedPaletteEntries = 0
for( i = 0; i < PredictorPaletteSize[ startComp]; i++ )
	if( PalettePredictorEntryReuseFlags[ i ] ) {
		for( cIdx =startComp; cIdx < (startComp + numComps); cIdx++ )
			CurrentPaletteEntries[ cIdx ][ numPredictedPaletteEntries ] =
 				PredictorPaletteEntries[ cIdx ][ i ]
				numPredictedPaletteEntries++
		}

for( i = 0; i < num_signalled_palette_entries[startComp]; i++ )

for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++)
		CurrentPaletteEntries[ cIdx ][ numPredictedPaletteEntries + i ] =
           new_palette_entries[ cIdx ][ i ]

(XX)

palette_escape_val_present_flag equal to 1 specifies that the current coding unit contains at least one escape coded sample. escape_val_present_flag equal to 0 specifies that there are no escape coded samples in the current coding unit.

The variable MaxPaletteIndex specifies the maximum possible value for a palette index for the current coding unit. The value of MaxPaletteIndex is set equal to CurrentPaletteSize + palette_escape_val_present_flag.

num_palette_indices_minus1 plus 1 is the number of palette indices explicitly signalled or inferred for the current block.

When num_palette_indices_minus1 is not present, it is inferred to be equal to 0.

palette_index_idc is an indication of an index to the palette table, CurrentPaletteEntries . The value of palette_index_idc shall be in the range of 0 to MaxPaletteIndex, inclusive, for the first index in the block and in the range of 0 to ( MaxPaletteIndex − 1 ), inclusive, for the remaining indices in the block.

When palette_index_idc is not present, it is inferred to be equal to 0.

The variable PaletteIndexIdc[ i ] stores the i-th palette_index_idc explicitly signalled or inferred. All elements of the array PaletteIndexIdc[ i ] are initialized to 0. copy_above_indices_for_final_run_flag equal to 1 specifies that the palette indices of the last positions in the coding unit are copied from the palette indices in the row above if horizontal traverse scan is used or the palette indices in the left column if vertical traverse scan is used. copy_above_indices_for_final_run_flag equal to 0 specifies that the palette indices of the last positions in the coding unit is explicitly coded in the bitstream or inferred. When copy_above_indices_for_final_run_flag is not present, it is inferred to be equal to 0.

palette_transpose_flag equal to 1 specifies that vertical traverse scan is applied for scanning the indices for pixels in the current coding unit. palette_transpose_flag equal to 0 specifies that horizontal traverse scan is applied for scanning the indices for pixels in the current coding unit.

copy_above_palette_indices_flag equal to 1 specifies that the palette index is equal to the palette index at the same location in the row above if horizontal traverse scan is used or the same location in the left column if vertical traverse scan is used. copy_above_palette_indices_flag equal to 0 specifies that an indication of the palette index of the sample is coded in the bitstream or inferred.

The variable CopyAboveIndicesFlag[ xC ][ yC ] equal to 1 specifies that the palette index is copied from the palette index in the row above (horizontal scan) or left column (vertical scan). CopyAboveIndicesFlag[ xC ][ yC ] equal to 0 specifies that the palette index is explicitly coded in the bitstream or inferred.

The variable PaletteIndexMap[ xC ][ yC ] specifies a palette index, which is an index to the array represented by CurrentPaletteEntries. The array indices xC, yC specify the location ( xC, yC ) of the sample relative to the top-left luma sample of the picture. The value of PaletteIndexMap[ xC ][ yC ] shall be in the range of 0 to ( MaxPaletteIndex – 1), inclusive.

The variable PaletteRun specifies the number of consecutive locations with the same palette index as the position in the above row (horizontal scan) or in the left column (vertical scan) when CopyAboveIndicesFlag[ xC ][ yC ] is equal to 1 or specifies the number of consecutive locations with the same palette index when CopyAboveIndicesFlag[ xC ][ yC ] is equal to 0.

The variable PaletteMaxRun represents the maximum possible value for PaletteRun. It is a requirement of bitstream conformance that the value of PaletteMaxRun shall be greater than0.

palette_run_prefix specifies the prefix part in the binarization of PaletteRun.

palette_run_suffix specifies the suffix part in the binarization of PaletteRun. When palette_run_suffix is not present, the value of palette_run_suffix is inferred to be equal to 0.

The value of PaletteRun is derived as follows:

- If palette_run_prefix is less than 2, the following applies:

PaletteRun = palette_run_prefix		(XX)

- Otherwise (palette_run_prefix is greater than or equal to 2), the following applies:

PrefixOffset = 1  <<  ( palette_run_prefix − 1 )

PaletteRun = PrefixOffset + palette_run_suffix
				(XX)

palette_escape_val specifies the quantized escape coded sample value for a component.

The variable PaletteEscapeVal[ cIdx ][ xC ][ yC ] specifies the escape value of a sample for which PaletteIndexMap[ xC ][ yC ] is equal to (MaxPaletteIndex – 1) and palette_escape_val_present_flag is equal to 1. The array index cIdx specifies the colour component. The array indices xC, yC specify the location ( xC, yC ) of the sample relative to the top-left luma sample of the picture.

It is a requirement of bitstream conformance that PaletteEscapeVal[ cIdx ][ xC ][ yC ] shall be in the range of 0 to (1  <<  ( BitDepthY + 1 ) ) − 1, inclusive, for cIdx equal to 0, and in the range of 0 to (1  <<  ( BitDepthC + 1 ) ) − 1, inclusive, for cIdx not equal to 0.

  - Decoding process for coding units coded in intra prediction mode

    - General decoding process for coding units coded in intra prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

Output of this process is a modified reconstructed picture before in-loop filtering.

The derivation process for quantization parameters as specified in clause 8.7.1 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight, and the variable treeType as inputs.

When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, the decoding process for luma samples is specified as follows:

- If cu[ xCb ][ yCb ].predMode = is equal to MODE_PLT, the following applies:

  - If treeType is equal to SINGLE_TREE, the following applies:

- The general decoding process for palette blocks as specified in clause XXX is invoked with the luma location ( xCb, yCb ), the variable startComp set equal to 0, the variable cIdx set to 0, the variable nTbW set equal to cbWidth, the variable nTbH set equal to cbHeight.

  - Otherwise, the following applies:

- The general decoding process for palette blocks as specified in clause XXX is invoked with the luma location ( xCb, yCb ), the variable startComp set equal to 0, the variable cIdx set to 0, the variable nTbW set equal to cbWidth, the variable nTbH set equal to cbHeight.

- The general palette predictor updating process for palette blocks as specified in clause 8.X.X is invoked with the luma location ( xCb, yCb ), the variable startComp set equal to 0, the variable numComps set equal to 1.

When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, the decoding process for chroma samples is specified as follows:

- If cu[ xCb ][ yCb ].predMode = is equal to MODE_PLT, the following applies:

  - If treeType is equal to SINGLE_TREE, the following applies

- The general decoding process for palette blocks as specified in clause XXX is invoked with the luma location ( xCb, yCb ), the variable startComp set equal to 0, the variable cIdx set to 1, the variable nTbW set equal to ( cbWidth / 2 ), the variable ( cbHeight / 2 ) set equal to cbHeight.

- The general decoding process for palette blocks as specified in clause XXX is invoked with the luma location ( xCb, yCb ), the variable startComp set equal to 0, the variable cIdx set to 2, the variable nTbW set equal to ( cbWidth / 2 ), the variable ( cbHeight / 2 ) set equal to cbHeight.

- The general palette predictor updating process for palette blocks as specified in clause 8.X.X is invoked with the luma location ( xCb, yCb ), the variable startComp set equal to 0, the variable numComps set equal to 3.

  - Otherwise, the following applies

- The general decoding process for palette blocks as specified in clause XXX is invoked with the luma location ( xCb, yCb ), the variable startComp set equal to 1, the variable cIdx set to 1, the variable nTbW set equal to ( cbWidth / 2 ), the variable ( cbHeight / 2 ) set equal to cbHeight.

- The general decoding process for palette blocks as specified in clause XXX is invoked with the luma location ( xCb, yCb ), the variable startComp set equal to 1, the variable cIdx set to 2, the variable nTbW set equal to ( cbWidth / 2 ), the variable ( cbHeight / 2 ) set equal to cbHeight.

- The general palette predictor updating process for palette blocks as specified in clause XXX is invoked with the luma location ( xCb, yCb ), the variable startComp set equal to 1, the variable numComps set equal to 2.

##### Decoding process for palette mode

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

- a variable startComp specifies the first colour component in the palette table,

- a variable cIdx specifying the colour component of the current block,

- two variables nTbW and nTbH specifying the width and height of the current block, respectively.

Output of this process is an array recSamples[ x ][ y ], with x = 0.. nTbW − 1, y = 0.. nTbH − 1 specifying reconstructed sample values for the block.

Depending on the value of cIdx, the variables nSubWidth and nSubHeight are derived as follows:

- If cIdx is equal to 0, nSubWidth is set to 1 and nSubHeight is set to 1.

- If startComp is equal to 1 and if cIdx > 1, nSubWidth is set to 1 and nSubHeight is set to 1.

- Otherwise, nSubWidth is set to SubWidthC and nSubHeight is set to SubHeightC.

The ( nTbW x nTbH ) block of the reconstructed sample array recSamples at location ( xCb, yCb ) is represented by recSamples[ x ][ y ] with x = 0.. nTbW − 1 and y = 0.. nTbH − 1, and the value of recSamples[ x ][ y ] for each x in the range of 0 to nTbW − 1, inclusive, and each y in the range of 0 to nTbH − 1, inclusive, is derived as follows:

- The variables xL and yL are derived as follows:

xL = palette_transpose_flag ? x * nSubHeight : x * nSubWidth	(8-69)

yL = palette_transpose_flag ? y * nSubWidth : y * nSubHeight	(8-70)

- The variable bIsEscapeSample is derived as follows:

- If PaletteIndexMap[ xCb + xL ][ yCb + yL ] is equal to MaxPaletteIndex and palette_escape_val_present_flag is equal to 1, bIsEscapeSample is set equal to 1.

- Otherwise, bIsEscapeSample is set equal to 0.

- If bIsEscapeSample is equal to 0, the following applies:

recSamples[ x ][ y ] = CurrentPaletteEntries[ cIdx ][ PaletteIndexMap[ xCb + xL ][ yCb + yL ] ]	(8-71)

- Otherwise, if cu_transquant_bypass_flag is equal to 1, the following applies:

recSamples[ x ][ y ] = PaletteEscapeVal[ cIdx ][ xCb + xL ][ yCb + yL ]	(8-72)

- Otherwise (bIsEscapeSample is equal to 1 and cu_transquant_bypass_flag is equal to 0), the following ordered steps apply:

- The derivation process for quantization parameters is invoked with the location ( xCb, yCb ) specifying the top-left sample of the current block relative to the top-left sample of the current picture.

- The quantization parameter qP is derived as follows:

- If cIdx is equal to 0,

qP = Max( 0, Qp′Y )		(8-73)

- Otherwise, if cIdx is equal to 1,

qP = Max( 0, Qp′Cb )		(8-74)

- Otherwise (cIdx is equal to 2),

qP = Max( 0, Qp′Cr )		(8-75)

- The variables bitDepth is derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC	(8-76)

- The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

- The following applies:

tmpVal = ( PaletteEscapeVal[ cIdx ][ xCb + xL ][ yCb + yL ] *
				levelScale[ qP%6 ] )  <<  ( qP / 6 ) + 32 )  >>  6	(8-77)

recSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, tmpVal )	(8-78)

##### Palette predictor update process for palette mode

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

- a variable startComp specifies the first colour component in the palette table,

- a variable numComps specifies the number of colour components in the palette table,

Output of this process is a variable PredictorPaletteSize[startComp] specifying the size of the updated palette predictor, and an array PredictorPaletteEntries [ cIdx ][ i ], with cIdx  = startComp, …, startComp+ numComps-1, i = 0.. PredictorPaletteSize − 1 specifying the updated palette predictor for the next block.

The variable PredictorPaletteSize and the array PredictorPaletteEntries are derived or modified as follows:

for( i = 0; i < CurrentPaletteSize; i++ )
	for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++ )
		newPredictorPaletteEntries[ cIdx ][ i ] = CurrentPaletteEntries[ cIdx ][ i ]
newPredictorPaletteSize = CurrentPaletteSize
for( i = 0; i < PredictorPaletteSize  &&  newPredictorPaletteSize < PaletteMaxPredictorSize; i++ )
	if( !PalettePredictorEntryReuseFlags[ i ] ) {
		for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++ )		(8-79)
			newPredictorPaletteEntries[ cIdx ][ newPredictorPaletteSize ] =
				PredictorPaletteEntries[ cIdx ][ i ]
		newPredictorPaletteSize++
	}
for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++ )
	for( i = 0; i < newPredictorPaletteSize; i++ )
		PredictorPaletteEntries[ cIdx ][ i ] = newPredictorPaletteEntries[ cIdx ][ i ]
PredictorPaletteSize = newPredictorPaletteSize

It is a requirement of bitstream conformance that the value of PredictorPaletteSize shall be in the range of 0 to PaletteMaxPredictorSize, inclusive.

| Table 9-5 – Association of ctxIdxOffset and syntax elements for each initializationType in the initialization process |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax structure | Syntax element | ctxTable | initType |  |  |
|  |  |  | 0 | 1 | 2 |
| coding_unit( ) | cu_skip_flag[ ][ ] |  |  | 0 | 3 |
|  | pred_mode_ibc_flag |  |  |  |  |
|  | pred_mode_flag |  |  |  |  |
|  | pred_mode_plt_flag |  |  |  |  |
|  | intra_bdpcm_flag[ ][ ] |  |  |  |  |
|  | intra_bdpcm_dir_flag[ ][ ] |  |  |  |  |
|  | intra_mip_flag[ ][ ] |  |  |  |  |
| palette_coding( ) | copy_above_indices_for_final_run_flag |  |  |  |  |
|  | palette_transpose_flag |  |  |  |  |
|  | copy_above_palette_indices_flag |  |  |  |  |
|  | palette_run_prefix |  |  |  |  |

| Table 9-11 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| coding_unit( ) | cu_skip_flag[ ][ ] | FL | cMax = 1 |
|  | pred_mode_ibc_flag | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | pred_mode_plt_flag | FL | cMax = 1 |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | intra_bdpcm_flag[ ][ ] | FL | cMax = 1 |
|  | intra_bdpcm_dir_flag[ ][ ] | FL | cMax = 1 |
| palette_coding( ) | palette_predictor_run | EG0 | - |
|  | num_signalled_palette_entries | EG0 | - |
|  | new_palette_entries | FL | cMax = cIdx = = 0 ? ( ( 1 << BitDepthY ) − 1 ) : ( (1 << BitDepthC ) − 1 ) |
|  | palette_escape_val_present_flag | FL | cMax = 1 |
|  | num_palette_indices_minus1 | 9.5.3.13 | MaxPaletteIndex |
|  | palette_index_idc | 9.5.3.14 | MaxPaletteIndex |
|  | copy_above_indices_for_final_run_flag | FL | cMax = 1 |
|  | palette_transpose_flag | FL | cMax = 1 |
|  | copy_above_palette_indices_flag | FL | cMax = 1 |
|  | palette_run_prefix | TR | cMax = Floor( Log2( PaletteMaxRun ) ) + 1, cRiceParam = 0 |
|  | palette_run_suffix | TB | cMax = ( PrefixOffset << 1 ) > PaletteMaxRun ? ( PalletMaxRun − PrefixOffset ) : ( PrefixOffset − 1 ) |
|  | palette_escape_val | EG3 | - |

#### Binarization process for num_palette_indices_minus1

Input to this process is a request for a binarization for the syntax element num_palette_indices_minus1, and MaxPaletteIndex.

Output of this process is the binarization of the syntax element.

The variables cRiceParam is derived as follows:

cRiceParam = 3 + ( ( MaxPaletteIndex + 1 )  >>  3 )	(9-30)

The variable cMax is derived from cRiceParam as:

cMax = 4  <<  cRiceParam		(9-31)

The binarization of the syntax element num_palette_indices_minus1 is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of num_palette_indices_minus1, prefixVal, is derived as follows:

prefixVal = Min( cMax, num_palette_indices_minus1 )	(9-32)

- The prefix bin string is specified by invoking the TR binarization process as specified in clause 9.3.3.2 for prefixVal with the variables cMax and cRiceParam as inputs.

When the prefix bin string is equal to the bit string of length 4 with all bits equal to 1, the suffix bin string is present and it is derived as follows:

- The suffix value of num_palette_indices_minus1, suffixVal, is derived as follows:

suffixVal = num_palette_indices_minus1 − cMax	(9-33)

- The suffix bin string is specified by invoking the k-th order EGk binarization process as specified in clause 9.3.3.3 for the binarization of suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1.

#### Binarization process for palette_index_idc

Input to this process is a request for a binarization for the syntax element palette_index_idc and the variable MaxPaletteIndex.

Output of this process is the binarization of the syntax element.

The variable cMax is derived as follows:

- If this process is invoked for the first time for the current block, cMax is set equal to MaxPaletteIndex.

- Otherwise (this process is not invoked for the first time for the current block), cMax is set equal to MaxPaletteIndex minus 1.

The binarization for the palette_index_idc is derived by invoking the TB binarization process specified in clause 9.5.3.4 with cMax.

| Table 9-17 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| pred_mode_plt_flag | 0 | na | na | na | na | na |
| palette_predictor_run | bypass | bypass | bypass | bypass | bypass | bypass |
| num_signalled_palette_entries | bypass | bypass | bypass | bypass | bypass | bypass |
| new_palette_entries | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_escape_val_present_flag | bypass | na | na | na | na | na |
| palette_transpose_flag | 0 | na | na | na | na | na |
| num_palette_indices_minus1 | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_index_idc | bypass | bypass | bypass | bypass | bypass | bypass |
| copy_above_palette_indices_flag and copy_above_indices_for_final_run_flag | 0 | na | na | na | na | na |
| palette_run_prefix | 0..7 (clause 9.5.4.2.10) |  |  |  |  |  |
| palette_run_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_escape_val | bypass | bypass | bypass | bypass | bypass | bypass |

##### 9.5.4.2.10 Derivation process of ctxInc for the syntax element palette_run_prefix

Inputs to this process are the bin index binIdx and the syntax elements copy_above_palette_indices_flag and palette_index_idc.

Output of this process is the variable ctxInc.

The variable ctxInc is derived as follows:

- If copy_above_palette_indices_flag is equal to 0 and binIdx is equal to 0, ctxInc is derived as follows:

ctxInc = ( palette_index_idc < 1 ) ? 0 : ( ( palette_index_idc < 3 ) ? 1 : 2 )	(9-63)

- Otherwise, ctxInc is provided by:

| binIdx | 0 | 1 | 2 | 3 | 4 | > 4 |
| --- | --- | --- | --- | --- | --- | --- |
| copy_above_palette_indices_flag = = 1 | 5 | 6 | 6 | 7 | 7 | bypass |
| copy_above_palette_indices_flag = = 0 | 0, 1, 2 | 3 | 3 | 4 | 4 | bypass |

# Reference

- Y.-H. Chao, V. Seregin, M. Karczewicz, Y.-C. Sun, T.-S. Chang, J. Lou, “CE8: Palette Mode in HEVC (CE8-2.1)”, JVET-N0258, 14th meeting, Geneva, CH, March 2019.

- W. Zhu, L. Zhang, J. Xu, K. Zhang, H. Liu, Y. Wang, “CE8-related: Palette Mode Coding”, JVET-N0258, 14th meeting, Geneva, CH, March 2019.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-N1028, 14th meeting, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).