| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0122 |
| --- | --- |

| Title: | CE7: Sign context, level mapping, and bitplane coding for TS residual coding (CE7-3.7, CE7-3.8, CE7-3.9, CE7-3.10, and CE7-3.11) |  |
| --- | --- | --- |
| Status: | Input document to JVET |  |
| Purpose: | Proposal |  |
| Author(s) or Contact(s): | Marta Karczewicz, Hongtao Wang, Muhammed Coban, Yung-Hsuan Chao, Yu Han (Qualcomm) | Tel: +1 858 658 3973 Email: martak@qti.qualcomm.com hongtaow@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |

_____________________________

# Abstract

This document reports the results of CE7-3.7~7-3.11 for transform skip (TS) residual coding on top of VTM-5.0.

In CE7-3.7, a local neighbor-based context derivation for sign coding is proposed. The simulation results show Class F: -0.44% AI, -0.37% RA, -0.30% LB BD-Rate, and Class SCC: -0.54% AI, -0.46% RA, -0.30% LB BD-Rate, versus the VTM-5.0 anchor under common test conditions (CTC).

In CE7-3.8, a local neighbor-based context derivation for gt1 level coding and level mapping are proposed. The simulation results show Class F: -0.31% AI, -0.28% RA, -0.23% LB BD-Rate, and Class SCC: -0.62% AI, -0.52% RA, -0.24% LB BD-Rate, versus the VTM-5.0 anchor under CTC.

In CE7-3.9, bitplane coding of TS residual element is proposed. The results show Class F: -0.03% AI, -0.06% RA, -0.07% LB BD-Rate, and Class SCC: -0.07% AI, -0.09% RA, -0.00% LB BD-Rate, versus the VTM-5.0 anchor under CTC.

In addition, CE7-3.10 and 7-3.11 report the combined test of the proposed methods. CE7-3.10 combines the local neighbor-based sign coding and level mapping, which shows Class F: -0.78% AI, -0.57% RA, -0.41% LB BD-Rate, and Class SCC: -1.23% AI, -0.89% RA, -0.57% LB BD-Rate, versus the VTM-5.0 anchor under CTC. CE7-3.11 combines all three proposed methods, which shows Class F: -0.79% AI, -0.62% RA, -0.46% LB BD-Rate, and Class SCC: -1.30% AI, -0.92% RA, -0.67% LB BD-Rate, versus the VTM-5.0 anchor under CTC.

# CE Description

## Sign context modelling

Sign coding context derivation in VTM5.0 is replaced by the following method that utilize the sign information of above neighboring coefficient and left neighboring coefficient to derive the sign coding context offset.

Figure 1: Current coefficient value X, neighboring left coefficient value X0 and neighboring above coefficient value X1

X0 is the left neigboring coefficient value, X1 is above neighboring coefficient value as shown in Figure 1. If RDPCM is not applied in the current coding unit. context offset 0 is used if both X0 and X1 are zero or nonzero but with opposite signs, context offset 1 is used if both X0 and X1 are non-negative, and context offset 2 is used otherwise. If RDPCM is applied in the current coding unit. context offset 3 is used if both X0 and X1 are zero or nonzero but with opposite signs, context offset 4 is used if both X0 and X1 are non-negative, and context offset 5 is used otherwise This can be summarized by the following table.

|  | (X0, X1) | Context offset |
| --- | --- | --- |
| intra_bdpcm_flag = = 0 | (0, 0), (+, -), (-, +) | 0 |
|  | (+, +), (+, 0), (0, +) | 1 |
|  | (-, -), (-, 0), (0, -) | 2 |
| intra_bdpcm_flag = = 1 | (0, 0), (+, -), (-, +) | 3 |
|  | (+, +), (+, 0), (0, +) | 4 |
|  | (-, -), (-, 0), (0, -) | 5 |

## Level mapping

If RDPCM is not applied in the current coding unit, the absolute coefficient level, absCoeffLevel, is mapped to a modified level to be coded by the following method that depends on the values of quantized residual samples to the left and above of the current residual sample. Let X0 denote the absolute coefficient level to the left of the current coefficient and let X1 denote the absolute coefficient level of above coefficient. For representing a coefficient with absolute coefficient level absCoeff, a mapped absCoeffMod is coded which is derived as follows:

pred = max(X0, X1);

if (absCoeff == pred)

{

absCoeffMod = 1;

}

else

{

absCoeffMod = (absCoeff < pred) ? absCoeff + 1 : absCoeff;

}

In addition, the context derivation for abs_level_gt1_flag in VTM5.0 is replaced by the following method utilizing the information of above neighboring coefficient and left neighboring coefficient ( X0 and X1 in Figure 1). If RDPCM is not applied in the current coding unit, context offset 0 is used if both X0 and X1 are zeros, context offset 1 is used if only one of the two neighboring coefficients is nonzero, and context offset 2 is used if both X0 and X1 are nonzero. If RDPCM is applied in the current coding unit, context offset 3 is used.

## Bitplane coding of TS residuals

Syntax elements sig_coeff_flag, coeff_sign_flag, abs_level_gt1_flag, par_level_flag, that are coded interleaved residual sample by residual sample in the first pass [1] followed by abs_level_gtX_flag bitplanes are converted to bitplane coding in sig_coeff_flag, coeff_sign_flag, abs_level_gt1_flag, abs_level_gtX_flag (X: 3,5,..) followed by par_level_flag bitplane order. Context modelling of syntax elements as well as abs_remainder coding is same as in VTM5.0.

# Simulation results

## Standard QPs (22, 27, 32, 37)

The proposed method is evaluated according to the CE7 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. IBC, HashME, and RDPCM are turned on only for class F and class TGM. Simulation results for standard QPs (22, 27, 32, 37) are shown in Table 1-5. Table 1 shows the results of sign context modelling described in 1.1 (CE7-3.7). Table 2 shows the results of level mapping described in 1.2 (CE7-3.8). Table 3 shows the results of bitplane coding described in 1.3 (CE7-3.9). Table 4 and Table 5 show the combined test results of sign context modelling and level mapping (CE7-3.10), and the combined tests results of sign contex modelling, level mapping, and bitplane coding (CE7-3.11).

Table 1: Results of the CE7-3.7 for standard QPs

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | 0.00% | 100% | 98% |
| Class A2 | 0.00% | -0.02% | -0.04% | 101% | 99% |
| Class B | -0.02% | -0.04% | 0.02% | 100% | 101% |
| Class C | -0.05% | -0.07% | -0.04% | 101% | 101% |
| Class E | -0.03% | -0.08% | -0.03% | 100% | 100% |
| Overall | -0.02% | -0.05% | -0.01% | 100% | 100% |
| Class D | -0.06% | 0.08% | -0.19% | 101% | 100% |
| Class F | -0.44% | -0.33% | -0.25% | 101% | 103% |
| TGM | -0.54% | -0.43% | -0.46% | 101% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.07% | 100% | 100% |
| Class A2 | 0.02% | 0.02% | 0.05% | 100% | 100% |
| Class B | -0.01% | -0.12% | -0.21% | 100% | 101% |
| Class C | -0.11% | -0.06% | -0.31% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.05% | -0.13% | 100% | 100% |
| Class D | -0.02% | -0.13% | 0.27% | 100% | 100% |
| Class F | -0.37% | -0.05% | -0.21% | 101% | 100% |
| TGM | -0.46% | -0.32% | -0.42% | 101% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.31% | 0.09% | 100% | 99% |
| Class C | -0.02% | 0.14% | -0.09% | 100% | 101% |
| Class E | -0.04% | 0.09% | -0.38% | 100% | 105% |
| Overall | -0.02% | 0.20% | -0.09% | 100% | 101% |
| Class D | -0.05% | -0.23% | 0.51% | 100% | 103% |
| Class F | -0.30% | -0.37% | -0.38% | 100% | 101% |
| TGM | -0.30% | -0.44% | -0.29% | 100% | 100% |

Table 2: Results of the CE7-3.8 for standard QPs

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | -0.04% | -0.01% | 101% | 101% |
| Class B | -0.01% | -0.06% | -0.07% | 101% | 102% |
| Class C | 0.00% | -0.08% | 0.05% | 101% | 98% |
| Class E | -0.01% | 0.00% | 0.16% | 101% | 100% |
| Overall | 0.00% | -0.04% | 0.02% | 101% | 100% |
| Class D | 0.00% | 0.21% | -0.01% | 101% | 99% |
| Class F | -0.31% | -0.31% | -0.40% | 102% | 99% |
| TGM | -0.62% | -0.62% | -0.65% | 104% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.03% | 0.09% | 101% | 101% |
| Class A2 | 0.00% | -0.06% | -0.09% | 101% | 101% |
| Class B | 0.02% | 0.10% | -0.20% | 100% | 101% |
| Class C | -0.01% | -0.08% | -0.05% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.08% | 101% | 101% |
| Class D | 0.04% | -0.17% | 0.04% | 101% | 101% |
| Class F | -0.28% | -0.25% | -0.33% | 101% | 100% |
| TGM | -0.52% | -0.44% | -0.58% | 103% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.11% | 0.10% | 101% | 99% |
| Class C | 0.02% | 0.21% | 0.03% | 100% | 98% |
| Class E | -0.04% | 0.14% | -0.57% | 100% | 101% |
| Overall | 0.00% | 0.06% | -0.09% | 100% | 99% |
| Class D | 0.04% | -0.45% | 0.73% | 101% | 103% |
| Class F | -0.23% | -0.31% | -0.87% | 100% | 99% |
| TGM | -0.24% | -0.41% | -0.39% | 103% | 101% |

Table 3: Results of the CE7-3.9 for standard QPs

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.03% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | -0.03% | -0.02% | 100% | 102% |
| Class C | 0.00% | -0.05% | -0.03% | 100% | 99% |
| Class E | 0.00% | 0.03% | 0.01% | 100% | 99% |
| Overall | 0.00% | -0.02% | -0.01% | 100% | 100% |
| Class D | -0.01% | 0.06% | -0.06% | 100% | 98% |
| Class F | -0.03% | 0.08% | 0.02% | 101% | 101% |
| TGM | -0.07% | -0.06% | -0.07% | 100% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.04% | -0.05% | 100% | 100% |
| Class B | 0.01% | 0.00% | 0.04% | 100% | 101% |
| Class C | 0.00% | -0.13% | -0.13% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.03% | 100% | 100% |
| Class D | 0.00% | -0.13% | 0.09% | 100% | 100% |
| Class F | -0.06% | 0.02% | 0.00% | 100% | 100% |
| TGM | -0.09% | -0.03% | -0.13% | 100% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.16% | 0.23% | 100% | 99% |
| Class C | 0.04% | 0.07% | -0.11% | 101% | 102% |
| Class E | 0.11% | 0.51% | 0.78% | 100% | 102% |
| Overall | 0.03% | 0.22% | 0.25% | 100% | 100% |
| Class D | 0.05% | -0.16% | 0.72% | 102% | 103% |
| Class F | -0.07% | 0.05% | -0.05% | 100% | 100% |
| TGM | 0.00% | -0.06% | 0.01% | 101% | 103% |

Table 4: Results of the CE7-3.10 for standard QPs

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | -0.02% | 101% | 101% |
| Class A2 | -0.02% | -0.03% | -0.02% | 101% | 99% |
| Class B | -0.03% | -0.05% | 0.02% | 100% | 102% |
| Class C | -0.03% | -0.08% | 0.01% | 101% | 98% |
| Class E | -0.06% | -0.08% | -0.04% | 101% | 100% |
| Overall | -0.03% | -0.05% | -0.01% | 101% | 100% |
| Class D | -0.08% | 0.16% | -0.02% | 102% | 102% |
| Class F | -0.78% | -0.62% | -0.82% | 102% | 100% |
| TGM | -1.23% | -1.13% | -1.15% | 104% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | 0.05% | 100% | 101% |
| Class A2 | 0.02% | 0.05% | 0.08% | 100% | 100% |
| Class B | 0.00% | 0.05% | -0.05% | 100% | 100% |
| Class C | -0.05% | -0.17% | -0.25% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.02% | -0.06% | 100% | 101% |
| Class D | -0.01% | 0.09% | 0.37% | 101% | 101% |
| Class F | -0.57% | -0.58% | -0.46% | 101% | 100% |
| TGM | -0.89% | -0.83% | -0.93% | 103% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.40% | 0.08% | 101% | 98% |
| Class C | 0.00% | 0.12% | -0.24% | 100% | 100% |
| Class E | -0.01% | -0.18% | 0.13% | 101% | 104% |
| Overall | 0.00% | 0.16% | -0.02% | 101% | 100% |
| Class D | 0.03% | -0.15% | 1.00% | 101% | 100% |
| Class F | -0.41% | -0.55% | -0.95% | 100% | 99% |
| TGM | -0.57% | -0.79% | -0.71% | 104% | 100% |

Table 5: Results of the CE7-3.11 for standard QPs

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.01% | 101% | 100% |
| Class A2 | -0.01% | -0.06% | -0.02% | 102% | 99% |
| Class B | -0.02% | -0.05% | 0.00% | 101% | 101% |
| Class C | -0.03% | -0.12% | -0.01% | 102% | 99% |
| Class E | -0.05% | -0.09% | -0.07% | 101% | 100% |
| Overall | -0.03% | -0.07% | -0.02% | 101% | 100% |
| Class D | -0.07% | 0.13% | -0.04% | 102% | 99% |
| Class F | -0.79% | -0.66% | -0.71% | 102% | 100% |
| TGM | -1.30% | -1.24% | -1.20% | 104% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.04% | 101% | 101% |
| Class A2 | 0.02% | 0.13% | 0.06% | 100% | 100% |
| Class B | 0.00% | 0.01% | -0.16% | 100% | 99% |
| Class C | -0.06% | -0.13% | -0.16% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.08% | 100% | 100% |
| Class D | 0.02% | 0.17% | 0.05% | 101% | 100% |
| Class F | -0.62% | -0.70% | -0.57% | 101% | 99% |
| TGM | -0.92% | -0.78% | -0.87% | 102% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.27% | -0.05% | 102% | 104% |
| Class C | -0.02% | 0.05% | -0.38% | 101% | 102% |
| Class E | -0.04% | 0.18% | 0.31% | 101% | 105% |
| Overall | -0.02% | 0.18% | -0.07% | 101% | 104% |
| Class D | -0.02% | -0.20% | 0.48% | 100% | 98% |
| Class F | -0.46% | -0.35% | -0.86% | 102% | 104% |
| TGM | -0.67% | -0.76% | -0.83% | 104% | 104% |

## Low QPs (2, 7, 12, 17)

Simulation results for low QPs (2, 7, 12, 17) are shown in Table 6-10. Table 6 shows the results of sign context modelling described in 1.1 (CE7-3.7). Table 7 shows the results of level mapping described in 1.2 (CE7-3.8). Table 8 shows the results of bitplane coding described in 1.3 (CE7-3.9). Table 9 and Table 10 show the combined test results of sign context modelling and level mapping (CE7-3.10), and the combined tests results of sign contex modelling, level mapping, and bitplane coding (CE7-3.11).

Table 6: Results of the CE7-3.7 for low QPs

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | 0.01% | 100% | 97% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.01% | 0.01% | 100% | 101% |
| Class E | -0.03% | 0.02% | 0.01% | 100% | 101% |
| Overall | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.04% | -0.06% | 101% | 101% |
| Class F | -0.14% | -0.12% | -0.15% | 100% | 99% |
| TGM | -0.38% | -0.30% | -0.30% | 101% | 101% |
|  | Random Access |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.06% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | -0.01% | -0.02% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | 0.02% | 100% | 101% |
| Class D | -0.04% | 0.03% | -0.07% | 100% | 99% |
| Class F | -0.66% | -0.15% | -0.09% | 100% | 100% |
| TGM | -15.55% | -0.17% | -0.16% | 100% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | -0.02% | 103% | 106% |
| Class C | -0.03% | 0.01% | 0.01% | 100% | 101% |
| Class E | -0.02% | -0.04% | -0.09% | 101% | 103% |
| Overall | -0.02% | -0.01% | -0.03% | 102% | 104% |
| Class D | -0.03% | -0.05% | 0.08% | 101% | 102% |
| Class F | -0.12% | -0.10% | -0.05% | 100% | 100% |
| TGM | -0.63% | -0.33% | -0.31% | 100% | 101% |

Table 7: Results of the CE7-3.8 for low QPs

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.02% | 0.05% | 103% | 103% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 96% |
| Class B | 0.01% | -0.01% | -0.01% | 101% | 100% |
| Class C | 0.01% | 0.02% | 0.00% | 102% | 100% |
| Class E | 0.03% | 0.03% | 0.01% | 101% | 100% |
| Overall | 0.02% | 0.01% | 0.01% | 101% | 100% |
| Class D | 0.00% | -0.01% | -0.05% | 101% | 99% |
| Class F | -0.13% | -0.21% | -0.24% | 103% | 103% |
| TGM | -1.22% | -0.89% | -0.93% | 104% | 97% |
|  | Random Access |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.02% | 0.00% | 0.03% | 101% | 100% |
| Class C | 0.03% | 0.00% | 0.04% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.04% | 0.13% | 0.07% | 101% | 102% |
| Class F | 0.08% | -0.09% | -0.15% | 102% | 101% |
| TGM | -1.53% | -0.81% | -0.85% | 103% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.01% | -0.01% | 103% | 102% |
| Class C | 0.01% | 0.03% | 0.00% | 101% | 97% |
| Class E | 0.03% | -0.04% | -0.07% | 102% | 108% |
| Overall | 0.02% | 0.00% | -0.02% | 102% | 102% |
| Class D | 0.06% | -0.01% | 0.06% | 103% | 100% |
| Class F | 0.03% | -0.15% | -0.10% | 102% | 103% |
| TGM | -1.00% | -0.64% | -0.71% | 105% | 101% |

Table 8: Results of the CE7-3.9 for low QPs

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.00% | 100% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class B | 0.00% | -0.01% | 0.00% | 100% | 99% |
| Class C | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.01% | 0.03% | -0.03% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | -0.02% | -0.02% | -0.05% | 101% | 100% |
| Class F | -0.10% | -0.16% | -0.09% | 100% | 100% |
| TGM | -0.37% | -0.29% | -0.29% | 100% | 102% |
|  | Random Access |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class B | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class C | 0.00% | -0.02% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.01% | 100% | 100% |
| Class D | -0.04% | 0.02% | 0.02% | 101% | 100% |
| Class F | -0.12% | -0.07% | -0.13% | 100% | 101% |
| TGM | -0.81% | -0.37% | -0.35% | 101% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | -0.01% | 0.00% | -0.02% | 101% | 102% |
| Class E | -0.01% | 0.02% | -0.12% | 100% | 102% |
| Overall | -0.01% | 0.01% | -0.04% | 100% | 102% |
| Class D | -0.01% | 0.00% | 0.05% | 101% | 101% |
| Class F | -0.04% | 0.00% | -0.05% | 100% | 100% |
| TGM | -0.92% | -0.39% | -0.41% | 100% | 101% |

Table 9: Results of the CE7-3.10 for low QPs

|  | All Intra |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.06% | 0.02% |  | 0.04% |  | 104% |  | 105% |  |
| Class A2 | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 98% |  |
| Class B | 0.00% | -0.01% |  | 0.01% |  | 101% |  | 100% |  |
| Class C | 0.01% | 0.02% |  | 0.00% |  | 102% |  | 101% |  |
| Class E | 0.01% | 0.01% |  | 0.01% |  | 101% |  | 100% |  |
| Overall | 0.01% | 0.01% |  | 0.01% |  | 102% |  | 101% |  |
| Class D | 0.00% | -0.01% |  | -0.04% |  | 101% |  | 99% |  |
| Class F | -0.26% | -0.38% |  | -0.38% |  | 103% |  | 100% |  |
| TGM | -1.63% | -1.27% |  | -1.28% |  | 105% |  | 99% |  |
|  | Random Access |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class A2 | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class B | 0.02% | -0.01% |  | 0.01% |  | 101% |  | 99% |  |
| Class C | 0.02% | -0.03% |  | 0.00% |  | 102% |  | 100% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class D | 0.05% | 0.15% |  | 0.12% |  | 102% |  | 102% |  |
| Class F | -0.56% | -0.23% |  | -0.34% |  | 102% |  | 99% |  |
| TGM | -1.07% | -1.19% |  | -1.21% |  | 103% |  | 99% |  |
|  | Low delay B |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% |  | -0.01% |  | -0.01% |  | 102% |  | 103% |
| Class C | 0.00% |  | 0.03% |  | -0.03% |  | 102% |  | 101% |
| Class E | 0.02% |  | 0.01% |  | -0.03% |  | 102% |  | 104% |
| Overall | 0.01% |  | 0.01% |  | -0.02% |  | 102% |  | 103% |
| Class D | 0.04% |  | -0.03% |  | 0.03% |  | 103% |  | 99% |
| Class F | -0.22% |  | -0.13% |  | -0.26% |  | 103% |  | 103% |
| TGM | -1.19% |  | -0.95% |  | -1.03% |  | 104% |  | 102% |

Table 10: Results of the CE7-3.11 for low QPs

|  | All Intra |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.06% | 0.02% |  | 0.04% |  | 102% |  | 101% |  |
| Class A2 | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 96% |  |
| Class B | 0.00% | -0.01% |  | 0.00% |  | 101% |  | 100% |  |
| Class C | 0.01% | 0.03% |  | 0.00% |  | 102% |  | 101% |  |
| Class E | 0.01% | 0.02% |  | 0.00% |  | 101% |  | 100% |  |
| Overall | 0.01% | 0.01% |  | 0.01% |  | 101% |  | 100% |  |
| Class D | -0.02% | -0.01% |  | 0.02% |  | 102% |  | 104% |  |
| Class F | -0.39% | -0.43% |  | -0.47% |  | 103% |  | 102% |  |
| TGM | -1.94% | -1.55% |  | -1.57% |  | 105% |  | 100% |  |
|  | Random Access |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class A2 | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class B | 0.01% | -0.01% |  | -0.01% |  | 101% |  | 100% |  |
| Class C | 0.02% | -0.02% |  | 0.02% |  | 101% |  | 100% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class D | 0.04% | 0.16% |  | -0.01% |  | 102% |  | 102% |  |
| Class F | -0.92% | -0.30% |  | -0.33% |  | 101% |  | 100% |  |
| TGM | -1.74% | -1.44% |  | -1.45% |  | 103% |  | 99% |  |
|  | Low delay B |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% |  | 0.00% |  | -0.01% |  | 101% |  | 101% |
| Class C | 0.00% |  | -0.01% |  | -0.01% |  | 101% |  | 99% |
| Class E | 0.01% |  | 0.02% |  | -0.09% |  | 101% |  | 100% |
| Overall | 0.01% |  | 0.00% |  | -0.03% |  | 101% |  | 100% |
| Class D | 0.08% |  | 0.03% |  | 0.09% |  | 102% |  | 100% |
| Class F | -0.30% |  | -0.35% |  | -0.37% |  | 101% |  | 102% |
| TGM | -1.50% |  | -1.19% |  | -1.30% |  | 103% |  | 101% |

# Reference

- H. Schwarz, M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding”, JVET-N1027, 14th meeting, Geneva, CH, March 2019.

- M. Karczewicz, M. Coban, “CE8-related: Sign context modelling and level mapping for TS residual coding”, JVET-N0455, 14th meeting, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).