| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0123-v3 |
| --- | --- |

| Title: | CE4-related: Alignment of BDOF refinement process with PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Chong Soon Lim 202 Bedok South Ave 1 #02-11 Singapore, 469332 | Tel: Email: | +65 6552 5423 jingya.li@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to align BDOF refinement process with that of PROF. The proposed method has coding impact of 0.00% luma BD-rate in RA over VTM-5.0 anchor.

# Introduction

JVET-N0236 proposed to refine the sub-block based affine motion compensated prediction with optical flow. The proposed method is illustrated in equation (1) and (2), wherein Ii, j indicates affine prediction, I'(i,j) indicates refined prediction, and the difference is calculated by sum of product of horizontal/vertical gradient and displacement.

I'(i,j)= Ii,j+ΔI(i,j)												   (1)

ΔIi,j= Clip(-213, 213-1,  ((gxi,j*Δvxi,j+gyi,j*Δvyi,j+1)≫1))	   	   (2)

When the predicted affine block is bi-prediction, the refined prediction is determined as:

I'i,j= Clip 0, 1023, (I0i,j+Clip( -213, 213-1,  ((gx0i,j*Δvx0i,j+gy0i,j*Δvy0i,j+1)≫1))+I1i,j+ Clip(-213, 213-1,  ((gx1i,j*Δvx1(i,j)+gy1i,j*Δvy1(i,j))+1)≫1))+16400)≫5  		   			   														   (3)

This prediction process is similar to BDOF’s refinement process as below:

I'i,j=

Clip 0, 1023, (I0i,j+I1i,j+((( gx1i,j- gx0i,j)*Δvxi,j+(gy1i,j-gy0i,j)*Δvyi,j+1)≫1) +16400)≫5

(4)

This contribution proposes to align BDOF’s refinement process (4) with PROF’s process (3) so that both refinement processes could share same processing logics to reduce hardware costs. Equation (4) is modified as:

I'i,j= Clip 0, 1023,  (I0i,j+Clip(-213, 213-1,  (((-gx0i,j)*Δvxi,j+(-gy0i,j)*Δvyi,j+1)≫1))+I1i,j+Clip(-213, 213-1, ((gx1i,j*Δvxi,j+gy1i,j*Δvyi,j+1)≫1)+16400)≫5  																		   (5)

# Experimental results

Coding performance of the proposed method is reported as below. The CTC (common test conditions) defined in [1] was followed.

CTC Coding impact for both RA and LDB are negligible.

Table 1: Simulation Results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.12% | -0.03% | 101% | 97% |
| Class A2 | 0.01% | 0.01% | 0.01% | 104% | 100% |
| Class B | 0.01% | 0.00% | -0.04% | 99% | 100% |
| Class C | -0.02% | 0.03% | 0.01% | 108% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.01% | 103% | 99% |
| Class D | 0.00% | 0.01% | 0.05% | 99% | 95% |
| Class F | -0.01% | 0.04% | 0.02% | 99% | 95% |

# Discussions

This contribution aligns of BDOF’s refinement process with that of PROF. Negligible coding performance difference is found. Encoder and decoding time reported is not reliable.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text

Changes of specification text is as below:

  - The horizontal and vertical motion offset of the current subblock are derived as:

vx = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres,	(8-783)
	−( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) )  :  0

vy = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres, ( ( sGydI << 3 ) − 	(8-784)
	( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGx2 ) ) )  :  0

  - For x =xSb − 1..xSb + 2, y = ySb − 1..ySb + 2, the prediction sample values of the current sub-block are derived as follows:

–	bdofOffset = Clip3( −1<<13, (1<<13−1), ( ((vx * ( − gradientHL0[ x + 1 ][ y + 1 ]) + vy * (− gradientVL0[ x + 1 ][ y + 1 ] ) +1)>>1))

+ Clip3(−1<<13, (1<<13−1),  ( ((vx * gradientHL1[ x + 1 ][ y + 1 ] + vy * gradientVL1[ x + 1 ][ y + 1 ] +1)>>1))

-

[Ed. (JC): Round() operation is defined for float input. The Round() operation seems redundant here since the input is an integer value. To be confirmed by the proponent]

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ]  + offset4 +	(8-786)
	predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset )  >>  shift4 )