| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O00124-v1 |
| --- | --- |

| Title: | CE11/AHG14: Test 3.1 - an approach for support of GRA/GDR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Ye-Kui Wang Jianle Chen Seungwook Hong | Email: | fhendry@futurewei.com ye-kui.wang@futurewei.com jianle.chen@futurewei.com seungwook.hong@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

This contribution describes test results for CE 11 test 3.1. The test is designed to provide implementation for support of gradual random access (GRA) / gradual decoding refresh (GDR) based on using coded region method in which the refreshed region and the unrefreshed region are coded in different slices. It is asserted that other than the need to treat the refreshed region boundaries in reference pictures as picture boundaries in certain cases, the approach implemented in test 3.1 relies only on high-level syntax features.

# Introduction

At the 14th JVET meeting in Geneva, it was agreed to normatively support gradual random access (GRA) / gradual decoding refresh (GDR), and a new NAL unit type has been defined, together with signalling of the recovery point in the slice header, while whether to specify any other normative feature for GRA support has been left for study in CE 11.

This contribution describes test results for CE 11 test 3.1 that as originally proposed in JVET-N0115. The proposed method can be summarized as follows (only the parts not yet included in the VVC draft specification are mentioned below):

- Progressive intra refresh for GRA is performed based on intra coding of entire slices.

  - A GRA picture and the pictures that follow the GRA picture in decoding order and before the recovery point are coded with at least two slices: at least one slice for the refreshed region, and at least one slice for the unrefreshed region.

  - A GRA picture contains at least one slice with refreshed_region_flag equal to 1 that is intra coded.

  - Only pictures within the GRA period starting from the recovery point may have only one slice (they can certainly also have multiple slices).

- A flag called refreshed_region_flag to specify whether a slice is part of the refreshed region or not is signalled in the slice header. All slices having refreshed_region_flag equal to 1 shall cover a connected region.

- The boundaries of the refreshed region in a picture are derived based on the values of the refreshed_region_flag in the slices of the picture.

- For encoding/decoding of a slice in the refreshed region (referred to as a refreshed slice) of a GRA period, the following applies:

  - The slice does not refer to reference pictures that belong to a different GRA period (i.e., does not refer to reference picture that precede the GRA picture in decoding order).

  - Coding blocks within the slice can only refer to reference blocks that belong to the refreshed region in the reference picture. For this, the boundaries of the refreshed region in the reference picture are treated as picture boundaries.

- For encoding/decoding of a slice in the unrefreshed region (referred to as an unrefreshed slice) of a GRA period, the following applies:

  - The slice may refer to reference pictures that belong to same GRA period and/or reference pictures that belong to a different GRA period.

  - When a reference picture belongs to a different GRA period, coding blocks within the slice can only refer to reference blocks that belong to the refreshed region in the reference picture. For this, the boundaries of the refreshed region in the reference picture are treated as picture boundaries.

  - When a reference picture belongs to the same GRA period, there is no special treatment needed.

It is asserted that other than the need to treat the refreshed region boundaries as picture boundaries, the approach implemented in test 3.1 relies only on high-level syntax features.

The text changes for the proposed method are provided in the attachment of this contribution, based on JVET-N1001-v8.

# Handling of refreshed and unrefreshed regions

As described above, coding blocks within a refreshed slice can only refer to reference blocks that belong to the refreshed region in a reference picture that belongs to the same GRA period. Coding blocks within an unrefreshed slice can refer to reference blocks that belong to the refreshed region in a reference picture that belongs to a different GRA period or any reference blocks in a reference picture that belongs to the same GRA period. Figure 1 illustrates the constraints for the refreshed and unrefreshed regions for supporting GRA.

Figure 1 – Coding block referencing for refreshed and unrefreshed regions.

To apply the above constraints, there are two options that can be used. The first option is to apply constraints on the encoder such that it performs motion search only within the given region (i.e., the refreshed region) and to ensure that no sample value outside the given region is used for prediction. The second option is to treat the boundaries of the refreshed region in a reference picture as picture boundaries. However, simulation results reported by the previous CE12 showed that the second option provides better coding performance.

In the VTM reference software implementation, treating boundaries of a region (e.g., tiles or slices) as picture boundaries can be implemented by applying padding to the area in the picture that is outside the given region as illustrated in Figure 2. Note that padding surrounding the picture boundaries has been implemented in the current VTM software, not part of the implementation of the proposed method.

Figure 2 – Padding of the unrefreshed region and picture surrounding.

# Simulation results

## BD-rate performance

Simulation for CE11-3.1 is conducted with test condition as agreed in JVET-N1031 with modification that ALF is turned off. The ALF is turned off for this test because there we found problem with the feature when running the based software with multiple slices and also the implementation of loop filter across tile boundary enabling / disabling is not implemented yet for ALF.

Table 1 tabulates the performance of CE11-3.1 against the agreed anchor.

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 2.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 12.28% | 18.25% | 18.97% | 101% | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).