| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0125-v1 |
| --- | --- |

| Title: | Non-CE4: Alignment of DMVR padding process with specification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Chong Soon Lim 202 Bedok South Ave 1 #02-11 Singapore, 469332 | Tel: Email: | +65 6552 5423 jingya.li@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to align DMVR padding process with specification. The proposed method has coding impact of 0.00% luma BD-rate in RA over VTM-5.0 anchor.

# Introduction

In VTM5.0’s specification JVET-N01001(v8), DMVR generates a refined MV from a MV candidate, In order not to access more reconstruction samples from reference pictures than normal MC, a padding process is applied to each DMVR sub-block along the reconstruction block of size (sbWidth+7)*(sbHeight+7) wherein sbWidth = min(16, cbWidth) and sbHeight = min(16, cbHeight). An example is illustrated in Figure 1.

Figure 1: Example of sample padding of DMVR processfor a sub-block in specification

It is noted that VTM5.0 has a different implementation. More specifically, in VTM5.0, the padding process only applies on CU level instead of sub-blocks as shown in Figure 2. Compared with Figure 1, additional reconstruction samples are needed for MC of a sub-block.

Figure 2: Example of sample padding of DMVR process for a sub-block inVTM5.0 implementation

It is proposed to align implementation with specification. By doing this, when hardware retrieves memory of reconstructed samples for a DMVR sub-block, additional logic could be saved for determining area of additional reconstructed samples to retrieve.

# Experimental results

Coding performance of the proposed method is reported as below. The CTC (common test conditions) defined in [1] was followed.

CTC Coding impact for both RA and LDB are negligible.

Table 1: Simulation Results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.13% | 0.02% | 100% | 102% |
| Class A2 | 0.00% | 0.06% | 0.06% | 103% | 104% |
| Class B | 0.02% | 0.03% | 0.04% | 102% | 103% |
| Class C | -0.01% | 0.03% | -0.06% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.01% | 102% | 102% |
| Class D | 0.00% | -0.02% | -0.03% | 101% | 99% |
| Class F | 0.01% | 0.01% | -0.03% | 103% | 103% |

# Discussions

This contribution aligns implementation of DMVR padding process with specification. Negligible coding performance difference is found. Encoder and decoding time reported is not reliable.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).