| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0126-v2 |
| --- | --- |

| Title: | Non-CE4: Align coding of BPWA index with normal truncated rice binarization process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Chong Soon Lim 202 Bedok South Ave 1 #02-11 Singapore, 469332 | Tel: Email: | +65 6552 5423 jingya.li@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to align coding of BPWA index with normal truncated rice binarization process in VTM5.0. The proposed method has coding impact of 0.01% luma BD-rate in RA and 0.03% luma BD-rate in LDB over VTM-5.0 anchor.

# Introduction

In VTM5.0, BPWA index coding is the only case that uses zero-based truncated rice binarization as shown in table 1 & 2.

Table 1: coding of BPWA index when NoBackwardPredFlag = = 1 in VTM5.0

| Bin String | Value | BPWA index |
| --- | --- | --- |
| 1 | 0 | 2 |
| 01 | 1 | 3 |
| 001 | 2 | 1 |
| 0001 | 3 | 4 |
| 0000 | 4 | 0 |

Table 2: coding of BPWA index when NoBackwardPredFlag = = 0 in VTM5.0

| Bin String | Value | BPWA index |
| --- | --- | --- |
| 1 | 0 | 2 |
| 01 | 1 | 3 |
| 00 | 2 | 1 |

It is proposed to align coding of BPWA index with normal truncated rice binarization process as below:

Table 3: proposed coding of BPWA index when NoBackwardPredFlag = = 1

| Bin String | Value | BPWA index |
| --- | --- | --- |
| 0 | 0 | 2 |
| 10 | 1 | 3 |
| 110 | 2 | 1 |
| 1110 | 3 | 4 |
| 1111 | 4 | 0 |

Table 4: proposed coding of BPWA index when NoBackwardPredFlag = = 0

| Bin String | Value | BPWA index |
| --- | --- | --- |
| 0 | 0 | 2 |
| 10 | 1 | 3 |
| 11 | 2 | 1 |

CABAC initial value for the first bin of BPWA index is also updated.

# Experimental results

Coding performance of the proposed method is reported as below. The CTC (common test conditions) defined in [1] was followed.

CTC Coding impact for both RA and LDB are negligible.

Table 1: Simulation Results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.13% | -0.01% | 98% | 98% |
| Class A2 | 0.01% | 0.09% | 0.06% | 98% | 98% |
| Class B | 0.02% | 0.08% | -0.09% | 98% | 99% |
| Class C | -0.02% | -0.16% | 0.03% | 97% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.02% | -0.01% | 98% | 97% |
| Class D | 0.02% | -0.09% | 0.08% | 94% | 92% |
| Class F | 0.00% | -0.04% | -0.04% | 100% | 94% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.29% | 0.26% | 99% | 98% |
| Class C | 0.08% | -0.03% | 0.04% | 105% | 105% |
| Class E | 0.01% | -0.03% | 0.18% | 98% | 98% |
| Overall | 0.03% | 0.10% | 0.16% | 101% | 100% |
| Class D | 0.01% | -0.14% | 0.28% | 102% | 101% |
| Class F | 0.06% | 0.11% | -0.14% | 101% | 101% |

# Discussions

This contribution aligns BPWA index coding with normal truncated binarization process. No coding performance difference is found and encoder and decoding time impact is negligible.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| for( i = 0; i < NumRefIdxActive[ 0 ]; i++ ) |  |
| luma_weight_l0_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < NumRefIdxActive[ 0 ]; i++ ) |  |
| chroma_weight_l0_flag[ i ] | u(1) |
| for( i = 0; i < NumRefIdxActive[ 0 ]; i++ ) { |  |
| if( luma_weight_l0_flag[ i ] ) { |  |
| delta_luma_weight_l0[ i ] | tu(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_weight_l0_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | tu(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| if( slice_type = = B ) { |  |
| for( i = 0; i < NumRefIdxActive[ 1 ]; i++ ) |  |
| luma_weight_l1_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < NumRefIdxActive[ 1 ]; i++ ) |  |
| chroma_weight_l1_flag[ i ] | u(1) |
| for( i = 0; i < NumRefIdxActive[ 1 ]; i++ ) { |  |
| if( luma_weight_l1_flag[ i ] ) { |  |
| delta_luma_weight_l1[ i ] | tu(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_weight_l1_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | tu(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |