      - 7.4.8.4	Coding tree semantics

When all of the following conditions are true, IsInSmr[ x ][ y ] is set equal to TRUE for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

- IsInSmr[ x0 ][ y0 ] is equal to FALSE

- cbWidth * cbHeight / 4 is less than 32

- treeType is not equal to DUAL_TREE_CHROMA

When IsInSmr[ x0 ][ y0 ] is equal to TRUE. the arrays SmrX[ x ][ y ], SmrY[ x ][ y ], SmrW[ x ][ y ] and SmrH[ x ][ y ] are derived as follows for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

SmrX[ x ][ y ] = x0		(7-96)

SmrY[ x ][ y ] = y0		(7-97)

SmrW[ x ][ y ] = cbWidth		(7-98)

SmrH[ x ][ y ] = cbHeight		(7-99)

The variables allowSplitQt, allowSplitBtVer, allowSplitBtHor, allowSplitTtVer, and allowSplitTtHor are derived as follows:

- The allowed quad split process as specified in clause 6.4.1 is invoked with the coding block size cbSize set equal to cbWidth, the current multi-type tree depth mttDepth, and treeType as inputs, and the output is assigned to allowSplitQt.

- The variables maxBtSize, maxTtSize and maxMttDepth are derived as follows:

- If treeType is equal to DUAL_TREE_CHROMA, maxBtSize, maxTtSize and maxMttDepth are set equal to MaxBtSizeC, MaxTtSizeC and MaxMttDepthC + depthOffset, respectively.

- Otherwise, maxBtSize, maxTtSize and maxMttDepth are set equal to MaxBtSizeY, MaxTtSizeY and MaxMttDepthY + depthOffset, respectively.

- The allowed binary split process as specified in clause 6.4.2 is invoked with the binary split mode SPLIT_BT_VER, the coding block width cbWidth, the coding block height cbHeight, the location ( x0, y0 ), the current multi-type tree depth mttDepth, the maximum multi-type tree depth with offset maxMttDepth, the maximum binary tree size maxBtSize, the current partition index partIdx, and treeType as inputs, and the output is assigned to allowSplitBtVer.

- The allowed binary split process as specified in clause 6.4.2 is invoked with the binary split mode SPLIT_BT_HOR, the coding block height cbHeight, the coding block width cbWidth, the location ( x0, y0 ), the current multi-type tree depth mttDepth, the maximum multi-type tree depth with offset maxMttDepth, the maximum binary tree size maxBtSize, the current partition index partIdx, and treeType as inputs, and the output is assigned to allowSplitBtHor.

- The allowed ternary split process as specified in clause 6.4.3 is invoked with the ternary split mode SPLIT_TT_VER, the coding block width cbWidth, the coding block height cbHeight, the location ( x0, y0 ), the current multi-type tree depth mttDepth, the maximum multi-type tree depth with offset maxMttDepth, the maximum ternary tree size maxTtSize, and treeType as inputs, and the output is assigned to allowSplitTtVer.

- The allowed ternary split process as specified in clause 6.4.3 is invoked with the ternary split mode SPLIT_TT_HOR, the coding block height cbHeight, the coding block width cbWidth, the location ( x0, y0 ), the current multi-type tree depth mttDepth, the maximum multi-type tree depth with offset maxMttDepth, the maximum ternary tree size maxTtSize, and treeType as inputs, and the output is assigned to allowSplitTtHor.

split_cu_flag equal to 0 specifies that a coding unit is not split. split_cu_flag equal to 1 specifies that a coding unit is split into four coding units using a quad split as indicated by the syntax element split_qt_flag, or into two coding units using a binary split or into three coding units using a ternary split as indicated by the syntax element mtt_split_cu_binary_flag. The binary or ternary split can be either vertical or horizontal as indicated by the syntax element mtt_split_cu_vertical_flag.

When split_cu_flag is not present, the value of split_cu_flag is inferred as follows:

- If one or more of the following conditions are true, the value of split_cu_flag is inferred to be equal to 1:

- x0 + cbWidth is greater than pic_width_in_luma_samples.

- y0 + cbHeight is greater than pic_height_in_luma_samples.

- Otherwise, the value of split_cu_flag is inferred to be equal to 0.

split_qt_flag specifies whether a coding unit is split into coding units with half horizontal and vertical size.

When split_qt_flag is not present, the following applies:

- If allowSplitQt is equal to TRUE, the value of split_qt_flag is inferred to be equal to 1.

- Otherwise, the value of split_qt_flag is inferred to be equal to 0.

mtt_split_cu_vertical_flag equal to 0 specifies that a coding unit is split horizontally. mtt_split_cu_vertical_flag equal to 1 specifies that a coding unit is split vertically

When mtt_split_cu_vertical_flag is not present, it is inferred as follows:

- If allowSplitBtHor is equal to TRUE or allowSplitTtHor is equal to TRUE, the value of mtt_split_cu_vertical_flag is inferred to be equal to 0.

- Otherwise, the value of mtt_split_cu_vertical_flag is inferred to be equal to 1.

mtt_split_cu_binary_flag equal to 0 specifies that a coding unit is split into three coding units using a ternary split. mtt_split_cu_binary_flag equal to 1 specifies that a coding unit is split into two coding units using a binary split.

When mtt_split_cu_binary_flag is not present, it is inferred as follows:

- If allowSplitBtVer is equal to FALSE and allowSplitBtHor is equal to FALSE, the value of mtt_split_cu_binary_flag is inferred to be equal to 0.

- Otherwise, if allowSplitTtVer is equal to FALSE and allowSplitTtHor is equal to FALSE, the value of mtt_split_cu_binary_flag is inferred as to be equal to 1.

- Otherwise, if allowSplitBtHor is equal to TRUE and allowSplitTtVer is equal to TRUE, the value of mtt_split_cu_binary_flag is inferred to be equal to !mtt_split_cu_vertical_flag.

- Otherwise (allowSplitBtVer is equal to TRUE and allowSplitTtHor is equal to TRUE), the value of mtt_split_cu_binary_flag is inferred to be equal to mtt_split_cu_vertical_flag.

The variable MttSplitMode[ x ][ y ][ mttDepth ] is derived from the value of mtt_split_cu_vertical_flag and from the value of mtt_split_cu_binary_flag as defined in Table 7-7 for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1.

Figure 7-1 – Multi-type tree spliting modes indicated by MttSplitMode (informative)

MttSplitMode[ x0 ][ y0 ][ mttDepth ] represents horizontal and vertical binary and ternary splittings of a coding unit within the multi-type tree as illustrated in Figure 7-1. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

Table 7-7 – Specification of MttSplitMode[ x ][ y ][ mttDepth ] for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1

| MttSplitMode[ x0 ][ y0 ][ mttDepth ] | mtt_split_cu_vertical_flag | mtt_split_cu_binary_flag |
| --- | --- | --- |
| SPLIT_TT_HOR | 0 | 0 |
| SPLIT_BT_HOR | 0 | 1 |
| SPLIT_TT_VER | 1 | 0 |
| SPLIT_BT_VER | 1 | 1 |

When all of the following conditions are true, IsInSmr[ x ][ y ] is set equal to TRUE for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

- IsInSmr[ x0 ][ y0 ] is equal to FALSE

- One of the following conditions is true:

- mtt_split_cu_binary_flag is equal to 1 and  cbWidth * cbHeight / 2 is less than 32

- mtt_split_cu_binary_flag is equal to 0 and  cbWidth * cbHeight / 4 is less than 32

- treeType is not equal to DUAL_TREE_CHROMA

When IsInSmr[ x0 ][ y0 ] is equal to TRUE. the arrays SmrX[ x ][ y ], SmrY[ x ][ y ], SmrW[ x ][ y ] and SmrH[ x ][ y ] are derived as follows for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

SmrX[ x ][ y ] = x0		(7-100)

SmrY[ x ][ y ] = y0		(7-101)

SmrW[ x ][ y ] = cbWidth		(7-102)

SmrH[ x ][ y ] = cbHeight		(7-103)

The variable cclmEnabledBtHor is derived as follows:

- If one of the following conditions is true, the value of cclmEnabledBtHor is set equal to TRUE:

- If treeType is equal to DUAL_TREE_CHROMA, cbWidth and cbHeight are equal to 32 and 16, respectively, and split_cu_flag is equal to 0.

- If treeType is equal to DUAL_TREE_CHROMA, cbWidth and cbHeight are equal to 32 and 16, respectively, and mtt_split_cu_vertical_flag and mtt_split_cu_binary_flag  are both equal to 1.

- Otherwise, the value of cclmEnabledBtHor is set equal to FALSE.

    - 8.4.4	Derivation process for chroma intra prediction mode

Input to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived.

A variable cclmEnabled is derived by invoking the sub-clause x.x.x Cross-component chroma intra prediction mode checking process.

The corresponding luma intra prediction mode lumaIntraPredMode is derived as follows:

- If intra_mip_flag[ xCb ][ yCb ] is equal to 1, lumaIntraPredMode is derived using IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] and sizeId as specified in Table 8-4 and assigning the value of candIntraPredModeX to lumaIntraPredMode.

- Otherwise, lumaIntraPredMode is set equal to IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ].

The chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode as specified in Table 8-5 and Table 8-6.

Table 8-5 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flag is equal to 0

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 0 | 50 | 18 | 1 | X |

Table 8-6 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flag is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 81 | 81 | 81 | 81 | 81 |
| 5 | 82 | 82 | 82 | 82 | 82 |
| 6 | 83 | 83 | 83 | 83 | 83 |
| 7 | 0 | 50 | 18 | 1 | X |

### x.x.x	Cross-component chroma intra prediction mode checking process

Input to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left luma sample of the current picture,

- a variable cqtDepthL specifying the cqtDepth of the collocated luma coding block covering the location ( xCb, yCb ),

- a variable cqtDepthC specifying the cqtDepth of the current chroma coding block,

- two variables cbWidthL and cbHeightL specifying the width and height of the collocated luma coding block,

- two variables cbWidthC and cbHeightC specifying the width and height of the current chroma block.

- a variable cclmEnabledBtHor specifying whether CCLM can be enabled when chroma VPDU is firstly binary horizontal split

Output to this process is:

- a flag cclmEnabled specifying if a cross-component chroma intra prediction mode is enabled or not for the current chroma coding block.

In this process, cclmEnabled is derived as follows.

- If sps_cclm_enabled_flag is equal to 0, cclmEnabled is set equal to 0.

- Otherwise the following applies.

  - cclmEnabled is set equal to 0.

  - If one of the following conditions is true, cclmEnabled is set equal to 1.

  - Both cqtDepthL and cqtDepthC are larger than or equal to 2.

  - Both cqtDepthL and cqtDepthC are equal to 1, and both cbWidthL and cbHeightL are equal to 32, and both cbWidthC and cbHeightC are equal to 16

  - cqtDepthL is larger than or equal to 2, and cqtDepth is equal to 1, and both cbWidthC and cbHeightC are equal to 32.

  - cqtDepthL is larger than or equal to 2, and cqtDepthC is equal to 1, and cclmEnabledBtHor is TRUE.