| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0131 |
| --- | --- |

| Title: | CE10-1.10/CE10-1.11: Evaluation results of CNN-based filtering with on-line learning model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura Kyohei Unno Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-6887-2592 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution reports the results by the convolutional neural network-based filtering (CNNF) with on-line learning model which was proposed in JVET-N0712. Simulation results reportedly show the BD-rates for luma in Class A1/A2 by -0.16/-0.17%, -1.39/-0.44%, -2.35/-0.43% and -2.35/0.04% in CE10-1.10a, CE10-1.10b, CE10-1.11a and CE10-1.11b, respectively. These differences are caused by bit-depth of CNNF parameter and the difference between in-loop and post processing filter. On the other hand, Simulation results reportedly show the BD-rates for luma in Class D by 0.42%, 6.66%, 6.14% and 6.59% in CE10-1.10a, CE10-1.10b, CE10-1.11a and CE10-1.11b, respectively.

Introduction

A convolutional neural network-based filtering (CNNF) with on-line learning model was proposed in JVET-N0712 [1]. This contribution reports the experimental results followed by the test conditions of Core Experiment 10 (CE10) description [2]. One of the CE10 purpose is to study the impact of CNNF parameter. Another purpose is to study the difference between in-loop and post processing filter. It is noted that experiments are performed only the first intra period (without second intra picture) to reduce the experimental burden for the bit-depth investigation.

Overview of the proposed CNNF

In this contribution, the boundary-based CNNF weight derivation is proposed based on JVET-M0159 [3]. The network structure is the same as JVET-M0159 but the method of learning and applying CNNF is different. In the applying stage, the weight is used for sum of residual signals by the CNNF and the signals just after deblocking filter to reduce the learning time. The basic weight is derived from the distance from the block boundary. We reuse the boundary definition and its strength derivation from the deblocking filter operation. When the boundary strength is larger than 0, weight is further modified by the block types like intra or inter coding. Figure 1 shows the actual weights for both intra and inter blocks. Thick and thin lines illustrate the block and pixel boundary. It is noted that the fair comparison between JVET-M0159 and this contribution is difficult under the current situation because the learning conditions is different.

Figure 1 Derived weights for intra (left) and inter (right) blocks when the BS is significant.

Experimental Results

Test condition

The proposed methods have been implemented on top of VTM-5.0 [4]. The common test conditions are used for evaluation [5] but the simulation results for CE10-1.10a/b are conducted in only Random Access 10 condition by the short frame which is defined as first intra period in JVET-N1030 [2].

The test conditions consist total of 4 items as described in Table 1. CE10-1.10a/b is the evaluation for the impact of CNNF bit-depth. CE10-1.11a/b is the evaluation for the difference between in-loop and post processing filter. It is noted that the CNNF is adaptively switching on/off in picture/CTU/32x32 block level in CE10-1.10a/b and CE10-1.11b (in-loop filtering), though CNNF is applied always in CE10-1.11b (post filtering). It is also noted that the bit-depth in CNNF parameter of CE10-1.11a/b is same as CE10-1.10b.

Table 1 Test conditions according to CE13 description

| CE10-1.10a | bit-depth | 6-bit (int) weights |
| --- | --- | --- |
| CE10-1.10b |  | 14-bit (int) weights |
| CE10-1.11a | in-loop / post filtering | CNNF as in loop filter |
| CE10-1.11b |  | CNNF as post filter |

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Simulation results

It is noted that encoding runtime does not include the learning time.

    - CE10-1.10a (6bit, in-loop filtering, 1rap)

Table 2 shows the simulation results of CE10-1.10a.

Table 2 The simulation results of CE10-1.10a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | 4.09% | -2.79% | 101% | 151% |
| Class A2 | -0.17% | -3.57% | -3.48% | 100% | 140% |
| Class B | 0.05% | -2.09% | -2.76% | 100% | 118% |
| Class C | 0.34% | -1.41% | -0.45% | 100% | 114% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.97% | -2.30% | 100% | 127% |
| Class D | 0.42% | 0.25% | 0.41% | 100% | 108% |
| Class F | 0.94% | -2.79% | -1.90% | 100% | 129% |

    - CE10-1.10b (14bit, in-loop filtering, 1rap)

Table 3 shows the simulation results of CE10-1.10b.

Table 3 The simulation results of CE10-1.10b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.39% | -5.52% | -12.38% | 100% | 219% |
| Class A2 | -0.44% | -16.37% | -11.83% | 100% | 182% |
| Class B | 0.45% | -22.30% | -20.03% | 100% | 210% |
| Class C | 1.69% | -12.85% | -8.01% | 100% | 155% |
| Class E |  |  |  |  |  |
| Overall | 0.23% | -15.24% | -13.66% | 100% | 190% |
| Class D | 6.66% | -4.53% | -3.72% | 100% | 132% |
| Class F | 3.37% | -9.10% | -6.26% | 100% | 177% |

    - CE10-1.11a (14bit, in-loop filtering, full-frame)

Table 4 shows the simulation results of CE10-1.11a.

Table 4 The simulation results of CE10-1.11a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.35% | -6.29% | -15.18% | 99% | 216% |
| Class A2 | -0.43% | -16.57% | -12.85% | 99% | 181% |
| Class B | -0.03% | -22.92% | -20.87% | 99% | 204% |
| Class C | 1.96% | -12.48% | -9.78% | 99% | 165% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -15.54% | -15.17% | 99% | 190% |
| Class D | 6.14% | -4.26% | -4.84% | 100% | 123% |
| Class F | 3.92% | -11.78% | -10.47% | 99% | 187% |

    - CE10-1.11b (14bit, post-filtering, full-frame)

Table 5 shows the simulation results of CE10-1.11b. It is noted that the results include the code amount for learning model which is placed on SEI.

Table 5 The simulation results of CE10-1.11b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.34% | -2.66% | -9.02% | 100% | 219% |
| Class A2 | 0.04% | -5.45% | -3.50% | 100% | 213% |
| Class B | 0.42% | -7.45% | -7.88% | 100% | 218% |
| Class C | 2.15% | -3.50% | -3.31% | 100% | 203% |
| Class E |  |  |  |  |  |
| Overall | 0.25% | -5.04% | -6.01% | 100% | 213% |
| Class D | 6.59% | 3.47% | 3.06% | 100% | 186% |
| Class F | 4.49% | 1.53% | 1.61% | 100% | 261% |

Conclusion

This contribution reported the experimental results followed by the test conditions of Core Experiment 10 (CE10) description [2]. Simulation results reportedly show the BD-rates for luma in Class A1/A2 by -0.16/-0.17%, -1.39/-0.44%, -2.35/-0.43% and -2.35/0.04% in CE10-1.10a, CE10-1.10b, CE10-1.11a and CE10-1.11b, respectively. (These differences are caused by bit-depth of CNNF parameter and the difference between in-loop and post processing filter. On the other hand, Simulation results reportedly show the BD-rates for luma in Class D by 0.42%, 6.66%, 6.14% and 6.59% in CE10-1.10a, CE10-1.10b, CE10-1.11a and CE10-1.11b, respectively. This means that it is a trade off between the coding performance improvement effect by CNNF and the code amount of the side information necessary for transmitting the learning model. Since the code amount of side information in class D is larger for the target image, it is considered that coding loss can be observed.

References

- K. Kawamura, Y. Kidani, S. Naito, “CE13-related: Adaptive CNN based in-loop filtering with boundary weights”, JVET-M0712, Geneva, CH, March 2019.

- Y. Li, S. Liu, K. Kawamura, “Description of Core Experiment 10 (CE10): Neural Network based Filter for Video Coding”, JVET-N1030, Geneva, CH, March 2019.

- Y.-L. Hsiao, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, and S.-M. Lei, “AHG9: Convolution neural network loop filter,” JVET-M0159, Marrakech, MA, January. 2019.

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 5 (VTM 5)”, JVET-N1002, Geneva, CH, March 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, March 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).