| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothernburg, SE, 3–12 July 2019 | Document: JVET-O0133-v2 |
| --- | --- |

| Title: | AHG8: Support for reference picture resampling - handling of picture size signalling, conformance windows, and DPB management |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seungwook Hong Ye-Kui Wang Jianle Chen (Futurewei) Tsui-Shan Chang Yu-Chen Sun Ling Zhu Jian Lou (Alibaba) | Tel: Email: | fhendry@futurewei.com seungwook.hong@futurewei.com ye-kui.wang@futurewei.com jianle.chen@futurewei.com tsuishan.cts@alibaba-inc.com yuchen.s@alibaba-inc.com qianling.zl@alibaba-inc.com j.lou@alibaba-inc.com |
| Source: | Futurewei Technologies, Inc. and Alibaba Group |  |  |

_____________________________

# Abstract

For support of the reference picture resampling (RPR) feature in VVC, the following aspects are proposed in this contribution:

- When RPR is enabled, additional picture sizes and conformance windows are signalled in the SPS. In the PPS, an index to the list of picture resolutions and conformance windows is signalled to specify the size of pictures referring to the PPS.

- For DPB management, reference pictures of different sizes are stored in the same DPB. For the determination of DPB fullness, each decoded picture is associated with the PictureSizeUnit variable that specifies the relative size of each picture. When a picture is added into the DPB, DPB fullness is incremented by PictureSizeUnit of the added picture; when a picture is removed from the DPB, DPB fullness is decremented by PictureSizeUnit of the removed picture.

The proposed text changes are included in an attachment to this contribution, with change marks relative to JVET-N1001-v8.

Note that JVET-O0134 includes a discussion and proposal on resampling, TMVP, DMVR, and BDOF for RPR support in VVC.

In v2 of this contribution, the attached specification text has been modified for the rounding process in the resampling process, to match what's implemented in the software, which is included as an attachment in JVET-O0134.

# Introduction

Methods for supporting reference picture resampling (RPR), which is also referred to as adaptive resolution change (ARC), has been studied for inclusion into VVC specification. At the 14th JVET meeting in Geneva, several contributions on RPR were submitted and from the discussion during the meeting it was agreed in principle that some support for RPR should be provided in VVC. The relevant meeting notes from the Geneva meeting are quoted as follows:

Decision: It was agreed in principle to use the following (at least as an initial approach):

- Having the resolution of the stored reference picture being different from that of the current picture

- A range of a factor of 4 in each dimension was suggested.

For further discussion to be performed in AHG8.

In this contribution, several aspects for supporting the above principle agreement are provided. The required modifications to the VVC draft specification have been implemented on top of JVET-N1001-v8 and are provided as an attachment to this contribution.

# Proposals

## Signalling of picture sizes

In the current VVC WD, picture size is signalled as follows:

- All pictures within the same coded video sequence (CVS) have the same picture size. Therefore, the picture size information is signalled in the SPS. The related syntax elements for picture size in the SPS are pic_width_in_luma_samples and pic_height_in_luma_samples.

- The signalled syntax elements are then used to derive variables such as PicWidthInCtbsY, PicHeightInCtbsY, PicSizeInCtbsY, PicWidthInMinCbsY, PicHeightInMinCbsY, PicSizeInMinCbsY, PicSizeInSamplesY, PicWidthInSamplesC, and PicHeightInSamplesC.

To support RPR, additional picture sizes need to be signalled and also the derivation of variables related to picture size should be moved out from the SPS. For that, we propose the following:

- When RPR is enabled, signal additional picture widths and heights in the SPS. Together with the existing signalling of picture size (i.e., pic_width_in_luma_samples and pic_height_in_luma_samples), lists associated with picture sizes (i.e., PicWidthInLumaSamples[ i ] and PicHeightInLumaSamples[ i ]) are derived.

  - The additional picture widths and heights are signalled in term of delta values for better signalling efficiency.

  - For further bits saving, pic_width_in_luma_samples and pic_height_in_luma_samples are associated with the picture width and picture height of the smallest picture size that may be present in the CVS.

- Signal an index, named pps_pic_size_idx, in the PPS to specify the index into the lists PicWidthInLumaSamples and PicHeightInLumaSamples. Based on the index, variables such as PicWidthInCtbsY, PicHeightInCtbsY, PicSizeInCtbsY, PicWidthInMinCbsY, PicHeightInMinCbsY, PicSizeInMinCbsY, PicSizeInSamplesY, PicWidthInSamplesC, and PicHeightInSamplesC are then derived.

## Handling of conformance windows

Similar to the signalling of picture size, in the current VVC WD, there is only one set of conformance window syntax elements signalled in the SPS. When multiple picture sizes are supported, it is straightforward that each picture size is associated with its own conformance windows. Based on that, we propose the following:

- When RPR is enabled, signal additional conformance windows in the SPS. Together with the existing signalling of conformance window (i.e., conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset), lists associated with conformance windows (i.e., ConfWinLeftOffset[ i ], ConfWinRightOffset[ i ], ConfWinTopOffset[ i ], and ConfWinBottomOffset[ i ]) are derived.

- Each entry in ConfWinLeftOffset[ i ], ConfWinRightOffset[ i ], ConfWinTopOffset[ i ], and ConfWinBottomOffset[ i ] correspond to PicWidthInLumaSamples[ i ] and PicHeightInLumaSamples[ i ]. In this way, the index pps_pic_size_idx is also used to refer the conformance window lists to specify the conformance window associated with pictures referring the PPS.

## Handling of DPB fullness management

Currently the DPB fullness is managed as follows: when a picture is added into the DPB, the DPB fullness is incremented by one; likewise, when a picture is removed from the DPB, the DPB fullness is decremented by one. Such management works when all the pictures in the CVS have the same size. However, with RPR, there can be multiple picture sizes of decoded pictures present in the CVS, which consequently means decoded pictures of different sizes may be stored in the DPB, and the current DPB fullness management does not work.

When pictures with different sizes may be stored in the DPB, pictures with a smaller size should be considered occupying less DPB's space to avoid waste of DPB memory. To achieve that, we propose the following:

- Each picture is associated with a variable PictureSizeUnit. The variable can be thought as how much space the picture will take when stored in the DPB. A picture with a smaller size has a smaller PictureSizeUnit.

- PictureSizeUnit is derived based on the ratio of the size of the picture to the base picture size. The base picture size is computed from the greatest common divisor (GCD) of all picture sizes that may be present in the CVS. For example, when pictures with sizes 960x540, 1920x1080, and 3840x2160 are present in the CVS, the base picture size is the GCD of all the three sizes, which is 960x540. Consequently, pictures with size 960x540 are associated with PictureSizeUnit equal to 1, pictures with size 1920x1080 are associated with PictureSizeUnit equal to 4, and pictures with size 3840x2160 are associated with PictureSizeUnit 16.

- PictureSizeUnit is used as follows:

  - When a picture is added into the DPB, the DPB fullness is incremented by PictureSizeUnit of the added picture.

  - When a picture is removed from the DPB, the DPB fullness is decremented by PictureSizeUnit of the removed picture.

# Patent rights declaration(s)

Futurewei Technologies, Inc. and Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).