| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothernburg, SE, 3–12 July 2019 | Document: JVET-O0134-v2 |
| --- | --- |

| Title: | AHG8: Support for reference picture resampling - handling of resampling, TMVP, DMVR, and BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsui-Shan Chang Yu-Chen Sun Ling Zhu Jian Lou (Alibaba) Hendry Seungwook Hong Ye-Kui Wang Jianle Chen (Futurewei) | Tel: Email: | tsuishan.cts@alibaba-inc.com yuchen.s@alibaba-inc.com qianling.zl@alibaba-inc.com j.lou@alibaba-inc.com fhendry@futurewei.com seungwook.hong@futurewei.com ye-kui.wang@futurewei.com jianle.chen@futurewei.com |
| Source: | Alibaba Group and Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

For support of the reference picture resampling (RPR) feature in VVC, the following aspects for handling of coding tools are proposed in this contribution:

- Reference pictures are stored in the DPB in its original size. Block-based resampling is applied when the resolutions of the current picture and the reference picture are different. The resampling reuses the existing interpolation filter.

- TMVP may be enabled when RPR is enabled; however, it is constrained that a reference picture with a resolution different from the resolution of the current picture shall not be used as the collocated picture for slices in the current picture.

- DMVR may be enabled when RPR is enabled; however, the use of DMVR for the current picture using a reference picture is disallowed when the resolutions of the current picture and the reference picture are different.

- BDOF may be enabled when RPR is enabled; however, the use of BDOF for the current picture using a reference picture is disallowed when the resolutions of the current picture and the reference picture are different.

The proposed text changes are included in an attachment to JVET-O0133, with change marks relative to JVET-N1001-v8. The simulation results and the source code of the software are included as attachments to this contribution.

Note that JVET-O0133 includes a discussion and proposal on picture size signalling, conformance windows, and DPB management for RPR support in VVC.

In v2 of this contribution, the attached source code of the software has been updated, for some minor bug fixes and clean-ups. These updates do not affect the simulations, thus the simulation results remain the same as in v1 of this contribution.

# Introduction

Methods for supporting reference picture resampling (RPR), which is also referred to as adaptive resolution change (ARC), has been studied for inclusion into VVC specification. At the 14th JVET meeting in Geneva, several contributions on RPR were submitted and from discussion during the meeting it was agreed in principle that some support for RPR should be provided in VVC. The relevant meeting notes from the Geneva meeting are quoted as follows:

Decision: It was agreed in principle to use the following (at least as an initial approach):

- Having the resolution of the stored reference picture being different from that of the current picture

- A range of a factor of 4 in each dimension was suggested.

For further discussion to be performed in AHG8.

From the discussion in the Geneva meeting, there are issues related to coding tools that needs to be addressed to support RPR. The issues includes:

- Handling of the resampling process, including whether it should be picture-based or block-based.

- Simplification of the use of certain coding tools. It was argued that the implementation of certain coding tools may not be possible or at least become more complicated when the resolutions of the current picture and the reference pictures are different.

In this contribution, we propose solutions to address the above issues. The required modifications to the VVC draft specification have been implemented on top of JVET-N1001-v8 and are provided as an attachment to JVET-O0133.

# Proposals

## On handling of the resampling process

For picture-based resampling, both the resampled version and the original version of the reference picture are stored in the DPB when the reference picture is resampled. A resampled reference picture is marked as "unused for reference" when the corresponding un-resampled reference picture is marked as "unused for reference".

For block-based resampling, reference pictures are stored in the DPB in the original size. When the resolutions of the current picture and the reference picture are different, several steps are applied to simply reuse the interpolation of the motion compensation. (1) Calculating the resolution scaling factor between the current picture and the reference picture. (2) Deriving the sub-pel locations in the reference picture pointed to by the MV according to the resolution scaling factor. (3) Applying normal interpolation of the motion compensation to get the target pixels.

The issue of whether to specify a picture-based or block-based resampling process for RPR relates to memory usage and computation resource requirements. Picture-based resampling requires relatively lower computational resource when compared to block-based resampling, since a reference picture may be resampled only once, stored in the DPB, and the resampled reference picture can be referred to by one or more pictures for inter prediction. However, since the resampled reference picture may need to be stored in the DPB in addition to the original decoded picture, picture-based resampling requires higher memory usage. Furthermore, DPB management needs to be updated in order to be able to handle the storage of both the original and resampled versions of some decoded pictures.

Block-based resampling, on the other hand, requires no additional memory and very little update to the current DPB management, since there is no additional copy of reference picture that needs to be stored in the DPB. The only update needed to DPB management is the management of DPB fullness to avoid memory waste. This issue is addressed in the JVET-O0133.

On the other hand, block-based resampling requires higher computational resource, as a reference coding block may need to be resampled more than once. The computation needed can be heavy at the encoder side during motion estimation. However, resampling at the encoder side may be implemented picture-based.

Based on the reasons above, we propose to specify a block-based resampling process for support of RPR in VVC where the resampling filter reuses the existing interpolation filter. For this, the text changes to the VVC draft specification is localized only in clause 8.5.6.3.1 within the section that handles fractional sample interpolation process.

## Handling of TMVP, DMVR, and BDOF

At the 14th JVET meeting in Geneva, JVET-N0118 and JVET-N0279 proposed that a simplification to the use of some coding tools is needed. JVET-N0118 suggested that temporal motion vector scaling is applied as needed. JVET-N0279 suggested the following:

- When slice_temporal_mvp_enabled_flag is equal to 1, the current picture and its collocated picture shall have the same size.

- When resolution change is allowed within a sequence, DMVR shall be turned off.

- When resolution change is allowed within a sequence, sps_bdof_enabled_flag shall be equal to 0.

Regarding the simplification to the use of the above three coding tools (i.e., TMVP, DMVR, and BDOF), we agree with the constraint proposed by JVET-N0279 for TMVP. Editorially, we can express the constraint such that a reference picture with a resolution different from the resolution of the current picture is not used as the collocated picture for slices in the current picture.

On the other hand, regarding the use of DMVR and BDOF, we think that the constraints suggested by JVET-N0279 are a bit too restrictive than necessary and could be relaxed. It is not needed to disabled the use of DMVR and BDOF for all coding of all pictures in the CVS when RPR is enabled. In practice, when RPR is enabled, not all pictures refer to reference pictures with different resolutions and for such pictures the use of DMVR and BDOF should be allowed to achieve better coding performance.

We therefore propose the following constraints for the use of DMVR and BDOF:

- DMVR is disabled for a CU using a reference picture when the resolutions of the current picture and the reference picture are different.

- BDOF is disabled for a CU using a reference picture when the resolutions of the current picture and the reference picture are different.

# Simulation results

The RPR software was implemented on top of VTM-5.0. Picture-based resampling and block-based resampling are both supported in the software. For studying this type of use cases, including video telephony and conferencing applications in which the bit rate may need to be reduced during certain period to adapt to the changes in network bandwidth, we did a simulation as follows:

- For the anchor, we ran VTM-5.0 with the low-delay configuration and inserted an IDR picture whenever a resolution change is needed.

- For RPR, we ran the modified software with an IDR picture only at the beginning of the bitstream, while an inter-coded picture referring to a reference picture with a different spatial resolution was inserted when a resolution change is needed.

- To simulate the network bandwidth change, the spatial resolution of coded pictures was reduced according to Table 1. The resolution was set to change at the 3rd second and restored back to its original width and height at the 7th second.

Table 2 and Table 3 show the simulation results of RPR with picture-based resampling and block-based resampling respectively. In general, the BD-rate performance of these two resampling methods are close while the decoding time of picture-based resampling is much more than the decoding time of block-based resampling. The reason is that in the current picture-based implementation, the reference picture is always resampled to generate a resampled version of reference picture rather than checking if it is needed first.

- Resolution change settings

| Original resolution | Resolution change setting |
| --- | --- |
| 1920x1080 | 1280x720 |
| 1280x720 | 832x480 |
| 832x480 | 416x240 |
| 416x240 | 208x120 |

- Picture-based RPR

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.02% | 0.10% | 2.75% | 113% | 195% |
| Class C | -0.74% | 0.11% | 0.91% | 113% | 178% |
| Class E | -7.75% | -16.50% | -14.98% | 108% | 265% |
| Overall | -3.03% | -4.05% | -2.29% | 112% | 204% |
| Class D | -0.51% | -0.44% | 0.47% | 111% | 173% |
| Class F | -10.29% | -9.51% | -9.10% | 108% | 243% |

- Block-based RPR

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.10% | 0.00% | 2.92% | 121% | 147% |
| Class C | -1.11% | -0.65% | 0.40% | 116% | 143% |
| Class E | -7.48% | -16.24% | -15.48% | 118% | 169% |
| Overall | -3.12% | -4.28% | -2.52% | 118% | 151% |
| Class D | -1.18% | -1.69% | -0.17% | 115% | 143% |
| Class F | -9.75% | -9.46% | -8.86% | 118% | 165% |

# Patent rights declaration(s)

Alibaba Group and Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).