| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0138-v1 |
| --- | --- |

| Title: | AHG12: Treating region boundaries as picture boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seungwook Hong Jianle Chen Ye-Kui Wang | Email: | fhendry@futurewei.com seungwook.hong@futurewei.com jianle.chen@futurewei.com ye-kui.wang@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

After the review of the CE12 tests during the 14th JVET meeting, it was noted that the JVET group was inclined to adopt method 1, which is the method of treating MCTS boundaries as picture boundaries, but stated that software implementation should be available before taking further action.

This contribution reports a software implementation for treating motion-constrained independent region (MCIR) boundaries as picture boundaries. Decoding of a subset of the MCIRs covering a rectangular region tested, and it is asserted that mismatch-free decoding was achieved, which proves the correctness of the implementation of processes for treating the boundaries of an MCIR as picture boundaries. The contributors plan to show some such decoded sequence at the meeting.

# Introduction

The idea of treating MCTS boundaries as picture boundaries was studied in CE12 and discussed at the 14th JVET meeting in Geneva. It was noted that the JVET group was inclined to adopt the idea but felt that the software implementation should be provided before taking the action. The relevant meeting notes from the Geneva meeting are quoted as follows:

"The group was inclined to adopt method 1, but felt that VTM software should be available before action. So this will be considered at the next meeting when software is expected to become available and the quality of the software and its speed can be confirmed. No CE was planned on this topic."

Following up the result of the discussion, we have implemented the idea on top of VTM-5.0. Simulation results shows that the implemented software provides similar results to the results reported earlier for CE12 method 1.

# Proposal

The proposal idea can be summarized as follows:

- A flag, motion_constrained_slice_flag, is added to the PPS, to specify whether the slice is an MCIR and its boundary is treated the same as picture boundary in the decoding process.

- When the boundaries of slices are indicated to be treated as picture boundaries, the following applies:

- In the derivation process for temporal luma motion vector prediction, the right and bottom picture boundary positions used in the process, indicated by pic_height_in_luma_samples − 1 and pic_width_in_luma_samples − 1, respectively, are replaced with the right and the bottom boundary positions, respectively, of the slice, in units of luma samples.

- In the luma sample interpolation process, the left, right, top, and bottom picture boundary positions used in the process, indicated by 0, pic_height_in_luma_samples − 1, 0, pic_width_in_luma_samples − 1, respectively, are replaced with the left, right, top, and bottom boundary positions, respectively, of the slice, in units of luma samples, respectively.

- In the chroma sample interpolation process, the left, right, top, and bottom picture boundary positions used in the process, indicated by 0, pic_height_in_luma_samples / SubWidthC − 1, 0, pic_width_in_luma_samples / SubWidthC − 1, respectively, are replaced with the left, right, top, and bottom boundary positions, respectively, of the slice, in units of chroma samples, respectively.

The text changes for the proposal method are provided in the Annex at the end of this document.

# Simulation results

For the simulation, we ran similar tests under the test conditions used for the previous CE12 described in JVET-M0870, with only the following difference: in the previous CE12 simulations, the original test sequences were split into sub-picture sequences before encoding, while in the simulations with the actual implementation, the test sequences were not split into sub-picture sequences before encoding.

Table 1 below tabulates test results against anchor results for the cases of 96 MCIRs.

Table 1 Performance of treating MCTS boundaries as picture boundaries 
against anchor tiles. (94 tiles; RA configuration)

| Test sequence | Y | U | V |
| --- | --- | --- | --- |
| ChairLift | 18.27% | 25.83% | 26.75% |
| GasLamp | 12.40% | 16.50% | 15.33% |
| Harbor | 9.96% | 14.94% | 15.04% |
| KiteFlite | 4.71% | 9.79% | 11.13% |
| SkateboardInLot | 24.42% | 40.13% | 42.27% |
| Trolley | 5.46% | 10.35% | 11.38% |
| Average | 12.54% | 19.59% | 20.32% |

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex

### Picture parameter set RBSP syntax and semantics

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| ... |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_brick_idx[ i ] | u(v) |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| } |  |
| } |  |
| if( rect_slice_flag ) |  |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { |  |
| motion_constrained_slice_flag[ i ] | u(1) |
| loop_filter_across_bricks_enabled_flag | u(1) |
| if( loop_filter_across_bricks_enabled_flag ) |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| ... |  |
| } |  |
| ... |  |
| } |  |

motion_constrained_slice_flag[ i ] equal to 1 specifies that the i-th rectangular slice is a motion constrained rectangular slice. motion_constrained_slice_flag[ i ] equal to 0 specifies that the i-th rectangular slice is not a motion constrained rectangular slice. When not present, the value of motion_constrained_slice_flag[ i ] is inferred to be equal to 0.

When a rectangular slice is a motion constrained rectangular slice, its boundary is treated the same as picture boundary in the decoding process described in clause 8 for pictures referring to the PPS.

### Slice header semantics

...

The variable NumBricksInCurrSlice, which specifies the number of bricks in the current slice, SliceBrickIdx[ i ], which specifies the brick index of the i-th brick in the current slice, SliceIdx, which specifies the index of the slice within the picture, TopLeftBrickIdx, which specifies the brick index of the top left brick of the slice, and BotRightBrickIdx, which specifies the brick index of the bottom right brick of the slice, are derived as follows:

if( rect_slice_flag ) {
	SliceIdx = 0
	while( slice_address  !=  slice_id[ SliceIdx ] )
		SliceIdx++
	NumBricksInCurrSlice = NumBricksInSlice[ SliceIdx ]
	TopLeftBrickIdx = brickIdx = top_left_brick_idx[ SliceIdx ]
	BotBrickIdx = brickIdx + bottom_right_brick_idx_delta[ SliceIdx ]
	for( bIdx = 0; brickIdx  <=  BotBrickIdx; brickIdx++ )	(7-40)
		if( BricksToSliceMap[ brickIdx ]  = =  SliceIdx )
			SliceBrickIdx[ bIdx++ ] = brickIdx
} else {
	NumBricksInCurrSlice = num_bricks_in_slice_minus1 + 1
	SliceBrickIdx[ 0 ] = slice_address
	for( i = 1; i < NumBricksInCurrSlice; i++ )
		SliceBrickIdx[ i ] = SliceBrickIdx[ i − 1 ] + 1
}

...

### General slice data semantics

end_of_brick_one_bit shall be equal to 1.

For each brick, the variables LeftBoundaryPos, TopBoundaryPos, RightBoundaryPos and BotBoundaryPos are derived as follows:

–	if motion_constrained_slice_flag[SliceIdx] is equal to 1, the following applies:

LeftBoundaryPos = BrickColBd[ TopLeftBrickIdx ]  <<  CtbLog2SizeY

TopBoundaryPos = BrickRowBd[ TopLeftBrickIdx ]  <<  CtbLog2SizeY

RightBoundaryPos = ( ( BrickColBd[ BotRightBrickIdx ] +
					BrickWidth[ BotRightBrickIdx ] )  <<  CtbLog2SizeY ) – 1
RightBoundaryPos = RefreshedRightBoundary > pic_width_in_luma_samples ? 	(7-41)
					pic_width_in_luma_samples : RightBoundaryPos

BotBoundaryPos  = ( ( BrickRowBd[ BotRightBrickIdx ] +
					BrickHeight[ BotRightBrickIdx ] )  <<  CtbLog2SizeY )  – 1
BotBoundary  = RefreshedBotBoundary > pic_height_in_luma_samples ?
					pic_height_in_luma_samples : BotBoundaryPos

–	Otherwise (i.e., motion_constrained_tile_group_flag is equal to 0), the following applies:

LeftBoundaryPos = 0		(7-xx)

RightBoundaryPos = pic_width_in_luma_samples − 1	(7-xx)

TopBoundaryPos = 0		(7-xx)

BotBoundaryPos = pic_height_in_luma_samples − 1	(7-xx)

### Derivation process for temporal luma motion vector prediction

...

- If yCb  >>  CtbLog2SizeY is equal to yColBr  >>  CtbLog2SizeY, yColBr is in the range from TopBoundaryPos to BotBoundaryPos less than pic_height_in_luma_samples and xColBr is in the range from LeftBoundaryPos to RightBoundaryPos less than pic_width_in_luma_samples, the following applies:

- The variable colCb specifies the luma coding block covering the modified location given by ( ( xColBr  >>  3 )  <<  3, ( yColBr  >>  3 )  <<  3 ) inside the collocated picture specified by ColPic.

- The luma location ( xColCb, yColCb ) is set equal to the top-left sample of the collocated luma coding block specified by colCb relative to the top-left luma sample of the collocated picture specified by ColPic.

- The derivation process for collocated motion vectors as specified in clause 8.3.2.9 is invoked with currCb, colCb, ( xColCb, yColCb ), refIdxLX , and control parameter controlParaFlag set equal to 0 as inputs, and the output is assigned to mvLXCol and availableFlagLXCol.

...

### Luma sample bilinear interpolation filtering process

...

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

–	If  motion_constrained_slice_flag[SliceIdx] is equal to 1, the following applies:

xInti = Clip3( LeftBoundaryPos, RightBoundaryPos, xIntL + i )	(8-458)

yInti = Clip3( TopBoundaryPos, BotBoundaryPos, yIntL + i )	(8-458)

–	Otherwise (motion_constrained_slice_flag[SliceIdx] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ) :	(8-435)
		xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )		(8-436)

...

### Luma sample interpolation filtering process

...

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

–	If motion_constrained_slice_flag[SliceIdx] is equal to 1, the following applies:

xInti = Clip3( LeftBoundaryPos,  RightBoundaryPos, xIntL + i − 3 )	(8-830)

yInti = Clip3( TopBoundaryPos,  BotBoundaryPos, yIntL + i − 3 )	(8-830)

–	Otherwise (motion_constrained_slice_flag[SliceIdx] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) :	(8-745) 
		xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )		(8-746)

...

### Chroma sample interpolation process

...

The chroma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

–	If motion_constrained_slice_flag[SliceIdx] is equal to 1, the following applies:

xInti = Clip3( LeftBoundaryPos / SubWidthC,
				RightBoundaryPos / SubWidthC, xIntL + i )	(8-844)

yInti = Clip3( TopBoundaryPos / SubHeightC,
				BotBoundaryPos / SubHeightC, yIntL + i )	(8-844)

–	Otherwise (motion_constrained_slice_flag[SliceIdx] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ? ClipH( xOffset, picWC, xIntC + i − 1 ) :
		xIntC + i − 1 )			(8-759)

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )		(8-760)

...

### Derivation process for subblock-based temporal merging candidates

...

- For xSbIdx = 0..numSbX − 1 and ySbIdx = 0 .. numSbY − 1, the motion vectors mvLXSbCol[ xSbIdx ][ ySbIdx ] and prediction list utilization flags predFlagLXSbCol[ xSbIdx ][ ySbIdx ] are derived as follows:

- The luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows:

xSb  =  xCb + xSbIdx * sbWidth		(8-523)

ySb  =  yCb + ySbIdx * sbHeight		(8-524)

- The location ( xColSb, yColSb ) of the collocated subblock inside ColPic is derived as follows:

    - The following applies:

yColSb = Clip3( yCtb, 
	Min( CurPicHeightInSamplesY − 1, yCtb + ( 1  <<  CtbLog2SizeY ) − 1 ),	(8-526)
 	ySb + ( tempMv[1]  >>  4 ) )

    - If motion_constrained_slice_flag[SliceIdx] is equal to 1, the following applies:

xColSb = Clip3( xCtb, 
	Min( RightBoundaryPos , xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ),	(8-642)
	xSb + ( tempMv[0]  >>  4 ) )

    - Otherwise (motion_constrained_slice_flag[SliceIdx] is equal to 0), the following applies:

xColSb = Clip3( xCtb, 
	Min( CurPicWidthInSamplesY − 1, xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ),	(8-525)
	xSb + ( tempMv[0]  >>  4 ) )

- The variable currCb specifies the luma coding block covering the current coding subblock inside the current picture.

...