| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0139 |
| --- | --- |

| Title: | CE3 Related: Low Memory and Computational Complexity Matrix Based Intra Prediction (MIP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar Seung-Hwan Kim 10225 Willow Creek Rd, San Diego, CA, U.S.A. 92131 | Tel: Email: | +1-858-635-5226 mehdi.salehifar@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

This contribution proposes several modification to the current MIP design in VTM5. Currently three different MIP index defined based on the block size (index 0: for W=H=4 index 1: for maxW,H=8, and index 2: formaxW,H>8). Also there are 19, 10, and 6 kernels which represent 35, 19, and 11 modes for each index 0,1, and 2 respectively. These MIP modes are divideded into MPM and non-MPM for signaling, where 3 MPM modes are coded as TU and non-MPM coded as fixed length code. This proposal suggests following modifications:

- Removing MIP for block size index 0 and 1.

- Reducing number of the kernels for MIP index 2 from 6 to 3.

- Encoder modification

- Removing the MPM derivation, and coding MIP mode with truncated binary code.

| Test # | Modification |
| --- | --- |
| Test 1 | a |
| Test 2 | a, c |
| Test 3 | a, b, c, d |

The reported result for test 1 is 0.08 %, 0.07 % and 0.04 % luma BD-rates differences with 94 %, 98 %, and 100 % encoding time for the AI, RA, and LB settings respectively, under common test conditions. Meanwhile it uses ~ 4 KB memory (50% less memory).

The reported result for test 2 is 0.00 %, 0.02 % and x.xx % luma BD-rates differences with 101 %, 101 %, and x % encoding time for the AI, RA, and LB settings respectively, under common test conditions. Meanwhile it uses ~ 4 KB memory (50% less memory).

The reported result for test 3 is 0.03 %, 0.03 % and 0.05 % luma BD-rates differences with 98 %, 99 %, and 99 % encoding time for the AI, RA, and LB settings respectively, under common test conditions. Meanwhile it uses ~2 KB memory (75% less memory).

# Introduction

In VTM5.0, three different MIP index is defined based on the block Width (W) and Height (H) [1]:

idxW,H=0for W=H=41for maxW,H=82formaxW,H>8.

Currently there are 35, 19, and 11 modes for MIP index 0,1, and 2 respectively.

There are 19, 10, and 6 kernels for index 0,1, and 2 respectively and the corresponding kernel is derived according to the following equations for each mode:

m=modefor W=H=4 and mode<18mode-17for W=H=4 and mode≥18modeformaxW,H=8 and mode<10mode-9formaxW,H=8 and mode≥10modeformaxW,H>8 and mode<6mode-5formaxW,H>8 and mode≥6.

Finally if an MIP mode is to be applied, the index predmode of the MIP mode is signaled using a MPM-list with 3 MPMS [1]. The derivation of the MPMs is performed using the intra-modes of the above and the left PU as follows. There are three fixed tables map_angular_to_alwipidx, idx∈{0,1,2} that assign to each conventional intra prediction mode predmodeAngular an MIP mode

predmodeALWIP= map_angular_to_alwipidx[predmodeAngular].

For each PU of width W and height H one defines an index

idxPU=idx(W,H)∈{0,1,2}

that indicates from which of the three sets the MIP-parameters are to be taken as in Section 1.3 above.

If the above Prediction Unit PUabove is available, belongs to the same CTU as the current PU and is in intra mode, if idxPU=idx(PUabove) and if MIP is applied on PUabove with MIP-mode predmodeALWIPabove, one puts

modeALWIPabove=predmodeALWIPabove .

If the above PU is available, belongs to the same CTU as the current PU and is in intra mode and if a conventional intra prediction mode predmodeAngularabove is applied on the above PU, one puts

modeALWIPabove=map_angular_to_alwipidx(PUabove)[predmodeAngularabove].

In all other cases, one puts

modeALWIPabove=-1

which means that this mode is unavailable. In the same way but without the restriction that the left PU needs to belong to the same CTU as the current PU, one derives a mode modeALWIPleft.

Finally, three fixed default lists listidx, idx∈{0,1,2} are provided, each of which contains three distinct MIP modes. Out of the default list listidx(PU) and the modes  modeALWIPabove and modeALWIPleft, one constructs three distinct MPMs by substituting -1 by default values as well as eliminating repetitions.

Based on our experiment first of all, MIP index 2 has the most contribution in the performance improvement. Secondly even inside of the MIP index 2 half of the kernels could be discarded without huge performance degradation.  Finally the whole MPM derivation process could be removed without significant performance change.

# Proposed Method

In this contribution we propose several modification for MIP.

First of all we remove applying MIP for blocks of index 0 and 1. So MIP would be applied for blocks with:

maxWidth,Height>8

Secondly we reduce number of the MIP modes for index 2  from 11 to 6 and keep 3 kernels out of the 6 (kernel 1,2,3 which correspond to mode 1,2,3,6,7,8).

Applying MIP for only index 2 reduces encoding time. So we had increased the encoder full RD check for MIP cases in order to bring back encoding time to the original level.

Finally as explaiend in the other proposal, we remove the whole MPM derivation process for MIP modes, and code the 6 modes using truncated binary code without any context coded bin.

Test 1 consists of removing MIP for blocks index 0 and 1.

Test 2 consists of removing MIP for blocks index 0 and 1, with encoder modification to have similar encoding time as anchor.

Test 3 consists of removing MIP for blocks index 0 and 1, reducing number of the mode for index 2 to 6 modes (3 kernels), removing MPM derivation and coding MIP mode with truncated binary, and encoder modification to have a similar encoding time as anchor.

# Experimental results

The proposed method has been implemented based on VTM 5.0 reference software according to common test conditions defined in JVET-N1010 [1].

Test1:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.01% | -0.05% | 95% | 100% |
| Class A2 | 0.03% | -0.09% | -0.05% | 95% | 100% |
| Class B | 0.04% | -0.08% | -0.02% | 94% | 100% |
| Class C | 0.17% | 0.00% | 0.14% | 94% | 99% |
| Class E | 0.12% | -0.02% | 0.03% | 94% | 98% |
| Overall | 0.08% | -0.04% | 0.02% | 94% | 100% |
| Class D | 0.25% | 0.23% | 0.02% | 94% | 100% |
| Class F | 0.09% | 0.14% | 0.09% | 96% | 99% |
| Class SCC | 0.02% | 0.03% | 0.00% | 96% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.15% | 0.10% | 99% | 100% |
| Class A2 | 0.06% | 0.02% | 0.01% | 99% | 100% |
| Class B | 0.05% | 0.00% | -0.08% | 98% | 100% |
| Class C | 0.13% | -0.07% | -0.16% | 97% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | -0.04% | -0.05% | 98% | 100% |
| Class D | 0.20% | -0.02% | 0.14% | 97% | 98% |
| Class F | 0.06% | -0.04% | 0.13% | 98% | 100% |
| Class SCC | -0.01% | -0.01% | -0.02% | 98% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.13% | 0.26% | 100% | #NUM! |
| Class C | 0.04% | 0.12% | -0.02% | 100% | 109% |
| Class E | 0.11% | -0.06% | -0.52% | 101% | 105% |
| Overall | 0.04% | 0.08% | -0.03% | 100% | #NUM! |
| Class D | 0.04% | -0.46% | 1.24% | 98% | 103% |
| Class F | 0.07% | -0.27% | -0.49% | 100% | 104% |
| Class SCC | -0.06% | -0.15% | -0.03% | 101% | 106% |

Test2:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.08% | -0.07% | 104% | 102% |
| Class A2 | -0.04% | -0.05% | -0.06% | 102% | 101% |
| Class B | -0.04% | -0.11% | -0.03% | 100% | 102% |
| Class C | 0.13% | -0.01% | 0.04% | 100% | 102% |
| Class E | 0.01% | -0.02% | 0.03% | 99% | 101% |
| Overall | 0.00% | -0.06% | -0.02% | 101% | 102% |
| Class D | 0.22% | 0.18% | 0.07% | 98% | 87% |
| Class F | 0.05% | 0.13% | -0.02% | 102% | 105% |
| Class SCC | 0.00% | 0.00% | -0.02% | 101% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.37% | -0.03% | 102% | 100% |
| Class A2 | 0.00% | 0.08% | 0.01% | 101% | 99% |
| Class B | 0.02% | -0.09% | -0.22% | 101% | 102% |
| Class C | 0.05% | -0.16% | -0.09% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.13% | -0.10% | 101% | 101% |
| Class D | 0.18% | -0.02% | 0.22% | 100% | 102% |
| Class F | 0.07% | -0.03% | 0.21% | 101% | 102% |
| Class SCC | 0.05% | 0.00% | -0.05% | 102% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.08% | 0.17% | -0.02% | 100% | 105% |
| Class E | 0.04% | 1.02% | 0.37% | 101% | 104% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.02% | 0.33% | 1.01% | 100% | 103% |
| Class F | -0.01% | -0.12% | -0.51% | 101% | 102% |
| Class SCC | 0.00% | -0.04% | 0.06% | 101% | 103% |

Test3:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.14% | -0.14% | 100% | 99% |
| Class A2 | 0.00% | -0.12% | -0.07% | 99% | 99% |
| Class B | -0.01% | -0.08% | -0.01% | 99% | 98% |
| Class C | 0.12% | -0.01% | 0.04% | 97% | 94% |
| Class E | 0.07% | 0.02% | -0.11% | 98% | 96% |
| Overall | 0.03% | -0.07% | -0.04% | 98% | 97% |
| Class D | 0.23% | 0.17% | -0.11% | 96% | 94% |
| Class F | 0.05% | 0.11% | 0.03% | 98% | 95% |
| Class SCC | 0.02% | 0.00% | -0.02% | 98% | 97% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.22% | 0.07% | 100% | 99% |
| Class A2 | 0.03% | -0.04% | -0.12% | 99% | 100% |
| Class B | 0.04% | -0.17% | -0.10% | 99% | 97% |
| Class C | 0.07% | -0.11% | -0.16% | 98% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.14% | -0.08% | 99% | 98% |
| Class D | 0.20% | 0.01% | 0.07% | 98% | 95% |
| Class F | 0.07% | 0.00% | 0.17% | 98% | 97% |
| Class SCC | -0.01% | 0.03% | -0.04% | 100% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.33% | -0.32% | 100% | 101% |
| Class C | 0.08% | 0.26% | -0.20% | 99% | 103% |
| Class E | 0.07% | 0.62% | 0.06% | 100% | 102% |
| Overall | 0.05% | 0.38% | -0.19% | 99% | 102% |
| Class D | 0.05% | 0.14% | 0.46% | 97% | 100% |
| Class F | -0.05% | -0.17% | 0.12% | 99% | 101% |
| Class SCC | 0.05% | -0.02% | 0.02% | 100% | 102% |

# Conclusion

In the contribution several modification on MIP is proposed, which reduces the memory and complexity with negligible loss.  It is recommended that the method be adopted in the next VTM.

# Reference

- J. Pfaff et al., “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0207, 14th JVET meeting, Geneva, CH, 19–27 March 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET meeting, Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).