| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothernburg, SE, 3–12 July 2019 | Document: JVET-O0142-v1 |
| --- | --- |

| Title: | AHG12: On turning off ALF filtering at brick and slice boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Jianle Chen Hendry 10180 Telesis Court San Diego, CA, 92121, USA | Tel: Email: | +1-908-803-3888 ye-kui.wang@futurewei.com +1-858-999-4243 jianle.chen@futurewei.com +1-858-375-8065 fhendry@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following aspects:

- Add the functionality that enables turning off ALF filtering at brick boundaries.

- Remove loop_filter_across_slices_enabled_flag and the associated decoding process functionalities.

Detailed specification text changes are provided in an attachment.

# Introduction

Currently, loop_filter_across_bricks_enabled_flag exists in the PPS syntax, and when equal to 0, it specifies that in-loop filtering operations, including ALF filtering, are not performed across brick boundaries in pictures referring to the PPS. However, the functionality of turning off ALF filtering across brick boundaries is missing.

Currently, loop_filter_across_slices_enabled_flag exists in the PPS syntax, and when equal to 0, it specifies that in-loop filtering operations are not performed across slice boundaries in pictures referring to the PPS. However, similarly, the functionality of turning off ALF filtering across slice boundaries is missing.

While the text changes to add the functionality of turning off ALF filtering across brick boundaries is easy to make, we found out that making text and software changes for enabling turning off ALF filtering across slice boundaries is rather difficult. This is due to that, unlike the boundaries of bricks that are rectangular-shaped, the shapes of slices are sort of random, and relationship between neighbouring slices can be complicated for specifying the clipping operations that are needed for turning of ALF filtering across the boundaries.

On the other hand, in our understanding, the group has had an intention to not use SEI messages for signalling of motion-constrained independent regions (MCIRs) like in HEVC, but to use some more normative mechanism such as a sub-picture design like in JVET-O141. That's why at the previous JVET meeting the group was close to adopt a proposal out of the previous CE 12 on treating MCTS boundaries as picture boundaries in the decoding processes.

With this intention, we believe then it is sufficient to enable turning off in-loop filtering, including ALF, across the boundaries of MCIRs, regardless whether the MCIR mechanism is a sub-picture design or not.

Therefore, it is proposed to add the functionality that enables turning off ALF filtering at brick boundaries and remove loop_filter_across_slices_enabled_flag and the associated decoding process functionalities.

# Proposal

It is proposed to add the functionality that enables turning off ALF filtering at brick boundaries and remove loop_filter_across_slices_enabled_flag and the associated decoding process functionalities.

The proposed text changes are in the attachment to this contribution, with change marks relative to JVET-N1001-v7.

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).