| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothernburg, SE, 3–12 July 2019 | Document: JVET-O0144-v1 |
| --- | --- |

| Title: | AHG12: On end_of_subset_one_bit, end_of_brick_one_bit, and byte alignment in the slice data syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Ye-Kui Wang 10180 Telesis Court San Diego, CA, 92121, USA | Tel: Email: | +1-858-375-8065 fhendry@futurewei.com +1-908-803-3888 ye-kui.wang@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a modification to the slice data syntax, related to the syntax elements end_of_subset_one_bit and end_of_brick_one_bit as well as the byte alignment syntax. It is asserted that the current signalling has a problem when wavefront parallel processing (WPP) is used for a slice that contains more than one brick.

# Problem description

At the 14th JVET meeting in Geneva, it was agreed to include the wavefront parallel processing (WPP) feature. Based on the decision to include WPP, the slice data syntax was updated to be as follows:

| slice_data( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < NumBricksInCurrSlice; i++ ) { |  |
| CtbAddrInBs = FirstCtbAddrBs[ SliceBrickIdx[ i ] ] |  |
| for( j = 0; j < NumCtusInBrick[ SliceBrickIdx[ i ] ]; j++, CtbAddrInBs++ ) { |  |
| ... |  |
| if( entropy_coding_sync_enabled_flag && ( ( j + 1 ) % BrickWidth[ SliceBrickIdx[ i ] ] = = 0 ) ) { |  |
| end_of_subset_one_bit /* equal to 1 */ | ae(v) |
| if( j < NumCtusInBrick[ SliceBrickIdx[ i ] ] − 1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |
| if( !entropy_coding_sync_enabled_flag ) { |  |
| end_of_brick_one_bit /* equal to 1 */ | ae(v) |
| if( i < NumBricksInCurrSlice − 1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |
| } |  |

We observed that the updated slice data syntax has a problem related to the signalling of end_of_subset_one_bit, end_of_brick_one_bit, and byte_alignment( ).

What is needed is as follows:

- Regardless of whether WPP is on, a bin equal to 1 needs to be present at the end of each brick, and byte_alignment( ) needs to be present at the end of each brick except for the last brick of the slice.

- When WPP is on, in addition to the above, a bin equal to 1 and byte_alignment( ) need to be present at the end of each CTU row within a brick except for the last CTU row within a brick.

According to the above slice data syntax:

- When WPP is off, there is no problem.

- However, when WPP is on, byte_alignment( ) is missing at the end of each brick. While that is correct for the last brick of the slice, it is not correct for other bricks in the slice.

# Proposal solution

To resolve the problem above, we propose the following modification:

| slice_data( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < NumBricksInCurrSlice; i++ ) { |  |
| CtbAddrInBs = FirstCtbAddrBs[ SliceBrickIdx[ i ] ] |  |
| for( j = 0; j < NumCtusInBrick[ SliceBrickIdx[ i ] ]; j++, CtbAddrInBs++ ) { |  |
| ... |  |
| if( entropy_coding_sync_enabled_flag && ( j < NumCtusInBrick[ SliceBrickIdx[ i ] ] − 1 ) && ( ( j + 1 ) % BrickWidth[ SliceBrickIdx[ i ] ] = = 0 ) ) { |  |
| end_of_subset_one_bit /* equal to 1 */ | ae(v) |
| if( j < NumCtusInBrick[ SliceBrickIdx[ i ] ] − 1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |
| if( !entropy_coding_sync_enabled_flag ) { |  |
| end_of_brick_one_bit /* equal to 1 */ | ae(v) |
| if( i < NumBricksInCurrSlice − 1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |
| } |  |

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).