| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothernburg, SE, 3–12 July 2019 | Document: JVET-O0145-v1 |
| --- | --- |

| Title: | AHG12: Miscellaneous AHG12 topics |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Hendry 10180 Telesis Court San Diego, CA, 92121, USA | Tel: Email: | +1-908-803-3888 ye-kui.wang@futurewei.com +1-858-375-8065 fhendry@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

The following aspects are proposed in this contribution:

- Remove the signalling of num_entry_point_offsets.

- Move offset_len_minus1 from the slice header syntax to the PPS syntax.

# On num_entry_point_offsets

When WPP is in use (i.e., when entropy_coding_sync_enabled_flag is equal to 1), the syntax element num_entry_point_offsets is signalled for the derivation of the variable NumEntryPoints. However, NumEntryPoints can be derived without such a signalling.

It is therefore proposed to remove and update the derivation of NumEntryPoints accordingly, as follows:

      - General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if ( entropy_coding_sync_enabled_flag ) |  |
| num_entry_point_offsets | ue(v) |
| if( NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| ... |  |
| } |  |

      - General slice header semantics

...

num_entry_point_offsets is used to specify the variable NumEntryPoints, which specifies the number of entry points in the current slice as follows:

NumEntryPoints = entropy_coding_sync_enabled_flag ? num_entry_point_offsets :
			NumBricksInCurrSlice − 1		(7-60)

The variable NumEntryPoints, which specifies the number of entry points in the current slice, is derived as follows:

if( !entropy_coding_sync_enabled_flag )
	NumEntryPoints = NumBricksInCurrSlice − 1
else {
	for( numBrickSpecificCtuRowsInSlice = 0, i =0; i < NumBricksInCurrSlice; i++ ) 	(7-60)
		numBrickSpecificCtuRowsInSlice  +=  BrickHeight[ SliceBrickIdx[ i ] ]
	NumEntryPoints = numBrickSpecificCtuRowsInSlice − 1
}

offset_len_minus1 plus 1 specifies the length, in bits, of the entry_point_offset_minus1[ i ] syntax elements. The value of offset_len_minus1 shall be in the range of 0 to 31, inclusive.

...

# On offset_len_minus1

Currently, offset_len_minus1 is signalled in the slice header syntax. However, often many slices would use the same value of offset_len_minus1 anyway, thus it would make more sense to signal it in the PPS syntax instead.

We therefore propose to move offset_len_minus1 from the slice header syntax to the PPS syntax, as follows:

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| ... |  |
| } |  |
| entropy_coding_sync_enabled_flag | u(1) |
| if( !single_tile_in_pic_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| offset_len_minus1 | ue(v) |
| ... |  |
| } |  |

      - General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| ... |  |
| } |  |

      - Picture parameter set RBSP semantics

...

offset_len_minus1 plus 1 specifies the length, in bits, of the entry_point_offset_minus1[ i ] syntax elements in slice headers referring to the PPS. The value of offset_len_minus1 shall be in the range of 0 to 31, inclusive.

...

      - General slice header semantics

...

offset_len_minus1 plus 1 specifies the length, in bits, of the entry_point_offset_minus1[ i ] syntax elements. The value of offset_len_minus1 shall be in the range of 0 to 31, inclusive.

...

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).