| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0146-v1 |
| --- | --- |

| Title: | AHG17: On unspecified NAL unit types |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 10180 Telesis Court San Diego, CA, 92121, USA | Tel: Email: | +1-908-803-3888 ye-kui.wang@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

Currently, the TemporalId of all the unspecified NAL unit types have to be equal to 0. This contribution proposes to reallocate NAL unit types such that some unspecified NAL unit types can have TemporalId greater than 0.

# Introduction

Currently, NAL unit types 28..31 are specified as "unspecified". Per the current NAL unit header syntax design, NAL unit types in the range of 16 to 31, inclusive, have zero_tid_required_flag equal to 1, and consequently have TemporalId equal to 0. Therefore, the unspecified NAL unit types all must have TemporalId equal to 0.

However, this imposes some undesriable burden in application systems using VVC as the codec, wherein it would be desriable to allow TemporalId greater than 0 for at least some systems data structures using unspecified NAL unit types.

For example, ISO/IEC 14496-15, which includes file formats for AVC, HEVC, and their extensions, includes the specification of aggregators and extractors that use NAL unit types unspecified in the video codec specifications. It is expected that aggregators and extractors would also be supported in the potential VVC file format. When used for NAL units of an access unit with TemporalId greater than 0, if the TemporalId of the aggregator or extractor has to have TemporalId equal to 0, then for extraction of a temporal subset of a stream containing the aggregator or extractor, all the aggregated or extracted NAL units in the aggregator or extractor must be parsed to figure out whether the extractor or extractor should be included in the extracted sub-btstream, which is not needed if TemporalId of the aggregator or extractor is allowed to be greater than 0.

It is therefore proposed to reallocate NAL unit types such that some unspecified NAL unit types can have TemporalId greater than 0.

# Proposal

The proposed text changes, for reallocation of NAL unit types, allowing some unspecified NAL unit types have TemporalId greater than 0, as well as other associated changes are provided below, wherein the changed parts are highlighted in yellow.

#### NAL unit header semantics

zero_tid_required_flag equal to 0 specifies that zero_tid_required_flag does not impose any additional constraints on the value of nuh_temporal_id1_plus1.

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit.

The value of nuh_temporal_id_plus1 shall not be equal to 0. When zero_tid_required_flag is equal to 1, the value of nuh_temporal_id_plus1 shall be equal to 1.

The variable TemporalId is derived as follows:

TemporalId = nuh_temporal_id_plus1 − 1		(7-1)

NOTE 1 – NAL unit types in the range of 16 to 31, inclusive, have zero_tid_required_flag equal to 1, and consequently have TemporalId equal to 0.

The value of TemporalId shall be the same for all VCL NAL units of an access unit. The value of TemporalId of a coded picture or an access unit is the value of the TemporalId of the VCL NAL units of the coded picture or the access unit.

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	If NalUnitType is equal to SPS_NUT, TemporalId is equal to 0 and the TemporalId of the access unit containing the NAL unit shall be equal to 0.

–	Otherwise, if NalUnitType is equal to APS_NUT, TemporalId shall be equal to that of the access unit containing the NAL unit.

–	Otherwise, when NalUnitType is not equal to EOS_NUT and not equal to EOB_NUT, TemporalId shall be greater than or equal to the TemporalId of the access unit containing the NAL unit.

NOTE 2 – When the NAL unit is a non-VCL NAL unit, the value of TemporalId is equal to the minimum value of the TemporalId values of all access units to which the non-VCL NAL unit applies. When NalUnitType is equal to PPS_NUT, TemporalId may be greater than or equal to the TemporalId of the containing access unit, as all picture parameter sets (PPSs) may be included in the beginning of a bitstream, wherein the first coded picture has TemporalId equal to 0. When NalUnitType is equal to PREFIX_SEI_NUT or SUFFIX_SEI_NUT, TemporalId may be greater than or equal to the TemporalId of the containing access unit, as an SEI NAL unit may contain information that applies to a bitstream subset that includes access units for which the TemporalId values are greater than the TemporalId of the access unit containing the SEI NAL unit.

nal_unit_type_lsb specifies the least significant bits for the NAL unit type.

The variable NalUnitType, which specifies the NAL unit type, i.e., the type of RBSP data structure contained in the NAL unit as specified in Table 7-1, is derived as follows:

NalUnitType = ( zero_tid_required_flag  <<  4 ) + nal_unit_type_lsb	(7-2)

NAL units that have NalUnitType equal to UNSPEC14, UNSPEC15, UNSPEC30, and UNSPEC31, for which semantics are not specified, shall not affect the decoding process specified in this Specification.

NOTE 3 – NAL unit types equal to UNSPEC14, UNSPEC15, UNSPEC30, and UNSPEC31 may be used as determined by the application. No decoding process for these values of NalUnitType is specified in this Specification. Since different applications might use these NAL unit types for different purposes, particular care must be exercised in the design of encoders that generate NAL units with these NalUnitType values, and in the design of decoders that interpret the content of NAL units with these NalUnitType values. This Specification does not define any management for these values. These NalUnitType values might only be suitable for use in contexts in which "collisions" of usage (i.e., different definitions of the meaning of the NAL unit content for the same NalUnitType value) are unimportant, or not possible, or are managed – e.g., defined or managed in the controlling application or transport specification, or by controlling the environment in which bitstreams are distributed.

For purposes other than determining the amount of data in the decoding units of the bitstream (as specified in Annex C), decoders shall ignore (remove from the bitstream and discard) the contents of all NAL units that use reserved values of NalUnitType.

NOTE 4 – This requirement allows future definition of compatible extensions to this Specification.

Table 7-1 – NAL unit type codes and NAL unit type classes

| NalUnitType | Name of NalUnitType | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 1 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 2 3 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 4 | APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |
| 6 5..7 | RSV_NVCL65.. RSV_NVCL7 | Reserved | non-VCL |
| 8 | TRAIL_NUT | Coded slice of a non-STSA trailing picture slice_layer_rbsp( ) | VCL |
| 9 | STSA_NUT | Coded slice of an STSA picture slice_layer_rbsp( ) | VCL |
| 10 | RADL_NUT | Coded slice of a RADL picture slice_layer_rbsp( ) | VCL |
| 11 | RASL_NUT | Coded slice of a RASL picture slice_layer_rbsp( ) | VCL |
| 12 13 | RSV_VCL_12 RSV_VCL_13 | Reserved non-IRAP VCL NAL unit types | VCL |
| 14 15 | UNSPEC14 UNSPEC15 | Unspecified | non-VCL |
| 16 | DPS_NUT | Decoding parameter set decoding_parameter_set_rbsp( ) | non-VCL |
| 17 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 18 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 19 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 20 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 21..23 | RSV_NVCL21.. RSV_NVCL23 | Reserved | non-VCL |
| 24 25 | IDR_W_RADL IDR_N_LP | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 26 | CRA_NUT | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 27 | GRA_NUT | Coded slice of a gradual random access picture slice_layer_rbsp( ) | VCL |
| 28 29 | RSV_VCL_28 RSV_VCL_29 | Reserved non-IRAP VCL NAL unit types | VCL |
| 30 31 | UNSPEC30 UNSPEC31 | Unspecified | non-VCL |

NOTE 6 – A clean random access (CRA) picture may have associated RASL or RADL pictures present in the bitstream.

NOTE 7 – An instantaneous decoding refresh (IDR) picture having NalUnitType equal to IDR_N_LP does not have associated leading pictures present in the bitstream. An IDR picture having NalUnitType equal to IDR_W_RADL does not have associated RASL pictures present in the bitstream, but may have associated RADL pictures in the bitstream.

nuh_layer_id specifies the identifier of the layer to which a VCL NAL unit belongs or the identifier of a layer to which a non-VCL NAL unit applies. The value of nuh_layer_id shall be in the range of 0 to 126, inclusive. The value of 127 may be specified in the future by ITU-T | ISO/IEC. For purposes other than determining the amount of data in the decoding units of the bitstream, decoders shall ignore all data that follow the value 127 for nuh_layer_id in a NAL unit.

NOTE 8 – The value of 127 for nuh_layer_id may be used to indicate an extended layer identifier in a future extension of this Specification.

The value of nuh_layer_id shall be the same for all VCL NAL units of a coded picture. The value of nuh_layer_id of a coded picture is the value of the nuh_layer_id of the VCL NAL units of the coded picture.

nuh_reserved_zero_bit shall be equal to '0'. The value 1 of nuh_reserved_zero_bit may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore (i.e. remove from the bitstream and discard) NAL units with nuh_reserved_zero_bit equal to '1'.

#### Order of NAL units and coded pictures and their association to access units

This clause specifies the order of NAL units and coded pictures and their association to access units for CVSs that conform to one or more of the profiles specified in Annex A and that are decoded using the decoding process specified in clauses 2 through 10.

An access unit consists of one coded picture, zero or more VCL NAL units and zero or more non-VCL NAL units. The association of VCL NAL units to coded pictures is described in clause 7.4.2.4.5.

The first access unit in the bitstream starts with the first NAL unit of the bitstream.

Let firstBlPicNalUnit be the first VCL NAL unit of a coded picture. The first of any of the following NAL units preceding firstBlPicNalUnit and succeeding the last VCL NAL unit preceding firstBlPicNalUnit, if any, specifies the start of a new access unit:

–	access unit delimiter NAL unit (when present),

–	DPS NAL unit (when present),

–	VPS NAL unit (when present),

–	SPS NAL unit (when present),

–	PPS NAL unit (when present),

–	APS NAL unit (when present),

–	Prefix SEI NAL unit (when present),

–	NAL units with NalUnitType equal to RSV_NVCL_5, RSV_NVCL_6, RSV_NVCL_21, or RSV_NVCL_22 (when present),

–	NAL units with NalUnitType equal to UNSPEC14 or UNSPEC15 (when present).

NOTE – The first NAL unit preceding firstBlPicNalUnit and succeeding the last VCL NAL unit preceding firstBlPicNalUnit, if any, can only be one of the above-listed NAL units.

When there is none of the above NAL units preceding firstBlPicNalUnit and succeeding the last VCL NAL preceding firstBlPicNalUnit, if any, firstBlPicNalUnit starts a new access unit.

The order of the coded pictures and non-VCL NAL units within an access unit shall obey the following constraints:

–	When an access unit delimiter NAL unit is present, it shall be the first NAL unit. There shall be at most one access unit delimiter NAL unit in any access unit.

–	When any DPS NAL units, VPS NAL units, SPS NAL units, PPS NAL units, APS NAL units, prefix SEI NAL units, NAL units with  NalUnitType equal to RSV_NVCL_5, RSV_NVCL_6, RSV_NVCL_21, or RSV_NVCL_22, or NAL units with NalUnitType equal to UNSPEC14 or UNSPEC15 are present, they shall not follow the last VCL NAL unit of the access unit.

–	NAL units having NalUnitType equal to SUFFIX_SEI_NUT, RSV_NVCL_7, or RSV_NVCL_23, or in the range of UNSPEC30..UNSPEC31 shall not precede the first VCL NAL unit of the access unit.

–	When an end of sequence NAL unit is present, it shall be the last NAL unit among all NAL units with in the access unit other than an end of bitstream NAL unit (when present).

–	When an end of bitstream NAL unit is present, it shall be the last NAL unit in the access unit.

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).