| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0147-v1 |
| --- | --- |

| Title: | AHG17: Constraints on IRAP pictures and leading pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Ye-Kui Wang 10180 Telesis Court San Diego, CA, 92121, USA | Tel: Email: | +1-858-375-8065 fhendry@futurewei.com +1-908-803-3888 ye-kui.wang@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add a set of constraints on IRAP, RASL, and RADL pictures. Most of the proposed constraints are asserted technically identical as in HEVC. One of the proposed constraints is asserted technically different than in HEVC, in order to address an issue related to coding of field pictures. That issue was raised at the 14th JVET meeting in Geneva when discussing the entire set of HEVC constraints on IRAP, RASL, and RADL pictures proposed for VVC in JVET-N0120.

# Proposed constraints that are the same as in HEVC

As part of the semantics of nal_unit_type_lsb, add the following constraints, which are technically identical as in HEVC:

Each picture, other than the first picture in the bitstream in decoding order, is considered to be associated with the previous IRAP picture in decoding order.

When a picture is a leading picture of an IRAP picture, it shall be a RADL or RASL picture.

When a picture is a trailing picture of an IRAP picture, it shall not be a RADL or RASL picture.

No RASL pictures shall be present in the bitstream that are associated with an IDR picture.

No RADL pictures shall be present in the bitstream that are associated with with an IDR picture having nal_unit_type equal to IDR_N_LP.

NOTE – It is possible to perform random access at the position of an IRAP access unit by discarding all access units before the IRAP access unit (and to correctly decode the IRAP picture and all the subsequent non-RASL pictures in decoding order), provided each parameter set is available (either in the bitstream or by external means not specified in this Specification) when it is referred.

Any picture that precedes an IRAP picture in decoding order shall precede the IRAP picture in output order and shall precede any RADL picture associated with the IRAP picture in output order.

Any RASL picture associated with a CRA picture shall precede any RADL picture associated with the CRA picture in output order.

Any RASL picture associated with a CRA picture shall follow, in output order, any IRAP picture that precedes the CRA picture in decoding order.

# Proposed constraint that is not the same as in HEVC

HEVC has the following constraint on leading pictures:

When a picture is a leading picture of an IRAP picture, it shall precede, in decoding order, all trailing pictures that are associated with the same IRAP picture.

An issue on this constraint related to coding of field pictures was raised at the 14th JVET meeting in Geneva. When interlace coding is used, the two fields of an IRAP picture may not both be coded as IRAP pictures, instead only the first field is coded as an IRAP picture while the second field is encoded as an inter-coded trailing picture for higher coding efficiency. However, according to the above constraint, it is not possible to have the first field picture coded as a CRA picture with associated RASL pictures (i.e., the classic open-GOP coding structure) while putting the second field picture, an inter-coded trailing picture, before the RASL pictures in decoding order.

To resolve this issue, instead of the above constraint as in HEVC, the following constraint is proposed:

If field_seq_flag is equal to 0 and the current picture is a leading picture associated with an IRAP picture, it shall precede, in decoding order, all non-leading pictures that are associated with the same IRAP picture. Otherwise, let picA and picB be the first and the last leading pictures, in decoding order, associated with an IRAP picture, respectively, there shall be at most one non-leading picture preceding picA in decoding order, and there shall be no non-leading picture between picA and picB in decoding order.

Note that when field_seq_flag is equal to 0, i.e., the pictures in the CVS are all frame pictures, the above constraint is the same as in HEVC. When field_seq_flag is equal to 1, then it is allowed to have at most one non-leading picture before all the leading pictures.

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).