| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0149-v1 |
| --- | --- |

| Title: | AHG17: On external decoding refresh (EDR) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Jianle Chen (Futurewei) 10180 Telesis Court San Diego, CA, 92121, USA Hualong Yu, Lu Yu (ZJU) | Tel: Email: | ye-kui.wang@futurewei.com jianle.chen@futurewei.com ylonge37@zju.edu.cn yul@zju.edu.cn |
| Source: | Futurewei Technologies, Inc., Zhejiang University |  |  |

_____________________________

# Abstract

This contribution proposes a design for support random accessing a bitstream from inter-coded pictures without a recovery period. Such random accessing needs a small number of reference pictures to be provided by an external means, thus named external decoding refresh (EDR).

The proposed design is summarized as follows:

- An EDR picture is a random accessible picture for which one or more external pictures are needed when random accessing from the picture.

- An EDR picture may start a CVS (hence may also start a bitstream).

- External pictures are provided in the form of coded pictures instead of decoded pictures.

- There is no change to RPL syntax, semantics, and derivation process.

- A new NAL unit type (EDR_NUT) is defined for EDR pictures.

- For each EDR picture, the POC MSB values of the external pictures and the EDR picture itself are signalled in the slice headers.

The proposed detailed spec text changes are provided in an attachment.

# Background

A video coding approach based on external decoding refresh (EDR), also referred to as cross RAP reference (CRR), has been proposed to VVC in JVET-M0360 and JVET-N0119.

The basic idea is of the EDR approach is as follows. Instead of coding random access points (except for the very first one in the bitstream) as intra-coded IRAP pictures, they are coded using inter prediction, to circumvent the unavailability of the earlier pictures than an IRAP picture. The trick is to provide a limited number of the earlier pictures, typically representing different scenes of the video content, through a separate video bitstream, which can be referred to as an external means. Such earlier pictures are referred to as the external pictures. Consequently, each external picture can be used for inter prediction referencing by pictures across the random access points. The coding efficiency gain basically comes from inter coding of the random access points.

At the 14th JVET meeting in April 2019 in Geneva, this was noted in the JVET meeting minutes as "For further study with systems experts" (See the notes in JVET-N0724-v8 under JVET-N0119), as there were some questions regarding how the EDR approach work with systems like in file format and in DASH.

# VVC with EDR support in systems environments

In response to the JVET meeting notes, at this meeting, besides proposing the same design as in JVET-N0119-v1, we have worked on systems mechanisms for applying VVC with EDR support in the ISO base media file format (ISOBMFF) and the DASH environments. These systems designs have been included in MPEG input documents m48080 and m48081, respectively.

It is asserted that these file format and DASH designs are fairly straightforward and simple. For example, the DASH content preparation operations and the DASH streaming operations are as follows:

DASH content preparation operations

- A video content is encoded into one or more representations, each is of a particular spatial resolution, temporal resolution, and quality.

- Each particular representation of a video content is represented by a main stream, and possibly also an external stream. The main stream contains coded pictures that may or may not include EDR pictures. When at least one EDR picture is included in the main stream, the external stream is also present and contains external pictures. When no EDR picture is included in the main stream, the external stream is not present.

- Each main stream is carried in a Main Stream Representation (MSR). Each EDR picture in an MSR is the first picture of a Segment.

- Each external stream, when present, is carried in an External Stream Representation (ESR).

- For each Segment in an MSR starting with an EDR picture, there is a Segment in the corresponding ESR having the same Segment start time derived from the MPD, carrying the external pictures needed for decoding of that EDR picture and the subsequent pictures in decoding order in the bitstream carried in the MSR.

- The MSRs of the same video content are included in one Adaptation Set (AS). The ESRs of the same video content are included in one AS.

DASH streaming operations

- A client gets the MPD of the DASH Media Presentation, parses the MPD, selects an MSR, and determines the starting presentation time from which the content is to be consumed.

- The client requests Segments of the MSR, starting from the Segment containing the picture having presentation time equal to (or close enough to) the starting presentation time.

  - If the first picture in the starting Segment is an EDR picture, the corresponding Segment (having the same Segment start time derived from the MPD) in the associated ESR is also requested, preferably before requesting of the MSR Segments. Otherwise, no Segment of the associated ESR is requested.

- When switching to a different MSR, the client requests Segments of the switch-to MSR, starting from the first Segment having Segment start time greater than that of the last requested Segment of the switch-from MSR.

  - If the first picture in the starting Segment in the switch-to MSR is an EDR picture, the corresponding Segment in the associated ESR is also requested, preferably before requesting of the MSR Segments. Otherwise, no Segment of the associated ESR is requested.

- When continuously operating at the same MSR (after decoding of the starting Segment after a seeking or stream switching operation), no Segment of the associated ESR needs to be requested, including when requesting any Segment starting with an EDR picture.

The EDR systems designs have been briefly reviewed and discussed at the face-to-face MPEG AHG meeting on Systems Technologies for VVC on May 20-21, 2019 in Berlin, with remote participations. As noted in the meeting minutes of that AHG meeting, "No major technical challenges to support storage of EDR has been identified." and "No major technical challenges to support DASH streaming of EDR has been identified."

So, basically, from MPEG Systems Subgroup's point of view, it's up to the JVET to consider whether an EDR support for VVC is a good design choice for VVC based on the coding performance improvement and its impact to encoder and decoder implementations. If an EDR support is adopted, the MPEG Systems Subgroup has no problem in making it work in systems environments.

# Coding performance discussion

As we know, the introduction of CRA pictures in HEVC was based on 1) that open-GOP based video coding can provide about 6% average coding performance improvement; and 2) allowing a CRA picture start a CVS provide a conforming way of handling random accessing and bitstream switching from open-GOP random access points.

Based on that EDR can provide significant coding performance improvement for practical streaming sequences, it is argued that EDR can provide a significantly improved capability of the VVC codec and the VVC ecosystem. As shown in Section 7, for CTC test sequences, the average coding gain is about 4.3%, (9.2% if class D and F are also included), while the number is about 24.3% for some long drama test sequences of sequence length ranging from 136s to 152s.

# Proposal

We propose to adopt the EDR feature into VVC. After that, the MPEG Systems Subgroup can work on the necessary systems components.

The proposed text changes are summarized in Section 5. The encoder operations, particularly on how to choose pictures to be used as external pictures, are described in Section 6. The simulations results based on VTM 5.0 are reported in Section 7.

# Proposed text changes

The proposed text changes are summarized as follows:

- An EDR picture is a random accessible picture for which one or more external pictures are needed when random accessing from the picture.

- It is specified that an EDR picture may start a CVS (hence may also start a bitstream).

- To allow reuse the existing RPL syntax unchanged, external pictures are provided in the form of coded pictures. External pictures are not output by the decoder; they are for inter prediction referencing only.

- Similarly as for CRA pictures, a new NAL unit type (EDR_NUT) is defined for EDR pictures.

  - When random accessing from an EDR picture, the EDR picture and all subsequent pictures in decoding order, except RASL pictures associated with the EDR picture, can be correctly decoded, provided the necessary external pictures are available through an external means.

  - For an EDR picture in a CVS that is not the first picture of the CVS, the external pictures are pictures in the CVS that precede the EDR picture in decoding order. Therefore, when decoding starts from an IRAP or EDR picture preceding the EDR picture in decoding order, no external pictures need to be provided for correct decoding of the EDR picture and all subsequent pictures in decoding order, except RASL pictures associated with the EDR picture.

- For each EDR picture, the POC MSB values of the external pictures and the EDR picture itself are signalled in the slice headers.

- RADL pictures and RASL pictures for allowed to be associated with an EDR picture.

The detailed spec text changes of the proposal are provided in an attachment to this contribution, with changes marked relative to JVET-N1001-v8.

# Encoding operations

The encoding operations are composed of the following steps:

- Selection of external pictures

There are two methods for selection of external pictures. One method is a two-pass process, where the first pass scans the whole sequence and select the optimal set of external pictures, and the second pass encodes the pictures of the whole sequence. The scanning process uses fast processes based on original picture samples values and does not do the actual encoding. The other method is a one-pass process, where selection of external pictures is based on the pictures that have been encoded. For now, only the two pass method is used in the simulation.

Selection of external pictures in the two-pass process is described below.

The selection of external pictures can be considered as a Crowdsourcing problem. Crowdsourcing obtains information or resources from multiple participants. Each participant can contribute to the target task with a cost. The target of crowdsourcing is to select an optimal set of participants that contributes the most information with minimal costs.

External pictures are supposed to provide the most reference information for pictures and cost the fewest bits. Therefore the solution of the crowdsourcing problem can be used to select external pictures, as described below.

A set of candidate pictures C=C1,…,CK are first selected. All pictures P=P1,…,PK that are to be used as random access points are selected as candidate pictures. The contribution of Ck providing to each random access segment (RAS) is estimated by simply calculating the contribution of Ck providing to each Pi. EPi is the residual energy of an IRAP picture, which is intra coded. EPiCk is the residual energy of Pi when it is inter coded by referencing to Ck. The contribution of Ci to Pi is VPiCk=EPi-EPiCk. The cost of Ck is represented as ECk.

EPiCk and ECk are computed by perform motion compensation and intra prediction by dividing Pi and Ck into blocks with a fixed size. Only the residues are required.

The selection processes of a set of external pictures L=L1,…,LN is as follows:

- Initialize L=Ck, where Ck=argmaxCk∈CUL, UL=VPL-EL, VPL=Pi∈PmaxLj∈LVPiLj, and EL=Lj∈LELj.

- If there exists a candidate picture Ck∈C\L, where UL∪Ck>1+εn2UL and n is the element number of C, update L=L∪Ck and repeat step b).

If such a Ck does not exist, go to step c).

- If there exists a candidate picture Ck∈L, where UL\La>1+εn2UL, update L=L∪Ck and go back to step b)

If such a Ck does not exist, go to step d).

- If US\L>UL, update L=S\L.

- Adaptive QP computation for external pictures

The QP of an external picture is computed based on a dependency factor ωi of the i-th block BiLn (fixed size) in external picture Ln. QP is computed as QP=QPInit-i=1M3log2ωi. The factor ωi is decided by the blocks BiPk in pictures Pk referencing to BiLn. The factor ωi is computed using the method proposed in [1].

- Encoding pictures by referencing external pictures

In the simulations, only EDR pictures refer to external pictures for inter prediction reference and each EDR picture Pi uses only one external picture Ln, where the selected external picture Ln satisfies Ln=argmaxLn∈LVPiLn, i.e., the external picture making the most contribution to the encoding EDR picture is selected.

# Simulation results

We have integrated the EDR coding approach into VTM5.0 and run simulations with the random access (RA) configuration of the VVC common test condition (CTC). The experiment results of VTM 5.0 with EDR compared to that without EDR are shown in Table 1.

Table 1 Results of VTM5.0 with EDR vs VTM5.0 without EDR.

|  |  | Y | U | V |
| --- | --- | --- | --- | --- |
| A1 | Tango2 | 1.29% | 5.79% | 2.94% |
|  | FoodMarket4 | 0.08% | 0.67% | 0.88% |
|  | Campfire | -2.65% | 2.10% | -2.41% |
| A2 | CatRobot1 | -5.26% | -2.59% | -3.10% |
|  | DaylightRoad2 | -3.05% | 5.05% | -2.71% |
|  | ParkRunning3 | -0.41% | 0.92% | 1.34% |
| B | MarketPlace | 1.09% | 3.01% | 3.25% |
|  | RitualDance | 0.05% | -0.16% | 2.64% |
|  | Cactus | -12.77% | -9.72% | -9.09% |
|  | BasketballDrive | -0.65% | 2.26% | 2.32% |
|  | BQTerrace | -9.61% | -1.61% | 10.07% |
| C | BasketballDrill | -22.48% | -32.44% | -28.97% |
|  | BQMall | -1.95% | 1.26% | 2.21% |
|  | PartyScene | -7.89% | -10.34% | -8.99% |
|  | RaceHorses | 0.04% | 1.76% | 2.15% |
| D | BasketballPass | -1.50% | -3.46% | -2.49% |
|  | BQSquare | -9.25% | -9.49% | -6.68% |
|  | BlowingBubbles | -3.93% | -4.01% | -3.97% |
|  | RaceHorses | -0.93% | -0.03% | 1.15% |
| E | FourPeople |  |  |  |
|  | Johnny |  |  |  |
|  | KristenAndSara |  |  |  |
| F | BasketballDrillText | -21.29% | -29.28% | -26.41% |
|  | ArenaOfValor | -8.67% | -8.49% | -8.10% |
|  | SlideEditing | -69.80% | -71.69% | -70.76% |
|  | SlideShow | -31.56% | -30.68% | -31.71% |
|  | A/B/C Average | -4.28% | -2.27% | -1.83% |
|  | All Average | -9.18% | -8.31% | -7.67% |

Additionally, 6 drama test sequences with length ranging from 136s to 152s were tested with the same test configuration. The random access period was set to 32 pictures. The parameters of drama test sequences are listed in Table 2 and the results are shown in Table 3.

Table 2 Drama test sequences with long duration

| Sequence | Resolution | Fps | Frame | Length (s) |
| --- | --- | --- | --- | --- |
| Bigbang | 640x360 | 30 | 4080 | 136 |
| Cards | 640x360 | 30 | 4337 | 144 |
| Sherlock | 640x360 | 30 | 4553 | 151 |
| Earthman | 640x360 | 30 | 4339 | 144 |
| Throne | 640x360 | 30 | 4336 | 145 |
| Girls | 640x360 | 30 | 4567 | 152 |

Table 3 Resuls of VTM5.0 with CRR vs VTM5.0 without CRR.

|  | Y | U | V |
| --- | --- | --- | --- |
| Bigbang | -18.95% | -17.65% | -17.87% |
| Card | -31.44% | -28.82% | -29.64% |
| Sherlock | -24.59% | -21.16% | -22.00% |
| ManFromEarth | -26.10% | -20.94% | -22.92% |
| Throne | -22.81% | -20.13% | -20.51% |
| Girls | -22.02% | -18.82% | -19.40% |
| Average | -24.32% | -21.26% | -22.06% |

# Patent rights declaration(s)

Futurewei Technologies, Inc. and Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y. Gao, C. Zhu, S. Li and T. Yang, "Source Distortion Temporal Propagation Analysis for Random-Access Hierarchical Video Coding Optimization," in IEEE Transactions on Circuits and Systems for Video Technology, vol. 29, no. 2, pp. 546-559, Feb. 2019.