| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothernburg, SE, 3–12 July 2019 | Document: JVET-O0155-v1 |
| --- | --- |

| Title: | AHG17: Resolving PPS parsing dependency on SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Ye-Kui Wang 10180 Telesis Court San Diego, CA, 92121, USA | Tel: Email: | +1-858-375-8065 fhendry@futurewei.com +1-908-803-3888 ye-kui.wang@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

This contribution asserted that there is dependency for parsing of some PPS syntax elements on SPS syntax in the current VVC working draft. The parsing dependency occurs in the parsing of the following syntax elements:

- Tiles and bricks related syntax elements. For deriving the values of NumTilesInPic and NumBricksInPic, which are needed for parsing other syntax elements that are also present in the PPS, the parsing process needs some information from SPS.

- Positions of virtual boundaries. For parsing syntax elements pps_virtual_boundaries_pos_x[ i ] and pps_virtual_boundaries_pos_y[ i ], the number of bits that represents those syntax elements depend on the picture width and height that are only available from SPS.

If the JVET decides to keep parsing independency between parameter sets, to solve the above parsing dependency issues, the following is proposed:

- For tile and brick signalling, the following is proposed:

  - When a picture is partitioned into more than one tile, regardless whether uniform tile partitioning is used or not, num_tile_columns_minus1 and num_tile_rows_minus1 are always signalled. The derivation process of colWidth[ i ] and RowHeight[ i ] is modified to incorporate the values of num_tile_columns_minus1 and num_tile_rows_minus1.

  - When one or more tiles are partitioned into more than one brick, regardless whether uniform brick partitioning is used or not, num_brick_rows_minus1[ i ] is always signalled. The derivation process of rowHeight2[ j ] is modified to incorporate the value of num_brick_rows_minus1[ i ]

- For signalling of virtual boundaries, the number of bits that represent the syntax elements pps_virtual_boundaries_pos_x[ i ] and pps_virtual_boundaries_pos_y[ i ] is signalled explicitly.

Note that the proposed modification is NOT needed if the JVET decides not to keep parsing independency between parameter sets, e.g., as proposed in JVET-O0113.

# On tile and brick signalling

## Problem description

At the 14th JVET meeting in Geneva, as the result of the adoption of JVET-N0498, the signalling of tile and brick was modified as follows:

- For tile signalling, the following applies:

  - If uniform_tile_spacing_flag is equal to 1, num_tile_columns_minus1 and num_tile_rows_minus1 are no longer present, instead, tile_cols_width_minus1 and tile_rows_height_minus1 are signalled and the number of tile columns and tile rows are derived.

  - Otherwise (i.e., uniform_tile_spacing_flag is equal to 0), num_tile_columns_minus1 and num_tile_rows_minus1 are present.

- For brick signalling, the following applies:

  - if uniform_brick_spacing_flag[ i ] is equal to 1, instead of signalling num_brick_rows_minus1[ i ], brick_height_minus1[ i ] is signalled and the number of brick rows within the i-th tile is derived.

  - Otherwise (i.e., uniform_brick_spacing_flag[ i ] is equal to 0), num_brick_rows_minus1[ i ] is present.

The motivation of such modification is to avoid derivation of values of the list of tile column widths based on the number of tile columns and the picture width which may result in alternating tile columns width. For example, given PicWidthInCtbsY is equal to 10 and number of tile column is 4, the derivation would give the values of the list of tile column width as [2, 3, 2, 3] while it is more desirable if the list of tile column width is [3, 3, 3, 1]. The same motivation applies for derivation of the list of tile row heights.

With the updated signalling, we assert that there is parsing dependency problem for tile and brick signalling in the cases when uniform_tile_spacing_flag is equal to 1 and when uniform_brick_spacing_flag is equal to 1. The parsing dependency issues is as follows:

- When uniform_tile_spacing_flag is equal to 1, the absent of num_tile_columns_minus1 and num_tile_rows_minus1 make it not possible to derive the value of NumTilesInPic without knowing the value of PicWidthInCtbsY, which is available from SPS. Note that NumTilesInPic is needed for parsing the brick partitioning within tiles.

- When uniform_brick_spacing_flag[ i ] is equal to 1, the absent of num_brick_rows_minus1[ i ] make it not possible to derive how many bricks are present in the i-th tile with only information signalled in the PPS. This in turn make it not possible to derive the value of NumBricksInPic. Note that NumBricksInPic is needed for parsing the value of top_left_brick_idx[ i ] since the number of bits representing that syntax element depends on the value of NumBricksInPic.

## Proposed solution

The following modifications are proposed to solve the parsing dependency:

- When a picture is partitioned into more than one tile, regardless whether uniform tile partitioning is used or not, num_tile_columns_minus1 and num_tile_rows_minus1 are always signalled. The derivation process of colWidth[ i ] and RowHeight[ i ] is modified to incorporate the values of num_tile_columns_minus1 and num_tile_rows_minus1.

- When one or more tile is partitioned into more than one brick, regardless whether uniform brick partitioning is used or not, num_brick_rows_minus1[ i ] is always signalled. The derivation process of rowHeight2[ j ] is modified to incorporate the value of num_brick_rows_minus1[ i ]

The proposed modification is implemented as follows:

Picture Parameter Set Syntax and Semantics

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| if( !uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| for( i = 0; brick_splitting_present_flag && i < NumTilesInPic; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| num_brick_rows_minus1[ i ] | ue(v) |
| if( !uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| ... |  |
| } |  |
| ... |  |
| } |  |

...

uniform_tile_spacing_flag equal to 1 specifies that tile column boundaries and likewise tile row boundaries are distributed uniformly across the picture and signalled using the syntax elements tile_cols_width_minus1 and tile_rows_height_minus1. uniform_tile_spacing_flag equal to 0 specifies that tile column boundaries and likewise tile row boundaries may or may not be distributed uniformly across the picture and signalled using the syntax elements num_tile_columns_minus1 and num_tile_rows_minus1 and a list of syntax element pairs tile_column_width_minus1[ i ] and tile_row_height_minus1[ i ]. When not present, the value of uniform_tile_spacing_flag is inferred to be equal to 1.

tile_cols_width_minus1 plus 1 specifies the width of the tile columns excluding the right-most tile column of the picture in units of CTBs when uniform_tile_spacing_flag is equal to 1. The value of tile_cols_width_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When not present, the value of tile_cols_width_minus1 is inferred to be equal to PicWidthInCtbsY − 1.

tile_rows_height_minus1 plus 1 specifies the height of the tile rows excluding the bottom tile row of the picture in units of CTBs when uniform_tile_spacing_flag is equal to 1. The value of tile_rows_height_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of tile_rows_height_minus1 is inferred to be equal to PicHeightInCtbsY − 1.

num_tile_columns_minus1 plus 1 specifies the number of tile columns partitioning the picture when uniform_tile_spacing_flag is equal to 0. The value of num_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. If single_tile_in_pic_flag is equal to 1, the value of num_tile_columns_minus1 is inferred to be equal to 0. Otherwise, when uniform_tile_spacing_flag is equal to 1, the value of num_tile_columns_minus1 is inferred as specified in clause 6.5.1.

num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the picture when uniform_tile_spacing_flag is equal to 0. The value of num_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. If single_tile_in_pic_flag is equal to 1, the value of num_tile_rows_minus1 is inferred to be equal to 0. Otherwise, when uniform_tile_spacing_flag is equal to 1, the value of num_tile_rows_minus1 is inferred as specified in clause 6.5.1.

...

uniform_brick_spacing_flag[ i ] equal to 1 specifies that horizontal brick boundaries are distributed uniformly across the i-th tile and signalled using the syntax element brick_height_minus1[ i ]. uniform_brick_spacing_flag[ i ] equal to 0 specifies that horizontal brick boundaries may or may not be distributed uniformly across i-th tile and signalled using the syntax element num_brick_rows_minus1[ i ] and a list of syntax elements brick_row_height_minus1[ i ][ j ]. When not present, the value of uniform_brick_spacing_flag[ i ] is inferred to be equal to 1.

brick_height_minus1[ i ] plus 1 specifies the height of the brick rows excluding the bottom brick in the i-th tile in units of CTBs when uniform_brick_spacing_flag[ i ] is equal to 1. When present, the value of brick_height_minus1 shall be in the range of 0 to RowHeight[ i ] − 2, inclusive. When not present, the value of brick_height_minus1[ i ] is inferred to be equal to RowHeight[ i ] − 1.

num_brick_rows_minus1[ i ] plus 1 specifies the number of bricks partitioning the i-th tile when uniform_brick_spacing_flag[ i ] is equal to 0. When present, the value of num_brick_rows_minus1[ i ] shall be in the range of 1 to RowHeight[ i ] − 1, inclusive. If brick_split_flag[ i ] is equal to 0, the value of num_brick_rows_minus1[ i ] is inferred to be equal to 0. Otherwise, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_rows_minus1[ i ] is inferred as specified in 6.5.1.

...

CTB raster scanning, tile scanning, and brick scanning process

The list colWidth[ i ] for i ranging from 0 to num_tile_columns_minus1, inclusive, specifying the width of the i-th tile column in units of CTBs, is derived, and when uniform_tile_spacing_flag is equal to 1, the value of num_tile_columns_minus1 is inferred, as follows:

if( uniform_tile_spacing_flag )
	tileColWidthInCtbY = Ceil( PicWidthInCtbsY ÷ (num_tile_columns_minus1 + 1 ) )
colWidth[ num_tile_columns_minus1 ] = PicWidthInCtbsY	(6-1)
for( i = 0; i < num_tile_columns_minus1; i++ ) {
	colWidth[ i ] = uniform_tile_spacing_flag ? tileColWidthInCtbY : tile_column_width_minus1[ i ] + 1
	colWidth[ num_tile_columns_minus1 ]  −=  colWidth[ i ]

if( uniform_tile_spacing_flag ) {
	remainingWidthInCtbsY = PicWidthInCtbsY
	i = 0
	while( remainingWidthInCtbsY > ( tile_cols_width_minus1 + 1 ) ) {
		colWidth[ i++ ] = tile_cols_width_minus1 + 1
		remainingWidthInCtbsY  −=  ( tile_cols_width_minus1 + 1 )
	}
	colWidth[ i ] = remainingWidthInCtbsY
	num_tile_columns_minus1 = i
} else {
	colWidth[ num_tile_columns_minus1 ] = PicWidthInCtbsY	(6-1)
	for( i = 0; i < num_tile_columns_minus1; i++ ) {
		colWidth[ i ] = tile_column_width_minus1[ i ] + 1
		colWidth[ num_tile_columns_minus1 ]  −=  colWidth[ i ]
	}
}

The list RowHeight[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, specifying the height of the j-th tile row in units of CTBs, is derived, and when uniform_tile_spacing_flag is equal to 1, the value of num_tile_rows_minus1 is inferred, as follows:

if( uniform_tile_spacing_flag )
	tileRowHeightInCtbY = Ceil( PicHeightInCtbsY ÷ (num_tile_rows_minus1 + 1 ) )
RowHeight[ num_tile_rows_minus1 ] = PicHeightInCtbsY	(6-2)
for( j = 0; j < num_tile_rows_minus1; j++ ) {
	RowHeight[ j ] = uniform_tile_spacing_flag ? tileRowHeightInCtbY : tile_row_height_minus1[ j ] + 1
	RowHeight[ num_tile_rows_minus1 ]  −=  RowHeight[ j ]
}

if( uniform_tile_spacing_flag ) {
	remainingHeightInCtbsY = PicHeightInCtbsY
	i = 0
	while( remainingHeightInCtbsY > ( tile_rows_height_minus1 + 1 ) ) {
		RowHeight[ i++ ] = tile_rows_height_minus1 + 1
		remainingHeightInCtbsY  −=  ( tile_rows_height_minus1 + 1 )
	}
	RowHeight[ i ] = remainingHeightInCtbsY
	num_tile_rows_minus1 = i
} else {
	RowHeight[ num_tile_rows_minus1 ] = PicHeightInCtbsY	(6-2)
	for( j = 0; j < num_tile_rows_minus1; j++ ) {
		RowHeight[ j ] = tile_row_height_minus1[ j ] + 1
		RowHeight[ num_tile_rows_minus1 ]  −=  RowHeight[ j ]
	}
}

...

The variable NumBricksInPic, specifying the number of bricks in a picture referring to the PPS, and the lists BrickColBd[ brickIdx ], BrickRowBd[ brickIdx ], BrickWidth[ brickIdx ], and BrickHeight[ brickIdx ] for brickIdx ranging from 0 to NumBricksInPic − 1, inclusive, specifying the locations of the vertical brick boundaries in units of CTBs, the locations of the horizontal brick boundaries in units of CTBs, the widths of the bricks in units of CTBs, and the heights of bricks in units of CTBs, are derived, and for each i ranging from 0 to NumTilesInPic − 1, inclusive, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_rows_minus1[ i ] is inferred, as follows:

for ( brickIdx = 0, i = 0; i < NumTilesInPic; i++ ) {
	tileX = i % ( num_tile_columns_minus1 + 1 )
	tileY = i / ( num_tile_columns_minus1 + 1 )
	if( !brick_split_flag[ i ] ) {
		BrickColBd[ brickIdx ] = tileColBd[ tileX ]
		BrickRowBd[ brickIdx ] = tileRowBd[ tileY ]
		BrickWidth[ brickIdx ] = colWidth[ tileX ]
		BrickHeight[ brickIdx ] = RowHeight[ tileY ]	(6-5)
		brickIdx++
	} else {
		if( uniform_brick_spacing_flag[ i ] )
			brickRowHeightInCtbsY = Ceil( RowHeight[ tileY ] ÷ ( num_brick_rows_minus1[ i ] + 1 ) )
		rowHeight2[ num_brick_rows_minus1[ i ] ] = RowHeight[ tileY ]
		for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) {
			rowHeight2[ j ] = uniform_brick_spacing_flag[ i ] ?
					brickRowHeightInCtbsY : brick_row_height_minus1[ i ][ j ]+ 1
			rowHeight2[ num_brick_rows_minus1[ i ] ]  −=  rowHeight2[ j ]

}
		if( uniform_brick_spacing_flag[ i ] ) {
			remainingHeightInCtbsY = RowHeight[ tileY ]
			j = 0
			while( remainingHeightInCtbsY > ( brick_height_minus1[ i ] + 1 ) ) {
				rowHeight2[ j++ ] = brick_height_minus1[ i ] + 1
				remainingHeightInCtbsY  −=  ( brick_height_minus1[ i ] + 1 )
			}
			rowHeight2[ j ] = remainingHeightInCtbsY
			num_brick_rows_minus1[ i ] = j
		} else {
			rowHeight2[ num_brick_rows_minus1[ i ] ] = RowHeight[ tileY ]
			for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) {
				rowHeight2[ j ] = brick_row_height_minus1[ i ][ j ]+ 1
				rowHeight2[ num_brick_rows_minus1[ i ] ]  −=  rowHeight2[ j ]
			}
		}
		for( rowBd2[ 0 ] = 0, j = 0; j  <=  num_brick_rows_minus1[ i ]; j++ )
			rowBd2[ j + 1 ] = rowBd2[ j ] + rowHeight2[ j ]
		for( j = 0; j < num_brick_rows_minus1[ i ] + 1; j++ ) {
			BrickColBd[ brickIdx ] = tileColBd[ tileX ]
			BrickRowBd[ brickIdx ] = tileRowBd[ tileY ] + rowBd2[ j ]
			BrickWidth[ brickIdx ] = colWidth[ tileX ]
			BrickHeight[ brickIdx ] = rowHeight2[ j ]
			brickIdx++
		}
	}
}
NumBricksInPic = brickIdx

...

# On virtual boundaries signalling

## Problem description

At the 14th JVET meeting in Geneva, method for signalling virtual boundaries within a picture is adopted into the VVC specification. We assert that there is parsing dependency problem for syntax element pps_virtual_boundaries_pos_x[ i ] and pps_virtual_boundaries_pos_y[ i ].

In the semantic of pps_virtual_boundaries_pos_x[ i ], it is specified that the number of bits used to represent pps_virtual_boundaries_pos_x[ i ] is Ceil( Log2( pic_width_in_luma_samples ) − 3 ). The value of pic_width_in_luma_samples is present in SPS which means the parsing of pps_virtual_boundaries_pos_x[ i ] has dependency to syntax element that is present in the SPS.

Likewise, in the semantic of pps_virtual_boundaries_pos_y[ i ], it is specified that the number of bits used to represent pps_virtual_boundaries_pos_y[ i ] is Ceil( Log2( pic_height_in_luma_samples ) − 3 ). The value of pic_height_in_luma_samples is present in SPS which means the parsing of pps_virtual_boundaries_pos_y[ i ] has dependency to syntax element that is present in the SPS.

## Proposed solution

To solve the above parsing dependency problem, we propose to signal the number of bits used to represent pps_virtual_boundaries_pos_x[ i ] and pps_virtual_boundaries_pos_y[ i ] explicitly in the PPS.

The proposed modification is implemented as follows:

Picture Parameter Set Syntax and Semantics

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| pps_loop_filter_across_virtual_boundaries_disabled_flag | u(1) |
| if( pps_loop_filter_across_virtual_boundaries_disabled_flag ) { |  |
| pps_virtual_boundaries_pos_len_minus1 | ue(v) |
| pps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_ver_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_x[ i ] | u(v) |
| pps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_hor_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_y[ i ] | u(v) |
| } |  |
| ... |  |
| } |  |

...

pps_virtual_boundaries_pos_len_minus1 plus 1 specifies the number of bits to represent syntax element pps_virtual_boundaries_pos_x[ i ] and pps_virtual_boundaries_pos_y[ i ].

...

pps_virtual_boundaries_pos_x[ i ] is used to compute the value of PpsVirtualBoundariesPosX[ i ], which specifies the location of the i-th vertical virtual boundary in units of luma samples. The number of bits used to represent pps_virtual_boundaries_pos_x[ i ] is pps_virtual_boundaries_pos_len + 1. pps_virtual_boundaries_pos_x[ i ] shall be in the range of 1 to Ceil( pic_width_in_luma_samples ÷ 8 ) − 1, inclusive.

The location of the vertical virtual boundary PpsVirtualBoundariesPosX[ i ] is derived as follows:

PpsVirtualBoundariesPosX[ i ] = pps_virtual_boundaries_pos_x[ i ] * 8	(7-37)

The distance between any two vertical virtual boundaries shall be greater than or equal to CtbSizeY luma samples.

...

pps_virtual_boundaries_pos_y[ i ] is used to compute the value of PpsVirtualBoundariesPosY[ i ], which specifies the location of the i-th horizontal virtual boundary in units of luma samples. The number of bits used to represent pps_virtual_boundaries_pos_y[ i ] is pps_virtual_boundaries_pos_len + 1. pps_virtual_boundaries_pos_y[ i ] shall be in the range of 1 to Ceil( pic_height_in_luma_samples ÷ 8 ) − 1, inclusive.

The location of the horizontal virtual boundary PpsVirtualBoundariesPosY[ i ] is derived as follows:

PpsVirtualBoundariesPosY[ i ] = pps_virtual_boundaries_pos_y[ i ] * 8	(7-38)

The distance between any two horizontal virtual boundaries shall be greater than or equal to CtbSizeY luma samples.

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).