| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0156-v1 |
| --- | --- |

| Title: | AHG12: On brick signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woong Lim, Gun Bang, Hui Yong Kim (ETRI) 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-1836 {woong.lim, gbang, hykim5}@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution proposes to skip the bottom-right brick index delta syntax element of the last slice which is partitioned into multiple bricks in a picture. The syntax element is signaled in PPS and by deriving the bottom-right brick index as the number of bricks in a picture for the last slice, it can be skipped. Nevertheless, the same partitioning structure can be kept logically.

Introduction

At the 14th JVET meeting, brick partitioning scheme was adopted in the Versatile Video Coding (VVC) Working Draft (WD) 5 [1]. In VVC WD5, partitioning a picture into multiple rectangular slices consisting of one or more bricks is allowed. In PPS, for each rectangular slice, top-left brick index and the delta value between top-left brick index and bottom-right brick index are signaled to indicate bricks which are included in the slice. In case of the first slice in a picture, the first top-left brick index is set equal to 0 and not signaled. The related syntax in tabular form, the semantic of bottom_right_brick_idx_delta and derivation of botRightBkIdx (bottom-right brick index) are shown below (yellow highlighted).

Table 1. A part of clause 7.3.2.4	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_brick_idx[ i ] | u(v) |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

Table 2. A part of clause 7.4.3.4	Picture parameter set RBSP semantics in WD5

bottom_right_brick_idx_delta[ i ] specifies the difference between the brick index of the brick located at the bottom-right corner of the i-th slice and top_left_brick_idx[ i ]. When single_brick_per_slice_flag is equal to 1, the value of bottom_right_brick_idx_delta[ i ] is inferred to be equal to 0. The length of the bottom_right_brick_idx_delta[ i ] syntax element is Ceil( Log2( NumBricksInPic − top_left_brick_idx[ i ] ) ) bits. It is a requirement of bitstream conformance that a slice shall include either a number of complete tiles or only a consecutive sequence of complete bricks of one tile. The variable NumBricksInSlice[ i ] and BricksToSliceMap[ j ], which specify the number of bricks in the i-th slice and the mapping of bricks to slices, are derived as follows: NumBricksInSlice[ i ] = 0 botRightBkIdx = top_left_brick_idx[ i ] + bottom_right_brick_idx_delta[ i ] for( j = 0; j < NumBricksInPic; j++) { if( BrickColBd[ j ] >= BrickColBd[ top_left_brick_idx[ i ] ] && BrickColBd[ j ] <= BrickColBd[ botRightBkIdx ] && BrickRowBd[ j ] >= BrickRowBd[ top_left_brick_idx[ i ] ] && (7-35) BrickRowBd[ j ] <= BrickRowBd [ botRightBkIdx ] ) { NumBricksInSlice[ i ]++ BricksToSliceMap[ j ] = i } }

Proposed method

This contribution proposes to skip the bottom-right brick index delta syntax element of the last slice which is partitioned into multiple bricks in PPS. In case of the first slice in a picture, the left-top brick index is already not signaled and set equal to i (0 for the first slice). Similarly, the bottom-right brick index delta value of the last slice can be skipped and the index can be determined using NumBricksInPic value which means the number of bricks in a picture. A part of syntax structure of PPS and the derivation process of the proposed method are shown below (highlighted in green).

Table 3. The proposed part of clause 7.3.2.4	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_brick_idx[ i ] | u(v) |
| if(i != num_slices_in_pic_minus1) |  |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

Table 4. The proposed part of clause 7.4.3.4	Picture parameter set RBSP semantics in WD5

bottom_right_brick_idx_delta[ i ] specifies the difference between the brick index of the brick located at the bottom-right corner of the i-th slice and top_left_brick_idx[ i ]. When single_brick_per_slice_flag is equal to 1, the value of bottom_right_brick_idx_delta[ i ] is inferred to be equal to 0. The length of the bottom_right_brick_idx_delta[ i ] syntax element is Ceil( Log2( NumBricksInPic − top_left_brick_idx[ i ] ) ) bits. It is a requirement of bitstream conformance that a slice shall include either a number of complete tiles or only a consecutive sequence of complete bricks of one tile. The variable NumBricksInSlice[ i ] and BricksToSliceMap[ j ], which specify the number of bricks in the i-th slice and the mapping of bricks to slices, are derived as follows: NumBricksInSlice[ i ] = 0 if(i != num_slices_in_pic_minus1) botRightBkIdx = top_left_brick_idx[ i ] + bottom_right_brick_idx_delta[ i ] else botRightBkIdx = NumBricksInPic - 1; for( j = 0; j < NumBricksInPic; j++) { if( BrickColBd[ j ] >= BrickColBd[ top_left_brick_idx[ i ] ] && BrickColBd[ j ] <= BrickColBd[ botRightBkIdx ] && BrickRowBd[ j ] >= BrickRowBd[ top_left_brick_idx[ i ] ] && (7-35) BrickRowBd[ j ] <= BrickRowBd [ botRightBkIdx ] ) { NumBricksInSlice[ i ]++ BricksToSliceMap[ j ] = i } }

Conclusion

This contribution proposes to skip the bottom-right brick index delta syntax element of the last slice which is partitioned into multiple bricks in PPS. A simple syntax structure change and derivation process for the brick index are described. Logically, the proposed method can skip the syntax element and some bit saving also can be expected with retaining picture partitioning as it was. It is recommended to consider these changes into the VVC specification and the next VTM software.

References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1001, Mar. 2019.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).